SB-919

R-5000 No Freq. Change thru Computer Interface                 7-21-87

R-5000 receivers below serial number 804xxxx may not change frequency when

operated with a personal computer/interface.  To allow correct operation,

rplace IC-52 on the Control Unit.  The correct part number for IC-52 is

MBM27C128-25JA2.

CAUTION:

The R-5000 incorporates CMOS technology.  Observe precautions for handling

electrostatic sensitive devices.

This modification may be performed under warranty.

Time required for this change is 1/2 hour or less.

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253