SB-934

TM-221A Final Module Hints    4/11/89

When the Final module is replaced in a TM-221A, there is a

possiblity that the transmit power will not go higher than

47 watts, even witht he APC turned OFF.  This is due to

variances in input impedance of Final Modules.  To correct

this condition, an 8 pF capacitor can be added between the

input of the module and ground.  With some modules, a value

between 8 pF and 15 pF will be required.  Try to choose a

value as close to 8 pF as possible to prevent oscillation.

The capacitor should be designated as C19 on the Final Unit

(X45-1360-02).

Required Part

8pF Ceramic Capacitor   Part # CC45CH1H080D

Alignment:

1.	Turn the APC off (TX-RX unit VR-6: Clockwise).

2.	Adjust TC3 TC4 (TX-RX) for maximum power supply current.

Since the APC is off, the Final will produce very high

power output.  It is important to perform this step as

quickly as possible.

3.	Adjust the APC for 45 Watts.

This modification may be performed under warranty.

Time required for this modification is 0.25 hrs or less.

(c) 42888TKC

Downloaded From P-80 International Information Systems 304-744-2253