SB-939

TH-25/45AT Automatic Power OFF Function                            8-02-88

Some users of the TH-25AT/45AT have expressed a desire to defeat the

automatic power off function.  The following modification will explain how

to do this.  It should be noted that this modification does not effect the

battery saver function.

Required Part:

Diode Kenwood Part #1SS133

1.   Disconnect the battery pack and antenna.

2.   Remove the Volume, Squelch, and Tuning Control knobs by pulling them

straight up from the top panel.

3.   Using a 7mm spanner wrench, remove the nut from the volume control and

the nut from the tuning control.

4.   Remove one screw located by the PTT switch.

5.   Reve one screw located by the speaker jack.

6.   Remove two screws from the battery terminal plate.

7.   Carefully pull the front panel up from the transceiver  (do not break

the wires connected between the front panel and the body of the

transceiver.)  Lay the front panel to the side of the transceiver (Keep

track of the F.LOCK cover if it comes off.)

8.   Remove the PTT cover.

9.   Remove one screw from the PTT switch unit.

10.  Gently lift the top panel from the transceiver by pulling it forward

and then up (the "O" ring on the BNC connector will produce some tension.)

11.  Carefully unfold the flex Printed Circuit Board (PCB) to expose IC2.

12.  Using a 45 watt (or less) soldering iron that has an isolated or

grounded tip, add a diode between pins 23 and 58 of IC2.

13.  Carefully assemble the transceiver by reversing step 1-11.  Pull the

BNC connector "O" ring up a little before installing the top panel.

14.  Reset the microprocessor by following the procedure in the instruction

manual (page 16) called CLEARING ALL MEMORY.

This is an optional change that is not covered under warranty.

Time required for this modification is 1 hour or less.

SYSOP NOTES:  Later versions of the TH-25/45AT came with a programmable

defeat for the Auto power off function.  Do not install this modification

if your manual provides a procedure for turning the power off function off!

Step 11 tells you to carefully unfold the flexible pc board.  It is

extremely important that you do not try and bend the board in a direction

that is opposite from its current bend.  To do so will break the board or

the foil traces inside the board!

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253