SB-947

TS-440S OPERATION WITH THE MC-85			   01/20/89

With some TS-440S transceivers, insufficient modulation may

occur when the MC-85 compression switch is turned on. This

may be caused by too much of a voltage drop across resistor

R172 on the IF unit of the transceiver . If this is found to

be the case, change the resistor from 1K ohm to 100 ohms.

REQUIRED PART

100 OHM, 1/6 WATT RESISTOR.......RD14CB2C101J

PROCEDURE:

1.	Disconnect the power supply and antenna from the

transceiver.

2.	Using a #2 Phillips screw driver, remove the 9 screws

from the top cover of the transceiver. Remove the cover

and unplug the speaker wire.

3.	Locate resistor R172 on the component side of the IF

board (Figure 1).

4.	Using a #1 Phillips screw driver, remove the 7 screws

that secure the IF unit to the chassis of the

transceiver.

5.	Pull the board up and rotate it towards the front of the

transceiver to expose the foil side of the board.

6.	Desolder and remove resistor R172 from the board.

7.	Install and solder the 100 ohm resistor in place of

R172.

8.	Assemble the transceiver by reversing steps 1 - 5.

This modification may be covered under warranty.

Time required for this modification is 0.5 hrs or less.

(C) 110888TKC

Downloaded From P-80 International Information Systems 304-744-2253