RDSSTV2.EXE

Niel Wiegand - WA5VLZ

Jan 10, 1987

HARDWARE REQUIREMENTS

- 48 KB user memory after DOS is loaded

- Color Graphics Card

- TV or RGB or composite video monitor (TV works best because individual

dots blend together better)

- Game Control Adapter and special cable

- One diskette drive

- Graphics printer

SETUP AND OPERATION

Use the "mode co80" DOS command to put your display adapter into the

graphics mode.

Connect audio tape recorder for recording SSTV video from SW receiver.

Transformer couple recorder audio/speaker output to game control adapter

pin 2 (a-#1 button) and pin 4 (ground).  Use an audio output transformer

such as a Radio Shack 273-1380.

_______________	_______________

RECORDER		    )||(		GAME CONTROL ADAPTER PIN 2

SPEAKER	       8    )||(_   1000	    (A-#1 BUTTON)

TERMINALS	      OHM   )||(    OHM

_______________)||(_______________ GAME CONTROL ADAPTER PIN 4

AUDIO OUTPUT		      (GROUND)

TRANSFORMER

Tune SSTV signals as normal SSB.

Use analyze audio input function to check SSTV signal quality and to

confirm that your receiver is properly set on frequency.  Tune for periodic

spikes down to the sync line with the rest a trace between black and

white.

Record sstv signal video.

Playback SSTV video while using input and view function.

Use change horz sync function to help jagged right edge due to loss of

sync.

Use change brightness and contrast to suit personal preference.

Use print function to obtain hard copy.

NOTE:  DO NOT USE ANALYZE OR INPUT FUNCTIONS UNLESS SSTV VIDEO IS PRESENT

AT THE GAME CONTROL ADAPTER.

FUNCTION DESCRIPTIONS

ANALYZE AUDIO INPUT:  Audio input through the game control adapter is

plotted on the screen.	The vertical scale is marked for approximate

periods of white, black, and sync.  600 cycles are plotted then the program

waits for the next command.  A typical SSTV signal will show periodic

spikes down to the sync line with the rest a trace between black and

white.	A gray scale would look like stair steps.  Use this function as a

tuning aid for setting your receiver on frequency.  USE OF THIS FUNCTION

WITHOUT AUDIO CONNECTED WILL HANG UP YOUR SYSTEM.

MENU DISPLAY: Display function menu.

INPUT AND VIEW SSTV FRAME:  Audio input through the game control adapter is

used to generate a sstv frame in three steps.  First, wait for vertical

sync pulse. Second, store audio cycle periods in audio buffer. Third,

process audio buffer to form sstv frame.  USE OF THIS FUNCTION WITHOUT

AUDIO CONNECTED WILL HANG UP YOUR SYSTEM.

VIEW SSTV FRAME:  Audio previously input and stored in the audio buffer is

processed to generate a sstv frame.

LOAD SSTV FRAME:  Load raw binary display data from data files on diskette.

(FILESPEC = [d:]filename[.PIC])

SAVE SSTVFRAME:  Save the displayed sstv frame as raw binary data in data

file on diskette. (FILESPEC = d:filename[.PIC])

CHANGE SSTV FRAME PARAMETERS:  Contrast, brightness, and horizontal sync

freq of the picture may be adjusted. This works in conjunction with view

sstv frame and the audio buffer. It has no effect on pictures loaded using

the load sstv frame function.

HELP AND INFORMATION FRAMES:  This information is available here.

PRINT SSTV FRAME:  Output displayed frame to the printer.  Requires

graphics printer and DOS that supports a graphics screen print.

HIDDEN FUNCTIONS:

SAVE AUDIO BUFFER (Press Y):  Copies digitized SSTV data from a 32KB

buffer to file a:audiobuf.dat. This function was implemented to support

program development.

LOAD AUDIO BUFFER (Press X): Copies file a:audiobuf.dat into a 32KB

program data buffer. This simulates the INPUT SSTV function (I). After

loading this buffer VIEW and CHANGE will both work. This gives a good

feel for the program.  This function was implemented to support program

development.

SSTV NOTES

Amateur SSTV is most likely to be heard near 28640, 7171, 14230, 21340 and

3845 KHz.  14230 is most popular.  This program is designed to function

best with 60 Hz area standard B/W SSTV.

- sync freq   1200 Hz

- black       1500 Hz

- white       2300 Hz

- sweep rate  15 Hz

For more information on amateur SSTV see The ARRL Handbook for the Radio

Amateur (American Radio Relay League, Newington, Ct, 06111).