SB-949

TS-440S PROTECTING Q33						   01/20/89

If pin 7 (RL) of the remote connector is accidently

connected to ground, current from the 14 volt line (14L)

will damage transistor Q33 on the IF unit. This will prevent

the radio from transmitting. To protect Q33, a 4.7 ohm

resistor should be installed in series with the RL line on

the foil side of the IF board. In the event that pin 7 is

connected to ground, the resistor will open, but the

transceiver will still be capable of transmitting.

REQUIRED PART:

4.7 OHM, 1/6 WATT RESISTOR ........ RD14CB2C4R7J

1.	Disconnect the power supply and antenna from the

transceiver.

2.	Using a #2 Phillips screw driver, remove the 9 screws

from the top cover of the transceiver. Remove the cover

and unplug the speaker wire.

3.	Locate connector 19 on the IF unit.

4.	Using a #1 Phillips screw driver, remove the 7 screws

that secure the IF unit to the chassis of the

transceiver.

5.	Pull the board up and rotate it towards the front of the

transceiver to expose the foil side of the board.

6.	On the foil side of the board, locate the trace that is

connected to pin 3 (brown wire) of connector 19.

7.	Using a craft knife, cut the trace comming from pin

three so as to open the foil trace.

8.	Solder the 4.7 ohm resistor across the now open trace

(i.e. in series with the trace).

9.	Assemble the transceiver by reversing steps 1 - 5.

This is an optional change that may not be covered under

warranty.

Time required for this modification is 0.5 hrs or less.

(C) 112188TKC

Downloaded From P-80 International Information Systems 304-744-2253