SB-953

MC-55 Change in Mic Element (T91-0328-08)               05/04/89

The new Mic element for the MC-55 comes in two pieces.  One piece is a

two pin ECM and the other piece is a spacer.  The spacer is required as

the new ECM is half the size of the old one.  The wiring configuration

is somewhat changed for the installation of the new Mic element.  The

figures below show the original configuration, the new configuration,

and the assembly.  The black wire is not used with the new ECM and a 330

ohm resistor is installed on the foil side of the microphone amplifier

board.  The leads of the resistor should be insulated with electricians

tape or shrink tubing.

(c) 103189TKC

(If you need a copy of this modification leave a note to the SYSOP with

your name and address and we will send one out to you.)

Downloaded From P-80 International Information Systems 304-744-2253