SB-931

TM-721A Low/No VHF Transmit Power                 4-11-88

Some early model TM-721A transceivers may develop low or no output power on

the VHF band.  This may be caused by coil L6 or L9 on the final unit.  The

coils should be inspected for burn marks on their insulation.  If any flaw

is detected in the insulation, replace the coild with a wider spaced coil.

Required parts:

Coil, 9.5T     L6,L9     Part Number L34-1238-05

1.   Disconnect the power supply and antenna from the transceiver.

2.   Remove the top and bottom covers (12 screws).  Do not damage the

speaker wires when removing the top cover.

3.   Remove the VHF Final unit shield (4 screws) from the top of the TM-

721A.

4.   Unsolder the Final module (Q1), antenna coax, and terminal pin W4 from

the Final board.

5.   Unplug the wires at connectors CN2, CN3, and CN4 on the VHF TX-RX unit

(top side of the transceiver.)

6.   Unplug the wires at connector CN10 on the UHF TX-RX unit (bottom side

of the transceiver.)

7.   Remove the 7 screws from the VHF Final unit.  Remove the Final unit

fromt he transceiver.

8.   Unsolder an remove the coil (L6 or L9).  Install and solder the new

coil.

9.   Assemble the transceiver by reversing steps 1 - 7.

This modification  may be performed under warranty.

Time required for this change is 1 hour or less.

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253