#A

#2#CB

#Radio Teletype Program

#For The IBM Personal Computer

#Version 1.2F

04-13-1986

Glenn E. Welman - KF4NB

3301 Pastern Ct.

Lexington, Ky 40513

(C) Copyright Welman Software 1983, 1984, 1985

IBM PC RTTY Ver 1.2F

INTRODUCTION

This program runs on the IBM Personal Computer and

interfaces to your Amateur Radio equipment through the

Asynchronous Communications adapter.  I wrote the program

to be as flexible as possible hoping that it will be

useable with any Terminal Unit (TU) and radio combination.

I have attempted to provide enough information to help you

make your system operational.  The program will work on the

IBM PCjr with one minor limitation which is explained

further on in this document.  I suspect that the program

will also work on the IBM Portable PC, most PC clones and

possibly the IBM PC AT, even though I have not had the

opportunity to test the program on those machines.

I intend for the program to be distributed by hams, rather

than retailers.  I suggest the following.

DISTRIBUTION GUIDELINES

Please share (copy) the program with others who are

interested in RTTY.

Do not sell or barter the program to anyone. (That's my

right!)

When you pass along the program, include only the

original unmodified version.

Send a SASE along with your current version number and

I will let you know if any known problems have been

fixed by a later version.

Don't remove these guidelines from the program or

document.

If you find the program of value, a small contribution

for my effort will be appreciated ($25 suggested).

Introduction                                               1

IBM PC RTTY Ver 1.2F

SUMMARY OF VERSION 1.2 CHANGES

�   Capability to clear the transmit buffer.

�   Capability to save the transmit buffer, clear it and

later restore it.

�   Up to 10 filenames can be specified for easy loading

into the transmit buffer from diskette.

�   The screen can now be used in color instead of white

only.  The colors can be modified to your own taste.

�   The program can now handle any oscillator frequency

input to the 8250 Async chip.  (e.g. The PCjr's

frequency is different from the PC.)

�   There are now separate baud rates for Baudot and ASCII

modes.  Additional baud rates can be user specified for

extended flexibility.

�   Several enhancements have been added for our European

friends

-   The European date format is supported. (e.g.

15.Nov.1984)

-   100 and 200 baud rates are now in the default ASCII

set.

-   The column for the warning beep is 56 and there is

an automatic CR/LF at column 64.  These values can

be changed and the automatic CR/LF turned off.

-   The Baudot character set is modified to support the

European standard.

�   Several characters have been defined to support unique

ASCII to Baudot conversions for MARS usage.

�   The ASCII to Baudot and Baudot to ASCII conversion

tables can be modified at program startup time to

support unique conversions.

�   The title screen will now go away after 10 seconds or

after the first keystroke.

�   The default time type can be specified in the

"PARMS.RTY" file and no prompting will occur.

�   The operation of the DTR and RTS outputs can be user

specified for transmit and receive.

Introduction                                               2

IBM PC RTTY Ver 1.2F

�   The number of ASCII data bits, parity bits and stop

bits can now be specified.

�   SELCAL has been implemented for unattended monitoring

operations.

�   A simple QSO logging capability has been added.

SUMMARY OF VERSION 1.2A CHANGES

�   Added MARS keyword support to the PARMS.RTY file

processor.

SUMMARY OF VERSION 1.2B CHANGES

�   Redefined some MARS special characters for ASCII to

Baudot translation.

�   Fixed a bug, in ASCII mode "+" would be sent 12 times,

now it is sent only once.

�   Enhanced the setting of baud rates in the PARMS.RTY

file.  You can now specify the rates positionally (you

define the order).

SUMMARY OF VERSION 1.2C CHANGES

�   Fixed bug when a transmit file with blank lines was

sent, two line feeds were sent for each blank line.

�   Fixed a bug in receive mode, with unshift on space

turned on, the program assumed an automatic unshift

with CRs and LFs.

�   Provided a method to make the program start up in the

ASCII mode.

�   Added the capability to specify the parms filename on

the command line.

�   Provide some special support for packet radio users.

Introduction                                               3

IBM PC RTTY Ver 1.2F

�   Enhanced the MARS support to include the automatic

sending of 12 letters functions when "NNNN" is sent.

SUMMARY OF VERSION 1.2D CHANGES

�   Made a change to the handling of blank lines.  Changed

the default to always send carriage return and line

feed for blank lines.  If you only want line feeds for

blank lines then specify "BLLF" in the parameter file.

SUMMARY OF VERSION 1.2E CHANGES

�   Fixed the scroll routine to handle foreground and

background colors.

�   Changed the program to allow a greater range of

foreground and background colors.

�   Added the capability to specify the SELCAL end string

and added an alternate string capability.

�   Provided the ability to start the program in the ART

mode and/or in the unshift-on-space mode.

�   Added the capability to inhibit the bell (beeps) in the

ART mode.

�   Added some enhancements for packet operation.  Packet

starts up in the transmit mode.  You can specify a

string to send the current time and date to your TNC.

You can inhibit sending of the time and date on

power-up and send it later with the use of the alt-D

character.  You can let the system send a note that you

are not around and then save data to a file for stored

messages.

�   I put the scrolling routine assembler source code in

the BAS file so there would be one less file to send

and keep track of.

Introduction                                               4

IBM PC RTTY Ver 1.2F

SUMMARY OF VERSION 1.2F CHANGES

�   Fixed a bug where the program would quit abruptly if

SELCAL was turned on and the SELCAL ending sequence was

received when the starting sequence had not been

received.

�   Fixed a problem by making the program ignore null

entries in the parms file.

�   Fixed a bug in the MARS option when "NNNN" is

received.

�   Added the capability to vary the receive and transmit

screen size.

�   Added the capability to customize the Date and Time

stamp to your own liking.

�   Added the capability for the program to append the

extension of "RTY" to the name of the parms file given

when RTTY is invoked.

�   Added the ability to specify the UTC time difference in

the parms file instead of being prompted for it.

�   Added the capability to send received data to both the

printer and diskette with the SELCAL option.

�   Added the function when receiving data to a file for

the program to save the file after every 15 lines of

text.  This will keep you data loss to a minimum if you

happen to lose power in the middle of a session or

someone accidentally resets the entire system for you.

�   The program now figures out if you are running on a

PCjr, and sets all of the correct parameters for you.

Introduction                                               5

IBM PC RTTY Ver 1.2F

PROGRAM OPERATION

The program is started by placing the program diskette in

the default drive (usually drive A:) and entering

RTTY     or      RTTY filename

In the above example, "filename" is the name of the

parameter file to use instead of "PARMS.RTY".  This feature

allows multiple runtime configuration files to be used for

various operation modes.  This and other files should be

personalized prior to running the program with your

callsign and other pertinent information.  These files are

identified further on in this document and may be changed

with any text editor.  The program is primarily written in

BASIC and compiled to achieve a useable operating speed.

There is one subroutine written in Assembly Language to do

things which I can't do well in BASIC.  Specifically, the

routine scrolls the receive and transmit screens.

The program takes advantage of the IBM Personal Computer's

capabilities.  The interrupt capabilities of the

Asynchronous Communications adapter and the keyboard allow

many things to occur simultaneously without the loss of

information.  The use of DOS files allows the storage and

retrieval of a large supply of RTTY data.  I hope you find

the program's many features useful.

In version 1.2 I have tried to make the program a little

more flexible.  The price that is paid for flexibility is a

little more complexity in setup.  One of the changes is to

allow the use of the program on the IBM PCjr.  The PCjr has

a slightly different input frequency to the 8250 Async chip

and it does not have DMA (direct memory access) during

diskette operations.  The significance of no DMA is that

data through the Asynchronous port will stop during any

diskette accesses.  Specifically, if you are sending the

received data to a diskette file, then about every 500

characters there is a chance of missing one or more

received characters.  During some tests which I performed

with a 5200 character test file, 2 characters were dropped

at 45.5 baud (BAUDOT mode), 8 characters were dropped at 74

baud (BAUDOT) and 110 baud (ASCII), 28 characters were

dropped at 300 baud (ASCII) and 35 characters were dropped

at 1200 baud (ASCII).  Note that this is a hardware

limitation as noted in the PCjr Technical Reference.

Characters are only lost when received characters are

placed in a diskette file, all other operations of the

program are functional with no known problems.

Program Operation                                          6

IBM PC RTTY Ver 1.2F

SYSTEM REQUIREMENTS

An IBM Personal Computer with at least 128K bytes of

memory and a minimum of one diskette drive or a true

IBM PC compatible computer.

An IBM Personal Computer Asynchronous Communications

Adapter or equivalent.  Note that for the compiled

BASIC program to work correctly, the Asynchronous

adapter must use an 8250 UART chip mapped into the same

I/O address space and interrupt level as the IBM PC

version.

IBM Personal Computer Disk Operating System (DOS)

version 1.1 or later (or equivalent).

A Terminal Unit (TU) for amateur radio teletype (RTTY)

applications.  Or a packet terminal node controller

(TNC) for packet radio operation.

A radio receiver or transceiver capable of operating

RTTY.

Program Operation                                          7

IBM PC RTTY Ver 1.2F

FEATURES

The program will run under DOS 1.1 or later.

The program will run on a monochrome display and both

the 80X25 and the 40X25 mode on a color display.

The program runs in a split screen mode with received

data displayed on the top and the transmit buffer

displayed on the bottom of the screen.  The number of

lines used by the receive and transmit screens can be

varied by using the RXLN keyword in the PARMS.RTY

file.

Run time information is displayed in the middle of the

screen, such as current time, date and elapsed time

since the last TTY ID was sent.

Prompts for operator input are also displayed in the

middle of the screen.

The definition and status of the function keys is

displayed at the bottom of the screen.

Errors and informational messages are also reported at

the bottom of the screen.

In transmit mode the program sends CR, CR, LF, and

letters at the end of a line.  This provides some

degree of compatibility with the old mechanical

teletype machines.

The program supports both BAUDOT and ASCII RTTY modes.

�   Common BAUDOT rates of 60, 67, 75, and 100 WPM are

supported.  Those are baud rates of 45.5, 50, 56.9,

and 74.2 respectively.

�   Common ASCII baud rates of 100, 110, 200, 300, 400

and 1200 BAUD are supported.

�   Additional baud rates can be user specified in the

"PARMS.RTY" file up to a maximum of 10 rates on

both modes.

The received input goes to the display and can be

directed to the printer and/or a DOS file.  File

compression is supported which can result in using less

diskette/disk file space.

While you are receiving, you can fill a 4000 character

transmit buffer from the keyboard or DOS files.

Program Operation                                          8

IBM PC RTTY Ver 1.2F

While the program is prompting you for information,

received characters are placed into a 4000 character

receive buffer.  If more than 4000 characters are

received while the program is prompting you, a

communications buffer overflow error will result.  The

program will attempt to recover as much as possible.

There are several predefined files which can be loaded

into the transmit buffer with the stroke of a function

key.  Up to 10 files can be user defined to be loaded

into the transmit buffer with the use of the "Alt" and

numeral keys.  Up to six files can be queued to be

placed into the transmit buffer at any one time.

A cursor is displayed at the current Transmit buffer

location.  When the cursor reaches column 56, the

program will sound a beep.  The user can define a new

warning column and can specify an ending column with an

automatic CR/LF insertion.

The transmit buffer can be edited minimally by using

the backspace key.  There is no full screen support.

In the transmit mode, a date and time stamp can be sent

over RTTY, as well as, a predefined RTTY ID field.

There is a key defined to allow the storage of the

callsign of the other station in the QSO.  The contents

of this storage field will be placed at the beginning

of the TTY ID when it is selected.  This buffer can be

changed at any time.

The transmit buffer is filled first by any transmit

files, second by characters resulting from special

keys, and finally from keyboard input.  The program

will stop placing characters in the transmit buffer

when it is full and will resume as characters are

transmitted from the transmit buffer.

A special Baudot mode can be selected for European

usage.

Program Operation                                          9

IBM PC RTTY Ver 1.2F

Some special characters have been defined for ASCII to

Baudot translation for the MARS program.  These

characters and their meanings are as follows

* causes a carriage return to be sent

= causes a line feed to be sent

+ causes 12 letters characters to be sent

@ causes the bell character to be sent

\ causes the null character to be sent (&H00)

> causes the figures character to be sent

< causes the letters character to be sent

Note that these characters may not operate correctly in

the European mode.

An additional MARS mode reduces the end-of-line

function to CR, CR, LF and does not send a letters

character automatically at the end-of-line or on the

space character.

SELCAL is now supported to automatically print or store

received signals when a user specified sequence of

characters is received.

The transmit buffer can be cleared or stored for those

times when you really want to send something other than

what's in the buffer.

A limited logging function can be executed with the use

of a few keystrokes.  You will be prompted for

pertinent information and it will be placed on

diskette.

When multiple blank lines are sent, the program only

sends line feed characters to speed things up.

Carriage returns are not sent for blank lines.

Special support has been added for packet radio

controller support.  I implemented a few things for use

with my Kantronics Packet Communicator.  I believe that

these are compatible with most TNCs on the market which

follow the TAPR command structure.  There is a DAY

command to set the clock in the TNC.  The program will

also send a message when you are away from the

keyboard.

Program Operation                                         10

IBM PC RTTY Ver 1.2F

SPECIAL KEYS

Backspace - Pressing the backspace key erases the last

character entered into the transmit buffer and the

cursor backs up.  If the last character was a line

feed, the cursor backs up to the previous line and the

character is displayed in reverse video.  When you

backspace beyond the top line of the transmit screen,

the previous line entered into the transmit buffer

appears on the top line.  Lines prior to the line at

the top of the screen are not visible.  Note that you

cannot back up beyond the last character sent to the

Asynchronous communications adapter.

ENTER - Pressing the "ENTER" key at the end of a line

causes a carriage return and line feed to be placed

into the transmit buffer.

Ctrl E - Ctrl E is a new added feature.  Typing a Ctrl

E into the transmit buffer will cause the program to

automatically return to the receive mode upon reading

the Ctrl E from the transmit buffer.  Additional

transmit buffer contents are retained but not

transmitted until the XMIT mode is entered again.  This

should be useful when you are transmitting a long file

and you want to take a break and may not make it back

in time to switch it over manually.

Ctrl S - Pressing Ctrl S causes the program to prompt

you for a line of characters which will be stored for

later retrieval.  The information is kept in memory

until the program is ended or another Ctrl S is

entered.

Ctrl R - This is the companion key of Ctrl S.  Pressing

Ctrl R retrieves the stored line and places it in the

transmit buffer.

Ctrl W - Ctrl W is pressed to instruct the program to

place only a carriage return in the transmit buffer.

Pressing the "ENTER" key immediately after a Ctrl W

will be ignored by the program.  Pressing a character

key will result in an overprint condition.  The Ctrl W

key will find primary application in sending RTTY art.

This character is placed in the receive file for any

carriage return.  An idea of what character may be

displayed by various editors can be found in appendix C

of the Technical Reference manual.

Note that the absence of the Ctrl W/Ctrl T at the end

of the line will result in the normal end of line

processing which send CR, CR, LF and ltrs.

Program Operation                                         11

IBM PC RTTY Ver 1.2F

Ctrl T - Ctrl T is the companion of the Ctrl W key and

results in a single line feed being placed in the

transmit buffer.  Pressing "ENTER" immediately after a

Ctrl T will be ignored by the program.  The Ctrl T key

will be used in RTTY art and may be used to replace a

line feed erased by backspacing.  This character is

placed in the receive file for any line feed.  An idea

of what character may be displayed by various editors

can be found in appendix C of the Technical Reference

manual.

Note that the absence of the Ctrl W/Ctrl T at the end

of the line will result in the normal end of line

processing which send CR, CR, LF and ltrs.

Ctrl X - Ctrl X is provided to force the sending of the

"Figures" character in Baudot mode only.  This key

could be useful in send a SELCAL prefix.  This key will

appear in the transmit buffer as a up arrow.

Ctrl Y - Ctrl Y can be used to force the sending of the

"(ltrs)" character in BAUDOT mode only.  This key can

also be used in sending a SELCAL prefix.  This key will

appear in the transmit buffer as a down arrow.

Alt A - Alt A can be used to toggle between the Art

mode and the No Art mode.  Art mode is for overstrike

lines to darken (fill in) the print.  Note that

multiple strikes are visible only on the printer, not

on the display.  The No Art mode is used for noisy

conditions (primarily text) where you want a CR/LF any

time a CR or LF is received for maximum copy.  The

status of the Art mode is included at the bottom of the

screen in the receive mode.  Art mode is indicated by

an "A" beside "RECV" and the No Art mode is indicated

by an "N".  At program start the default is the No Art

mode.  The "BELL" file on the diskette is an example of

RTTY Art.

Note that files which are received in the Art mode will

have Ctrl-W and Ctrl-T characters in the file.  Files

received in the no Art mode will not contain the Ctrl-W

and Ctrl-T characters.

Alt C - This option can be used to clear the transmit

buffer.  The buffer is cleared, all loading from

diskette files is stopped, and you can start again with

a clean slate.

Program Operation                                         12

IBM PC RTTY Ver 1.2F

Alt D - This option will send the current time and date

over the Asynchronous port in the format specified by

the PDAT keyword.  The PDAT keyword is relatively free

format to allow for different TNC implementations.  See

the PARMS.RTY description for additional details.

Alt T - This option saves the transmit buffer in a file

named "TXBUFFER.RTY" and then clears the transmit

buffer just like the "Alt C" option.  If the transmit

buffer was previously saved (this session) it will

first clear the transmit buffer and then load the file

"TXBUFFER.RTY".  Note that once the file has been

restored it will reset the flag indicating that the

transmit buffer was previously saved.

Alt S - Alt S is used to toggle the SELCAL (Selective

Call) option on and off.  When the SELCAL option is

turned on, you will be prompted to find out if you want

to send the selected data to the printer, diskette or

both.  A filename will be asked for when the diskette

option is selected.  A message will appear at the

bottom of the screen when the option is turned off.

The character string to search for to start saving the

data can be specified in the "PARMS.RTY" file.  Because

there is no letters or figures character in ASCII, I

have included the capability to specify a separate

SELCAL ID for both the BAUDOT and ASCII mode.  SELCAL

saving is terminated by the receipt of the sequence

"NNNN" in the default mode.  You can use the ESEL and

GESL keywords to specify your own unique SELCAL

terminator.

Alt L - This option will place information into a

logging file with a filename of "LOG.RTY".  You will be

prompted for the callsign of the station worked and for

any other pertinent data.  You may want to include such

things as signal reports, frequency, names and so on.

The date, time and your data will be appended to the

"LOG.RTY" file.

Alt N - This option toggles the not-at-keys

enable/disable flag.  The current status is displayed

on the error line for approximately 5 seconds.  When

turned on, the program will send the file "AWAY.MSG"

when a string is received which matches the NKEY

string.  All incoming data will be stored in the file

"WHILE.OUT" or the already active received data file.

This option is primarily for packet use.

Alt 0-9 - This key sequence will load the file defined

in the "PARMS.RTY" file corresponding to the file

numbers 1 to 10.  If the filename has not been

specified, then no action will be taken.

Program Operation                                         13

IBM PC RTTY Ver 1.2F

Note that some of the special keys may be the same as the

control keys for some TNCs.  The RTTY program will take

action on the special key and will generally not pass any

code over the Asynchronous lines.  To circumvent the

inherent problem, my suggestion is to respecify the control

key to the TNC, making sure that you do not pick another

RTTY special key.

Program Operation                                         14

IBM PC RTTY Ver 1.2F

FUNCTION KEYS

The ten function keys are used during program operation to

direct the flow of information in the computer system.  The

definition and status of the function keys is displayed at

the bottom of the screen.  The function keys are defined as

follows:

General Function Keys

F1  -  KEYS

This key toggles the definition of function keys 3

through 10 and changes the status line.

F2  -  RECV/XMIT

This key toggles the program between receive mode

and transmit mode.  When the program begins, this

key defaults to RECV.  When the key is pressed to

go from XMIT to RECV, the program will wait until

the basic communication buffer is empty before

returning to the RECV mode.  BASIC buffers up to

128 characters independent of the program.  So

don't get too anxious if the transmitter doesn't

stop immediately.

There is a short delay of about one half a second

from the time the program turns the transmitter on

and the first data is sent.  This should give the

other station a chance to lock onto your signal.

Additionally, an A or N will appear with RECV to

indicate the current status of the Art mode (See

Alt A for additional information).

Program Operation                                         15

IBM PC RTTY Ver 1.2F

Function Key Set 1

F3  -  RX FLE

Receive a File.  Pressing this key prompts you for

a filename in which to place the received

information.  The information is appended to the

file.  Pressing only "ENTER" results in no file

being opened.  Pressing this key while a file is

currently open will close the file.  Another

filename must be entered to continue saving data.

Entering the same name as an existing file will

result in that data being appended to the file.

When a filename is entered, you are asked if you

want the file to be saved as a compressed file.

Compressed files will take up less room on your DOS

diskette, but are not readily alterable in an

editor.  A utility is provided to decompress or

compress a file offline.

When a receive file is open, a reminder is

displayed on the middle status line of the screen.

The form of the display is "RX-filename".  The

program will close and then reopen the file after

every 15 lines of input.  This forces the computer

to update the file status and prevents the loss of

the entire file should you happen to lose power in

the middle of a session or if the computer managed

to get reset in the middle of a session.

F4  -  UOS ON/UOS OF

This key toggles between "Unshift On Space" being

turned on and turned off.  This key only applies to

received data.  When "unshift on space" is on, the

program will shift back to "letters" mode any time

a space is received.

Independent of this key, the program will transmit

a "(ltrs)" character in the BAUDOT mode as required

with a space.  This makes the program compatible

with either type of teletype which may be in the

other station.  When the program begins, this key

defaults to UOS OF.

F5  -  END

This key provides a means to terminate the program

gracefully.  You will be prompted for confirmation

of your request to quit.

Program Operation                                         16

IBM PC RTTY Ver 1.2F

F6  -  PRT OF/PRT ON

This key toggles output to the printer on and off.

When PRT ON is displayed, all characters received

will also be printed on the matrix printer.  Bell

characters are not sent to the printer to keep down

the noise pollution.  When the program begins, this

key defaults to PRT OF.

F7  -  Baud Rate Selector

This key toggles through the baud rates supported

by the program.  The status line is updated with

each change.  In Baudot mode, the baud rates of

45.5, 50.0, 56.9, and 74.2 correspond to character

rates of 60, 67, 75, and 100 words per minute (WPM)

respectively.  When the program begins, this key

defaults to 45.5.

For ASCII the baud rates of 100, 110, 200, 300,

400, and 1200 are supported.  When the ASCII mode

is selected this key defaults to 110 baud.

Up to 10 baud rates per mode are possible by adding

baud rates through the use of the BAUD and ASCI

options of the "PARMS.RTY" file.  The default rates

can also be changed.

F8  -  BAUDOT/ASCII

This key toggles between Baudot and ASCII modes.

Any combination of baud rate and mode is

obtainable.  When the program begins, this key

defaults to BAUDOT unless specified otherwise in

the parms file.  Each time this key is pressed, the

baud rate selection will change to the first baud

rate for that mode, the default if 45.5 baud for

Baudot and 110 baud for ASCII.

F9  -  NEW LN

This key provides the ability to force a CR/LF (new

line) in the middle of received text.  Such an

operation might be needed if the original CR/LF

were lost in the noise.

F10 -  LTRS

This is the "letters" key.  During Baudot reception

it is occasionally desirable to force the receive

mode back into the "letters" mode.  This key

accomplishes that task.

Program Operation                                         17

IBM PC RTTY Ver 1.2F

Function Key Set 2

F3  -  TX FLE

Transmit a File.  Pressing this key allows the

contents of a DOS file to be placed into the

transmit buffer for transmission.  You will be

prompted for the filename of the file to transmit.

Pressing just "ENTER" will result in no file being

transmitted.  Entering the word "stop" will cause

the current file being processed to be closed.

The filename is placed in a buffer que which will

eventually be opened and read.  This allows for

placing several transmit files into the transmit

buffer in sequential order.  The filename of the

file currently being read is displayed on the

center display line.  The form of the display is

"TX-filename".

If an error is encountered in opening the file, an

error status is displayed for about five seconds at

the bottom of the screen and a beep is sounded.

Note that the program will not handle tab

characters.  The file should contain spaces to get

to the correct column.  For example, if the file is

created using the Personal Editor program, be sure

to use the "notabs" keyword when the file is

saved.

F4  -  TX CQ

Transmit a CQ.  Pressing this key causes the file

"CQ.RTY" to be loaded into the transmit buffer.

This file is queued just like a transmit file and

is displayed on the middle status line of the

screen.  Note that you must personalize the

"CQ.RTY" file with your callsign and name.

F5  -  RESET

This is the reset key.  Pressing this key will

close all transmit files, clear the screen and

start the program over.  There is no prompting to

make sure you mean it.  Note, BASIC sometimes

decides to clean up the string area at the most

inappropriate times.  When it does this, the system

may appear to lock up for several seconds (maybe

many), so try to be patient.

Program Operation                                         18

IBM PC RTTY Ver 1.2F

F6  -  TX RYS

Transmit RYs.  Pressing this key will result in the

"RYS.RTY" file to be placed into the transmit file

que and eventually into the transmit buffer.  This

is handy for placing a bunch of RYs at the start of

your transmission, which gives the guy at the other

end a chance to lock back onto your signal.

F7  -  PSE OF/PSE ON

Transmit Pause Off / Transmit Pause On.  This key

is used to temporarily stop the program from

sending any more characters out of the transmit

buffer.  This is useful for sending a steady

carrier signal for tuning purposes or whatever.  It

should be noted, that BASIC can buffer up to 128

characters.  These characters cannot be stopped

from being sent.  Thus, you may not be able to make

the program pause exactly where you want.  When the

program begins, this key defaults to PSE OF.

F8  -  TTY ID

Teletype ID.  Pressing this key causes the contents

of the "TTYID.RTY" file to be placed into the

transmit buffer.  The contents of the QSO ID buffer

is placed at the beginning of the line.  Note that

this line of data is read from the file at the

beginning of the program and stored in program

memory.  Also note that the "TTYID.RTY" file must

be personalized with your name and callsign.  The

little block at the end of the TTYID when it is

placed in the buffer is used by the program to

reset the time since last ID.

F9  -  QSO ID

This key prompts you for an entry of the current

QSO station id.  The data entered is placed at the

beginning of the TTY ID when that key is pressed.

F10 -  DT&TM

Date and Time stamp.  This causes the current date

and time to be placed into the transmit buffer.

Note that the time sent is the time the key is

pressed, not the time the characters are actually

sent.

Program Operation                                         19

IBM PC RTTY Ver 1.2F

PROGRAM FILES

There are three files which the program expects to find on

the default DOS drive.  If any of these files are absent

when they are accessed, an error message will be

displayed.  There is also an optional file which may be

absent with no adverse effects.  There are also two files

which will be created if absent and will be written to by

the program.  The files are described below.

Required Files

TTYID.RTY - This file is required to have one line

only.  The line contains the characters to be placed in

the transmit buffer for your RTTY ID (e.g. DE KF4NB

"GLENN" IN LEXINGTON, KY).  Note, you must personalize

this file with your callsign, name, location and

whatever.

CQ.RTY - This file contains all the information you

would like to send for a RTTY CQ.  Note, you must

customize this file with your callsign, name, and

location.  You may optionally change the format of the

CQ.  This file may contain any number of lines.

RYS.RTY - This file is simply a line of RYs which will

be sent in response to a function key being pressed.

This file could be changed in just about any way you

desired.  Multiple lines are allowed.

Program Operation                                         20

IBM PC RTTY Ver 1.2F

Optional Files

LOG.RTY - This file will be appended to by the "Alt L"

command.  The information placed in the file is the

date, time, Station ID worked and your comments.  If

the file does not exist, a new file will be created.

TXBUFFER.RTY - This file will be created or read by the

"Alt T" command.  The transmit buffer will be dumped

into the file on the first use of the "Alt T" command

and the file will be reloaded into the transmit buffer

on the second occurrence of the "Alt T" command.

AWAY.MSG - This file will be sent when the not-at-keys

string is received.  This is primarily intended for

packet operation to allow you to tell someone to leave

you a message when they connect.   All received input

up until the time the end string is received

(disconnect) will be placed into the file "WHILE.OUT".

Both the start and stop strings can be user defined.

PARMS.RTY - This file can have one or more of the

following keywords on a single line separated by commas

or on separate lines.  The first three or four

characters of the words are compared and when a match

is found, the pertinent information is obtained from

the text string.  Any extra information between commas

or at the end of the line is ignored.  Blank lines are

ignored.  Commas can be imbedded in a keyword string by

placing the entire keyword string in double quotes.

For example, "DTTM=HH:MM Z  MONTH DD, YYYY".

The filename can be overridden by specifying the

alternate filename on the command line (e.g. RTTY

ASCII.RTY).  The contents of the file may be any valid

keywords for the PARMS.RTY file.  The program will

automatically attempt to append the RTY extension, if

it is unable to find the filename specified.  When an

alternate filename is specified on the command line,

the program will prompt for a new filename anytime

during that session that the RESET key is used.

As the file is processed, each parameter specification

is displayed on the screen.  When the file has been

processed, the program will wait approximately three

seconds and then clear the screen and continue.  This

waiting period can be bypassed by pressing any key.

During processing of the file any parameters which are

incorrectly specified will be displayed on the screen

and the program will wait for you to press "Enter" to

continue.

Program Operation                                         21

IBM PC RTTY Ver 1.2F

AGSL - This keyword is used to set the general

SELCAL search string for ASCII mode selection.  The

format is AGSL=QST.  Up to 10 characters can be

specified.  The default is "QST".

ALCR - This keyword can be specified to override

the default value put into the ASCII Line Control

Register of the 8250 Async chip.  This value will

determine format of the ASCII data.  It controls

the number of data bits, the number of stop bits,

use of the parity bit and the type of parity if

used.  The format of the option is ALCR=&H3E.  The

default value is &H3E, which sets the ASCII mode to

7 data bits, 2 stop bits and parity bit stuck low.

Further information on the possible values of this

parameter can be found in the Technical Reference

Manual.

ART  - This keyword allows you to start the RTTY

program in the ART mode.  If it is not specified

the program defaults to NoART mode.

ASCI - This keyword is used to add extra values to

the ASCII baud rate selection list.  The format is

ASCI=nnnn where "nnnn" is any baud rate number you

want to be able to run.  Up to 10 baud rates are

allowed.  A second form of the keyword allows you

to redefine the baud rate for any of the rates in

the list (even the default values).  The format is

ASCIxx=nnnn where "xx" is a number from 1 to 10 and

"nnnn" is the desired baud rate.  The number "xx"

must correspond to a previously defined rate or

just one more than the total number of rates

defined.

When the baud rate is specified, the program will

set the 8250 Async chip timer divisor rate to the

closest value to achieve the specified rate.  The

divisor rate of the 8250 is set to the integer

value of the crystal frequency divided by the baud

rate.  A total of four ASCII baud rates can be

specified.  Refer to the Technical Reference manual

for further information on divisor rates.

ASEL - This keyword is used to set the specific

SELCAL search string for ASCII mode selection.  The

format is ASEL=KF4NB.  Up to 10 characters can be

specified.  The default is "??????????".

Program Operation                                         22

IBM PC RTTY Ver 1.2F

ATCR - This keyword is used to turn on the

automatic carriage return and line feed (CR/LF)

option.  When this option is on, a CR/LF will be

added to the transmit data if the data reaches

column 64 or the column specified in the LLEN

option.  This option defaults to off.

BAUD - This keyword is used to add extra values to

the Baudot baud rate selection list.  The format is

BAUD=nnn.n where "nnn.n" is any baud rate number

you want to be able to run.  Note that the baud

rate is different from the words per minute rate.

Up to 10 baud rates are allowed.  A second form of

the keyword allows you to redefine the baud rate

for any of the rates in the list (even the default

values).  The format is BAUDxx=nnn.n where "xx" is

a number from 1 to 10 and "nnn.n" is the desired

baud rate.  The number "xx" must correspond to a

previously defined rate or just one more than the

total number of rates defined.

When the baud rate is specified, the program will

set the 8250 Async chip timer divisor rate to the

closest integer value to achieve the specified

rate.  The divisor rate of the 8250 is set to the

integer value of the crystal frequency divided by

the baud rate.  A total of six Baudot baud rates

can be specified.  Refer to the Technical Reference

manual for further information on divisor rates.

BDLW - This keyword provides the capability to

change the Baudot to ASCII conversion table of the

program.  The BDLW command is used to specify the

letters mode Baudot character to ASCII character

relationship.  The format of the command is

BDLWnn=xxxx.  Where "nn" is a number from 0 to 31

representing the 5 bit Baudot code and "xxxx" is

the value of the ASCII character.  For example,

"BDLW10=&H52" causes the receipt of a Baudot

character of 10 in the letters mode to be

translated to an ASCII character of &H52 which is

the correct representation of the letter "R".  Any

number of BDLW commands may be contained in the

file.

Program Operation                                         23

IBM PC RTTY Ver 1.2F

BDUP - This keyword provides the capability to

change the Baudot to ASCII conversion table of the

program.  The BDUP command is used to specify the

figures mode Baudot character to ASCII character

relationship.  The format of the command is

BDUPnn=xxxx.  Where "nn" is a number from 0 to 31

representing the 5 bit Baudot code and "xxxx" is

the value of the ASCII character.  For example,

"BDUP10=&H34" causes the receipt of a Baudot

character of 10 in the figures mode to be

translated to an ASCII character of &H34 which is

the correct representation of the number "4".  Any

number of BDUP commands may be contained in the

file.

BDOT - This keyword provides the capability to

change the ASCII to Baudot conversion table of the

program.  The BDOT command is used to specify the

ASCII character to Baudot character relationship.

Part of this command requires the capability to

specify if the Baudot character should be sent in

letters mode, figures mode or don't care mode.

This is accomplished by adding 64, 128 or 192

(decimal) respectively to the Baudot character.

This can also be represented by &H40, &H80, and

&HC0 (hexidecimal).  The format of the command is

BDOTnn=xxxx, where "nn" is the value of the ASCII

character and "xxxx" is the value of the Baudot

character with the added value for the proper

mode.  For example, "BDOT52=&H8A" causes an ASCII

character of 52 ("4") in the transmit buffer to be

sent as a "figures" followed by a Baudot 10.  Note

that if the figures mode was already valid then no

figures character would have been sent.  Any number

of BDOT commands may be contained in the file.

BGSL - This keyword is used to set the general

SELCAL search string for Baudot mode selection.

The format is BGSL=(ltrs)N(ltrs)N(ltrs)N where the

"(ltrs)" indicates the letters Baudot character.

Up to 10 characters can be specified.  The default

is "(ltrs)N(ltrs)N(ltrs)N".  Note that "(ltrs)" is

defined to be "Ctrl Y" and "(figs)" is defined to

be "Ctrl X" during program operation.

BLLF - This keyword is used to suppress the sending

of carriage returns when a blank line is

encountered.  Only line feeds will be sent for

blank lines when this option is used.

Program Operation                                         24

IBM PC RTTY Ver 1.2F

BSEL - This keyword is used to set the specific

SELCAL search string for Baudot mode selection.

The format is BSEL=(ltrs)KF4NB where the "(ltrs)"

indicates the letters Baudot character.  Up to 10

characters can be specified.  The default is

"??????????".  Note that "(ltrs)" is defined to be

"Ctrl Y" and "(figs)" is defined to be "Ctrl X"

during program operation.

COLO - This keyword is used to set the values for

the color characters for a color display.  The

colors can be further modified independently

through other options of the "PARMS.RTY" file.  Any

further changing must occur after the COLO

statement.  The default is to black and white

characters.

COM2 - This keyword will cause the program to use

the second asynchronous communications adapter for

RTTY.  The program default is the first adapter

(COM1).  This option may be useful if the first

adapter is primarily used for some other reason

such as a telephone modem or a serial printer.

This keyword must be used with a PCjr if an

internal modem option is installed.

COMM - This keyword specifies the base I/O address

of the 8250 chip (serial port).  The default is

COMM=&H3F8.  For the PCjr the parameter should be

specified as COMM=&H2F8.  If the PCjr also contains

the internal modem option, the COMM keyword must

come after the COM2 keyword in the PARMS.RTY file.

Note that beginning in version 1.2F, the program

recognizes that you are running on a PCjr and

correctly sets the proper COMM value to &H2F8.

DIDL - This keyword is provided for those folks who

like to send something (even null) while the

transmit buffer is empty.  Null characters are sent

about once per second while the transmit buffer is

empty.

DTR  - This keyword can be used to specify what

logic level the DTR (Data Terminal Ready) line out

of the Asynchronous adapter should be set to for

receive and transmit modes.  The format is DTR=xy,

where xy can be 00, 01, 10, or 11.  The number

corresponding to "x" is the logic level during

receive and the number corresponding to "y" is the

logic level during transmit.  "0" corresponds to a

low logic level and a "1" corresponds to a high

logic level.  The default is DTR=01.

Program Operation                                         25

IBM PC RTTY Ver 1.2F

DTTM - This keyword lets you specify the format of

the date and time stamp which can be sent using

function key F10.  The normal default is "HH:MM:SS

TMT MONTH DD, YYYY", where HH is hours, MM is

minutes, SS is seconds, TMT is the time type (e.g.

EST), MONTH is the month name, DD is the day of the

month and YYYY is the year.  The default time will

change if the MARS or the EURO keywords are

specified.  By using DTTM you can customize your

time stamp to anything you would like it to be.

The following keys will result in the indicated

substitution actions:

HH    -  hours as two decimal digits

MM    -  minutes as two decimal digits

SS    -  seconds as two decimal digits

TMT   -  time type as specified by TIME keyword

MO    -  month as two decimal digits

MON   -  month as a three letter abbreviation

MONTH -  month as the actual month name

YY    -  year as two decimal digits

YYYY  -  year as four decimal digits

Any other characters entered along with the keyword

will be displayed as they appear, no substitution.

The form of the keyword is DTTM=HH:MM:SS Z  MO/DD/YY.

If you want to imbed a comma within the date and time

stamp, then the entire keyword must be placed within

double quotes.  For example,

"DTTM=HH:MM:SS TMT   MONTH DD, YYYY".

Program Operation                                         26

IBM PC RTTY Ver 1.2F

ECHO - This keyword causes each character sent to

the BASIC "COMn:" port to be displayed on the

receive portion of the screen.  This option is

provided for those who have TUs which do not place

the transmitted data back onto the receive line

coming back to the computer.  The program default

is no ECHO.

Note - BASIC has a 128 byte transmit buffer.  When

this keyword is active, the receive screen shows

the character when it is placed into the BASIC

buffer.  In other words, just because you see it on

the screen doesn't mean it has been sent yet!  The

program contains a check to make sure that the

Asynchronous adapter has finished before turning

off the transmitter.

If the receive buffer seems to be repeating

characters while in the transmit mode, then the

most likely problem is that you have specified echo

and your TU is also feeding back into the receive

input.  ECHO should be removed from the "PARMS.RTY"

file.

ERRB - This keyword is used to set the error line

background color.  The format is ERRB=7.  The

defaults are ERRB=7 for the monochrome mode and

ERRB=0 for the color mode.  Refer to the BASIC

manual for colors associated with possible values.

ERRF - This keyword is used to set the error line

foreground color.  The format is ERRF=0.  The

defaults are ERRF=0 for the monochrome and ERRF=12

for the color mode.  Refer to the BASIC manual for

colors associated with possible values.

ESEL - This keyword is used to set the specific

SELCAL search string for turning off SELCAL and/or

packet not-at-keys data storage.  The format is

ESEL=* DISCON.  UP to 10 characters are allowed.

The default is "NNNN".

Program Operation                                         27

IBM PC RTTY Ver 1.2F

EURO - This keyword is used to specify the European

mode.  The date and time stamp format is changed to

default to the form "HH:MM:SS TMT    DD.MON.YYYY",

where HH is hours, MM is minutes, SS is seconds,

TMT is the time type (e.g. EST), DD is the day, MON

is the three letter month name and YYYY is the

year.  The time format can be changed using the

DTTM keyword.  This EURO mode uses the automatic

CR/LF option and changes the Baudot to ASCII

conversion table to fit the European standards

instead of the U.S. standards.  The differences are

for figures mode only and are as follows

Baudot  European

code   character

05       '

09       #

11      bell

13       [

17       +

20       !

26       ]

30       =

Note that any parameter specifications using the

BDLW, BDUP or BDOT commands should occur after the

EURO command in the file.

FILE - This keyword is used to specify the names of

files to be transmitted with the use of the "Alt n"

keys, where "n" is one of the number keys at the

top of the QWERTY section of the keyboard.  The

format of the command is FILEnn=filename, where

"nn" is a number from 1 to 10 and filename is any

valid DOS filename.  For example, FILE1=PARMS.RTY

will cause the "PARMS.RTY" file to be loaded into

the transmit buffer when "Alt 1" is pressed.  Note

that "Alt 0" will cause FILE10 to be loaded.  Note

that the file must be in the current directory.

GESL - This keyword is used to set the general

SELCAL search string for turning off SELCAL and/or

packet not-at-keys data storage.  The format is

GESL=QUIT.  UP to 10 characters are allowed.  The

default is "NNNN".

KEYB - This keyword is used to set the keys status

line background color.  The format is KEYB=7.  The

defaults are KEYB=7 for the monochrome mode and

KEYB=3 for the color mode.  Refer to the BASIC

manual for colors associated with possible values.

Program Operation                                         28

IBM PC RTTY Ver 1.2F

KEYF - This keyword is used to set the keys status

line foreground color.  The format is KEYF=0.  The

defaults are KEYF=0 for the monochrome and the

color mode.  Refer to the BASIC manual for colors

associated with possible values.

LLEN - Use this keyword to specify the column at

which you want the end of the line to be.  This is

used in conjunction with the ATCR command.  The

format is LLEN=nn, where nn is the column number of

the end of the line.  The default is 64.

LPT2 - This keyword allows the printer output to go

to the printer attached to the second printer

adapter.  The default is the first printer adapter

(LPT1).

LPT3 - This keyword allows the printer output to go

to the printer attached to the third printer

adapter.  The default is the first printer adapter

(LPT1).

MARS - This keyword modifies the operation of the

program slightly for compatibility with the MARS

format rules.  Specifically, end-of-line processing

is set to CR, CR, and LF.  A letters character is

not sent automatically at the end of the line or

after the "space" character if a number preceded

it.  Also the bell character is represented by an

"@" in the receive file.  The letters character

after space can be reenabled by specifying

BDOT32=&H44 after the MARS keyword in the PARMS.RTY

file.  Twelve consecutive characters will be sent

after "NNNN" is transmitted.  The characters will

be the letters function or any alternate character

specified using the NCHR keyword.

MODE - This keyword can be used to make the program

start up in the ASCII mode.  The form of the

keyword is "MODE=A".  The normal default is to

start in the Baudot mode.

NCHR - This keyword is used in conjunction with the

MARS keyword to specify the character to send when

"NNNN" is sent.  The character will be sent twelve

times.  The form of the keyword is NCHR=nn, where

nn is the value of the character to be sent.  The

default is NCHR=60 which causes twelve letters

functions to be sent.

Program Operation                                         29

IBM PC RTTY Ver 1.2F

NKEY - This is the keyword to use to specify the

search string for the not-at-keys message during

packet operation.  When the string is found, all

received data up to the reception of the stop

search string will be saved in the file WHILE.OUT,

unless a receive file is currently open.  When this

string is received, the file AWAY.MSG is sent out

over the Asynchronous lines.  The format of the

keyword is NKEY=** CONNECT.  Up to 10 characters

are allowed.  The default is "* CONNECTE".

NOBP - This keyword is used to disable the bell

character (beep) on the PC while in the ART mode

only.  It is only controllable at program startup

time.

NPDT - This keyword tells the program to not send

the packet time/date over the asynchronous lines

when the program is just starting or has just been

reset.  To send the packet time/date, use the alt-D

key during normal program operation.

PACK - This keyword is used to make the controller

compatible with some packet radio TNCs.  The

program is modified to send only one CR at the end

of the line instead of the normal two CRs.  Also

when backspace is used, the program will send the

backspace character to the TNC for any character

already sent up to the last CR.  The program will

also erase the characters on the last line which

have been sent.  The program will come up in the

transmit mode.  Most TNCs are semi-full duplex, so

there is no need to run in the half duplex mode.

(You can send data at any time without tying up the

channel.)

PDAT - This keyword is used to specify the packet

time/date command format, which is sent by the use

of the alt-D key.  You can specify just about any

form you want with the exception of no imbedded

commas.  The format of the keyword is

"PDAT=MO-DD-YYYY AT HH:MM:SS EST".  Where the

program will substitute in the proper numbers from

the date and time commands.  The parameters to use

are YYYY for the full 4 digit year, YY for just the

last 2 digits of the year, MO for the month, DD for

the day of the month, HH for the hours digit, MM

for the minutes and SS for the seconds.  Any other

characters in the string will be sent as is with no

substitutions.  Any of the parameters can be left

out of the string.  The default is

"PDAT=DAY YYMODDHHMMSS".

Program Operation                                         30

IBM PC RTTY Ver 1.2F

QB   - This keyword causes a bell character to be

sent any time a quote character is sent in the

transmit mode.  This is possibly useful in RTTY

ART, since some machines have the quote and the

bell reversed.  No action is taken in the receive

mode.  The default is no QB.

RTS - This keyword can be used to specify what

logic level the RTS (Request To Send) line out of

the Asynchronous adapter should be set to for

receive and transmit modes.  The format is RTS=xy,

where xy can be 00, 01, 10, or 11.  The number

corresponding to "x" is the logic level during

receive and the number corresponding to "y" is the

logic level during transmit.  "0" corresponds to a

low logic level and a "1" corresponds to a high

logic level.  The default is RTS=01.

RXB  - This keyword is used to set the receive

buffer background color.  The format is RXB=0.  The

defaults are RXB=0 for the monochrome and RXB=2 for

the color mode.  Refer to the BASIC manual for

colors associated with possible values.

RXF  - This keyword is used to set the receive

buffer foreground color.  The format is RXF=7.  The

defaults are RXF=7 for the monochrome mode and

RXF=14 for the color mode.  Refer to the BASIC

manual for colors associated with possible values.

RXLN - This keyword is used to change the number of

lines used for the receive data area of the screen

and the transmit data area of the screen.  The form

of the keyword is RXLN=n, where n is the number of

lines you would like to use for the receive data

area.  The number is value checked to be within the

range of 2 to 20 (19 in 40 column mode).  The

remaining lines will be used for the status line,

key definition line(s), error reporting lines and

for the transmit data area lines.  Increasing the

receive lines will decrease the transmit lines.

The default number of receive lines is 11.

SPLF - This keyword is used to suppress the sending

of line feeds over the communications link.  This

may prove useful when the program is used to send

data to a packet radio TNC which cannot handle the

CR/LF normally sent at the end of a line.

Program Operation                                         31

IBM PC RTTY Ver 1.2F

STSB - This keyword is used to set the status line

background color.  The format is STSB=7.  The

defaults are STSB=7 for the monochrome mode and

STSB=6 for the color mode.  Refer to the BASIC

manual for colors associated with possible values.

STSF - This keyword is used to set the status line

foreground color.  The format is STSF=0.  The

defaults are STSF=0 for the monochrome and the

color mode.  Refer to the BASIC manual for colors

associated with possible values.

TIME - This keyword is used to set the time-type at

program startup time.  When this keyword is used,

the program will assume that the time and date are

set according to the time-type and no prompting for

the time will be done at program startup time.

(Generally people always set their time the same

way every time.)  The format is TIME=EST.  If

TIME=? is used then prompting will occur.

TXB  - This keyword is used to set the transmit

buffer background color.  The format is TXB=0.  The

defaults are TXB=0 for the monochrome and TXB=1 the

color mode.  Refer to the BASIC manual for colors

associated with possible values.

TXF  - This keyword is used to set the transmit

buffer foreground color.  The format is TXF=7.  The

defaults are TXF=7 for the monochrome mode and

TXF=11 for the color mode.  Refer to the BASIC

manual for colors associated with possible values.

UOS  - This keyword will cause the program to begin

operation with the unshift-on-space option turned

on.  The default is to start with the option turned

off.

WARN - Use this keyword to specify the column at

which you want the program to beep at you in the

transmit buffer to warn you that you are

approaching the end of the line.  The format is

WARN=nn, where nn is the column number at which you

would like to be warned.  The default is 56.

Program Operation                                         32

IBM PC RTTY Ver 1.2F

XTAL - This keyword is used to specify the crystal

input frequency to the 8250 Async chip.  The format

of the command is XTAL=1.8432.  For the IBM PCjr

XTAL is set to 1.78977.  Note that the complete

number must be given and that any inaccuracies will

affect the accuracy of the baud rate.  The default

is set to 1.8432 MHz.

ZULU - This keyword is used to specify that time is

to be sent in a military format.  The default time

format is "DDHHMMZ MON YY", where DD is the day, HH

is the hours, MM is the minutes, MON is the first

three letters of the month and YY is the last two

digits of the year.  If the time displayed on the

screen is anything other than "UTC", you will be

prompted at the start of the program for the amount

of time in hours to be added to the local time to

convert it to UTC time.  You can specify the

default time difference in the PARMS.RTY file and

you will not be prompted for the adder.  The form

of the keyword is ZULU=n where n is the number of

hours to add to the default time.  The time format

can be changed by using the DTTM keyword.  Anytime

the timestamp is placed in the transmit buffer, the

program will convert the displayed time to "UTC"

time, correcting for any overflow into the next

day, month and/or year.  Leap years are also taken

into account.  Additionally, when this keyword is

specified the timestamp will also be appended to

the end of your "TTY ID".

Program Operation                                         33

IBM PC RTTY Ver 1.2F

UTILITY PROGRAM

A utility program has been provided to perform some of the

normal program functions offline.  The utility is started

by entering

RTTYU

The program uses the same "PARMS.RTY" file to determine

which printer to use for output and to decide whether or

not to add a bell character after a quote.  The program

prompts you for the name of the file to be processed.  A

name must be entered and if any problems are encountered,

the program terminates.

You are then asked if you want the processed data sent to

another file.  If you answer in the affirmative, you will

be asked the name for the file.  Responding with only an

"ENTER" for the filename will be interpreted as meaning you

do not really want to save the data in a new file.  If the

filename is OK, you are then asked if you would like the

data in the new file compressed.  Here again as in other

places, responding with anything other than a word starting

with the letter "Y" will be taken to mean "NO".  This

feature should be good for uncompressing a file so it can

be edited or for compressing a file to save diskette

space.

You will also be asked if you want the file printed on the

matrix printer.  Answer in the affirmative if you would

like hard copy.  Again, this routine does not send the

noisy bell character to the printer.

Regardless of the choices you make in processing the file,

the information is displayed on the display so you can keep

current with the progress of the program.

Program Operation                                         34

IBM PC RTTY Ver 1.2F

BASIC COMPILING HELP

COMPILING THE RTTY PROGRAM

I am changing my stance on sending out the source code from

my original position.  Originally, I intended only for the

compiled program to be sent to the general public and the

source code to stay under relatively tight control.  Well,

as luck would have it, some well meaning soul took the

liberty of placing "My source code" on the public access

disk of CompuServe and Lord knows where else.  Well, if you

can't lick 'em. . . join 'em!  This little episode along

with some requests for help in getting it to run on some PC

clones has helped change my mind.

So much for the philosophical statements, now to the good

stuff.  There is one assembly language routine which must

be assembled.  This routine is a basic subroutine to scroll

half screens.  It must be properly linked into the basic

program before it will work.  If it isn't done correctly or

the BASIC program is run in the interpretive mode, chances

are the PC will lock up really well when the receive screen

is full.  Because the execution speed of the interpretive

mode is totally unacceptable (by my standards) I won't even

try to explain how to link in the assembly language routine

to the interpretive mode.  The following lines show how to

compile the RTTY program and properly link the appropriate

programs to end up with the executable RTTY file.

BASCOM RTTY,,,/X/W/O/C:4096/S       (Assumes source in B: and

LINK RTTY+RTTYSUBS+A:IBMCOM,,,A:      BASIC compiler in A:,

B: is primary)

BASIC Compiling HELP                                      35

IBM PC RTTY Ver 1.2F

COMPILING THE RTTYU PROGRAM

The RTTYU program is similarly compiled as follows

BASCOM RTTYU,,,/E/O                 (Assumes source in B: and

LINK RTTYU,,,A:                       BASIC compiler in A:,

B: is primary)

GETTING AND ASSEMBLING THE RTTYSUBS PROGRAM

Starting with version 1.2E, the Assembly language for the

Scrolling routine to support the half screens is included

in the RTTY.BAS file.  In order to properly assemble the

program, first of all copy the RTTY.BAS program to a file

called RTTYSUBS.ASM.  Next using an editor of some flavor,

preferably a full screen editor such as Personal Editor,

remove the first several hundred lines of the basic program

up to the start of the assembly code.  Next, remove the

BASIC line numbers and the quote mark and leading blank

characters from the remaining statements.  Once this has

been accomplished, you can use the macro assembler to

generate the object module necessary to link the Compiled

BASIC RTTY program.

BASIC Compiling HELP                                      36

IBM PC RTTY Ver 1.2F

INTERFACE INFORMATION

INTERFACE SIGNALS

This program interfaces to the Amateur Radio equipment

through the Asynchronous Communications adapter.  The

specifications for the voltage levels are those for a

standard RS-232C interface.  In particular, the standard

specifies voltage levels of +12 volts and -12 volts to

represent the logic states of the signal.  Further

information can be obtained in the Technical Reference

Manual.  The signals which the program uses are as

follows.

Pin  2 - Transmit Data (Tx data)   - characters to TU

Pin  3 - Receive Data  (Rx data)   - characters from TU

Pin  8 - Carrier Detect (CD)       - CW data in from TU

Pin  4 - Request to Send (RTS)     - Transmitter

control

Pin 20 - Data Terminal Ready (DTR) - CW ID control

Pin  7 - Ground (Gnd)              - reference level

For Tx data and Rx data, mark is -12v and space is +12v.

CD is tied to Rx data and is not required for RTTY,

however, I included it for future use if I ever get around

to writing a Morse receive routine.

RTS and DTR are -12v in receive mode and +12v in normal

transmit mode.

Hopefully this is enough information to help you interface

to your terminal unit.

Interface Information                                     37

IBM PC RTTY Ver 1.2F

KANTRONICS 'THE INTERFACE' INFORMATION

While I will give information here on how I interfaced the

Kantronics TU to my system, I must add that it was a

decision based on availability and price at the time of

purchase more than anything else.  If you are trying to

make a decision, check around and see what is available and

what seems to work well.  While the Kantronics does well in

many cases, it does seem to have difficulty with weak

and/or fading signals.  The Kantronics interface does not

provide an RS-232C level interface and also requires an

external power supply.  Additionally, the RTTY transmit

line requires a normal space condition for receive to

operate correctly.  Based on these considerations, I

designed a special RS-232C to Kantronics interface box.

The schematic is included below.

Since my original purchase of a TU, Kantronics has come out

with some new models which seem to have addressed some of

the above mentioned problems.  In particular, they have

increased the sensitivity of the circuits and have provided

an RS-232C level interface.  So remember, it pays to

inquire before you purchase.

Next, just a few words about the circuit.  The interface

chips used are the same as those used in the IBM PC,

namely, the 75150 line driver and the 75154 line receiver.

The 1488 line driver and 1489 line receiver from Radio

Shack could also be used but the pinouts are different.

The 74LS00 is used to gate the Tx data signal with the

transmitter control, to invert the RTTY transmit signal and

to invert signals for the normal/reverse switch.  The

circuitry needs +5v, +12v and -12v to run.  I used the

7805, 7812 and the 7912 voltage regulators to get my

voltages, but you can use your favorite regulator.  Just be

sure to have enough filter capacitance on the input and

enough heat sink on the regulators.  There is nothing worse

than intermittent problems when things get hot and heavy.

Also note, the signal ground from the asynchronous adapter,

the special interface ground and the TU ground should all

be tied together.

Due to popular demand, I have also included a schematic

using the 1488 and 1489 which should be readily available

at Radio Shack.

Interface Information                                     38

IBM PC RTTY Ver 1.2F

KANTRONICS RS-232C INTERFACE SCHEMATIC

+5v

�

����������Ŀ  �

Ĵ1         �� � ���������������������������������������� CWID

Ĵ          ���� �

Ĵ          ��   �              ������������������������� KEYIN

DTR ��������Ĵ          ������              �  ����������Ŀ

Tx data ����Ĵ          ����Ŀ              ��Ĵ1         �� +5v

RTS ��������Ĵ          ����������������������Ĵ          ����������� DECODE

Ĵ   75154  ��   �              ��Ĵ          �����   �

Gnd Ĵ          ��Ŀ �              ��Ĵ          �����Ŀ �

������������  � �         �������Ĵ          ���Ŀ � �

������������������ �         �������Ĵ  74LS00  ���Ĵ � �

�                  �         �� Gnd Ĵ          �Ŀ � � �

�                  �         ��      ������������ � � � �

�                  �         ��                   � � � �

�                  �         ������������������������������ RTTY

�                  �����������������������Ŀ      � � � �

�����������������������������Ŀ  �            /o��������� � �

�   �                         �  ������������o|       �   � �

�   �                         �               |o�������   � �

�   �  ����������Ŀ           �               |           � �

�   ��Ĵ1         �� +12v     �               /o�������������

������Ĵ          ���������Ŀ ���������������o            �

������Ĵ   75150  �������Ŀ �                  o�����������

Gnd Ĵ          �� -12v � �

������������       � �           Normal/Reverse

� �           DPDT switch shown

Rx data ������������������������� �           in normal position

CD ��������������������������������

Gnd ����������������������������������������������������������������� Gnd

Computer                                                        Kantronics

Interface                                                       Interface

�

Note: Non-Intersections are indicated by  ���

�

All grounds are connected together.

An IBM Graphics Printer is required for this schematic

to print correctly.

Kantronics RS-232C Interface

Interface Information                                     39

IBM PC RTTY Ver 1.2F

DTR ���������������������������Ŀ

�

����������Ŀ +5v   �

Tx data ����Ĵ1         ����    �      ������������������������������ CWID

Ĵ          ���������      �

������Ĵ          ��             �    ������������������������� KEYIN

RTS ��������Ĵ          ����������������    �  ����������Ŀ

������     Ĵ          ��                  ��Ĵ1         �� +5v

�    ������Ĵ   1489   ��     ���������������Ĵ          ����������� DECODE

�    � Gnd Ĵ          ��     �            ��Ĵ          �����   �

�    �      ������������      �            ��Ĵ          �����Ŀ �

�    �                        �       �������Ĵ          ���Ŀ � �

�    �                        �       �������Ĵ  74LS00  ���Ĵ � �

�    ��������������������������       �� Gnd Ĵ          �Ŀ � � �

�                                     ��      ������������ � � � �

�                                     ��                   � � � �

�                                     ������������������������������ RTTY

���������������������������������������������������Ŀ      � � � �

��������������������������������Ŀ  �            /o��������� � �

�                                �  ������������o|       �   � �

�                                �               |o�������   � �

�         ����������Ŀ           �               |           � �

�   -12v Ĵ1         �� +12v     �               /o�������������

���������Ĵ          ���������������������������o            �

Rx data ����Ĵ          ��                           o�����������

Ĵ          �����Ŀ

Ĵ          ��    �               Normal/Reverse

Ĵ   1488   ��    �               DPDT switch shown

Gnd Ĵ          ��    �               in normal position

������������     �

CD ����������������������������

Gnd ����������������������������������������������������������������� Gnd

Computer                                                        Kantronics

Interface                                                       Interface

�

Note: Non-Intersections are indicated by  ���

�

All grounds are connected.

An IBM Graphics Printer is required for this schematic

to print correctly.

Kantronics RS-232C Interface

Interface Information                                     40

IBM PC RTTY Ver 1.2F

A TERMINAL UNIT DESIGN COMPLIMENTS OF BOB AUSTIN, N4CLH

#W1#GAFSK MODULATOR WITH XR-2206

#W0#H

** ALL PARTS ARE AVAILABLE AT RADIO SHACK **

����Ŀ

������������������������������������������Ĵ1 16�

R1= 220    �  AUDIO OUT���������������ĴC5�����������Ĵ    �

R2= 33K    �           ���������ĴR8�����������������Ĵ    ��ĴR1������Ŀ

R3= 20K    �           �              ���������������Ĵ    ��������������

R4= 20K    �           �              �   ����ĴC3���Ĵ    ���������������Ŀ

R5= 33K    �           �    +12V�����Ĵ   �����������Ĵ    �               �

R6= 4.7K   �           �              �       �������Ĵ    ������������Ŀ  �

R7= 4.7K   �           �              �       �    ��Ĵ8  9����Ŀ       �  �

R8= 50K    �           �              �       �    �  ������    ���TTL  �  �

�    �����������ĴR7��������       R5   R2           �   IN  �  �

C1= 1uf    �    �      �                      �    �            �     + �  �

C2= .001uf �  + �      �                      �    �            �       C1 �

C3= .01uf  �    C4     R6                     R4   �            C2      �  �

C4= 10uf   �    �      �                      �    R3           �       �  �

C5= .1uf   �    �      �                      �    �            �       �  �

���  ���    ���                    ���  ���          ���     ������

-----------------------------------------------------------------------------

NOTES:

C3 MUST BE STABLE CAPACITOR

R3/R4 SHOULD BE MULTI-TURN POTS

R8 VARIABLE POT TO ADJUST AUDIO OUTPUT

C1/C4 ARE ELECTROLITIC

SETUP:

POWER TO PIN 4, ADJUST R3 FOR 2125 HZ AT AUDIO OUTPUT

POWER TO PIN 4, GROUND PIN 9, ADJUST R4 FOR 2295 HZ AT AUDIO OUTPUT

ADJUST R8 FOR DESIRED AUDIO OUTPUT LEVEL TO MICROPHONE CONNECTION

NORMALLY LESS THAN 1 VOLT PEAK.

NOTE**

THE XR2211 WILL BLOW IF MORE THAT 2V PEAK  AUDIO IS APPLIED. IF YOU HOOK UP

THE RELAY CIRCUIT TO ECHO XMIT BUFFER DO NOT ADJUST R8 FOR MORE THAN THE

2V PEAK.

Interface Information                                     41

IBM PC RTTY Ver 1.2F

From Bob Austin, N4CLH

#W1#GAFSK DEMODULATOR USING XR-2211

#W0#H

�����Ŀ

IN      +12V��������Ĵ1  14�����ĴC6����Ŀ

AUDIO  �������ĴC11���Ĵ     ���������������

����������������������������������Ĵ     �������������Ŀ

�   �   ��������������������������Ĵ     �����ĴR9�������ĴR10����Ŀ

�   �   �                          �     ���ĳ��������������Ŀ     �

�   �   �       TTL OUT�����������Ĵ     �   ���������Ŀ     �     �

�   �   �                       ��Ĵ7   8����ĴR12����Ĵ     �     �

�   �   �       +5V����ĴR15���Ĵ  ������� �           �     �     �

�   �   �                       �          �           �     �     �

�   �   �                       ���ĴR13��Ĵ           �     �     �

�   �   �                                  �           C7    C8    R11

R14  C10 �                                  �           �     �     �

�   �   �                                  C9          �     �     �

�   �   �                                  �           �     �     �

��� ��� ���                                ���         ���   ���   ���

-----------------------------------------------------------------------------

R9 = 220K

R10= 15K         NOTES: PINS 10 AND 11 DO NOT CONNECT

R11= 20K               R11 IS MULTITURN POT

R12= 100K              C6 MUST BE STABLE CAPACITOR

R13= 470K              C9 IS FOR 110 ASCII/BAUDOT OR LESS

R14= 470K                 I SWITCH TWO CAPS AT C9 FOR LOW SPEED AND HIGH

R15= 10K                  SPEED USING .047 AND .0047(HIGH) 300 ASCII/BAUDOT

C6 = .022        ADJUSTMENT:

C7 = .0047                 POWER TO PIN 1, ADJUST R11 FOR CLEAN COPY

C8 = .1                   (I USE A STRING OF *U FOR ADJUSTMENT AT 110 ASCII)

C9 = .047

C10= .05

C11= .1

Interface Information                                     42

IBM PC RTTY Ver 1.2F

From Bob Austin, N4CLH

#W1#GTTL TO EIA INTERFACE

#W0#H

I LIKE TO USE A 7404 OR 74LS04 BETWEEN THE EIA LINE DRIVER/RECEIVER

AND THE EXAR CHIPS TO LET ME USE SOME L E D'S FOR MARK AND SPACE.

YOU MAY ELIMINATE THIS CIRCUIT AND CONNECT THE EXAR CHIPS TO THE

LINE DRIVER/RECEIVER DIRECT.

TTL IN  FROM XR2211-PIN6

���

�        7404 OR 74LS04

�         �����Ŀ

N ��������������Ĵ1  14���������Ĵ+5V

�������� S1        ��Ĵ     �

�    R ��������������Ĵ     �

�        �         ��Ĵ     �

�        �      �����Ĵ     �

�       R16     �     �     �

�        �      �   �Ĵ7  8 �

�        �     R17  � �������

�        �      �   �                    S1= SPDT FOR NORMAL AND REVERSE

�        D1     �   �                        SIGNALS

�        �      �   �                    R16= 390

�        �      D2  �                    R17= 390

�        �      �   �                    D1=  LED  (MARK)

�       �ſ    �ſ �ſ                   D2=  LED  (SPACE)

------�---------------------------------------------------------------------

�     1488 DRIVER

��������������������������Ŀ               1489 RECEIVER

�

+12V    -12V      EIA PIN6  �          PIN4    PIN9  PIN2        +5V

�       �            �     �            �EIA    �    �EIA        �

�       �����Ŀ      �     �            �       �XR  �           �

�     �����Ŀ �      �     �            �       �2206�           �

�����Ĵ1  14���      �     �            �       �    �  �����Ŀ  �

�Ĵ     ���Ŀ    �     �            �       �    ��Ĵ1  14����

�Ĵ     ��������ĳ�Ŀ  �        ���ĳ��������������Ĵ     �

�Ĵ     ����������  �  �        �   �       �������Ĵ     �

�Ĵ     ���Ŀ       �  �        �   ���������������Ĵ     �

�Ĵ     �����������ĳ���        �      ������������Ĵ     �

#        �Ĵ7  8 ����Ŀ      �           �      �      �����Ĵ     �

� �������    �      �          C11     �      �   �Ĵ7  8 �

�            �      �           �      �      �   � �������

�  EIA   �����      �           �     C12    RELAY�

�  PIN3             �           �      �     DRIVE�

�ſ                 �ſ         �ſ    �ſ        �ſ

C11= .1

C12= .1

Interface Information                                     43

IBM PC RTTY Ver 1.2F

From Bob Austin, N4CLH

#W1#GPTT CIRCUIT

#W0#H

FOR THE RELAY TRANSISTOR, I USED A PNP 2N3906.

E

PIN6              �������� +5V

1489 ������ĴR18��ĴB   �

RCVR               �������� RELAY COIL (OTHER END GND)

C

R18= 470

RELAY= DPDT 5VOLT MINIATURE.

** THERE ARE SEVERAL WAYS TO HOOK THE RELAY UP TO YOUR PTT.

I COUPLED MY AUDIO OUTPUT TO THE AUDIO INPUT TO ECHO

MY XMITTED BUFFER BACK TO ME.(THIS ALLOWS ME TO SEE BOTH XMIT AND

RECEIVE IN MY SAVED FILES)

IF YOU USE THE ECHO FUNCTION OF A RTTY PROGRAM YOU DO NOT HAVE TO

HOOK THIS UP.. A SPST RELAY IS SUFFICIENT FOR PTT

AUDIO HOOKUPS ON THE RELAY ARE MADE WITH COUPLING CAPS C5 AND C11 BETWEEN

XR CHIPS AND RELAY.

XR2206               COM �����ĿCOM

AUDIO INPUT     ��������Ĵ     ��������Ĵ PTT INPUT

XR2211             N/OPEN�     � N/OPEN

AUDIO OUTPUT    ��������Ĵ     �����������Ŀ

N/CLOSE�     � N/CLOSE   �

RIG AUDIO OUT   ��������Ĵ     ��������    �

(SPKR)                   �������           �

�

�

�

�ſ

#W1#G73's N4CLH#W0#H

Interface Information                                     44

#