SB-009

TS-820 Speech Processor Alignment                            11-17-76

The following alignment procedure may be used in lieu of the one in the

Service Manual.

Test Equipment required:

Audio Signal Generator

Audio VTVM

Oscilloscope or RF VTVM

1.	Preliminary control settings:

Screen grid switch off.

Mode switch to USB.

Comp Level to USB.

Processor SW to Pull "ON".

Meter switch ot "Comp".

Send/Rec switch to "Send" as required when making adjustments.

2.	Apply 1500 Hz audio signal to the mic jack at 10mv level.

Connect RF VTVM or scope to TP-2.

Adjust T-11, 12, 13, for maximum level on the S-meter.

Adjust T-14 for maximum level on scope or VTVM at TP-2.

3.	Turn compression level to min, (CCW).

Reduce audio sig at mic jack to 0.3 mv.

Adjust TC-4 (Freq response for speech processor) so that level at

TP-2 is not changed when changing audio gen from 400 Hz to

2000Hz.

4.	Turn compression level to max, (CW)

Set audio gen to 1500Hz at 0.3 mv.

Adjust TC-3 and VR-6 (carrier bal for speech proc) for maximum level

at TP-2.

5.	Set audio gen to 10 mv at 1500Hz

Send/Rec switch to send

Adjust VR-7 (comp level for speech processor) so that the level at

TP-2 is not changed when turning the processor on and off.  VR-7

should be adjusted for the particular microphone the customer

will be using with his TS-820.

6.	Finally check for non distortion of signal, and that comp level of

20db can be obtained at full comp on.

(c) 111776TKC

Downloaded From P-80 International Information Systems 304-744-2253