LAN-LINK VERSION 1.56 PAGE 1

LAN-LINK Copyright (c) 1988, 1989, 1990 by G3ZCZ

All Rights Reserved

Joe Kasser G3ZCZ

POB 3419

Silver Spring

Md., 20918, U.S.A.

(301) 593 6136

Compuserve ID. 70531,1405

U.S.A. Packet BBS Routing G3ZCZ @ N4QQ

World Packet BBS Routing G3ZCZ @ N4QQ.MD.USA.NA

_______

____|__     |                  (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

LAN-LINK  is  a Personal Packet Terminal Program  for  the  TNC1,

TNC2, KPC-2 and most of all a smart multimode digital  communica#

tions controller for the KAM, MFJ1278 and the PK-232.

LAN-LINK is written and supported by Joe Kasser, G3ZCZ.

The program is distributed as a Shareware product. You may freely

copy  and  share  the product for noncommercial  use,  with  your

friends,  associates and other radio hams. If you decide  to  use

the  product, you are asked to become a registered user  by  com#

pleting  the  registration  form and sending it,  and  $35.00  or

equivalent in foreign currency to the author.

Upon  receipt  of your registration, you will  receive  one  free

update disk, telephone and mail (electronic and regular) support.

This product may not be sold or distributed with another  product

without the express written permission of Joe Kasser, G3ZCZ.

Joe  Kasser,  G3ZCZ will only support unmodified copies  of  this

software.  Your comments and suggestions for changes are  however

welcome.  If you are the first to suggest a change that is imple#

mented,  you will be sent a complimentary copy of the  disk  with

the change incorporated.

Potential Commercial Users please contact Joe Kasser directly for

modifications and/or details of Site licensing.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 2

TABLE OF CONTENTS

1.0  INTRODUCTION

1.1  PROGRAM HIGHLIGHTS

1.1.1  Packet Radio

1.1.2  AMTOR

1.1.3  Morse/BAUDOT/ASCII

2.0  Getting Started

2.1 Bringing LAN-LINK Up the First Time

2.2 Connecting to Other Stations

2.3 Upgrading from YAPP

2.4 Using Different TNCs

2.5 Using Different HF and VHF Configuration Files

2.6 Using the AMTOR/PACKET Robot/Automatic Contest Station

3.0  Things You Should Know

3.1 Memory Requirements

3.2 PC to TNC Communications

3.3 Open Loop

3.4 Problems

3.5 The Contents of the Status Window

4.0  Function Keys

4.1  Capture to Disk Toggle

4.2  Send Brag Tape

4.3  Transmit Callsign Sequence

4.4  Send/talk Brag Tape

4.5  Show Packet MHlist and Point and Shoot Connect

4.6  Auto CQ Call

4.7  Activate Type Ahead Buffer/AMTOR Relink

4.8  Disconnect/Receive

4.9  Put TNC in CMD Mode

4.10 Put TNC in CONVERSE /Transmit Mode /AMTOR FEC

4.11 Alternate Function Keys

4.12 Shift Function Keys

4.13 Control Function Keys

5.0  Main Menu

6.0  BBS Menu

6.1 Download Text Files

7.0 Call Menu

7.1  Alert Call/ARQ Call

7.2  Connect/Call

7.3  Enter Call

7.4  reMember Path

7.5  Packet Cluster

7.6  Call CQ

7.7  Retry

7.8  Show Directory

7.9  Target Call

7.10 aUto CQ

7.11 Toggle DX Flag

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 3

7.12 Zap BBS

8.0 Edit Menu

8.1   Edit Brag Tape

8.2   Edit Call Directory File

8.3   Edit any other file

8.4   Edit Logbook File

8.5   Answer Incoming Messages (Split Screen)

8.6   Leave Note for Someone

8.7   Edit Capture-to-Disk File

8.8   Edit Two files

8.9   Edit LAN-LINK.SYS File

8.10  Edit LAN-LINK.001 through LAN-LINK.010 files

9.0 Event/Clock Menu

9.1  Set Alarm

9.2  Connect

9.3  Display Event

9.4  Periodic Event

9.5  Turn Robot ON/OFF

9.6  Turn Contest Mode ON/OFF

9.7  Time Display ON/OFF

9.8  Cancel Event

9.9  Zap BBS

10.0 Files Menu

10.1  Send Brag Tape

10.2  File Directory

10.3  Erase File

10.4  Rename File

10.5  Send File

10.6  Send/Talk File

10.7  Send Uppercase File

10.8  View File

11.0 Help Menu

12.0 Jump to Dos

13.0 LAN-LINK Menu

13.1 :QBU: External Protocols

13.2 :QBM: File

13.3 :QSP: Message

13.4 :QDB: File

13.5 :QRU: Messages

13.6 :QRA: Who is "ON-LINE"

14.0 Log Menu

14.1 Change Logbook File

14.2 Show last Few Entries

14.3 Log Entry

14.4 Print Log

14.5 Scan Log

14.6 Export Log

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 4

14.7 Clear Up Log

15.0 Message Menu

15.1 BBS Command Files

15.2 Current Messages

15.3 Scan Message

15.4 Leave Note

15.5 Old Messages

15.6 Time Tag

16.0 TNC Menu

16.1 Set TNC Date/Time

16.2 Flush TNC Buffer

16.3 Initialize TNC

16.4 Show Calls Monitored List

16.5 Show Connect Status

16.6 Change CTEXT

17.0 AMSAT-OSCAR Menu

17.1 UoSAT ASCII Beacon

17.2 Phase 3 RTTY Beacon

17.3 SAREX Menu

17.3 1 Attack Mode ON/OFF

17.3.2 Change SAREX Call

17.3.3 SAREX Packets Capture to Disk ON/OFF

17.4 SAREX Orbiter Features

18.0 Parameter Menu

18.1  Turn Beacon Mode ON/OFF

18.2  Clear Connect Count

18.3  Turn Digipeat Detect ON/OFF

18.4  Turn Beacon Control ON/OFF

18.5  Turn Command Echo ON/OFF

18.6  Change Keyboard Beep Count

18.7  Set Message Count

18.8  Turn NODE Drop Link ON/OFF

18.9  Turn Printer ON/OFF

18.10 Turn QTC Snatch ON/OFF

18.11 Turn :QSP: Relay Flag ON/OFF

18.12 Turn Sound ON/OFF

18.13 Turn Contest Mode ON/OFF

18.14 Turn DCD Flag ON/OFF

18.15 Turn BBS Snatch ON/OFF

18.16 Turn Time Id. ON/OFF

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF

18.18 Change BBS Parameters

18.18.1 Local BBS Callsign

18.18.2 Mail-Snatch Trigger Word

18.18.3 Read Request Command

18.19 Change Callsigns

18.19.1 Callsign

18.19.2 The SELCAL

18.19.3 The PacketCluster Call

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 5

18.20 Change Directories

18.20.1 QBM Download Directory

18.21 Enter Common Calls

18.22 Change File Names

18.22.1 Change Capture to Disk File (LAN-LINK.RUN)

18.22.2 VHF Logbook File (VHF.LOG)

18.22.3 HF Logbook File (HF.LOG)

18.22.4 Path Directory File (LAN-LINK.DIR)

18.22.5 Brag Tape (LAN-LINK.TXT)

18.22.6 QBU Upload File Name (QBU-TX.BAT)

18.22.7 QBU Download File Name (QBU-RX.BAT)

18.23 Change LAN-LINK Parameters

18.23.1 Change CQ Text

18.23.2 The Default Band

18.23.3 The Default Transmitter Power

18.23.4 Set CQ Delay Time Seconds

18.23.5 Set CQ Delay Time Minutes

18.23.6 Set Number of Lines in a Non Contest CQ Call

18.23.7 Set Shut Down Mode (QRT Flag)

18.23.7.1 QRT

18.23.7.2 QRV

18.23.7.3 QRV + CTEXT TOGGLE

18.23.7.4 BBS

18.23.7.5 SOLO

18.23.7.6 BBS+SOLO

18.24 Change PC Communications Parameters

18.24.1 The Communications Port

18.24.2 The Terminal Baud Rate

18.24.3 TNC Data Bits

18.24.4 TNC Parity

18.24.5 Number of TNC Stop Bits

18.25 Change Colors

18.25.1   InWindow Color

18.25.2   OutWindow Color

18.25.3   StatusWindow Color

18.25.4   PromptWindow Color

18.25.5   LogWindow Color

18.25.6   LogScanwindow Color

18.25.7   ScrollBackWindow color

18.25.8   MHWindow Color

18.25.9   Alarmwindow Color

18.25.10  BottomWindow Color

18.25.11  Emphasis Color

18.25.12  Prompt Color

18.25.13  Option Color

18.25.14  Connected Text Color

18.25.15  Computer Color

18.25.16  VHF Header Color

18.25.17  HF Header Color

18.25.18  Alert Call Color

18.25.19  Outgoing Text Color

18.25.20  Multi I/O Stream Color[1]

18.25.21  Multi I/O Stream Color[2]

18.25.22  Multi I/O Stream Color[3]

18.25.23  Multi I/O Stream Color[4]

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 6

18.25.24  Multi I/O Stream Color[5]

18.25.25  Multi I/O Stream Color[6]

18.25.26  Multi I/O Stream Color[7]

18.25.27  Multi I/O Stream Color[8]

18.25.28  Multi I/O Stream Color[9]

18.25.29  Multi I/O Stream Color[10]

18.26 Change SAREX Parameters

18.26.1 SAREX CallSign

18.26.2 Change SAREX Capture to Disk File

18.27 Update LAN-LINK.SYS

18.28 Verify Changes

18.29 Change Scan Words

18.30 Turn Misc. Flag ON/OFF

18.30.1 All Modes:- Debug or state-sequence numbers

18.30.2 BAUDOT Mode:- Navy MARS Protocol Verify

19.0 Q Codes [NC/L]

20.0 CommunicationS Modes

20.1 VHF PACKET

20.2 HF PACKET

20.3 ASCII (RTTY)

20.4 BAUDOT (RTTY)

20.5 MORSE (CW)

20.6 AMTOR (STANDBY RECEIVE)

20.7 AMTOR (ALIST [CHIRPCOPY])

20.8 SIGNAL

20.9 NAVTEX

21.0 Terminal Mode Menu

21.1 Packet Terminal Menu

21.1.1 Change Packet Baud Rate

21.1.2 CQ/Beacons

21.1.3 Everything

21.1.4 Read the Mail

21.1.5 Just My Station (Solo)

21.1.6 All Traffic

21.1.7 Single User Connects

21.1.8 Multiple User Connects

21.1.9 Conference

21.1.10 Conference Bridge On/Off

21.2 AMTOR Terminal Menu

21.2.1 Force Break [ACHG]

21.2.2 Start Mode B [FEC]

21.2.3 Force LETTERS case

21.2.4 Read the Mail [ALISTEN]

21.2.5 Override ROBOT/Mailbox

21.2.6 Standby Receive [ARQ]

21.2.7 Set ROBOT/Mailbox

21.2.8 CW Identification

21.2.9 Set ROBOT infinite Delay

21.3 Baudot Terminal Menu

21.3.1 Set Baud Rate

21.3.2 Select Shift

21.3.3 NAVY MARS Message Protocol

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 7

21.3.4 USOS On/Off

21.3.5 SELCAL set/reset

21.3.6 CW Identification

21.4 ASCII Terminal Menu

21.4.1 Set Baud Rate

21.4.2 Select Shift

21.4.3 SELCAL On/Off

21.4.4 CW Identification

21.5 CW Terminal Menu

21.5.1 Lock to Receive Signal

21.5.2 Set Morse Speed

21.5.3 Unlock Morse Speed

22.0  eXit to DOS

23.0 Hot Key Combinations

23.1  Alt-A - Alert Call (Packet)/ARQ Call/Talk (AMTOR)

23.2  Alt-B - Send Break/AMTOR Changeover

23.3  Alt-C - Call/Connect to

23.4  Alt-D - Disconnect or Receive

23.5  Alt-E - Enter Call

23.6  Alt-F - Flush TNC Buffer

23.7  Alt-J - Jump to DOS Shell

23.8  Alt-K - Jump to DOS Shell with Quick Return

23.9  Alt-L - Edit Log

23.10 Alt-M - Message Scan

23.11 Alt-N - Multi Connect Stream Next

23.12 Alt-O - Turn Robot On

23.13 Alt-P - Turn Printer ON/OFF

23.14 Alt-Q - Call QRZ

23.15 Alt-R - Retry Call/Connect

23.16 Alt-S - Scan Log

23.17 Alt-X - Exit to DOS

23.18 Alt-Y - Zap Packet Cluster

23.19 Alt-Z - Zap BBS

23.20 Alt-= - Toggle Miscellaneous Flag

23.21 End   - > +?  [Over]

23.22 PgUp  - Scrollback (Scrolls back incoming text)

23.23 The HOME Key

23.24 The DOWN ARROW Key

23.25 Shift/Baudrate Control keys

23.26 Control-Z (Ctrl-Z  or ^Z)

24.0  LAN-LINK disk files

24.1  LAN-LINK.EXE

24.2  LAN-LINK.SYS

24.3  LAN-LINK.RUN

24.4  LAN-LINK.LOG

24.5  LAN-LINK.DIR

24.6  CONFIG.SYS

24.7  *.MBX

24.8  *.BBS

24.9  LAN-LINK.001 .. LAN-LINK.010

24.10 QBU-RX.BAT

24.11 QBU-TX.BAT

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 8

25.0 Mailbox features

25.1 Answering Machine

25.2 Mail Beacon (Annunciator)

25.3 How to leave a message

25.4 Remote Beacon shutdown

APPENDIX 1. Anomalies and Bugs

A1.1  Command/Converse Mode anomalies

A1.2  Buffer messages in AMTOR Beacon CQ Mode

A1.3  FEC +? responses in AMTOR Beacon CQ Mode

A1.4  Using a CGA card with a monochrome monitor

A1.5  The Packet Conference Mode

A1.6  Packet Conference Mode disconnects

A1.7  Strange Events

A1.8  *** DISCONNECTED

A1.9  *** CONNECTED

A1.10 Greek Characters on Incoming Screen

A1.11 Dumb TNC

APPENDIX 2. Local Area Network Capability

A2.1 LAN Protocol (G3ZCZ Version)

A2.2 Message Format

A2.3 File Uploading/Downloading

A2.4 Path Determination to a DX Station

APPENDIX 3. NC/L Dictionary

A3.1  :QBM: File Download

A3.2  :QDB: File Upload

A3.3  :QMH: Monitor Heard List

A3.4  :QSM: Send Repeat of Message

A3.5  :QSP: Send Message

A3.6  :QRA: Who is About?

A3.7  :QRT: Shut Up

A3.8  :QRU: Upload Messages in Batch

A3.9  :QNO: Negative Response

A3.10 :QJG: Task Completed Response

A3.11 :QRV: Ready Response

A3.12 :QSL: Confirmation Response

A3.13 :QTC: List of Messages

A3.14 :QYU: YAPP Binary File Transfer Upload

A3.15 :QYD: YAPP Binary File Transfer Download

APPENDIX 4. USING OTHER TNCs

A4.1 TNC2 (MFJ 1270) Version 1.1.2

A4.1.1 Default Commands

A4.1.2 Program differences

A4.2 TNC1 (HD-4040)

A4.2.1 Default Commands

A4.2.2 Program differences

A4.3 Kaypro KPC-2

A4.3.1 Default Commands

A4.3.2 Program differences

A4.4 KAM

A4.4.1 Default Commands

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 9

A4.4.2 Program differences

A4.5 PK-232 EPROM Versions JUN 1987 and later

A4.6 MFJ1278 EPROM Version 3.3

A4.6.1 Default Commands

A4.6.2 Program differences

APPENDIX 5. UPDATES/REVISION HISTORY

APPENDIX 6. Registration Form

APPENDIX 7. Problem Reporting Form

APPENDIX 8. Other PC Software by Joe Kasser G3ZCZ

APPENDIX 9. How Shareware Works

ACKNOWLEDGMENTS

Thanks to Milt, 4X1AA.  If it wasn't for Milt, Version 1.00 would

never have been written.

Thanks  to  Jim, WB8QJL, who loaned me his  MFJ1278  for  several

months but died in a plane accident before he got to use LAN-LINK

on his MFJ1278.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 10

1.0  INTRODUCTION

LAN-LINK  is a Personal Digital Communications  Terminal  Program

for a Packet Radio TNC.  The program is designed to optimize  the

configuration  of  the  TNC in each communications  mode  and  to

provide some smart terminal features.  It takes advantage of  the

extra   features  of  the  PK-232  which  is  made  by   Advanced

Electronics  Applications Inc. It is designed to allow anyone  to

use and get the most out of their packet TNC as well as from  the

PK-232,  the MFJ1278 and the KAM for Morse, ASCII, BAUDOT,  AMTOR

as well as Packet Radio communications without having to keep the

manual handy.

LAN-LINK  is  a  sophisticated program.  In its  basic  state  it

allows  you to use the TNC in an optimal manner.   It  configures

the  TNC  (it  types  the  commands)  for  you  to  maximize  the

communications  efficiency  in the communications  mode  of  your

choice.   That means, for example, when working Packet on HF  you

need  to program the TNC parameters to different values than  you

would  use  on  VHF  to  make  maximum  use  of  the  mode.   One

significant  difference is the length of the packet  itself,  for

the  longer it is, the greater the probability of QRM  destroying

it. This program will adjust the packet parameters for you.

Since  the  computer is now involved, other  features  have  been

added  to  simplify  operation, and several  features  have  been

automated.   All these operations are performed using  menus  and

function keys as documented below.  It will take a while to learn

how  to  use this program in a manner that suits you,  read  this

document and have fun, after all, isn't that one of the  purposes

of Amateur Radio.

This manual is organized by menu.  It describes the operation  of

each  menu option and function key in each  Communications  Mode.

It is not organized in a how to do something manner.

Any  Terminal  Node  Controller  (TNC)  is  a  complex  piece  of

equipment.   It can operate in a Command Mode, in which you  tell

it to do something, or in a Converse Mode in which you are  using

it  to talk to other stations.  Many people confuse the two  when

first  getting on Packet.  If you monitor the channels  you  will

recognize Command Mode TNC instructions on the air, and when  you

use  the  TNC you will receive the "error" reply  when  you  type

something  thinking  that you are in the Converse  Mode  but  are

really in the Command Mode.  LAN-LINK is designed to make  Packet

operation simple using Menus and high level (Function and Hot key

combinations) commands.

Morse,  ASCII, BAUDOT communications modes are HALF DUPLEX.   One

station  is transmitting at any time.  You thus have to  use  the

Transmit/Converse  and Receive/Disconnect function keys  to  turn

the transmitter on and off respectively.

AMTOR is also HALF DUPLEX but has some of the attributes of  full

duplex.   In  this  Mode  you  control  who  is  talking  through

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 11

software,  only one station can send data at a time, but you  can

type  characters  in to the buffer and they will  be  transmitted

automatically  when the other station turns the QSO over to  you.

Once   you  have  linked  with  someone,  you  do  not  use   the

Receive/Disconnect function UNTIL THE QSO IS OVER.  Both stations

are  communicating automatically thanks to the TNC.  If you  want

the  other operator to respond type the +? key sequence  (or  use

the 'End' Key).

Packet  Radio communications  on the other hand is a full  duplex

mode  of communications.  Once you have connected  with  someone,

you  do not use the Receive/Disconnect function UNTIL THE QSO  IS

OVER.   Both stations are communicating automatically  thanks  to

the  TNC.  If you want the other operator to respond type  the  >

key followed by the 'Enter' Key, better yet, use the 'End' Key.

Note  that  Morse,  AMTOR, ASCII and BAUDOT  are  character  mode

communications  in  that each character you type is sent  as  you

type them.  In Packet Communications, the TNC stores them up  and

sends out a burst or packet when you type the 'Enter' Key or  you

type  enough characters to fill up a packet.  Studies have  shown

that the human attention span when sitting at the keyboard is  of

the  order  of 2 seconds. If you are in a  keyboard  to  keyboard

packet  radio connect, remember this and don't forget to use  the

'Enter' Key frequently, especially before you stop to think about

what to send next.

Note   that   AMTOR  and  PACKET  are  full   duplex   modes   of

communications.  When you connect with someone, there is no  need

to use the transmit and receive function keys.  In fact you  will

have  problems  if you do.  Do not disconnect or break  the  link

until the QSO is over.

1.1  PROGRAM HIGHLIGHTS

Function key and Menu driven.

Automatic logbook entries for Packet and Mailbox/Beacon  Mode

AMTOR  Connects,  semiautomatic  logbook  entries  for  other

modes.

Log file can be processed by the Logbook Package of PC-HAM by

G3ZCZ  for  indexed  listings, tracking  of  DXCC  and  other

AWARDS, etc.

Contest  operation, sends standard message and  automatically

increments QSO count.

Automatic  optimized configuration of the TNC for  each  com#

munications mode.

All mode Function key  'OVER' feature (End).

There  are  10 files with fixed names  (LAN-LINK.001  through

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 12

LAN-LINK.010) which may be viewed and transmitted by means of

function keys.  They may also be edited from the Edit Menu.

Set up of TNC for AMSAT-OSCAR Telemetry reception.

Exports log file to data base programs.

Time display and event scheduler.

ASCII Text Editor.

Customizable Colors.

Access  to the TNC Command Mode is provided in case the  user

wishes to override any defaults.

1.1.1     PACKET RADIO

Automatic  capture  to  disk of all  packet  radio  connects.

Automatic indication of the number of Packet connects.

Local   Area   Network  (LAN)  message  store   and   forward

capability.

Capable of automatic connect attempts to download a QTC  from

another station in the LAN.

Capable  of  automatic connect attempts to a  packet  BBS  to

download  your incoming messages, when your callsign  appears

on the BBS mail beacon annunciator.

Capable  of  automatically requesting Bulletins  on  subjects

that interest you from your local packet BBS.

Digipeat monitoring and capture.

Alert signal to let you know when a predetermined call  shows

up in a packet header on frequency.

Conference Mode in multiconnect situations.

Bridge Mode in multiconnect situations.

Path determination to DX station via :QMH:.

Indicator that a specific station designated as the  'target'

call   connected   in  Packet  Mode,  or  linked   to   AMTOR

Beacon/Mailbox while you were away.

Automatic  NET/ROM and KA Node path set up from  LAN-LINK.DIR

call/path directory file.

Selective  answering machine and MAILBOX using  NC/L  command

dialogue.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 13

Screen  indication  of  connect by  desired  station  (target

call).

Automatic Beacon Mode CQ caller

Automatic contest (DX-pedition) mode

Will  call  CQ repetitively and either work the  connect  and

keep  going  after disconnect or signal you when a  reply  is

received.

SAREX special features

:QRA: trigger to determine who else is on channel

Single keystroke Packet Cluster connect

1.1.2     AMTOR

Automatic SELCAL determination.

Automatic  CQ caller.  Will call CQ repetitively  and  signal

you when a reply is received.

Local   Area   Network  (LAN)  message  store   and   forward

capability.

Selective  answering machine and MAILBOX using  NC/L  command

dialogue.

Function  key  change from monitoring FEC CQ's  to  QSO's  in

progress (chirpcopy).

Automatic Beacon Mode CQ caller

Automatic contest (DX-pedition) mode

Color changes between incoming and outgoing (echoed as  sent)

text.

1.1.3     MORSE/BAUDOT/ASCII

Automatic  CQ caller.  Will call CQ repetitively  and  signal

you when a reply is received.

RTTY SELCAL

NAVY MARS RTTY file transmitting protocols for the PK-232.

2.0  Getting Started

MAKE  A  BACK UP COPY OF THIS DISK, BEFORE YOU REMOVE  THE  WRITE

PROTECT TAB.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 14

If  you don't have a PK-232 then rename the LAN-LINK.SYS file  to

PK232.SYS  and  rename your TNC.SYS file  as  LAN-LINK.SYS.   For

example,  if  you  are  using  a  TNC2,  perform  the   following

operations in DOS.

RENAME LAN-LINK.SYS PK232.SYS

RENAME TNC2.SYS LAN-LINK.SYS

For details of the rest of the customization possibilities in the

'LAN-LINK.SYS' file, read Section 23.2.  YOU WILL NOT BE ABLE  to

change  the TNC Type from within LAN-LINK.  YOU MUST  rename  the

SYS files BEFORE you begin the customization sequence.

2.1 Bringing LAN-LINK Up the First Time.

2.1.1    Turn on the TNC.

2.1.2   If  the date and time in the PC are not set  to  UTC

(GMT),  set  them  using  the  DOS  date  and   time

commands.

2.1.3   Load the program by typing LAN-LINK in the way  that

you normally load a program.

2.1.4   WAIT until the program says that it has  initialized

the  TNC and shows the band and power in the  status

window at the top of the screen.

2.1.5   If  there is no battery in your TNC do what the  TNC

manual  tells you to do to synchronize the  TERMINAL

baud rate to 1200 baud.  In the case of the  PK-232,

LAN-LINK will send the PK-232 three * characters  to

set the PK-232 to 1200 baud.

2.1.6   After  LAN-LINK has loaded, use the Parameters  Menu

option  "2" to replace G8BTB with your callsign  and

Selcal  information.  If your TNC does  not  support

AMTOR  you  still need a dummy entry  in  that  line

(which  is  taken care of automatically).   You  may

also  change the colors at this time.  Use  the  "U"

option to save the updated file.  After reading  the

rest  of this manual, you may want to  change  other

defaults to set up LAN-LINK the way you like it.

2.1.8   Use  the  "N"  and "I" options in the  TNC  Menu  to

configure the TNC.  Then select the desired mode  of

operation  from the displayed menu by  pressing  the

appropriate key.

2.1.9   Exit from LAN-LINK by using the Alt-X Key or the Esc

Key followed by the X Key.

2.1.10  Reload  LAN-LINK the same way you did before.   This

will load the edited LAN-LINK.SYS file with the  new

settings.  You are now ready to use LAN-LINK.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 15

NOTE:  If you have batteries in the TNC, make sure that  the

TNC is set for a terminal baud rate of 1200, and try to  run

the  program.   If you see Greek characters  or  it  doesn't

work, remove the batteries and try again (Refer to  Appendix

1.10).  You can then replace the batteries and from then  on

the program will be correct. Make sure you leave the TNC on

when  you replace the batteries or your initialization  will

revert to the TNC default settings - the batteries are there

to power the TNC RAM while your primary power is off.

2.2 Connecting to other stations

LAN-LINK  treats  incoming connects  and  outgoing  connects

differently.  To connect to another station, Use  the  Alt-C

hot  key, Function key 5, or bring up the Call Menu via  the

'Esc  C' character sequence as described in Section  7.   DO

NOT  TYPE 'C Callsign' in the command mode.  If you do,  the

connect will still take place (the TNC won't know if you  or

LAN-LINK  issued the command) but LAN-LINK will treat it  as

an  incoming  connect and issue the "[ZCZ]  LAN-LINK  1.56>"

handshaking signal (if the LAN-LINK flag is in its  default,

or  enabled state) as described in Section 18.  If you  con#

nect  to  a BBS or a Node this way, you will  get  an  error

message back from the BBS or node, since it does not  recog#

nize LAN-LINK's handshake.

When you use the Alt-C or F5 approach you can also make  use

of the 'path memory' feature in the LAN-LINK.DIR file.

2.3 Upgrading from YAPP

A  YAPP.CNF file is provided to facilitate  conversion  from

YAPP to LAN-LINK. YAPP.CNF is a configuration file for  YAPP

Version  2 that will configure the PK-232 for YAPP and  then

back to LAN-LINK.

If you are currently using YAPP, replace your YAPP.CNF  file

with this one and load YAPP.  Then terminate the program  in

the  normal manner by using the Alt-X Key.  You should  then

be able to load and run LAN-LINK without any problems.  Each

time you subsequently want to run YAPP, YAPP will set up the

PK-232,  execute  and then reconfigure the PK-232  back  for

LAN-LINK.

If you are using a YAPP.DIR file, rename it to LAN-LINK.DIR.

2.4 Using Different TNCs

LAN-LINK  is normally loaded by typing LAN-LINK followed  by

the  'Enter'  key.   In  this  instant,  LAN-LINK  gets  its

configuration information from the LAN-LINK.SYS file.   LAN-

LINK.SYS  is the default SYSTEM or configuration  file.   If

you have more than one TNC you may want to set up  different

configuration files for each TNC.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 16

You must set up your different configuration files with  the

filetype ".SYS", examples being TNC1.SYS, TNC2.SYS, KAM.SYS,

PK232.SYS  as  well as the default LAN-LINK.SYS.   Edit  the

sample  files on the distribution disk to customize them  to

your requirements.

To load LAN-LINK with the optional configuration file,  type

'LAN-LINK'  followed  by the name of the file  (without  the

filetype)  on the command line then press the  'Enter'  Key.

For   example,  if  you  have  a  PK-232  and  a  TNC2   and

occasionally use the TNC2, set up the LAN-LINK.SYS file  for

the  PK-232.   Then  type  'LAN-LINK'  to  run  the  program

normally.   On the rare occasions when you want to  use  the

TNC2, you may type 'LAN-LINK TNC2' to run LAN-LINK with  the

TNC2.SYS configuration file.

2.5 Using Different HF and VHF Configuration Files

You  can  also  use two SYS files, one for HF  and  one  for

VHF/UHF.  If you name them HF.SYS and VHF.SYS you can  bring

the relevant one up by typing

'LAN-LINK VHF' or  'LAN-LINK UHF'

as desired.  You can also use a different one for a contest,

or for any reason you wish.

2.6 Using the AMTOR/PACKET Robot/Automatic Contest Station

In  order  to use the Robot in the AMTOR Mode, you  need  to

have  a  Brag Tape file. The default name for this  file  is

LAN-LINK.TXT.  If this file does not exist, you will not  be

able to activate the Robot.

You   activate  the  Robot  by  using  the  Alt-O  hot   key

combination.  The  Robot will then call CQ and  wait  for  a

reply.   If  no reply is received, it will call  again.   It

will  continue to call at preset intervals until a reply  is

received  (you  may change the interval via  the  Parameters

Menu).  In AMTOR, the Brag Tape will be transmitted and  one

(and  one  only)  transmission  accepted  from  the  linking

station,  unless the linking station uses a NC/L command  on

the  same  line  as the +?.  In that case,  the  Robot  will

respond  to it.  All incoming text will be captured to  disk

in the Capture-to-disk file (LAN-LINK.RUN).  The Robot  will

then  sign off and drop the link.  If it has not managed  to

recognize a valid call from the linking station it will  ask

for a retransmission of the callsign.

In  the Packet Communications Mode, the CTEXT will  ONLY  be

transmitted  if you have set CMSG to ON (in the TNC  Command

Mode),  the Brag Tape will not be transmitted. NC/L will  be

active.  When the connecting station disconnects, the  Robot

will restart the CQ cycle.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 17

In AMTOR turn the answerback off if the Robot is on. In this

way  other  stations  will  not  be  able  to  mess  up  the

sequencer.

When  you touch the F8, F3, Alt-D, Alt-Q or 'End' keys,  you

turn the Robot off.

The  PK-232's  flush command is used before  each  robot  CQ

message is transmitted to synchronize the text to the  Robot

software.  The KAM however, does not have a way to flush its

internal  transmit buffer.  Under QRM conditions or if  sta#

tions do not sign on correctly, the sequence can get out  of

order due to the wrong text being transmitted.

To  use the Automatic Contest mode, enable the contest  mode

from  the Parameter menu, then activate the Robot using  the

Alt-O function key.  To turn the Automatic Contest Mode off,

deselect  (turn  off) the contest mode  from  the  Parameter

Menu.

3.0  Things You Should Know

3.1 Memory Requirements

LAN-LINK  requires a minimum of 360k of RAM.  You will  need

more  RAM if you wish to use the Editor.  The  Editor  files

will each take a maximum of 64K additional RAM.

3.2 PC to TNC Communications

All communications between the computer and the TNC normally

use  the 8 BIT NO PARITY and 1 stop bit RS-232  Serial  Port

configuration. You CAN THUS SEND FOREIGN LANGUAGE CHARACTERS

FROM THE KEYBOARD OR FILES.  If you want to limit the  char#

acter  set  to the 7 bit ASCII set, set the  TNC  parameters

accordingly.   The baud rate defaults to 1200, but  you  may

set it to 2400, 4800 9600 or even 19200 if your CPU clock is

fast enough and the TNC will respond correctly.

3.3 Open Loop

IF YOU USE THIS PROGRAM, YOU STILL MUST READ THE TNC MANUAL.

WARNING  ....  THIS PROGRAM OPERATES IN AN OPEN  LOOP  MODE.

IT REMEMBERS WHAT INSTRUCTIONS OR COMMANDS IT GAVE THE  TNC.

IF  FOR  SOME  REASON, THE TNC IS NOT  SYNCHRONIZED  TO  THE

PROGRAM YOU WILL NOT GET CORRECT DISPLAYS ON THE SCREEN.

3.4 Problems

In case you have problems, always check the default settings

using  the 'Verify Changes' option of the  Parameters  Menu.

Further information is contained in Appendix 1.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 18

3.5 The Contents of the Status Window

The indicators in the status line depend on the state of the

LAN  Link and are described in the sections that  deal  with

the functions they area associated with.  This section is  a

summary of the displays in the Status Window.

Some typical status window displays are shown below.

2  Meters 2 Watts      PACKET 1200 W [R] TRFC ->N4QQ

20 Meters 100 Watts    MORSE TEST[4]

20 Meters 100 Watts    PACKET 300 N TRFC

CAPTURE TO DISK        PACKET 300 N [C1][Z] TRFC ->N4QQ

If the automatic Capture-to-disk is turned off, a yellow bug

will be flashing on the left side of the line.

If the Printer Flag is set, a yellow [P] will be flashing on

the left side of the line.

If  the miscellaneous flag is set, an asterisk '*'  followed

by  two  numbers  and a ']' will be  displayed.   The  first

number is the BBS Sequence State, the second is the  Connect

Sequence State.  These numbers reflect the internal state of

LAN-LINK and should be quoted when describing problems.

If  a message transfer is taking place, the callsign of  the

station involved will be displayed.

When  the Capture-to-Disk is turned on, a 'CAPTURE TO  DISK'

will  be  shown,  if it is turned off,  the  operating  band

(Meters) and power (Watts) will be shown.  If a ZapBBS is in

operation, the Bulletin Request Count will then be shown  in

square brackets ('[]').

The communications mode will be shown next, it may be one of

the following;

PACKET    AMTOR     AMTOR-(MAIL)   AMTOR-FEC

BAUDOT    ASCII     MORSE          NAVTEX

SIGNAL    ????      DEFAULT {boot up}

A flashing yellow musical note following the  Communications

Mode indicates that the sound has been inhibited.

The  next  number to be displayed is the  RF  communications

baudrate.   The  letter 'W' or 'N' indicates that  the  Wide

shift or the Narrow shift has been selected.  A 'down arrow'

will  be shown if the shift is normal. If the shift  is  re#

versed a flashing yellow 'up arrow' will be displayed.

In   the   BAUDOT  mode,  'USOS'  will   be   displayed   if

'Unshift-on-space' is selected.

If  the Contest Mode is selected, the word 'TEST' will  then

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 19

be displayed followed by the Contest QSO number in '[]'.

In certain Robot and AutoCQ states, namely while the comput#

er  is waiting for a reply, a flashing cumulative  CQ  count

will be displayed next.

If  the  RTTY  SELCAL is active the word  'SELCAL'  will  be

displayed next.

If  the Navy MARS BAUDOT RTTY protocol is enabled,  a  '[n]'

will be displayed.

In the packet and AMTOR Robot modes, if at least one station

has  connected  or linked, a flashing yellow '[C]'  will  be

displayed next.  The number in the square brackets with  the

'C'  is the number of stations that  have  connected/linked.

If the target call station was one of those station, a happy

face will also be displayed in the square brackets.

If at least one QTC snatch has taken place a flashing yellow

'[Q]'  will  be displayed next.  The number  in  the  square

brackets  is  the  number of QTC Snatches  that  have  taken

place.

In the packet communications mode, if a ZapBBS, a QTC Snatch

or a PacketClusterZap is in progress a flashing yellow '[Z]'

will be displayed next.

In the packet or AMTOR communications mode, a '[R]' will  be

displayed  next  if LAN-LINK is configured  for  'store  and

forward' (:QSP:).

In the packet and AMTOR communications modes, if the  beacon

mode is enabled a '[B]' will be displayed next.  if  someone

has  connected and sent a :QRT: sequence, the '[B]' will  be

flashing.   The '[B]' will be followed by a '[M]'  if  there

are  messages  pending.  The number in the  square  brackets

with  the  '[M]'  is the number of  messages  (*.OUT  files)

pending.

In  the  packet communications mode, if the  SAREX  mode  is

enabled,  a  '[S]' will be shown next.  The  '[S]'  will  be

flashing  while  capture to disk is in progress.   If  SAREX

packets  have been copied a happy face will be displayed  in

the square brackets together with the 'S'.

In  the packet communications mode, if the  digipeat  Detect

mode is enabled a '[D]' will be displayed next.

In  the packet communications mode, the  terminal  sub-modes

(see Section 21) are then displayed as follows;

SOLO      This lets you only see messages addressed to you.

CQ/B      This lets you see beacons and CQ calls as well.

TRFC      This  lets you see packets containing messages  as

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 20

well.

EVRY      This lets you see everything on channel, including

the link control packets.

MAIL      This  lets  you monitor packets  from  a  specific

station.   In the PK-232,  you can do  it  without

displaying the headers.

The  next  item  to be displayed is a  callsign,  under  the

following  conditions;   the call of the  station  connected

with,  will be prefixed by a '*>' when connected, and  by  a

'->' when a connect request is in progress.  if a disconnect

is  in progress, the callsign will be replaced by a  '$$$$'.

The  callsign will be seen as flashing when a connect is  in

progress.

In  the MAIL mode, the callsign of the station that you  are

"reading the mail" on will be displayed, otherwise the  call

sign displayed will be that of the one you are connected to.

If  an  Alert Call has been entered, it  will  be  displayed

next,  similarly  for a target call.   The display  will  be

slightly different of the calls have been heard/worked.

The  next  items to be displayed are the packet  multi  user

mode  displays.  If the conference bridge is up, a  flashing

yellow bridge will be shown.  If only the conference mode is

set,  the word 'CONF' will be flashing, if the  basic  multi

user  mode is set, the word 'MULT' will be seen.  The  call#

sign will be prefixed by the I/O channel number/letter,  and

will be in the color allocated to the traffic on that  chan#

nel.

In the non packet modes, the callsign displayed will be that

of the station being worked (entered by the AMTOR robot, the

automatic CQ call recognition, or by the operator using  the

Alt-C  or  Alt-E keys).  In AMTOR, the SELCAL of  the  other

station  will  also be displayed sometimes, usually  if  you

entered the call.

An  '[L]' will be displayed in the non packet modes  when  a

callsign is entered.  The '[L]' will flash to remind you (to

log it) until the QSO is logged.

4.0  Function Keys

Function  keys  are  used to perform  operations  common  to  all

communications modes, or to allow fast performance of a  function

without having to go through several layers of menus.  The action

of  each  of  the function keys is  described  in  the  following

sections.   The function key number corresponds to the  paragraph

number, namely 4.1 refers to Function Key 1 (F1), etc.

4.1- Capture to Disk Toggle (F1)

This  key toggles the "capture-to-disk" command.   When  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 21

"capture-to-disk"  is active, all incoming  text  (including

'Echo as sent' outgoing text) is stored on a disk (floppy or

hard)  in a file called 'LAN-LINK.RUN'.  If this  file  does

not  exist  the first time that LAN-LINK is  loaded,  it  is

created.   From that time on, successive activations  append

new data to the contents of the file.

Each time the command is activated a datestamp (or  timetag)

will be written to the file.

When  a  Packet station connects "capture-to-disk"  will  be

activated automatically unless you instruct LAN-LINK not  to

do so with the Capture-to-disk flag (Section 18.17).  Packet

connects  will thus be recorded in their entirety  automati#

cally  unless you turn off the "Capture-to-disk" during  the

course  of  the QSO. "Capture-to-disk" will  be  stopped  at

disconnect time.

You  may later edit the file at your convenience  with  your

favorite word processor or the included Editor.  However, if

your word processor can only handle files smaller than  64k,

make  sure that the 'capture-to-disk' file remains  smaller.

You can do this by renaming the file to something like 'LAN-

RUN.001' and change the number each time, or use a date code

such as L890101.RUN.

LAN-LINK will automatically close the "capture-to-disk" file

when you terminate the program and return to DOS.

Capture-to-disk is active when you scroll back.  you can use

this  feature to save data that has already scrolled by  the

window.

4.2  Send Brag Tape and Talk (F2).

This  command lets you transmit the Brag Tape (Default  LAN-

LINK.TXT. In the non packet modes, it will leave the  trans#

mitter on after sending the file.

4.3- Transmit Callsign Sequence (F3)

This command lets you transmit the following sequence <other

call> DE <your call>.  For example if you are 4X6AA and  you

are  working  G3ZCZ  then this command  would  transmit  the

sequence  'G3ZCZ de 4X6AA'.  You use this at the  start  and

end of a transmission in any mode except Packet.

4.4  Send Brag Tape (F4)

This  command lets you transmit the Brag Tape (Default  LAN-

LINK.TXT.  In the non packet modes, it will turn the  trans#

mitter off after sending the file.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 22

4.5 Show Packet 'MH'list and Point and Shoot Connect (F5).

This  command  lets  you see what calls the  TNC  has  heard

recently on the Packet channels.  You can see up to 18, with

the  data and time they were last logged by the TNC.   Refer

to the 'MH' command in the TNC manual for the particulars of

the display.

The  MH list shows up in a special window and freezes  until

you depress any key.  While the list is frozen, LAN-LINK  is

not monitoring received data, so don't freeze it too long.

If  the MH list is blank, or information comes in  from  the

TNC  between the time you entered the command and  the  time

the  list is sent back,  subsequent text from the  TNC  will

show up in the MH window until the next 'Cmd:' appears or 18

lines of text have been received.

You  can also use this command in a point and  shoot  method

for  attempting  a connect with a station  you  have  heard.

Bring  up the MH Window and move the cursor to  the  desired

callsign.  When the cursor is positioned on any character in

the  call,  press  the 'Enter' Key and  that  call  will  be

recognized  as the callsign.  If the call is joined  to  the

word "Cmd:" or the cursor is positioned on a space or blank,

the connect attempt will be inhibited.

This command does not work on the TNC1.

4.6  Auto CQ Call (F6)

This  command will send a CQ sequence, wait a  preset  delay

and then repeat the CQ sequence until it receives your  call

back from someone else.  When it does so, it will alert  you

with  a  Connect  Alarm signal, reset the  delay  timer  and

revert  to the normal or listening mode.  In AMTOR  it  will

revert  to the mode in which it is waiting for an  ARQ  call

and is able to copy FEC signals on frequency.

When   the  Automatic  CQ  sequence  is  in  operation,   an

indication  will  be  shown  in  the  status  window.    The

indicator  will be steady while the transmitter  is  sending

the CQ text, and will blink while the receiver is  listening

for a reply.

The time delay between calls is set using the 'Q' command in

the Parameters Menu (Set CQ delay time).  It is also set  as

a  defined default by you when you setup  your  LAN-LINK.SYS

file.

The automatic CQ command will only work in the TNC nonpacket

modes if the 'Echo as Sent Mode' is set to 'ON' per the LAN-

LINK.SYS file.  In other words, you must be able to see  the

CQ  text slowly echo in the incoming window for the  command

to work.  The reason for this is that the "K K K"  (or  "+?"

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 23

in AMTOR) sequence is used to arm the software to look for a

reply.

In  AMTOR  you  will get the connect  alarm  only  when  the

replying station transmits the '+?' character combination.

The check for the detection of your callsign and the connect

alarm  sequence only occur after a line feed  character  has

been received.

Under  most conditions in AMTOR, many in RTTY, and  some  in

MORSE,  LAN-LINK will recognize the callsign of the  station

answering  your CQ and display it in the status window.   In

the  event  that it fails completely, the  sequence  '--?--'

will  show up in the status window.  Since  the  recognition

algorithm  used just looks for the first word following  the

'DE space' sequence and assumes that it has the callsign, be

careful  to  check it when operating under  QRM  conditions.

You can override the automatic call selection with the Enter

Callsign command in the Call Menu.

The automatic CQ sequence is canceled when an incoming  call

is detected (only if the Beacon or Robot/Mailbox Mode is not

selected), if you push the disconnect command key, or if you

change communications modes.

4.7 Activate Type Ahead Buffer/AMTOR Relink (F7)

This key is not available in the Packet Communications Mode.

It  also  functions somewhat differently in  the  AMTOR  and

other  communications  modes as described in  the  following

sections.

4.7.1 Activate Type Ahead Buffer

The  use of this key sets LAN-LINK to the "Type  Ahead"

Mode.   This is necessary for the PK-232 only, the  KAM

and the MFJ1278 have the feature built in.  Any charac#

ters then typed at the keyboard will be placed into the

TNC  transmit buffer.  The contents of the buffer  will

be  transmitted over the air when the transmit  command

is invoked with Function Key 10 [F10].

This  feature  allows you to 'type-ahead'  as  you  can

enter  text  into  the transmit  buffer  while  viewing

incoming words in the INPUT window in the screen.   Any

characters  typed from the keyboard will be  echoed  in

the OUTGOING window on the screen.

4.7.2 AMTOR Relink

If  you  return  to the Command  Mode  when  linked  to

another  station in AMTOR, use this key to get back  to

the Converse Mode.  If the link times out, you can  try

to relink with this key if you have entered the  SELCAL

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 24

of the other station.

4.8 Disconnect, Disengage or Receive (F8)

This  function  key  returns  the  station  to  the  receive

communications  (TNC command) mode.  It may also be used  to

abort  the automatic CQ sequence.  If you use it to abort  a

CQ  sequence while the transmission is in progress, you  may

also want to flush the PK-232  transmit buffer by using  the

Flush  command  in  the  TNC  Menu  or  the  Alt-F  hot  key

combination.

In  a multiconnect situation the program will prompt you  to

tell it which I/O channel to disconnect.

In  AMTOR  this  command key toggles  LAN-LINK  between  the

Chirpcopy  Mode  (listening  to  a  QSO  in  progress)   and

monitoring FEC transmissions (CQ calls).

4.9 Put TNC in CMD Mode (F9)

This  command puts the TNC into the Command Mode.   At  this

time, the program is transparent and acts as a dumb terminal

(except  for  placing  incoming and  outgoing  text  in  the

relevant windows).  You use this Mode for overriding any  of

the TNC default conditions.  (SEE I TOLD YOU THAT YOU HAD TO

READ THE TNC MANUAL).

4.10 Put TNC in CONVERSE /Transmit Mode /AMTOR FEC (F10)

This  command places the TNC into the transmit  or  Converse

Mode.  The transmitter is turned on and the contents of  the

transmit buffer are transmitted.  In the non packet communi#

cations  modes, the outgoing characters will be echoed  back

from the TNC and displayed in the INCOMING window on the CRT

as they are transmitted on the air.

In the AMTOR modes this key puts the TNC in the FEC Mode and

turns the transmitter on.

4.11 Alternate Function Keys

These  keys  transmit the files with the  fixed  names  LAN-

LINK.001 through LAN-LINK.010 (the suffix number corresponds

to  the  function  key) which must be located  in  the  same

directory as the LAN-LINK.EXE program.

4.11.1  Send/Talk file LAN-LINK.001

4.11.2  Send/Talk file LAN-LINK.002

4.11.3  Send/Talk file LAN-LINK.003

4.11.4  Send/Talk file LAN-LINK.004

4.11.5  Send/Talk file LAN-LINK.005

4.11.6  Send/Talk file LAN-LINK.006

4.11.7  Send/Talk file LAN-LINK.007

4.11.8  Send/Talk file LAN-LINK.008

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 25

4.11.9  Send/Talk file LAN-LINK.009

4.11.10 Send/Talk file LAN-LINK.010

In  the non packet communications modes, the transmitter  is

left on when the file has been sent.  In AMTOR, the  change#

over character pair (+?) is not sent.

4.12 Shift Function Keys

These function keys show the contents of the files with  the

fixed  names LAN-LINK.001 through LAN-LINK.010  (the  suffix

number  corresponds  to  the function  key)  which  must  be

located in the same directory as the LAN-LINK.EXE program.

4.12.1  Show contents of file LAN-LINK.001

4.12.2  Show contents of file LAN-LINK.002

4.12.3  Show contents of file LAN-LINK.003

4.12.4  Show contents of file LAN-LINK.004

4.12.5  Show contents of file LAN-LINK.005

4.12.6  Show contents of file LAN-LINK.006

4.12.7  Show contents of file LAN-LINK.007

4.12.8  Show contents of file LAN-LINK.008

4.12.9  Show contents of file LAN-LINK.009

4.12.10 Show contents of file LAN-LINK.010

4.13 Control Function Keys

These  function keys show the first lines of the files  with

the  fixed  names  LAN-LINK.001  through  LAN-LINK.010  (the

suffix number corresponds to the function key) which must be

located in the same directory as the LAN-LINK.EXE program.

5.0  Main menu

When  you  press the Escape Key, the Main Menu  appears  on  your

screen  and provides a number of choices which are  described  in

the following sections.

B BBS Menu

C Call Menu

D eDit Menu

E Event Menu

F Files Menu

H Help Menu

J Jump to DOS Shell

K LAN-LINK Menu

L Log Menu

M Message Menu

N TNC Menu

O AMSAT-OSCAR Menu

P Parameter Menu

Q Q Codes [NC/L]

S communicationS Mode Menu

T Terminal Mode Menu

X eXit LAN-LINK

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 26

Type in the single letter to make your choice.

NOTE:

Different options will show up in each menu depending on the

connect  state (in packet) or the type of TNC that  you  are

using.   For  example,  if you are using  a  TNC  that  only

supports packet, the non packet menus will not be seen.   An

other  example is that the BBS Menu is only active when  you

are connected to another station.

Consider each of the menus in turn.

6.0 The BBS Menu

This menu gives you smart file transfer capabilities with a  BBS.

The menu is only active when you are connected to another  packet

station.  The BBS Menu provides the following capabilities.

D Download Text Files.

Type in the single letter to make your choice.

6.1 Download Text Files

This option lets you download a text file from the BBS  into

a  separate  text  file on your disk.  Enter  the  full  BBS

download command at the prompt line.

To download a file  from a W0RLI BBS such as FILENAME.TYP in

the  A  subdirectory  you'd  enter  'DA  FILENAME.TYP'.   To

download  the  same file from the PACKET subdirectory  of  a

WA7MBL BBS you'd enter the command 'D \PACKET\FILENAME.TYP'.

If you use the 'D' command to download a text file, it  will

end up in a separate file on your disk ONLY IF a file of the

same name does not exist on your drive.  If a file with  the

selected name already exists on your disk drive you will get

an error message.

7.0 The Call Menu

The call menu is used to enter calls, either to call/connect/link

a  station  or  to remember a call or path.  Each  time  you  are

prompted for a call, LAN-LINK will remember the previous 15 calls

and  you may use the Up and Down arrows to move the cursor  to  a

remembered call. If you wish to capture a call from the  incoming

text  window, push Function Key 10 (F10) to enter the window  and

move  the  cursor to the desired callsign.  When  the  cursor  is

positioned on any character in the text, push the 'Enter' Key and

that  word  will be recognized as the callsign.   Any  characters

joined  to the call (such as a '.') will be picked up as part  of

the  call.  If the cursor is positioned on a space or blank,  the

connect attempt will be inhibited.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 27

In  AMTOR, whenever you are asked to enter a  callsign,  LAN-LINK

will prompt you with its guess at the SELCAL.  If you agree  with

it,  just  press  the 'Enter' Key, if you  disagree,  enter  your

choice over the prompt version.

The Call Menu provides the following choices.

A Alert Call (packet) or ARQ Call Talk (AMTOR)

C Connect/Call

D Show Directory

E Enter Call

M reMember Path

P Packet Cluster

Q Call CQ

R Retry

T Target Call

U aUto CQ

X Set DX Flag On/Off

Z Zap BBS

Type in the single letter to make your choice.

7.1 Alert Call/ARQ Call

7.1.1 Alert Call

The Alert call feature is used in the Packet  Communications

Mode  when disconnected and the terminal set for  'TRFC'  or

'CQ/BEACON'.  LAN-LINK will scan the packet headers received

from  the  TNC, and, when it sees a  packet  originated  (or

digipeated if the MRPT parameter in the TNC is set to 'ON'),

by  the station whose call you have entered as  the  'Alert'

call,  it  will sound an alarm at the  console.    The  line

containing the packet header will also be displayed in light

green.

This  option toggles the Alert call feature on and off.   If

it  is off, you are prompted for the call to  be  monitored,

and, if it is on, it is turned off.  The toggle is set  when

you  enter  a  callsign, and cleared when  you  depress  the

'Enter' Key instead of a callsign.

The  Alert call will be shown flashing in the status  window

when in the disconnected state.

NOTE: If the terminal is set for 'CQ/BEACON' the Alert  will

only  sound if the specified station transmits CQ or  BEACON

packets.

7.1.2 ARQ Call

In  AMTOR, this option will attempt to send an ARQ  call  to

the  desired  station, transmit the callsign  sequence  when

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 28

linked and allow you to continue typing text (Note that  the

C option sends the '+?' after the callsign sequence).

7.2 Connect/Call

This  option  works in almost the same manner  in  both  the

packet  and non packet communications modes as described  in

the following sections.

7.2.1 Non packet Modes

You  use  this option to call a station  heard  on  the

band.   In AMTOR the link request will be sent  in  the

ARQ Mode to the SELCAL of the desired station.

7.2.2 Packet Mode

You  use this option to attempt a connect with  another

packet  station.  In a multiconnect situation the  LAN-

LINK  will automatically select a free I/O  channel  to

try the connect.

7.2.2.1 Names, Handles and Paths

If  you  so  desire, you can create  a  directory  file

(default name is LAN-LINK.DIR) and keep a list of names

and  calls.  The computer will look up a name and  call

the station by its callsign.  For example if you put

Joe G3ZCZ

as  a line in the file, when you tell the  computer  to

connect  to Joe, it will try to connect to  G3ZCZ.   If

you enter

4X4HF 4X4HF V 4X1AA, 4Z4ZB, 4X4IL

you  only  need to type '4X4HF' to set up  the  correct

connect path.

You  can  also use this feature to automate  a  NET/ROM

and/or a KA Node path connection.

7.2.2.2 NET/ROM and TheLink

In  the manual NET/ROM path setting up mode, you  would

first  connect  to your local node, then when  you  are

connected with it, connect to the next node, and so  on

through  any intermediate nodes within the  whole  path

before  you  finally  connect  with  your   destination

station.   You may also have digipeaters in any of  the

paths.

For  example if you wish to connect to K1HTV who is  in

range  of the SSCT node, and you are in Washington  DC,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 29

you  might first have to connect to your local  NET/ROM

node   (ELK)  via  a  digipeater,  then  each  of   the

intermediate nodes, SNJ3, NNJ, WMA before you could try

to connect with K1HTV.

Consider  the manual sequence to make  the  connection.

K1HTV  is  the station the contact is  being  attempted

with,  and  he  is located within  range  of  the  SCCT

NET/ROM  node.  The station attempting the  connect  is

located  in  the Washington DC area one  digipeat  away

from the ELK NET/ROM node.

The  connection to the local node is  performed  first.

The  operator  has to type 'C ELK v WB4APR-5'  to  make

that link. When the connect with the node is  achieved,

the operator must type 'C SNJ3' to connect to the  next

node.   Each time the connection is made, the  operator

must  type in the command to make the next  one,  until

finally the last node is reached and a connect  request

can  be made to the destination station (either  direct

or  via  a  digipeater).  Sometimes  there  may  be  an

intermediate   non-NET/ROM   connection   between   two

stations somewhere in the path.

This  procedure is tedious and time consuming,  because

it  may take several seconds to complete each stage  of

the link.  LAN-LINK automates this sequence by scanning

the  line in the LAN-LINK.DIR file associated with  the

callsign  entered and transmitting each section of  the

line  when it recognizes the 'Connected to' reply  from

each NET/ROM node in the path.  When it finally gets to

the destination it sounds a chime to alert you that the

connect has been achieved.

The  LAN-LINK.DIR  file  entry  for  this  path  is  as

follows:

K1HTV ELK v WB4APR-5!SNJ3!NNJ!WMA!SCCT!K1HTV

where the '!' separates each stage of the link.

The key word is K1HTV.  LAN-LINK then scans the line to

see if there is a '!' in it.  If there is it knows that

it  is dealing with a NET/ROM path.  It then tells  the

TNC to attempt a connect with the characters  following

the  key  word up to the first '!'.  In  this  case  it

tells the TNC to connect to ELK via WB4APR-5. When  the

TNC  sends back the connected message,  LAN-LINK  tells

the TNC to try to connect to the next node in the path,

namely  in this example, SNJ3. The  sequence  continues

each  time  a  connection is made  until  the  link  is

complete.

To  set up a directory entry just use the format  shown

above.   Place  a '!' sign after the  uplink  path  and

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 30

different  node  segments. Do not place one  after  the

downlink path.

Depending  on the configuration, LAN-LINK will issue  a

'disconnect'  command  to  the TNC  in  the  event  the

NET/ROM or KA-Node connect attempt fails.  This feature

is  designed  for  known paths  not  for  general  path

determination.

7.2.2.3 KA-Nodes

You can connect or crossconnect via KA-Nodes using  the

directory file.  You must still use the ! separators as

well as two new symbols %  and &.  Use % for 'C' and  &

for 'X' in KA-Node links..

For example, given that a path to W9TNN-2 is first to a

local  KA node [JRW] then to a second node on the  same

VHF frequency [W3EAX-7] and lastly via a cross  connect

on  40 Meters to W9TNN-2, the line in the  LAN-LINK.DIR

file would be as follows.

w9tnn-2  jrw!%w3eax-7!&w9tnn-2

You can mix NET/ROM and KA-Node paths as in:

n7dva!g3zcz-9!&WA6IEL-4!n7dva v kl7vz-3

The different symbols are used for the KA node to  tell

LAN-LINK  which replies to look for as well as what  to

prefix the call with.

Depending  on  the Node Drop Link  Flag  configuration,

LAN-LINK  will issue a 'disconnect' command to the  TNC

in  the  event the NET/ROM or KA-Node  connect  attempt

fails.   This feature is designed for known  paths  not

for general path determination.

7.2.2.4 Loop Backs

In the Packet Communications Mode, if you want to  loop

back through someone else you would have to type

C <yourcall> V <hiscall>.

For  example if I (G3ZCZ) wanted to loop  back  through

4X6AA, I would have to type the command

C G3ZCZ V 4X6AA

to the TNC.

In  LAN-LINK you just have to enter the  '/'  character

followed  by the call of the station you want  to  loop

back through.  For example if I (G3ZCZ) wanted to  loop

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 31

back through 4X6AA, I just need to enter

/4X6AA

as  the call to connect to at the correct  prompt,  and

LAN-LINK  will tell the TNC to try to connect to  G3ZCZ

via  4X6AA, namely issue the command 'C G3ZCZ V  4X6AA'

to the TNC.

7.3 Enter Call

This  option lets you enter the call of any station  calling

you into the program. If you type the Enter Key in  response

to  the  prompt,  you can later use  Function  Key  (F3)  to

transmit the sequence "DE <yourcall>". You may also use  the

Retry feature to call that station.

In  AMTOR,  you use the feature to enter  the  callsign  and

SELCAL  of the station you are listening to and may want  to

call later.

7.4 ReMember Path

You may use this option to put an entry in the  LAN-LINK.DIR

file.

7.5 Zap Packet Cluster

This  option  firsts prompts you with the  callsign  of  the

PacketCluster. If you enter a callsign, the entered callsign

will become the new PacketCluster call. LAN-LINK then issues

a connect request to the PacketCluster.  When the connection

is achieved, and the PacketCluster sends you a '>' character

at the end of a line of text, LAN-LINK will issue a

'SHOW/DX'

command to the PacketCluster and return to manual operation.

PacketCluster  is a specialized BBS/conference  by  Pavilion

Software for DX chasers.

7.6 Call CQ

You  use  this  command  key to  call  CQ.   In  the  Packet

Communications  Mode, the TNC will transmit a line  of  text

along with the CQ packet.  The line of text is set up in the

LAN-LINK.SYS file described in Section 22.

In  the  AMTOR  Communications Mode,  your  SELCAL  will  be

transmitted along with the CQ call in the FEC Mode.

7.7 Retry

This  option  allows you to retry a connect  that  for  some

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 32

reason did not go through the first time.  It saves wear and

tear  on  your fingers particularly if the  desired  connect

path is via a number of digipeat relays or through a  number

of NET/ROM NODES.

In the non packet modes, it allows you to retransmit a  call

to another station without having to reenter the call.

If  you  have  previously used the Enter feature  to  put  a

callsign in the program when you hear that station, you  can

then use the Retry command to call him later when the QSO in

progress terminates.

7.8 Show Call Directory

This  option  displays the contents of  the  call  directory

file (LAN-LINK.DIR).

7.9 Target Call

The Target call feature is used in the Packet Communications

Mode when disconnected. LAN-LINK  will monitor the  stations

connecting  to  you. When it sees a connect by  the  station

which you have entered as the 'Target' call, it will set the

flashing Connect Count display to show a 'happy face'.

This option toggles the Target call feature on and off.   If

it is off, you are prompted for the call to be monitored, if

it  is  on, it is turned off.  The toggle is  set  when  you

enter a callsign, and cleared when you press the 'Enter' Key

instead of a callsign.

The Target call will be shown flashing in the status  window

when in the disconnected state.

7.10 AUto CQ

This option is the same as Function Key 6 (F6).

7.11 Toggle DX Flag

LAN-LINK allows you to call either 'CQ' or 'CQ DX'.  If this

flag  is  set, all CQ calls will be to "CQ DX".   If  it  is

cleared, all CQ calls will be plain CQ's.

7.12 Zap BBS

The BBS Mail Snatch can be initiated in two ways. It can  be

initiated when your incoming screen displays a "Mail" beacon

(received  from your local BBS) with your call in the  first

line  following  the  header,  or  by  LAN-LINK's  Zap   BBS

function.

In the event of a BBS Mail-Snatch, if the *.BBS file  exists

for  that BBS, the file will be processed and  the  contents

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 33

sent to the BBS.  Thus if you don't get a chance to send the

mail  before  LAN-LINK downloads the mail from the  BBS,  it

will send it for you.

When the BBS has accepted the commands the name of the  file

is  changed  from '*.BBS' to '*.n', where 'n'  is  a  number

increasing sequentially each time the function is performed.

For  example,  the  file name KE8X.BBS will  be  changed  to

KE8X.001  the first time it happens.  In this way, you  will

have  a  copy of the outgoing file on your  disk  until  you

choose to delete it.

In the real world however, message formats are  nonstandard,

and  can  contain  any set  of  characters.   The  algorithm

developed  here  transmits the next line in the  *.BBS  file

when  LAN-LINK sees a 'trigger' character-string unless  one

of the 'inhibitor' character-strings is also present on  the

line of the message text received by LAN-LINK.

These  'trigger'  character-strings  are '>'  (as  the  last

character  of the line of text ), 'Enter', 'Subject',  'Sj:'

and 'Msg:'.  The 'inhibitor' character-strings are 'R:', '<'

and 'Message'(R: as in routing headers and < and Message  as

in Message-Id: <message number> in message headers).

This should cover most simulated manual BBS message  reading

(WA7MBL and W0RLI/VE3GYQ) and avoid false triggering due  to

'>'s showing up in message and routing headers.

NOTE that when LAN-LINK finishes processing the *.BBS  file,

it  will  send  a 'B' command to the BBS  to  terminate  the

session.  This feature is designed to minimize the time  you

spend  connected  to  the BBS by allowing you  to  scan  the

message list when automatically receiving your mail, prepare

your  replies and download requests off-line and then  later

read  the  incoming messages or files without tying  up  the

BBS.  It thus positively disconnects you from the BBS.

In the Packet Communications Mode, the Alt-Z key initiates a

BBS  Mail Snatch.  You use this key if you connect to a  BBS

before you hear its beacon, or if you have disabled the  BBS

Mail Snatch feature.  If you are connected to a BBS, the key

must be used BEFORE the first '>' is received.  Note: it  is

that  first  '>' that triggers the sequence.   If  you  have

already  received that first '>', send the BBS any  command,

and  the sequence should begin when the BBS signals that  it

has  completed  processing  that command.  If  you  are  not

connected,  LAN-LINK will first try to connect to  the  BBS,

then perform the sequence.

When  you use this command you will be asked if you want  to

upload  or download. If you want to download, just push  the

'Enter'  Key.   A Download (Mail from the BBS to  you)  will

automatically  be  followed by an Upload if the  *.BBS  file

exists.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 34

The download sequence performed by the BBS Mail Snatch is as

follows:

RM        To read your messages, (Configurable)

L         To let you know what is new on the BBS,

LM        To list ALL your messages on the BBS,

B         To log you off the BBS.

LAN-LINK will wait for the '>' as the last character of  the

line of text being received from the BBS, before issuing the

next command in the sequence.  Your messages will be  placed

in a message file <yourcall>.OUT as if someone else had left

a message for you.

The 'RM' may be customized in the LAN-LINK.SYS file to  'RN'

or 'VM' depending on the software in your local BBS and your

preference.

During  the  time  that the BBS is  responding  to  the  'L'

command  with a list of new messages, LAN-LINK  is  scanning

the  lines  containing the messages to see if it  finds  one

containing a Scan Word.  If it does it will queue a  request

to download that message.

If  you wish to add a 'KM' to the sequence, place it in  the

BBS.BBS file. If you use 'KM' as you may delete mail  BEFORE

reading it.

If you use the 'D' command to download a text file, the file

will  end up in a separate file on your disk ONLY IF a  file

of the same name does not exist on your drive.

Do  delete 'read' mail from your local BBS.   Don't  clutter

the  BBS's  disk with your read mail. The  'LM'  command  is

built  into  LAN-LINK  to remind you of what  you  have  out

there.

The BBS 'S' command (including its variations such as  'SP')

is the only multiple line command allowed.  You can thus put

all  the single line commands such as D, H, ?, L, and  R  in

the BBS.BBS file.

A  'multiple  command'  is defined in  this  instance  as  a

command which allows you to enter multiple lines before  the

BBS  will send back a '>' character (i.e. one that  the  BBS

tells you to type in text and terminate it with the  Control

Z character).

Each  message must be in the format expected by the BBS  and

must  end with the " :EOF: " character string, but there  is

no need for any special end of BBS file character.

A typical message could be as shown below.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 35

SP G3ZCZ @ N4QQ

LAN-LINK

I  think  it is great. However, why don't ......  (add  your

comments here).

:EOF:  {last line may also be '/EX'}

8.0 Edit Menu

This menu provides the following choices.

B Brag Tape

D Call Directory File

F Any other File

L Logbook

M Answer Incoming Mail

N Note

R Capture to Disk File

S LAN-LINK.SYS

T Two Files

1,2 - 0 LAN-LINK.001 - 010

Type in the single letter to make your choice.

LAN-LINK  contains an ASCII text editor suitable for  files  less

than  64k in size.  It is built based on Borland's  Turbo  Pascal

Editor Toolbox and the commands are compatible with Sidekick  and

Wordstar.  A summary of the editor commands is shown below.

F10      Switch Windows (if more than one window open)

Backspace Delete left char

Del       Delete Char

Enter     New line

Ins       Toggle insert mode

PgUp      Move cursor up one page

PgDn      Move cursor down one page

^A        Move cursor left one word

^C        Move cursor down one page

^D        Move cursor right one character

^E        Move cursor up one line

^F        Move cursor right one word

^G        Delete one character

^H        Delete left one character

^I        Tab

^L        Find/replace next occurrence

^P        Insert a control character into the text

^M        New line

^N        Insert line

^R        Move cursor up one page

^S        Move cursor left one character

^T        Delete one word after cursor

^V        Toggle insert mode

^W        Scroll up

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 36

^X        Move cursor down one line

^Y        Delete line at cursor

^Z        Scroll down

^K^B      Mark beginning of block

^K^C      Copy block to position of cursor

^K^D      Save file and exit edit

^K^H      Hide block marker

^K^K      Mark end of block

^K^Q      Abandon file and exit edit

^K^R      Read file into screen

^K^T      Mark single word as block

^K^V      Move block to position of cursor

^K^W      Write block to disk file

^K^Y      Delete block

^K 1..9   Set marker 1 .. 9

^Q^A      Find text and replace

^Q^B      Move to beginning of block

^Q^C      Move to end of file

^Q^D      Move to right of line

^Q^E      Move to top of window

^Q^F      Find text

^Q^I      Toggle autoindent mode

^Q^K      Move to end of block

^Q^R      Move to top of file

^Q^S      Move to left of line

^Q^X      Move to bottom of window

^Q^Y      Delete to end of line

^Q 1..9   Jump to marker 1..9

The  following options can be set in the Find/find  and  Re#

place operations.

# locates #th occurance

G global replace

N replace without Y/N question

U ignore upper case/lower case

W match whole words only

Notes

1    The ^ key in front of a character identifies that  character

as  a  'control' character.  To activate it, hold  down  the

Control key AND the character key.

2    Some of the commands require two keystrokes.

WHILE  USING THE EDITOR, LAN-LINK can't receive  characters  from

the TNC. All other features ARE INHIBITED OR LOCKED OUT.  THUS IF

SOMEONE CONNECTS TO YOU, THE TNC WILL KNOW BUT LAN-LINK WILL NOT.

BE WARNED !!!

The Edit menu allows you to call up the various files for editing

as follows.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 37

8.1  Edit Brag Tape

This option allows you to create and edit the Brag Tape File

(LAN-LINK.TXT).

8.2  Edit Call Directory file

This option allows you to create and edit the Call Directory

File (LAN-LINK.DIR).

8.3  Edit any other file

This option allows you to create and edit any other file.

8.4  Edit Logbook file

This  option allows you to create and edit the Logbook  File

(LAN-LINK.LOG).

8.5  Answer Incoming Messages (Split Screen)

This  option  allows you to answer incoming  messages  in  a

split  screen  mode. The contents of the  incoming  messages

will  be displayed in the top window, and the reply file  in

the  lower  window.  The reply file is assumed to  be  named

according to the Zap convention (*.BBS file).  You will have

to  set up the replies/outgoing messages in the  form  shown

below.

SP Call @ BBS

Subject of the message

content lines

End the message with either

':EOF:'  , '/ex' or '/EX'

as  the first word on a separate line (without the ').   For

example,

SP G3ZCZ@N4QQ

Test

This is a test

73

/ex

8.6  Leave Note for Someone

In  all  modes  you can leave a  brief  note  (message)  for

another  station.  When you invoke this option, you will  be

prompted for the call of the station that you want to  leave

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 38

a note for.  The date/time header is written to the  message

file, then as you enter the text it is stored on the disk in

the  usual  format.   Use Function key 10  (F10)  to  switch

windows.   At  this time you cannot  transfer  text  between

windows.

After you have left the note and received the 'QSL'  message

from LAN-LINK, if you are in the Packet Communications Mode,

press  Function Key 8 (F8) or Alt-D to load the callsign  of

the recipient into the beacon.

When  you  leave a note, leave it for the root call  of  the

connectee, for example to leave a note for G3ZCZ/W3 leave it

for G3ZCZ, similarly to leave a note for  WB8WKA-1 leave  it

for  WB8WKA.  LAN-LINK is smart enough to extract  the  root

call from any callsign it links/connects with.

8.7  Edit Capture-to-Disk file

This  option  allows you to edit  the  Capture-to-Disk  file

(LAN-LINK.RUN) as long as it is smaller than 64k.

8.8  Edit Two files

This  option  allows you to create and edit any  two  files.

Use  Function Key 10 (F10) to switch windows.  At this  time

you cannot transfer text between windows.

8.9  Edit Configuration file

This option allows you to edit the Configuration File  (LAN-

LINK.SYS).

8.10  Edit LAN-LINK.001 through LAN-LINK.010 files

This option allows you to create and edit the 10 files (LAN-

LINK.001 to LAN-LINK.010).  Use the suffix number to  access

the  respective LAN-LINK.* file, use the number 0 to  access

the LAN-LINK.010 file .

9.0 Event/Clock Menu

LAN-LINK contains some time dependent features which allow you to

schedule  events.  These features are controlled from  this  menu

and are listed below.

A Set Alarm

C Connect/Call

D Display Event

P Periodic Event

R Turn Robot ON/OFF

S Turn ConteSt ON/OFF

T Turn Time Display ON/OFF

X Cancel Event

Z Zap BBS

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 39

Type in the single letter to make your choice.

To set a scheduled event, first select the event, then enter  the

time.   You  may set the event to occur once a day,  or  once  an

hour.  If you wish the event to occur once an hour, enter an hour

value  greater than 23 when asked to enter the hour.   Each  time

the event occurs, the feature will be cleared.

9.1  Set Alarm

This option allows you to set an alarm clock.

9.2  Connect

This  option allows you to set LAN-LINK to issue a  call  or

connect request at some later time.

9.3  Display Event

This option allows you to display the scheduled event.

9.4  Periodic Event

This option allows you to set a periodic connect request  or

call  event.  You can set the start time, the end  time  and

the  time interval between the connect/call  attempts.   The

first  connect  that  goes  through  will  clear  the  event

scheduler.

9.5  Turn Robot ON/OFF

This option allows you to turn the AMTOR/Packet Robot on  or

off at a fixed time.

9.6  Turn Contest ON/OFF

This  option allows you to turn the  AMTOR/Packet  Automatic

Contester  on or off at a fixed time. You will  be  prompted

for the time, a starting QSO number (in case you a  restart#

ing  the contest) and the Contest Text (in case you wish  to

change it).

9.7  Turn Time Display ON/OFF

This option allows you to turn the clock display on or off.

9.8  Cancel Event

This option allows you to cancel a scheduled event.

9.9  Zap BBS

This  option  allows  you  to  schedule  a  BBS  Zap  at   a

predetermined  time (like in the middle of the  night).  You

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 40

may  for example set it to attempt to connect to a BBS at  a

fixed  time or hourly at any desired minute after the  hour.

Should  the  connect be made, the event  scheduler  will  be

cleared and no further connect attempts will be made.

10.0 Files Menu

The Files Menu provides the following choices.

B Send Brag Tape

D File Directory

E Erase File

R Rename File

S Send File

T Send/Talk File

U Send Uppercase File

V View file

Type in the single letter to make your choice.

During  the file transfer, the outgoing text will appear  in  the

OUTwindow.  If the text is long enough and fills the TNC  buffer,

you will see it stop, and then continue when the TNC is ready  to

accept  more text, the message sequence will resume.  NOTE  hard#

ware flow control NOT ^S/^Q software is used.

When you transmit a file (B or S) you are returned to the receive

mode  after the file is sent except in the Packet  Communications

Mode.  In the Packet Communications Mode, when the file has  been

loaded into the TNC the computer will add the ^Z character to the

outgoing text to signal to the recipient that the file  transmis#

sion is over.

The computer will signal you (using the bell character) when  the

file has been loaded into the TNC,  Note that is not the same  as

the  time that the TNC has transmitted the file, because the  TNC

incorporates a character buffer.

10.1  Send Brag Tape

This  command  allows you to transmit a standard  text  file

historically  known as a "Brag Tape".  The default  name  of

the brag tape file is LAN-LINK.TXT, and it should be created

with a word processor in the non-document mode.

10.2  File Directory

This command shows you the directory of the disk drive.

10.3  Erase File

This command allows you to erase or delete a file.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 41

10.4  Rename File

This  command  allows you to rename a file.  You  could  for

example rename a document file to a "<callsign>.OUT" file so

that  someone will receive it automatically next  time  they

connect.

10.5  Send File

This  option  lets  you  transmit  an  ASCII  file.  In  the

non  packet  modes, it will turn the transmitter  off  after

sending the file.

10.6  Send/Talk File

This  option  lets  you  transmit  an  ASCII  file.  In  the

non  packet  modes, it will leave the transmitter  on  after

sending the file.

10.7  Send Uppercase File

This  option  lets you transmit an ASCII file  and  converts

each   line  of  text  in  the  file  to  UPPERCASE   before

transmitting  it.  The file itself is not  changed.  In  the

non  packet  modes, it will turn the transmitter  off  after

sending the file.

10.8  View file

This option lets you view the contents of a file.  Use it if

you must on long files.  On files with less than 64k charac#

ters, use the Edit Menu.

11.0 Help Menu

This option will activate the HELP display. Help displays a brief

summary of the function and hot keys.  This window will remain on

display until a character is typed at the keyboard.

12.0 Jump to DOS Shell

This  option  allows  you to enter a DOS shell.  If  you  do  not

restore  the  system  to the directory  which  contains  LAN-LINK

before you EXIT from the shell, LAN-LINK will do so for you.

You may use  the DOS shell while in QSO when you have loaded  the

computer/TNC output buffers and output will continue to flow.

To  exit from the DOS shell back to LAN-LINK, enter  the  command

'EXIT'.  LAN-LINK  will automatically return you to  the  correct

subdirectory.

13.0 LAN-LINK Menu

The LAN-LINK menu provides different options depending on whether

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 42

you  are  connected  to another station or  not.   The  following

options are available when connected.

B :QBU: External Protocols

M :QBM: File

P :QSP: Message

Q :QDB: File

U :QRU: Messages

The following option is available when not connected.

W Who is "ON-LINE"

Type  in  the  single letter to make your choice.

Each  of  the features (except the :QBU:) may also  be  performed

manually by typing the full sequence of Network Control  Language

(see Appendix 3) character strings.

13.1 The :QBU: Command

This is a Sequence for external operations and only shows up

in  the LAN-LINK Menu when connected. It requires  that  the

batch  file 'QBU-TX.BAT' be present in the  initiating  sta#

tion, and that the batch file 'QBU-RX.BAT' be present in the

receiving  station.  When the QBU sequence is  executed  the

following occurs:-

The transmitting station sends a :QBU:

The receiving station sends a :QRV: and an amount of  memory

left on disk, enters the transparent mode, jumps to dos  and

executes the batch file 'QBU-RX.BAT'.

Upon  receipt  of the :QRV:, the transmitting  station  then

puts  the TNC in the transparent mode and jumps to  Dos  and

executes the batch file 'QBU-TX.BAT'. The batch file  should

exit and return to LAN-LINK. LAN-LINK then exits the  trans#

parent mode.

When  the receiving station batch file finishes and  returns

to  LAN-LINK. LAN-LINK then exits the transparent  mode  and

transmits  a :QSL: if the DOS Error level was 0 and a  :QNO:

error if it wasn't.

At this time both stations are back in the converse mode.

You  may  use this technique for  transmitting  pure  binary

files  using  external (to LAN-LINK)  protocols).   You  may

change  the  default names of the batch files  in  the  LAN-

LINK.SYS configuration file.

13.2 :QBM: File

You use this option to download a file from another LAN-LINK

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 43

station.    It  is  the  same  as  manually  typing   ':QBM:

FILENAME'.

13.3 :QSP: Message

You  use this option to store a message on another  LAN-LINK

station.  It is the same as manually typing

':QSP: <callsign>'

LAN-LINK contains store and forward routines for messages in

a Local Area Network (LAN) as described in Appendix 2.

When connected to another LAN-LINK station, you may use  the

NC/L :QSP: command to leave a message at that station.  This

QSP  command  automates that feature for you.  It  lets  you

just  enter the callsign of the station you wish to leave  a

message  for.   It will then send the  text  sequence  :QSP:

<callsign>.   The  other LAN-LINK HOST station  will  either

reply  :QNO: followed by a number or :QRV: <callsign>.   The

'QNO'  message means that the other computer  cannot  accept

the message, the :QRV: message means that it is waiting  for

you to send it.

If  you get the :QRV: reply, go ahead and type the  message,

or  send the text file containing the message.  If you  type

the  message,  use a control Z (^Z) character  and  carriage

return  to terminate the message.  (In AMTOR, LAN-LINK  will

translate the ^Z to :EOF: +? to terminate the message).  You

will  then  receive either a :QSL: from  the  other  station

meaning that the message is accepted, or a :QNO: followed by

a message that something went wrong and the message was  not

stored.

13.4 :QDB: File

You  use  this option to upload a file to  another  LAN-LINK

station.  It is the same as manually typing

':QDB: FILENAME'.

This option saves you typing the file name twice.

13.5 :QRU: Messages

The  NC/L  QRU  command allows you to  upload  a  number  of

messages  to  the other station.  You use it when  you  have

messages  pending on your system and want to pass them to  a

third  computer for storage in the LAN, while you take  your

computer off-line.

You  use  this  option  to begin  the  upload   sequence  to

transfer  messages for a number of stations to another  LAN-

LINK station.  It is the same as manually typing :QRU:.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 44

You  can only use this option with a station which you  have

previously  designated as a store and forward mail box,  for

specific  stations, namely one for which you have set  up  a

<callsign>.MBX  file.  This command may be triggered by  you

using this choice or by the connecting station sending you a

:QRU: command.

When the sequence is over, the computer sending the messages

will transmit the NC/L command word :QJG: to signal that the

sequence is complete.  Should you receive it before you  get

any messages, then there weren't any pending.

If  you  want  to download messages  from  another  LAN-LINK

system,  connect to it and type the sequence :QRU:  followed

by the 'Enter' character.  That system will then respond  to

you as if the operator there had initiated a QRU command.

Appendix 2 contains more information on the LAN capabilities

of LAN-LINK.

13.6 Who is "ON-LINE"

You  use this option to see who is 'ON-LINE' on the LAN.  It

is  the  same as going manually into the Converse  Mode  and

transmitting an unconnected packet containing the  character

string :QRA:.

14.0 Log Menu

The Log Menu provides the following choices.

C Change Logbook File

F Show last Few Entries

L Log Entry

P Print Log

S Scan Log

X eXport Log

Z Clean Up Log

Type in the single letter to make your choice.

14.1 Change Logbook File

This  option lets you temporarily change the  logbook  file.

If you want to change it permanently, change it in the  LAN-

LINK.SYS file.

14.2 Show last Few Entries

This  command displays the last few log entries made  during

the  active  session. If no QSO's have been made  since  the

program was loaded the display window will be blank.  Up  to

5 log entries will be displayed in the window.  You use this

command for a quick look at which stations have connected to

you most recently in your absence.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 45

14.3 Log Entry

LAN-LINK  performs automatic logbook entries for Packet  and

AMTOR  Beacon  mode  Connects,  and  semiautomatic   logbook

entries for other modes.  The Log file is compatible to that

of  the CONTEST program in the G3ZCZ PC-HAM package and  can

be  processed  by  the DBASE  Logbook  Package  for  indexed

listings,  tracking of DXCC and other AWARDS, etc.  This  is

ideal  for special event stations and DX-peditions, for  the

whole QSL process and further statistical analyses of  their

operation.

For  this  feature to work, the file  LAN-LINK.LOG  must  be

present  on the disk.  If it isn't, it will be  created  for

you.  After one logging operation has occurred, edit out the

initial  text.   You  must perform  this  editing  operation

before  appending  the contents of the logbook file  to  the

logbook  database of the PC-HAM LOGBOOK Package.   Edit  the

log  before appending it to the DBASE package, as there  may

be some null characters at the end of the file which may  be

recognized as a blank entry by DBASE.

It is also advisable not to let the logbook file get too big

as you can only use the editor on files smaller than 64K.

When  an  AMTOR beacon mode or packet  connect  occurs,  the

connect  is automatically logged.  In the Packet  Mode,  the

connect  path  is  placed in the  comments  section  of  the

logbook entry.  In the non packet modes, the log data  entry

is  set up for you with default 599 values for  the  reports

exchanged.

You invoke this option to enter the data into the log or set

the  band and power for subsequent automatic logging.   When

you invoke the command you will be presented with a  typical

menu in the menu window and a current entry line in the  log

window.

You  should choose the character associated with each  item.

For  example,  if  you want to change the  Band,  enter  the

letter  'B', to change the Comments, enter the  letter  'O'.

Enter  the text when prompted.  You will then  be  presented

with an updated entry.

To  enter  the data into the logbook, use the 'L'  key.   To

save  the  data  but  not  make a  log  entry  use  the  '*'

character,  for  example you would do that if  you  want  to

change the band or power information.

You  may use the 'QSL sent' entry to tag the fact  that  you

have written out a QSL card.

To  Flag  an entry for Deletion, the 'F' option  places  the

'/*'  character pair in the callsign position of  the  entry

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 46

and  writes it to disk.  The logbook is a  sequential  file,

and there is no easy way to delete an entry once written  to

disk.  This technique flags the bad entry, and the clean  up

option  described below (par. 14.7) is used to clean up  the

log.   Do not place the '/*' character pair in the  comments

field.

The  band and power information are set up from the  default

values stored in the configuration file LAN-LINK.SYS.   They

are  displayed in the status window.  If you are  using  the

logging  command,  you  should  change  them  when  changing

bands/powers  and  exit  the logging command  with  the  '*'

character  so  that  you don't put a blank  entry  into  the

logbook.

You  can also use this command to enter the odd SSB  contact

into the logbook file.  If you do, use the characters  'SSB'

or 'FM' for those voice modes to remain compatible with  the

rest of the DBASE logbook package.

Do  not use the comma character ',' in the  comments  field.

If you examine the logbook file you will see that all  items

are  separated by comma's.  If you use a comma character  in

the comments you will screw up the transfer of data into the

logbook package.

The   comments  field  in  the  LAN-LINK  logbook  file   is

restricted  to a maximum of 15 characters.  If you  try  and

enter more than 15, the surplus will be ignored.

14.4 Print Log

This command prints the contents of the LAN-LINK.LOG file in

a formatted manner to the printer. It requests the  callsign

of  the  log  entry to be displayed. If you  want  to  print

entries for particular calls or parts of calls, enter  those

calls or parts of calls.  Valid entries are G, G3, G3Z  etc.

If  you  want  to print every entry in  the  log  enter  the

asterisk '*' character.

14.5 Scan Log

This command displays the contents of the LAN-LINK.LOG  file

in  a  formatted  manner on the  screen.   It  requests  the

callsign  of the log entry to be displayed. If you  want  to

see  entries  for particular calls or parts of  call,  enter

those  call or parts of calls. Valid entries are G, G3,  G3Z

etc.  If  you want to see every entry in the log  enter  the

asterisk '*' character.

14.6 Export Log

This command prompts you for the name of the file to  export

contents  of the LAN-LINK.LOG file to. It then  exports  the

contents of the LAN-LINK.LOG file to an ASCII file delimited

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 47

with  double quotation marks (") suitable for  loading  into

many data base programs.  It also deletes the first line  of

LAN-LINK.LOG files (if it is the sign on line).

If  you are using the G3ZCZ PC-HAM LOGBOOK Package, use  the

'V'  option  in the Miscellaneous Menu of the  G3ZCZ  PC-HAM

LOGBOOK to import the exported log. The G3ZCZ PC-HAM LOGBOOK

Package  assumes  that the log to be imported has  the  file

type  ".LOG", so it is recommended that you change the  name

of the exported log file to TEMP.LOG.

The  default  structure of the blank  G3ZCZ  PC-HAM  LOGBOOK

Package database logbook is as shown below.

Field  Field Name  Type      Length    Dec

1  DATE        Character      8

2  TIME        Character      4

3  BAND        Character      3

4  CALL        Character     10

5  RX          Character      3

6  TX          Character      3

7  MODE        Character      4

8  POWER       Character      4

9  QSLSENT     Character      1

10  QSLRX       Character      1

11  COMMENTS    Character     10

-----------------------------------------------

** Total **                      52

The size of the COMMENTS field is a compromise.  Each  field

is  a  fixed  size  so  that each  log  entry  takes  up  52

characters  worth  of  space on the disk  even  if  all  the

character  space is unused. You may change the size  of  the

field if you wish in dBASE.

14.7 Clean Up Log

This   option  searches  through  the  logbook   for   lines

containing  the '/*' character pair.  Should it find such  a

line  it, deletes both that line and the previous line.   At

the  end of the operation, it displays the total  number  of

lines read, and the number of entries deleted (the line  and

the '/*').  The logbook file is renamed to 'LOGBOOK.BAK' and

the  cleaned up logbook takes its place.  You can  also  use

the included Editor to perform this function if you wish.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 48

15.0 Message Menu

The Message Menu provides the following choices.

B BBS Command Files

C Current Messages

M scan Message

N leave Note

O Old Messages

T Time tag

Type in the single letter to make your choice.

15.1 BBS Command Files

This option shows you a list of the BBS command files on the

default directory.  In other words, all files with the  file

extension of '.BBS'.

15.2 Current Messages

This  option shows you a list of the outgoing messages  that

have not been sent. In other words, all files with the  file

extension of '.OUT'.

15.3 Scan Message

This  feature allows you to scan messages in  the  computer.

After  depressing the key you will be prompted for INcoming

or  OUTgoing (I/O) messages.  If you request  Incoming  mes#

sages you will see the messages addressed to your  callsign.

After  scanning your messages,  the file will be renamed  to

'<yourcall>.n' where 'n' is an incrementing number from 1 to

999.

15.4 Leave Note

This feature allows you to leave a note for another  station

on the LAN in your system.  It works in the manner described

in Section 8.6.

15.5 Old Messages

This feature shows you a list of the messages that have been

sent.  In other words, all files with the file extension  of

'.OLD'.

15.6 Time Tag

This option is used to place a date time header on a message

file  previously prepared by a word processor.  The name  of

the file must be in the format 'CALLSIGN.OUT' (For  example,

G3ZCZ.OUT).   It requests the callsign of the message  file.

It then opens a new file, writes the header to the new  file

and copies the contents of the original '*.OUT' file to  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 49

new one.  When all is done, the original filename is changed

to '*.BAK' and the new one becomes '*.OUT'.

16.0 TNC Menu

The TNC Menu provides the following choices.

D Set TNC Date/Time

F Flush TNC Buffer

I Initialize TNC

M calls Monitored List

S Show Connect Status

Q Change CQ Text

X Change CTEXT

Type in the single letter to make your choice.

16.1 Set TNC Date/Time

This  feature puts the TNC into the Command Mode  and  loads

the date into it from the computer time of day clock.

This command does not work on the TNC1.

16.2 Flush TNC Buffer

This  option  is used to flush the program's  outgoing  text

buffer and that of the PK-232.  When used with other TNCs it

will only flush contents (if any) of the outgoing buffer in

the PC.

16.3 Initialize TNC

This feature sets up the defaults for the TNC from the  file

LAN-LINK.SYS.   If  you have put batteries in the  TNC,  you

should normally only have to run this command when you first

power  up  the TNC or change the  batteries.   However,  the

batteries must be removed initially since they are there  to

retain the default settings of the TNC (particularly the PK-

232).  Once the initialization is complete, you can  replace

the  batteries but leave the TNC on since the  battery  pro#

vides power to the RAMs after the TNC power is turned off.

It  will also set the date and time within the TNC from  the

computer  clock, and, when it has loaded  the  configuration

file,  may prompt you to set the TNC Terminal mode.  If  you

are  using a TNC1 or a TNC2, it will not prompt you  because

it assumes that they only operate on VHF packet. If you  are

using a KPC-2, you may select 'H' or 'V', and of course,  if

you have a PK-232, you may chose any of the options.

16.4 Show Calls Monitored List

This  feature performs the same operation as Function Key  5

(F5).

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 50

16.5 Show Connect Status

When you are in the Packet Communications Mode, this command

lets you see the state of the connection.  It will show

you if there are any unacknowledged packets en route to  the

other station.  If you have been sending text, it is advisa#

ble  to  use this command before you disconnect,  to  ensure

that all the data has been received and acknowledged by  the

other  party.   You may use this command in either  the  TNC

Command or the TNC Converse Mode.

In a multiconnect situation, it will show you the status  on

all the I/O channels.

You  may  also use this command to reset the  status  window

display  after  a relink or any other event  which  was  not

reflected in the status window.

16.6 Change CTEXT

This  feature allows you to change the message that will  be

received by another packet station when it connects to  you.

Refer to the 'CTEXT' command in the TNC manual.

17.0 AMSAT-OSCAR Menu

This menu allows you to configure the KAM, MFJ1278 or the  PK-232

to  copy signals from AMSAT-OSCAR Spacecraft  telemetry  beacons.

The following options are available.

A UoSAT ASCII Beacon

B Phase 3 RTTY Beacon

C SAREX Menu (only if you are in the Packet  Communica#

tions Mode)

Type in the single letter to make your choice.

17.1 UoSAT ASCII Beacon

This  option is only available on the PK-232. It  configures

the  PK-232 for receiving UoSAT 1200 Baud ASCII  signals  in

the following manner.

ASCII

AB 1200

VHF ON         (Bell 202 tones)

RXR OFF

TXR OFF

WI ON          (850 Hz shift)

Signals from UoSAT-OSCAR 11 may be copied in this configura#

tion.  Note that although UoSAT-OSCAR 11 transmits Bell  202

compatible  tones, the data sense is reversed.   A  hardware

modification is required in the PK-232 to reverse  the sense

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 51

of  the  data at the modem output BEFORE it  passes  to  the

digital section of the unit.

17.2 Phase 3 RTTY Beacon

This  option configures the KAM, the MFJ1278 and the  PK-232

for  receiving  the  AMSAT-OSCAR 10 and  13  RTTY  telemetry

beacons in the following manner.

BAUDOT

WI OFF         (170 Hz shift)    [ KAM : SH 170]

TXR OFF

RXR OFF

RB 50          (50 baud)

USOS OFF       (Unshift on space off)

17.3 SAREX Menu

This menu is only active in the Packet Communications  Mode.

It is designed for use with the Shuttle Amateur Radio Exper#

iment  (SAREX) and the packet radio MicroSats.   Apart  from

the  Attack  Mode, it can also be used as a  diagnostic  LAN

tool for monitoring LAN traffic.

17.3.1 Attack Mode ON/OFF

If the Attack Mode is set, this option will cause  LAN-

LINK  to  issue  a connect request to  the  SAREX  Call

whenever  a  packet sent to or from it is heard.   The

mode is cleared when the connect is made (and does  not

retry out) or when the 'A' option is selected a  second

time, or when the Alert Call is cleared.  If this  mode

is  enabled, the Alert/SAREX Call prefix shown  in  the

Status Window will indicate accordingly.

A  happy face will be displayed next to the '>'  before

the call once the connect has been achieved.

Be careful using this feature, as it has the  potential

to  cause a great deal of QRM.  It can also be  cleared

by another station connecting to you and telling you to

':QRT:'.

17.3.2 Change SAREX Call

This option lets you temporarily change the SAREX call.

17.3.3 SAREX Packets Capture to Disk ON/OFF

This option allows you to configure LAN-LINK to capture

to  disk  any packets addressed to or  from  the  SAREX

callsign.  A [S] will be displayed in the status window

when  this option is active.  The capture to disk  file

is opened by a packet header containing the SAREX call,

and closed by another packet header not containing  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 52

call.   When  the  file is open, the  [S]  will  blink.

Packet  headers are considered to be lines with  a  '>'

character in them.  LAN-LINK thus considers both of the

lines below as packet headers.

N4QQ*>G3ZCZ

N4QQ BBS>

If you use this feature for monitoring BBS traffic, the

BBS   prompt lines ('E.G. N4QQ BBS>') will not be  cap#

tured to disk.  Do not get your SYSOP to remove the '>'

from the prompt line, or the ZAP feature will then  not

work.

A  happy  face will be displayed in the [S]  after  the

SAREX call has been heard.

If you want to see an ORBITER menu, temporarily  change

the SAREX call to be the same as your call and bring up

the AMSAT-OSCAR Menu.

17.4 SAREX Orbiter Features

You  need the ROBOT EPROM for this menu to actually do  any#

thing.  The  EPROM may be available from The  Radio  Amateur

Satellite Corporation (AMSAT) for DX-pedition use.

In  the Orbiter mode, LAN-LINK does not scan  incoming  text

for  the  'CONNECTED' and 'DISCONNECTED' messages  when  the

SAREX  Robot  is active.  This inhibits the  bells  and  the

capture-to-disk.   It also means that the Robot must be  OFF

for any :Q: code uploads or downloads to occur.

17.4.1 Status Window Display

A flashing '>>' appears in the status window when the  robot

is  active.  The SAREX Configuration boots up assuming  that

the SAREX Robot is active (Default).

17.4.2 Alt-O Function key.

The Alt-O Function Key brings up the Orbiter menu.  This  is

a faster route than the  'Esc O' sequence.

17.4.3 The Meta Beacon

The  Meta beacon is loaded from the contents of one  of  the

META.00n files.  These files may be created by the  LAN-LINK

editor.  When  loading, LAN-LINK will insert a pass  charac#

ter  (^V) into the text before a carriage return, so as  not

to  end the packet before at least 240 characters,  and  not

more  than 255 characters have been output to the TNC.   The

maximum  number  of characters that can be loaded  into  the

Meta  beacon  is 1776.  The maximum number of packets  in  a

Meta  beacon is 7.  You can also end force a  hard  carriage

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 53

return  or 'end of packet' in the Meta beacon by  using  the

'\' (backslash) character, which will not be transmitted.

17.4.4 SAREX Orbiter Menu

The following commands are active in the Orbiter Menu.

17.4.4.1 Activates Beacons/SAREX Robot.

This  option activates the SAREX Robot and  loads  Meta

Beacon.   This  command is designed for  use  when  the

SAREX is returned to the regular frequency.

17.4.4.2 Set QRZ/QSL Beacon Interval

This  command  allows you to change the  beacon  timer.

The default value is 120.

17.4.4.3 Set SAREX Call.

This  command  allows  you to change  the  SAREX  call.

Don't  use  this command on orbit (its for  ground  use

only)

17.4.4.4 Turn SAREX ROBOT OFF

The  Robot must be turned On and Off this way  to  syn#

chronize LAN-LINK to the TNC.  When the Robot is turned

OFF the following commands are sent to the TNC by  LAN-

LINK.

USERS 1

|A

RO OFF

CMSG OFF

NO OFF

17.4.4.5  Set SAREX ROBOT Heard Count

This command lets you set the Heard Count number.

17.4.4.6 Show I/O Status

Shows who is on each stream, the SAREX Robot state, the

number of USERS, the beacon timer value, the metabeacon

timer value and the CTEXT.

17.4.4.7 ChecK Count

Shows both heard and worked counts.

17.4.4.8 List Heard Calls

Lists calls of the last few stations heard.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 54

17.4.4.9 Load Metabeacon

This command loads the Metabeacon.  You will be prompt#

ed  for  a  text file number in  the  META.00n  series.

Enter a number 1-9 corresponding to  META.001-META.009.

The  0  value is file META.010. Note the  delays  after

loading the file to allow the TNC to stabilize.

17.4.4.10 Show Worked Calls

Shows the last 35 or so calls worked.

17.4.4.11 Turn SAREX ROBOT ON

The SAREX Robot must be turned On and Off via the menu.

When the Robot is turned ON the following commands  are

sent to the TNC by LAN-LINK.

USERS 9

CONOK ON

CMSG ON

RO ON

NO ON

17.4.4.12 Stop Metabeacon

This  command stops the Meta beacon.   When  activated,

the  contents of the beacon are dumped to  the  UNPROTO

address  (default  SAREX)  by the TNC,  rather  like  a

regular  BBS  dumping packets when a  user  disconnects

abnormally.

17.4.4.13 Set Metabeacon Interval

This  command allows you to set the Meta Beacon  timer.

It  will be loaded into the TNC the next time that  the

Meta Beacon is loaded.  Even though loaded into the TNC

immediately, it does not take effect till then.

17.4.4.14 Set SAREX ROBOT Worked Count

Lets you set the number.

17.4.4.15 Set Connect TeXt

This is the text (CTEXT) that will be transmitted to an

incoming   station  when the ROBOT mode is  off.   This

option lets you set up a new text string to go with the

QSO number.

17.4.4.16 Selects Silent Mode

This command turns off the beacons, the SAREX Robot and

also  stops the Meta beacon.  This command is  designed

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 55

to be used just before any prearranged packet activity.

17.4.4.17 Restart TNC

This command RESTARTs the TNC, loads the date from  the

computer's  clock and turns the 'EC OFF'.  It seems  to

have  no  effect on the SAREX ROBOT and  counters,  but

does kill the Meta beacon.

17.4.4.18 Capture QSL/QRZ Calls

This  command initiates the log-to-disk  operation.  It

turns the capture-to-disk 'on' and sends the  following

commands  to the TNC :- 'DA', 'SERW',  'SERH','MW',MH'.

When  the calls have stopped  scrolling,  use  Function

key 1 to turn the capture-to-disk off.

18.0 Parameter Menu

The Parameter Menu provides the following choices.

B Turn Beacon Mode ON/OFF      1 Change BBS Parameters

C Clear Connect Count          2 Change Callsigns

D Turn Digipeat Detect ON/OFF  3 Change Directories

E Turn Command Echo ON/OFF     4 Enter Common Calls

I Turn Beacon ON/OFF           5 Change File Names

K Change Keyboard Beep Count   6 Change LAN-LINK Parameters

L Turn Ctdsk ON/OFF            7 Change PC Communications Params

M Set Message Count            8 Change Colors

N Turn NODE Drop Link ON/OFF   9 Change SAREX Parameters

P Turn Printer ON/OFF

Q Turn QTC Snatch ON/OFF       U Update LAN-LINK.SYS

R Turn :QSP: Relay Flag ON/OFF V Verify Changes

S Turn Sound ON/OFF            W Change Scan Words

T Turn Contest Mode ON/OFF

X Turn DCD Flag ON/OFF

Y Turn BBS Snatch ON/OFF

Z Turn Time Id. ON/OFF         + Turn Misc. Flag ON/OFF

The colors used for the option letter differentiate between those

options that are "remembered" in the SYS file and those that  are

only set temporarily for the session.

Type in the single letter to make your choice.

18.1 Turn Beacon Mode ON/OFF

This command toggles the beacon option for the automatic  CQ

caller  in the AMTOR and Packet Communications Modes.   When

it  is active a [B] will be displayed in the status  window.

In  this mode, the Automatic CQ sequence will  resume  after

the  AMTOR link is dropped or a Packet  Communications  Mode

disconnect.

In  the  Beacon Mode, the last few logbook entries  will  be

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 56

displayed in the logpage window which overlays the  outgoing

text window at the bottom of the screen.

In the Packet Mode, the CTEXT message is the only  transmis#

sion from you that the other station will receive,      so make

it good and don't forget to set 'CMSG ON'.

In  the AMTOR Mailbox/Beacon Mode, the contents of the  LAN-

LINK.TXT file will be transmitted to the station who  called

you.   The capture-to-disk will then be turned on to  record

one  transmission from that station.  The QSO will  then  be

logged  to  the  logbook file.  Don't forget  to  check  the

entries  in  the LAN-LINK.LOG and text in  the  LAN-LINK.RUN

files in case the computer made a mistake in recognizing the

call sign of the other station.

THE  BEACON MODE IS DESIGNED TO ENCOURAGE PEOPLE TO QSY  OFF

THE  CROWDED PACKET HF CHANNELS. DO NOT USE IT ON A  CROWDED

HF CHANNEL OR ON AN ACTIVE VHF CHANNEL.  IF YOU DO, YOU WILL

SOON FIND THAT OTHER STATIONS HAVE SET THEIR TNCS TO  REJECT

YOUR  PACKETS, AND YOU WILL NOT WORK ANYBODY, YOU WILL  JUST

BE A SOURCE OF QRM AND WILL BE KNOWN AS SUCH.

THE  MODE IS ALSO DESIGNED FOR ROBOT BEACONS FOR THE  HIGHER

FREQUENCIES.  FOR EXAMPLE IF YOU LEAVE IT RUNNING ON 6 OR 10

METERS YOU NEVER KNOW WHEN A SHORT OPENING WILL ALLOW A  QSO

TO  SOME  PART OF THE WORLD,  A DX CONTACT  YOU  WOULD  HAVE

OTHERWISE  MISSED.    DO NOT LEAVE IT RUNNING ON  20  METERS

WHEN THE BAND IS OPEN.  FURTHER APPLICATIONS ARE IN  METEOR#

SCATTER AND OTHER "EXOTIC" MODES OF COMMUNICATIONS.

If you are using this Mode in a special event station or DX-

pedition  you may type characters into the transmit  buffer,

while the other station is sending to you.  When he, she  or

even  it  turns  it back to you,  you  will  transmit  those

characters before the sign-off sequence.

In both the Packet and AMTOR communications modes, the robot

may  be shut down by stations connecting to you and  telling

you to QRT.

18.2 Clear Connect Count

You  use  this option to reset the PACKET and  AMTOR  Beacon

Mode  "connect  has taken place indicator"  and  QTC  Snatch

count.   These  are  flashing [C] and [Q]  displays  in  the

status  window.  Whenever someone else connects or links  up

to  you  the [C] display will be  incremented.   The  number

shown with the 'C' is the number of connects that have taken

place since the Indicator was last reset.  If a 'happy face'

is  flashing  along  with the connect  count,  your  "target

station"  was  amongst those connecting. The  [Q]  indicator

alerts  you  to the fact that LAN-LINK tried to  connect  to

another  station  after  recognizing your  call  in  a  mail

beacon.   Both  are  reset  by  this  option.   If  you  are

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 57

monitoring  Packet traffic on a channel, you may not  notice

that a connect has previously taken place.  These indicators

alert  you to check the contents of the Incoming  mail  file

('yourcall.OUT') for any incoming messages.

If the 'Q' count is much greater than the 'C' count you  may

be  copying a beacon from a station that you cannot  connect

with directly.

18.3 Turn Digipeat Detect ON/OFF

This option toggles the digipeat display command on and off.

If  the Digipeat Display is active, any  signals  digipeated

through  you will be stored to the LAN-LINK.RUN file  and  a

'beep'  will  be sounded for each packet  being  digipeated.

The status display window will also contain a [D].

18.4 Turn Beacon Control ON/OFF

This  option toggles the LAN-LINK Handshake  features.  When

they  are on (default), the Connect handshake message

[ZCZ] *** LAN-LINK 1.56>

will  be transmitted on incoming connects. The  beacon  text

and time will be controlled by LAN-LINK as a function of the

messages you have posted on your system.  When they are off,

LAN-LINK  does not do anything to your beacon, nor  does  it

transmit the handshake.

18.5 Turn Command Echo ON/OFF

This option toggles the 'Command ECHO ON/OFF' flag.  If  the

Command  Echo  flag is set, the commands and  (contents  of)

files  that LAN-LINK sends to the TNC will be  displayed  in

the  outgoing  window.  Set this flag to  see  how  LAN-LINK

commands the TNC in the different modes.

18.6 Change Keyboard Beep Count

LAN-LINK  counts the number of characters entered on a  line

at  the  keyboard.  When it gets to a preset value  it  will

beep, signaling you to start a new line. This option changes

the value that the keyboard beep counter is compared with to

sound the audio signal.

18.7 Set Message Count

As  an  alternative to Function Key 8 (F8) or  Alt-D,  using

this  option  will  also load a QTC 'Mail'  list  into  your

Packet  Communications  Mode Beacon and  initiate  a  beacon

transmission every 30 minutes (Refer to the BTEXT command in

the  TNC manual).  If no messages are present, or  the  only

message   is   to  you  (incoming   or   snatched),   beacon

transmissions   are  inhibited.   This  conforms   to   good

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 58

operating practice on crowded Packet channels.

18.8 Turn NODE Drop Link ON/OFF

When connecting via NET/ROM, the normal way of operating  is

to  disconnect  if  the  connect  attempt  fails.   You  may

configure the NET/ROM connect attempt flag to stay connected

at the node where the failure occurred.  Use the normal mode

when  trying  to get through on a known path, and  the  stay

alive mode when exploring NET/ROM circuits.

18.9 Turn Printer ON/OFF

This  option  toggles  the printer on  and  off.   When  the

printer  is turned on, any incoming data will be printed  in

real  time.  If you intend to print files received  at  1200

baud in the Packet Communications  Mode, use a print spooler

or  you may lose data.  The printer is automatically  turned

off in the Packet Mode when a disconnect occurs.

18.10 Turn QTC Snatch ON/OFF

This option toggles the QTC Snatch flag on and off.  If  the

QTC  Snatch flag is enabled, LAN-LINK scans the contents  of

packets  appearing in the Inwindow on the screen  while  the

terminal  is  disconnected.  Should it recognize  the  three

text  strings 'LAN-LINK',  ':QTC:', and 'your  callsign'  in

the  contents  of  a packet, it will then  issue  a  connect

request  to  the  callsign  of  the  LAN-LINK  station  that

originated the beacon transmission.  If the connect is made,

the  other station will send you the  message  automatically

which  LAN-LINK  will "capture to disk".  The  connect  will

time out and terminate a few minutes later.

The  normal  connect  request will be  made  DIRECT  to  the

station.  If you expect messages from stations you can  only

connect  with via a digipeater, place the path in your  LAN-

LINK.DIR  directory file.  It should be there anyhow if  you

connect  to  the  station regularly.  For  example,  if  you

expect  messages  from  4Z4ZB  whom you  can  only  work  if

digipeated via 4X1AA then place an entry like

4Z4ZB   4Z4ZB V 4X1AA

in the LAN-LINK.DIR  file.

The  QTC  Snatch  leaves your messages  in  a  message  file

<yourcall>.OUT.   It  is  triggered by the first  >  in  the

initial  connect response of LAN-LINK 1.55R> (note the  '>')

and  terminated by the 'over' at the end of the message  ea.

G3ZCZ/W8>  (note the '>'). If you train stations  not  using

LAN-LINK who want to leave you messages to trigger your QTC-

Snatch, by transmitting the following unaddressed packet

LAN-LINK :QTC: <yourcall>

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 59

your  system will connect to them.  (For example to leave  a

message for me, they would send

LAN-LINK :QTC: G3ZCZ

as  an  unaddressed  packet  [in  Converse  Mode  when   not

connected, or in their beacon text].)

If they then send you a '>' on a line by itself, followed by

the  message  itself, and terminated by another '>'  as  the

last  character  of the line, the message will end up  in  a

message  file rather than in the LAN-LINK.RUN file  and  you

can  scan it fast using Alt-M (Message Scan  Menu  Options).

It  goes without saying that the message itself  should  not

contain  the  '>' character.  If it does, by  the  way,  the

first part of the message will be in the message file, while

the remainder will be in the LAN-LINK.RUN file.

18.11 Turn :QSP: Relay Flag ON/OFF

This  option  toggles the Store and  forward  Message  Relay

Capability of your system on and off.  If the Message  Relay

Capability is active (toggled on), any one who connects  can

store a message in your system using the ':QSP:  <callsign>'

NC/L command. If the Message Relay Capability is not  active

anyone  who  attempts to ':QSP: a message'  will  receive  a

':QNO:  1' "error" message.  The status display window  will

show a [R] when the Message Relay Capability is active.  The

default  state  for the Message Relay Capability  is  loaded

from the LAN-LINK.SYS file.

When the Store and forward capability is active, the  suffix

'R' will be added to the

'[ZCZ] *** LAN-LINK 1.56' ie., '[ZCZ] *** LAN-LINK 1.56R'

message  received  by anyone connecting to  your  system  to

notify  them that they can store messages.  In  this  manner

they  don't  have  to try to leave a message  and  then  get

':QNO:'  error  message  to find out  that  the  feature  is

toggled off.

Remember,  if  you  configure your  computer  to  store  and

forward  messages,  you have a moral obligation  to  deliver

them.

18.12 Turn Sound ON/OFF

This  option toggles the sound on and off.  If the sound  is

off,  a flashing indicator will be displayed in  the  Status

Window.   When the sound is toggled on again, a  chime  will

sound to verify the audio.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 60

18.13 Turn Contest Mode ON/OFF

This option first toggles the Contest Mode.  If the  Contest

Mode  is active, the word 'TEST' and the number of the  next

QSO will be displayed in the status window. When the Contest

Mode is toggled on, this command then prompts you to set the

starting  QSO number and the standard message that  will  be

transmitted  as  part of the exchange of messages  during  a

contest.   The QSO number will be incremented each time  you

log a contact.

There are two special character sequences that you can enter

in  the  contest text string.  If you enter   a  double  '#'

sign, as in '##', it will be replaced by the QSO number when

the text is transmitted.  You may also use '#1' to represent

the callsign of the station being worked in the contest text

string.   For  example, use the following  entries  for  the

following contests;

'##1 UR ##A' ... in the Sweepstakes,

'5905'  ... in the WAZ,

'59 MD' ... in the ARRL DX, and

'59 ##' ... in the WPX.

As  an example, if I (W3/G3ZCZ) was in the Sweepstakes  con#

test,  and my Contest text line was,

'##1 UR ##A W3/G3ZCZ 68 MDC'.

If  the next QSO number is 345, and I am working KW3C,  LAN-

LINK  would  transmit the following line when  I  touch  the

'End' key;

'KW3C UR 345A W3/G3ZCZ 68 MDC'.

The contest feature is toggled off the next time you  select

this feature.

Note  that  while  this does  give  you  some  semiautomatic

contest  operation  by  using  command  keys,  there  is  no

checking of callsigns for duplicates built into the program.

You may do that by using the Alt-S hot key.

In  a contest, you enter the callsigns  and call  the  other

station  in  the normal manner, then use the  'End'  Key  to

transmit the exchange (QSO number and standard text).   LAN-

LINK will only transmit the contest text in this manner.  If

you  want to add a 'K' or 'KN', you will need to put  it  in

the  contest  text string.  It is also advisable  to  use  a

different  log file for each contest to cut down the  search

time.   The contest QSO number increments each time you  log

the QSO (Alt-L).

You  may  put  the Contest Mode into automatic  if  you  are

operating AMTOR or packet.  To do so, use the Alt-O function

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 61

key.

18.14 Turn DCD Flag ON/OFF

If you are using a KAM or a PK-232 then this flag should  be

set to ON.  If you are using another TNC, if you can  jumper

the TNC so that pin 8 on the RS-232 interface changes  state

when  connected,  do  so and set this flag to  ON.   If  you

can't, then set this flag to OFF.  If this flag is OFF,  the

multiple  connect  modes will not work so don't  attempt  to

invoke them.  If this flag is OFF, the words '*** CONNECTED'

and  '***  DISCONNECTED'  when seen in  the  LAN  may  cause

spurious apparent connects and disconnects in LAN-LINK.

18.15 Turn BBS Snatch ON/OFF

If the BBS Mail Snatch is enabled, when a BBS sends a packet

announcing 'Mail', if your callsign is in the first line  of

that  message, LAN-LINK will issue a connect request  (again

via the path in LAN-LINK.DIR) to that BBS.  When  connected,

it will act as if you have initiated a ZapBBS command.   The

Zap counter will be incremented each time a  BBS Mail Snatch

takes place.

A  typical KAM display which triggers a BBS Mail Snatch  for

any of the calls listed below follows.

N4QQ>MAIL/V [09/03/89  18:49:24]:

LstMsg# 19428 Mail: N4FG KL7VZ WA6IEL N7DVA W4TNX G3ZCZ

Note if the BBS Mail beacon has an extra line in it (between

the  header and the list of calls) as follows, the BBS  Mail

snatch will not work.

N4QQ>MAIL/V [09/03/89  18:49:24]:

BBS Serving Silver Spring Md.

LstMsg# 19428 Mail: N4FG KL7VZ WA6IEL N7DVA W4TNX G3ZCZ

If a message is posted for any call containing the character

string  "N4FG" such as N4FGA, N4FG will respond to the  mes#

sage.  This gives rise to a lock out situation, because N4FG

will  respond  to the beacon, go through  the  sequence  de#

scribed  in the Zapbbs section and disconnect.   The  beacon

message will not be cleared because it was not read, so  the

next  mail beacon from the BBS will contain the call  N4FGA,

and N4FG will once again respond to it.

Should  you be unlucky and happen to be in  this  situation,

disable the BBS Snatch flag and set a Periodic Event to Zap

your BBS sometime in the middle of the night.

18.16 Turn Time Id ON/OFF

This option is only active in the PK-232.  It allows you  to

determine   if  the  callsign  sequence  transmitted   using

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 62

Function key 3 (F3) will contain a date/time stamp.  If  the

Id is set ON a date/time stamp will be transmitted.

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF

18.18 Change BBS Parameters

This  sub-menu allows you to change the BBS  Parameters,  as

described in the following paragraphs.

18.18.1 Local BBS Callsign

This option lets you change the call of the local  BBS.

This is the call that LAN-LINK will show in the  prompt

for the 'Zap' command.

18.18.2 Mail-Snatch Trigger Word

You  can  set the Mail-Snatch Trigger in the  BBS  Mail

beacon.   Most  BBS's use word 'Mail' and a  string  of

calls  to indicate the presence of messages.   If  your

BBS uses something else, you may customize LAN-LINK  on

this line to tell it what word your BBS uses.

Note that if your BBS uses 'QTC' you may end up calling

yourself if you disconnect while traffic is flowing  on

the LAN.

18.18.3 Read Request Command

This is the command LAN-LINK issues to the BBS during a

Zap  sequence to request your mail.  The default  value

is RM for Read Mine. On a WA7MBL BBS you may change  it

to  VM for a Verbose Read of the full  packet  headers.

On a W0RLI BBS you may change it to RN to only  request

new messages.  If you do change it, use the 'V'  option

in the Parameters Menu to make sure it is correct.   If

it isn't correct, the Zap BBS feature will not work.

18.18.4 Read Bulletin Request Command

This is the command LAN-LINK issues to the BBS during a

Zap  sequence to read bulletins identifed by  the  scan

words. The default value is R for Read. On a WA7MBL BBS

you  may change it to V for a Verbose Read of the  full

packet  headers.  On a W0RLI BBS you may change  it  to

RH.

18.19 Change Callsigns

This  sub-menu allows you to change the Callsign  and  AMTOR

SELCAL, as described in the following paragraphs.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 63

18.19.1 Callsign

The  default  callsign  is G8BTB.  Please  do  not  use

G8BTB.   Replace  G8BTB   with yours.  You  may  use  /

characters  if  you wish.  This is the call  sign  that

will be used in the non packet modes.  You may thus use

"portable  designators"  such  as  G3ZCZ/A,   G3ZCZ/W3,

W3/G3ZCZ or G3ZCZ/4X, which you cannot do in Packet.

This  is  the call that will be monitored  for  in  the

Digipeat  Monitoring  Mode.   In this  Mode,  which  is

toggled  on  and off by the selection of the  D  Option

included  in  the Parameter Menu  (PK-232  only),    if

someone  uses  you as a digipeater while  you  are  not

connected  to anyone else, the event will be logged  to

disk.   This  Mode will only work if your  callsign  as

stored in this line is a valid (uppercase only)  packet

callsign.

For  example, in the PK-232, if the callsign  is  G3ZCZ

then  the  sequence  >G3ZCZ> will be  recognized  as  a

digipeat and the event logged.  Thus 4X6AA*>G3ZCZ>4Z4ZB

would be logged as a digipeat event.

Store  your callsign using UPPERCASE characters if  you

want  the DIGIPEAT DETECT option to work  (EA.  G3ZCZ).

If  you  use lower case (ea. g3zcz),  recognition  will

never  take place. The TNC automatically converts  case

to upper case in the non packet communications modes.

If  you are using an SSID (e.g. G3ZCZ-8) in the  Packet

Communications  Mode, you should enter the  whole  call

INCLUDING the SSID.

18.19.2 The SELCAL

This  is your AMTOR SELCAL. LAN-LINK will try to  guess

it  and  prompt you with a SELCAL configured  for  your

callsign.   If you want a different one, overwrite  the

prompted version.

If  you are not using a PK-232 or KAM or  the  MFJ1278,

the SELCAL must still be in the Configuration file.

18.19.3 The PacketCluster Call

This option allows you to change the PacketCluster Call

without conecting to it.

18.20 Change Directories

This  sub-menu allows you to change the Directories used  by

LAN-LINK, as described in the following paragraph.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 64

18.20.1 QBM Download Directory

This is the directory the :QBM: command will search  to

find  a  file to be downloaded.  The default  value  is

\LAN-LINK\FILES.   Set them up using DOS.  If  you  are

using   a  hard  disk,  put  LAN-LINK  in  a   LAN-LINK

subdirectory.

18.21 Enter Common Calls

Each  time you call another station, LAN-LINK remembers  the

last 16.  You can use this option to add calls, delete calls

or edit calls in the list.  Use the 'Delete' key to delete a

call.

18.22 Change File Names

This  sub-menu allows you to change the default  file  names

used by LAN-LINK, as described in the following paragraphs.

18.22.1 Capture to Disk File (LAN-LINK.RUN)

This option lets you temporarily change the Capture  to

Disk file.  Don't change it while capture to disk is in

progress,  or  the old one will stay open and  may  not

close  properly when LAN-LINK is terminated,  resulting

in data loss.

18.22.2 VHF Logbook File (VHF.LOG)

This  is  the name of the file to be used as  your  VHF

Packet logbook.

18.22.3 HF Logbook File (HF.LOG)

This  is  the name of the file to be used  as  your  HF

logbook.  It is used for all modes, including HF  pack#

et.

18.22.4 Path Directory File (LAN-LINK.DIR)

This  is  the  name  of the  file  used  as  your  call

directory.

18.22.5 Brag Tape (LAN-LINK.TXT)

This is the name of the file used as your brag tape.

18.22.6 QBU Upload File Name (QBU-TX.BAT)

This  is the name of the batch file used to initiate  a

QBU.

18.22.7 QBU Download File Name (QBU-RX.BAT)

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 65

This  is the name of the batch file used when  a  :QBU:

command is received from a remote station.

18.23 Change LAN-LINK Parameters

This sub-menu allows you to change the LAN-LINK  Parameters,

as described in the following paragraphs.

18.23.1 Change CQ Text

This  feature allows you to change the contents of  the

CQ text message string. This is the message string that

is  transmitted along with the CQ call.  Think of  this

message as a CQTEXT line.

18.23.2 The Default Band

The  default  value is 2 for 2 Meters.  Use  the  Meter

value for the band to keep the display correct.  It  is

displayed  in  the  status  window.   If  you  normally

operate  on  another band change the default  value  to

reflect your usual default state.

18.23.3 The Default Transmitter Power

The default value is 10 for 10 watts.  It is  displayed

in  the  status  window.  If your  transmitter  uses  a

different  power  level  change the  default  value  to

reflect your usual default state.

18.23.4 Set CQ delay time Seconds

This  option  allows you to change the  time  (seconds)

that the LAN-LINK waits for a response after a CQ  call

until  the following one is transmitted.   The  default

value (in minutes and seconds) is loaded from the  LAN-

LINK.SYS file.

18.23.5 Set CQ delay time Minutes

This  option  allows you to change the  time  (minutes)

that the LAN-LINK waits for a response after a CQ  call

until  the following one is transmitted.   The  default

value (in minutes and seconds) is loaded from the  LAN-

LINK.SYS file.

18.23.6 Set Number of lines in a Non Contest CQ Call

This option allows you to change the number of lines in

a CQ call from the default value of 3 to any number you

wish.   Note that in the contest mode, this option  has

no effect.  This parameter is not remembered when  LAN-

LINK is terminated, so if you want to use a value other

than  3, you will have to set it each time you  operate

this program.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 66

18.23.7 Set Shut Down Mode (QRT Flag)

This  option  allows  you to customize  the  shut  down

sequence  of the TNC.  You have the choice of QRT,  QRV

and  QRV + CTEXT Toggle as described in  the  following

sections.

18.23.7.1 QRT

Turns  the  TNC into a VHF  Digipeater  and  turns

CONOK  OFF (CFROM NONE for the PK-232).   In  this

condition the TNC will reject connect requests.

18.23.7.2 QRV

Allows you to stay alive on turnoff. It will send

MON OFF (MON 0 for the PK-232)

to   the   TNC  at   shutdown   without   touching

CONOK/CFROM {PK-232}, and

at  turn on. You will then have to initialize  the

terminal  and  Communications Mode  after  reading

your mail.  Connects should then be logged at  the

time you read the mail, not at the time they  took

place.

18.23.7.3 QRV + CTEXT TOGGLE

This  is  the same as the QRV state  but  CMSG  is

toggled  on/off.   When stations  connect  to  you

while  LAN-LINK is active, they will  not  receive

the CTEXT message.  When other stations connect to

you  while  LAN-LINK  is  not  active,  they  will

receive the CTEXT message

18.23.7.4 BBS

This option lets you activate the PBBS in the  TNC

when you close LAN-LINK. It inhibits the PBBS when

you bring LAN-LINK up the next time.

18.23.7.5 SOLO

This option puts the TNC in the SOLO mode when you

shut  down,  so as not to fill the buffer  in  the

TNC.

18.23.7.6 BBS+SOLO

This option combines the two previous ones.

Set this value as follows,

0 = QRT,                1 = QRV,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 67

2 = QRV + CMSG toggle,  3 = BBS,

4 = SOLO,           and 5 = BBS+SOLO.

18.24 Change PC Communications Parameters

This sub-menu allows you to change the Communications Param#

eters  between the PC and the TNC, as described in the  fol#

lowing paragraphs.

18.24.1 The Communications Port

This  is the default Communications port.  The  default

value  is 1. You may set it at any value between 1  and

4.   If you only have one serial port in your  PC,  set

the parameter to '1' even if your card is configured as

COM2.   LAN-LINK will ALWAYS assume COM1 for  a  single

card.

DON'T  change this to a non existent port, or  LAN-LINK

may take a long time to exit.

18.24.2 The Terminal Baud Rate

This Parameter tells LAN-LINK what baud rate the TNC is

set  to.  This  is the baud rate  on  the  RS-232  line

between the TNC and the computer.  Valid baud rates for

LAN-LINK  are 300, 1200, 2400, 4800, 9600  and  19,200.

If you specify something else LAN-LINK will default  to

1200.

You  may  experience  problems when  using  baud  rates

greater  than 1200.  It depends on the CPU  clock  rate

and the I/O card in the PC.  LAN-LINK scans the  incom#

ing text constantly, something which requires time.  My

PK-232,  when used with my XT Clone at 10MHz, does  not

like  baud  rates faster than 2400, and  sometimes  not

even that. The   PK-232 seems to drop the first charac#

ter  of some lines at 4800 baud.   Use 1200  until  you

have tested and are sure that a faster one will work.

This program has been tested on a TNC2 using a terminal

baud  rate of 9600 baud and worked correctly.  It  also

works correctly with a KAM at 2400 Baud.

18.24.3 TNC Data Bits

You can set the number of data bits on the RS-232  line

between  the computer and TNC to 7 or 8.  Use 7 with  a

TNC 2 if you have problems with Greek characters.

Set this value to either 7 or 8.

18.24.4 TNC Parity

Use this parameter to set the Parity bit on the  RS-232

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 68

line between the computer and TNC as described below.

0 = None, 1 = Odd, 2 = Even, 3 = Mark, 4 = Space.

18.24.5 Number of TNC Stop Bits

You can set the number of stop bits on the RS-232  line

between  the computer and the TNC to 1 or 2.  Use 1  at

all terminal (not packet) baud rates except 300.

Set this value to 1 or 2.

18.25 Change Colors

This  option lets you change the default colors.  A  display

of all of the combinations is shown on the left side of  the

screen.  You will then be prompted to sequence through  each

window/message  color  that can be  customized.   Enter  the

number  shown  in  the combination window  followed  by  the

'Enter'  key  to set a value.  If you don't wish to  make  a

change, just press the 'Enter' Key.

The following identifications are made to the color choices:

18.25.1   InWindow Color

This is the color of the text in the CW, RTTY and  non-

connected/linked packet and AMTOR communications  modes

shown in the incoming window.

18.25.2   OutWindow Color

This  is the color of the text (typed at the  keyboard)

in the outgoing window.

18.25.3   StatusWindow Color

This is the color of the Status Window.

18.25.4   PromptWindow Color

This is the color of most of the menus.

18.25.5   LogWindow Color

This  is the color of the text in the log window.

18.25.6   LogScanWindow Color

This is the color of the text in the window showing the

scanned log pages (initiated by the Alt-S key).

18.25.7   ScrollBackWindow color

This  is  the  color of the text  in  the  Scroll  back

window.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 69

18.25.8   MHWindow Color

This  is  the color of the text in  the  monitor  heard

calls (MH) window.

18.25.9   Alarmwindow Color

This  is the color of the text in the Error  and  Alarm

window.

18.25.10  BottomWindow Color

This is the color of the text in the prompt line at the

bottom of the screen.

18.25.11  Emphasis Color

This  is the color of the emphasized text in  the  Menu

windows.

18.25.12  Prompt Color

This  is  the  color of the prompt  text  in  the  Menu

windows.

18.25.13  Option Color

This  is the color of selected option text in the  Menu

windows.

18.25.14  Connected Text Color

This  is the color of the text in the  incoming  window

when connected in the packet communications modes.

18.25.15  Computer Color

This is the color of the computer generated text in the

outgoing  window when the Command Echo Flag  option  is

enabled.

18.25.16  VHF Header Color

This  is  the  color of the VHF  Header  lines  in  the

Incoming window when not connected.

18.25.17  HF Header Color

This  is  the  color  of the HF  Header  lines  in  the

Incoming window when not connected.

18.25.18  Alert Call Color

This is the color of the packet header lines containing

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 70

the  Alert  call  in  the  Incoming  window  when   not

connected.   It is also the color of the  non-permanent

Parameter Menu prompts.

18.25.19  Outgoing Text Color

This  is the color of the outgoing text echoed  in  the

Incoming window in the AMTOR Communications Mode.

18.25.20  Multi I/O Stream Color[1]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 1.

18.25.21  Multi I/O Stream Color[2]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 2.

18.25.22  Multi I/O Stream Color[3]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 3.

18.25.23  Multi I/O Stream Color[4]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 4.

18.25.24  Multi I/O Stream Color[5]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 5.

18.25.25  Multi I/O Stream Color[6]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 6.

18.25.26  Multi I/O Stream Color[7]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 7.

18.25.27  Multi I/O Stream Color[8]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 71

stream 8.

18.25.28  Multi I/O Stream Color[9]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 9.

18.25.29  Multi I/O Stream Color[10]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 10.

NOTE that LAN-LINK does not overwrite all existing  windows.

Thus  if you want to be sure of seeing the  changed  colors,

save  the  color changes using the Update option,  and  then

exit from and reload LAN-LINK.

18.26 Change SAREX Parameters

This sub-menu allows you to change the SAREX Ground  Station

parameters, as described in the following paragraphs.

18.26.1 SAREX CallSign

This is the callsign of the SAREX station.

18.26.2 Change SAREX Capture to Disk File

This  option  lets  you change the name  of  the  SAREX

Capture-to-disk  file.  Don't change file  names  while

capture-to-disk  is  in progress (flashing [S]  in  the

status  window), or the old one will stay open and  may

not close properly when LAN-LINK is terminated, result#

ing in data loss.

18.27 Update Configuration File

Use  this  option to save the changes (permanently)  in  the

configuration file (LAN-LINK.SYS).

18.28 Verify Changes

This feature lets you display how LAN-LINK was configured by

the  LAN-LINK.SYS file.  It also displays the state  of  any

flags  you have set/reset if they are not already  shown  in

the  Status  window or Parameter Menu.  YOU  SHOULD  USE  IT

AFTER  EDITING  THE  LAN-LINK.SYS FILE  TO  MAKE  SURE  THAT

EVERYTHING IS CORRECT.

18.29 Change Scan Words

In  the ZAP BBS feature you can ask the program to scan  the

lines read from the BBS and if it sees any of the scan words

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 72

in  the line, it will queue a request for that  bulletin  by

number.   You  can only set 16 such Scan  words  (should  be

enough) and request up to 63 messages/bulletins at a time.

Use the 'Delete' key to delete a word from the list.

NOTE  that  words that show up in the BBS  prompt  line  are

ILLEGAL.  For example if your BBS prompts you with

N4QQ BBS>

then 'N4QQ' and 'BBS' should not be used as Scan Words.

DON'T  LEAVE  A BLANK LINE IN THE SCAN WORD SECTION  OF  THE

CONFIGURATION FILE.

18.30 Turn Misc. Flag ON/OFF

The Miscellaneous Flag performs several different  functions

in the different Communications modes as follows.

18.30.1 All Modes:- Debug or state-sequence numbers

Shows  debug  or state-sequence numbers  in  left  hand

corner of the status window.  These are used to  figure

out what went wrong in the automatic sequences.

18.30.4 BAUDOT Mode:- Navy MARS Protocol Verify

When  the flag is set, and the BAUDOT Navy MARS  option

is  enabled,  the  character  string  ':CR:'  will   be

displayed  in the outgoing window each time a  carriage

return  character is received in the  incoming  window.

Similarly,   every  time  a  line  feed  character   is

received,  a ':LF:' will be displayed in  the  incoming

window.

You may use this feature to verify that another station

is  sending files using the correct protocol.   If  you

do,  make  sure that you tell the PK-232 not to  add  a

line feed to incoming carriage returns by setting 'ALFD

OFF'  in the Command Mode.  You will have to set  'ALFD

ON'  again when you change communications modes or  all

incoming lines will overwrite each other.

19.0 Q Codes [NC/L]

This  option  displays  help information about  the  use  of  the

Network Communications Language (NC/L) "Q" codes.

20.0 CommunicationS Modes

This  menu  allows you to change the communications mode  of  the

TNC.   If  your TNC hardware cannot support  those  modes,  don't

bother to try them.  This is software not hardware, and in  spite

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 73

of  what  people  tell you, software can't  do  everything.   The

following communications modes are available.

V VHF PACKET

H HF PACKET

A AMTOR (STANDBY RECEIVE)

B BAUDOT (RTTY)

C MORSE (CW)

M AMTOR (ALIST [chirpcopy])

S ASCII (RTTY)

G SIGNAL

N NAVTEX

Type  in the single letter to make your choice.  Don't forget  to

select the correct Radio port on the PK-232.

20.1 VHF PACKET

This  is the 1200 baud VHF Packet Mode. LAN-LINK will  opti#

mize the TNC link parameters for this mode.

20.2 HF PACKET

This is the 300 baud HF Packet Mode. LAN-LINK will  optimize

the TNC link parameters for this Mode.

20.3 ASCII (RTTY)

This is the ASCII RTTY Mode. LAN-LINK will configure the TNC

parameters for this Mode at 110 baud.

20.4 BAUDOT (RTTY)

This  is the BAUDOT RTTY Mode. LAN-LINK will  configure  the

TNC parameters for this Mode at 45 baud.

20.5 MORSE (CW)

This is the CW Mode. LAN-LINK will configure the TNC parame#

ters for this Mode at 20 wpm (10 wpm for the KAM).

In this Mode the ALT-D Key will disengage the receive  speed

lock  inside the PK-232 to facilitate locking  onto  another

signal  and  Function Key 8 (F8) will turn  the  transmitter

off.

20.6 AMTOR (STANDBY RECEIVE)

When  you select the AMTOR Standby Mode you are placed  into

the  'Standby  Receive' Mode.  If you are in  AMTOR  Standby

Mode,  you  will  not  be able to  display  signals  of  the

Chirpcopy  Mode  (which means that you will not be  able  to

display a QSO in progress).

The  Alt-D  Key or Function Key 8 (F8) toggles  between  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 74

AMTOR ARQ and the ALIST communications modes.

20.7 AMTOR (ALIST [chirpcopy])

If  you  want to read the mail on an existing AMTOR  QSO  in

progress, select the AMTOR ALIST or Chirpcopy Mode.

The  Alt-D  key or Function key 8 (F8) toggles  between  the

AMTOR ARQ and the ALIST communications modes.

20.8 SIGNAL

Use  the 'G' option to select the 'SIGNAL' Mode.   Read  the

PK-232  documentation carefully to understand how this  mode

works.  Once the PK-232 has identified the digital signal it

is  tuned  to,  use the Communications Mode  Select  Menu  a

second  time  to select the relevant mode and then  set  the

baud rates and shift to display the data if necessary.

20.9 NAVTEX

Use the 'N' option to select the 'NAVTEX Mode.  Read the PK-

232   documentation  carefully to understand how  this  Mode

works.

21.0 Terminal Mode Menu

This menu allows you to set the Terminal Configuration Mode.  The

Terminal  Menu  provides  different choices  depending  on  which

communications mode you have selected.

21.1 Packet Terminal Menu

The  Packet Terminal Mode of operation will be shown in  the

Status window at the top of the screen.  If you are  reading

the mail on a station, the callsign of that station will  be

shown  next  to the word "MAIL".  If you  are  connected  to

someone,  the call (and path) will be shown prefixed by  the

"*>" sequence.

In  the Packet Communications Mode, you are  presented  with

the following choices.

B Change Packet Baud Rate

C CQ/Beacons

E Everything

M Read the Mail

S Just My Station (Solo)

T All Traffic

0 Multiple User Connects

1 Single User Connects

F Conference

R Conference Bridge On/Off

Type in the single letter to make your choice.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 75

21.1.1 Change Packet Baud Rate

In the PACKET Mode you may change the RF baud rate   between

300, 600, 1200 and 2400 baud.

21.1.2 CQ/Beacons

In  this  Mode, you will see CQ and BEACON  packets  on  the

channel.  (This corresponds to 'MONITOR 1'on the PK-232).

This Mode only works on the PK-232 and the KAM.

21.1.3 Everything

You can set the TRAFFIC Mode to 'Everything'. It corresponds

to 'MONITOR 6' on the PK-232.

21.1.5 Read the Mail

You  can  read the mail on a BBS or other  station  and  get

bulletins without connecting to that station yourself.   You

use  this Mode to do so.  You will be prompted for the  call

whose  packets you wish to monitor.  Remember to  enter  the

call of the station sending the packets, not the call of the

one  receiving them.  (This corresponds to 'MONITOR  0'  and

'MBX'  callsign). This Mode only works without  showing  the

packet  headers  on  the PK-232.  On  the  other  TNCs  the

headers will also be displayed.

You  can also use this command to monitor both  sides  (read

the mail) on a QSO on a busy packet channel.

You may want to turn the Digipeat Data Display Mode off when

reading the mail.

21.1.6 Just My Station (Solo)

In  this Mode, you will only see messages addressed to  you.

You  will only get messages from people who connect to  you.

(This corresponds to 'MONITOR 0' or 'MONITOR OFF').

21.1.7 All Traffic

In this Mode you will see all of the data packet traffic  on

channel.   You  can use this Mode to check that the  TNC  is

working. (This corresponds to 'MONITOR 4' or 'MONITOR ON').

21.1.8 Single User Connects

This is the normal one-on-one Mode of operation ('USERS 1').

21.1.9 Multiple User Connects

When you are in a Multiple Connect Mode, an asterisk will be

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 76

shown in the status window next to the word 'PACKET'.   Each

I/O  stream  will  show up in the Inwindow  in  a  different

color.

LAN-LINK supports two multiple connect modes, Individual and

Conference.

An  extract of an actual 'off the air' CRT display  (showing

received  text  only) in a multi-user mode is  shown  below.

Note  each line of text as received by the TNC  is  prefixed

(by the PK-232) with the I/O channel number and callsign  of

the station its connected to.

1:4X4IL: *** CONNECTED to 4X4IL via 4X4IC, 4X6JP

0:4X4IC: joe do u read?

2:4Z4ZB: *** CONNECTED to 4Z4ZB

1:4X4IL: Look for me on PR via 4X6JP 73s and Shalom.

2:4Z4ZB: GE JOE I WILL DIS THEN U CAN GO VIA THE REP....>

1: *** DISCONNECTED: 4X4IL via 4X4IC, 4X6JP

0:4X4IC: UNPROTO  CQ

2:4Z4ZB: GO AHEAD BOYS...DE ALON..>

0:4X4IC:

2: *** DISCONNECTED: 4Z4ZB

This is the normal Multiconnect Mode as described in the TNC

manual.   Here you  are connected to up to 10  stations  and

will send different traffic to each of them.  Each time  you

wish  to  send something to a particular station,  you  must

select  the I/O channel the station is connected  on  before

typing the text or sending the file.

The stream change character for all TNCs in the VHF multiple

connect modes is '|' ($7C). In the KAM, it is '\' on HF  and

'|' on VHF.  They are both on the same key but one is in the

shift  case.  Be careful on HF as the '\' shows up  in  file

directories  and  confuses the computer.  THESE  VALUES  ARE

HARD CODED INTO LAN-LINK. IF YOU CHANGE THEM, THE COLOR  AND

OTHER MULTIPLE CONNECT FEATURES WILL NOT WORK CORRECTLY.

You  select the I/O channel using the Alt-N key  and  moving

the  cursor  to  the desired stream, or by  typing  the  '|'

character  followed by (in the TNC) the channel  letter/num#

ber.  For example to do it manually, if you are connected to

G8BTB  on I/O channel 0 and to G3ZCZ on I/O channel 1,  then

to  talk to G8BTB you would type '|0' before any  text,  and

similarly  to send something to G3ZCZ you would type a  '|1'

before  the  text you're going to send me.  If you  are  not

using a PK-232, read the TNC manual to determine if the  I/O

streams  are identified by numbers or by letters.  The  TNC2

standard configuration uses letters (|A etc.).

Use the Alt-N key and move the cursor up and down to  select

the I/O channel automatically.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 77

21.1.10 Conference

In  the Conference Mode on the other hand,  everything  that

you  type  at the keyboard is automatically  transmitted  to

each  station that you are connected with.  Thus if you  are

linked  to two stations each line will be packeted twice  by

the  TNC.  You don't have to worry about sending  the  wrong

thing to the wrong person, as they will all get the stuff.

You  enter  this  Mode by selecting the 'F'  option  in  the

Terminal Menu.  You terminate this Mode by selecting the '1'

option.

You  connect in the usual manner using Alt-C or the  Connect

Option in the Call Menu.  You MUST WAIT FOR A CONNECTION  TO

BE  ESTABLISHED OR TIME OUT before attempting to connect  to

the  next  station.  If you do not, you will  get  an  error

message back from the TNC.

In  the Conference Mode, as the same text is transmitted  on

all I/O channels, the throughput will be reduced.  Slow down

your  typing speed or use short packets.  Should  you  check

the link status or call monitored list, you may not be  able

to  get  back into the Converse Mode if you  have  too  many

packets outstanding. If that happens, wait a while and  then

try  to  set the Converse Mode with Function Key  10  (F10).

You  may have to try it a few times, depending on  how  good

your links are.

DON'T  TRY TO SEND A FILE IN THE CONFERENCE MODE, IT IS  NOT

DESIGNED FOR IT. KEEP THE PACKETS SHORT.

BE ALSO WARNED THAT IF YOU USE THE CONFERENCE (BRIDGE)  MODE

ON  BAD LINKS (SUCH AS CROWDED HF CHANNELS AND  OTHERS  WITH

LOTS  OF RETRIES), THE KEYBOARD MAY SEEM TO LOCK  UP,  WHILE

LAN-LINK  WAITS  FOR SPACE IN THE TNC BUFFER TO  OUTPUT  THE

TEXT.

The Conference Mode will only work in the PK-232 if  DCDCONN

is  set to ON.  So if you are using a PK-232, LAN-LINK  will

do it for you when you enter the Conference Terminal Mode in

case you forgot or/and haven't set the LAN-LINK.SYS file  to

include   the  line  'DCD  ON'.   At  the  same   time   set

LAN-LINK.SYS  default  states  to show the  calls  of  other

stations  on  each incoming line in the  Multiconnect  modes

(read the TNC manual on how to do this).

The Conference Mode will only work in the TNC2 if DCDCONN is

jumpered so that Pin 8 of the RS-232 Interface changes state

when you connect to someone else.  If you get error messages

and see the characters '|J' on your screen, then the  jumper

probably isn't set.

The Conference Mode does not work on the TNC1.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 78

In  the  Conference  Mode,  if  any  one  of  the   stations

disconnects,  the TNC will return to the Command  Mode,  and

you must use Function Key 10 (F10) to return to the Converse

Mode before you type another character of text.

21.1.11 Conference Bridge On/Off

This option toggles the Conference Bridge on and off. In the

Bridge Multiconnect Mode you are acting as a central switch.

You  can  link together up to 10 stations  in  one  multiway

connection.  In this Mode any packets arriving on any stream

are echoed to all other streams that you have 'connects'  on

except the one that they came in on.  Anything you type will

be sent to all connected streams.

In  this  manner, all stations will not see their  own  text

echoed  back.  Each line of text output by LAN-LINK in  this

Mode  will contain the callsign of the  station  originating

the  packet. Your packets will contain your call as  entered

in  line 1 of the LAN-LINK.SYS file.  Thus for  example,  if

your  call was KB8RP/4X and normally shows up in the  packet

header as KB8RP here it will show in the data as KB8RP/4X.

Just remember that the Bridge puts out a lot of packets,  so

do  multiway connects on a quiet frequency.  You must be  in

the  Conference  Mode  to  turn  the  bridge  on,  and   any

disconnect  will take the bridge down.  Should that  happen,

toggle  it on again.  A bridge will be shown in  the  status

window whenever the Conference Bridge is up.

21.2 AMTOR Terminal Menu

In the AMTOR Communications Mode, you are presented with the

following choices.

C Force Break [ACHG]

F Start Mode B [FEC]

L Force LETTERS case

M Read the Mail [ALISTEN]

O Override ROBOT/Mailbox

S Standby Receive [ARQ]

R Set ROBOT/Mailbox

W CW Identification

Type in the single letter to make your choice.

21.2.1 Force Break [ACHG]

This  option forces a changeover or 'break in'.  If for  any

reason  you want to interrupt the other station you  send  a

'BREAK' signal in software this way.

21.2.2 Start Mode B [FEC]

This option places the terminal into the FEC Transmit  Mode.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 79

This  puts  you  on  the air,  anything  you  type  will  be

transmitted.  Function Key 10 (F10) does the same thing.

21.2.3 Force LETTERS case

This  option  lets  you force the  PK-232  receiver  to  the

letters  case  if for some reason you appear to  be  in  the

numbers case.  It will return you to the Converse Mode.

21.2.4 Read the Mail [ALISTEN]

This option lets you read the Mail on a QSO.  Unfortunately,

when you tune in a chirping AMTOR QSO in the ARQ or  Standby

Mode you won't get any copy on the QSO in progress.  You can

however read the mail on one by entering the ALISTEN Mode.

21.2.5 Override ROBOT/Mailbox

This  option lets you temporarily override the ROBOT  BEACON

and chat to a station that has linked to the ROBOT.  Use  it

after the message in LAN-LINK.TXT has been sent, namely when

the other station is sending a message to you.

21.2.6 Standby Receive [ARQ]

This option returns you to the ARQ Standby Mode. You must be

in this Mode to receive signals from another station who  is

calling  you.   When  you  tune for  CQ  calls,  they  sound

somewhat like fast RTTY with a ringing sound similar to  the

connect  alarm  used by LAN-LINK (but at  a  faster  ringing

rate).

21.2.7 Set ROBOT/Mailbox

This  option lets you turn on the Mailbox during  a  regular

AMTOR  QSO so that the other station can leave a message  or

download  a  file remotely. You must only use  it  when  the

other station is sending text.

21.2.8 CW Identification

This  option is used to send a CW identification.  The  CWID

is sent in the FEC Mode.

The  identification  consists  of  the  sequence  'DE  <your

callsign>' where your call sign is that entered into the PK-

232 as  'MYCALL'.

You  use  this  feature  if your  license  requires  you  to

identify in Morse code as well as in the operating mode.

21.2.9 Set ROBOT Infinite Delay

You  use  this  option  to  prevent  the  Robot  from   ever

transmitting  when  not linked. In the  normal  manner,  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 80

Robot counts down the CQ delay parameter, and, when it  gets

to  0 puts a CQ call on the air.  If the Infinite  Delay  is

set,  the CQ call will be inhibited. This option  thus  puts

the  Robot into an indefinite wait mode where it  thinks  it

has sent out a CQ call and is waiting for a reply.

You would use this Mode if you want to set up a MailBox on a

known frequency for some group or organization.

21.3 Baudot Terminal Menu

In  the Baudot Communications Mode, you are  presented  with

the following choices.

B Set Baud Rate

F Select Shift

N NAVY MARS Message Protocol

U USOS Off

S SELCAL On/Off

W CW Identification

Type in the single letter to make your choice.

21.3.1 Set Baud Rate

This feature allows you to set a desired communications baud

rate  directly.  You will be prompted to enter  the  desired

rate.  Use this command to avoid having to shift through too

many intermediate states, if for example, you want to make a

direct change from 45 to 100 baud.

21.3.2 Select Shift

This  feature lets you select Wide (850 Hz) or  narrow  (170

Hz)  shift  directly.  The PK-232 will copy 170  Hz  signals

using its internal 200 Hz circuitry.  If you have a KAM, you

may also select the Commercial Shift (425 Hz).

The  shift is displayed in the status window at the  top  of

the screen. It is shown as 'W', 'N' or 'C' for wide,  narrow

or commercial respectively.

21.3.3 NAVY MARS Message Protocol

The Navy Mars option toggles the Navy MARS Mode on and  off.

This  feature is only present on the PK-232.   When  active,

the feature provides the following protocol.

21.3.3.1  Any  Carriage  Return  or  Enter  (CR)  from   the

keyboard  is  sent as CR, CR. (two  sequential  CR

characters.

21.3.3.2 In  a file transmission lines are terminated  by  a

CR,CR,LF (Line Feed) sequence.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 81

21.3.3.3  In a file transmission, blank lines are sent as  a

LF (without a CR).

21.3.3.4  In  a  file transmission, a plus sign '+'  in  the

file  is  replaced by a sequence of  12  "letters"

characters.  There can be more than one + sign  on

a line.

21.3.3.5  In  a  file transmission, the sequence  'NNNN'  is

transmitted followed by a sequence of 12 "letters"

characters.  There can only be one 'NNNN' sequence

on a line.

When the NAVY MARS feature is active a [N] will be displayed

in the status window.

21.3.4 USOS On/Off

This  option toggles the Unshift on space (USOS) feature  on

and  off.   When  the USOS is on, the  TNC  reverts  to  the

letters shift each time a 'space' character is received.  It

helps when displaying noisy copy.

If USOS is active, 'USOS' will be shown in the status window

at the top of the screen.

21.3.5 SELCAL On/Off

This  command  key toggles the SELCAL feature  on  and  off.

When the SELCAL is activated, as shown in the status window,

the incoming text from the TNC is scanned.  When either  the

letters  "ZCZC" or 'your callsign' (as entered in line 1  of

the LAN-LINK.SYS file) is received the "capture-to-disk"  is

turned  on.  All subsequent text will be stored in the  LAN-

LINK.RUN  file  until the letters "NNNN" are  received.   At

that time the "capture-to-disk" is turned off.

You  may  use this feature when monitoring  commercial  RTTY

broadcasts  to  record news and other information,  or  when

calling CQ to start recording all incoming text and keep the

QSO on disk.

You may use Function Key 1 (F1) to turn the capture to  disk

permanently  on  to override the automatic  turnoff  if  you

wish.

21.3.6 CW Identification

This  option is used to send a CW identification.   You  use

this if your license requires you to identify in Morse  Code

as well as in the operating mode.

The  identification  consists  of  the  sequence  'DE  <your

callsign>' where your call sign is that entered into the PK-

232 as  'MYCALL'.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 82

21.4 ASCII Terminal Menu

In the ASCII Communications Mode, you are presented with the

following choices.

B Set Baud Rate

F Select Shift

S SELCAL set/reset

W CW Identification

Type in the single letter to make your choice.

21.4.1 Set Baud Rate

This feature allows you to set a desired communications baud

rate  directly.  You will be prompted to enter  the  desired

rate.  Use this command to avoid having to shift through too

many intermediate states, if for example, you want to make a

direct change from 110 to 1200 baud.

21.4.2 Select Shift

This  feature lets you select Wide (850 Hz) or  narrow  (170

Hz)  shift  directly.  The PK-232 will copy 170  Hz  signals

using its internal 200 Hz circuitry.  If you have a KAM, you

may also select the Commercial Shift (425 Hz).

The  shift is displayed in the status window at the  top  of

the screen. It is shown as 'W', 'N' or 'C' for wide,  narrow

or commercial respectively.

21.4.3 SELCAL On/Off

This  command  key toggles the SELCAL feature  on  and  off.

When the SELCAL is activated, as shown in the status window,

the incoming text from the TNC is scanned.  When either  the

letters  "ZCZC" or 'your callsign' (as entered in line 1  of

the  LAN-LINK.SYS file) is received the  capture-to-disk  is

turned  on.  All subsequent text will be stored in the  LAN-

LINK.RUN  disk file until the letters "NNNN"  are  received.

At that time the capture-to-disk is turned off.

You may use Function Key 1 (F1) to turn the capture to  disk

permanently  on  to override the automatic  turnoff  if  you

wish.

21.4.4 CW Identification

This  option is used to send a CW identification.   You  use

this if your license requires you to identify in Morse  code

as well as in the operating mode.

The  identification  consists  of  the  sequence  'DE  <your

callsign>' where your call sign is that entered into the PK-

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 83

232 as  'MYCALL'.

21.5 CW Menu

In  the CW Communications Mode, you are presented  with  the

following choices.

L Lock to Receive Signal

S Set Morse Speed

U Unlock Morse Speed

Type in the single letter to make your choice.

21.5.1 Lock to Receive Signal

This option locks the PK-232 to the speed of the incoming CW

signal.

21.5.2 Set Morse Speed

This option lets you set the CW speed.

21.5.3 Unlock Morse Speed

This option unlocks the PK-232 from the incoming CW  signal.

It  performs the same task as the Alt-D option keys.  It  is

used  to  speed up capture of a new signal or to  break  the

lock when the PK-232 appears to have locked up at the  wrong

speed.

22.0  eXit to DOS

When  you  exit the program and return to DOS, if the  Shut  down

(QRT) flag is set to 0, the TNC is put into the VHF default  mode

in  which it rejects calls from any other stations.  (Read  about

the  'CFROM'  command in the TNC manual).  You can thus,  if  you

wish,  leave  it  powered, and if you also leave  the  VHF  radio

powered  up, you will be active as a digipeater, but at the  same

time  anyone trying to connect with you will receive a busy  sig#

nal.  The MBX command (Reading the Mail) is also cleared at  this

time.

If you want to leave the TNC set up to accept messages when  LAN-

LINK is not active, customize the shut down state as described in

the Section 18.23.

23.0 Hot Keys

LAN-LINK  also  contains a number of "Hot" keys to allow  you  to

perform  common  operations with a minimum of key  strokes.   The

keys  perform  related  but  slightly  different  tasks  in  each

communications mode.  A list of those available is shown in Table

23.   Should you require any more hot keys, I suggest you use  an

external  program  with  a  Keyboard  Macro  capability  such  as

Borland's Superkey.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 84

Consider the action of each key in detail.

23.1 Alt-A - Alert Call (Packet) and AMTOR ARQ call/talk

In  the  Packet Communications Mode,  this  key  combination

performs the same task as the Alert Call option in the  Call

Menu.

In  the  AMTOR  Communications Mode,  this  key  combination

performs the same task as the AMTOR ARQ call/talk option  in

the Call menu.

23.2 Alt-B - Send Break/AMTOR Changeover

In  the  Packet  Communications Mode,  activating  this  key

combination sends a 'BREAK' signal to the TNC.  Use this key

combination to get out of the TNC's transparent mode.

In  the AMTOR Communications Mode, use this key  combination

to force a changeover.  You can type text into the  outgoing

buffer and then force a changeover without losing that text.

23.3 Alt-C - Call/Connect to

This  key  combination performs the same task  as  the  Call

option in the Call Menu.

23.4 Alt-D - Disconnect or Receive

This key combination performs the same task as Function  Key

8 (F8) EXCEPT in MORSE and AMTOR.

In  Morse this key disengages the receive speed lock in  the

PK-232 to unlock the PK-232 from the last signal copied.  It

lets you lock onto another signal faster.

In  AMTOR this key toggles the receive mode between ARQ  and

FEC signals (Qso's in progress and Cq calls).

23.5 Alt-E - Enter Call

This  key  combination performs the same task as  the  Enter

Call option in the Call Menu.

23.6 Alt-F - Flush TNC Buffer

This key combination performs the same task as the Flush TNC

Outgoing buffer option in the TNC Menu.

23.7 Alt-J - Jump to DOS

This  key  combination lets you "Jump to a DOS  Shell".   It

performs  the  same function as the 'J' option in  the  Main

Menu.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 85

Table 23 Summary of Hot Keys

____________________________________________________________________

Hot                 Communications Mode

Key

_______________________________________________________________

Packet         AMTOR        BAUDOT/ASCII        CW

____________________________________________________________________

A    Set ALERT      ARQ Call/Talk     ------           ------

Call

____________________________________________________________________

B    Send 'Break'   Force Changeover  ------           ------

to TNC

____________________________________________________________________

C    Connect        ARQ Call          Call             Call

____________________________________________________________________

D    Disconnect     Toggle Receive    Receive         Disengage

Modes             Mode            Speed Lock

____________________________________________________________________

E    Enter Call     Enter Call and   Enter Call        Enter Call

Selcal

____________________________________________________________________

F          Flush TNC Buffer (all communications modes)

____________________________________________________________________

H                      Display Help Screen

____________________________________________________________________

J            Jump to DOS (all communications modes)

____________________________________________________________________

K            Jump to DOS quick return (all communications modes)

____________________________________________________________________

L          Edit Log Entry (all communications modes)

____________________________________________________________________

M           Scan Message (all communications modes)

____________________________________________________________________

N  Next I/O Stream

____________________________________________________________________

O    Turn Robot ON  Turn Robot ON     ------           ------

____________________________________________________________________

P         Turn Printer ON/OFF (all communications modes)

____________________________________________________________________

Q    call QRZ       call QRZ in FEC   call QRZ         call QRZ

____________________________________________________________________

R    Retry Connect    Retry last Call (entered).

(not ZAP)

____________________________________________________________________

S           Scan Logbook (all communications modes)

____________________________________________________________________

X            Exit to DOS (all communications modes)

____________________________________________________________________

Y    Zap PacketCluster

____________________________________________________________________

Z    Zap BBS        Drop Link after       Receive mode after

sending contents       sending contents

of buffer                 of buffer

____________________________________________________________________

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 86

23.8 Alt-K - Jump to DOS, Quick Return

This  key  combination lets you "Jump to a DOS  Shell".   it

performs  the  same function as the 'J' option in  the  Main

Menu.  When the command is executed, control is returned  to

LAN-LINK without the 'type exit to quit the shell sequence'.

23.9 Alt-L - Edit Log

This  key  combination performs the same task  as  the  Edit

option in the Log Menu.

23.10 Alt-M - Message Scan

This  key combination performs the same task as the  Message

Scan choice in the Message Menu.

23.11 Alt-N - Next Stream

In  the  Packet Communications Mode, if you  are  using  the

multiple  connect capability of LAN-LINK, this key  combina#

tion  prompts  you to change I/O streams,  and  display  the

state  of the I/O streams showing which ones were last  used

to  send or receive something.  To change I/O streams,  move

the cursor to the desired stream then push the 'Enter'  Key.

The  calls  in the Next Stream window update  when  stations

connect  and  disconnect.  In the event of the window       not

being correct, you may manually enter or delete calls  using

the  'Insert' Key to enter a call, and the 'Delete'  Key  to

remove a call.

The data about the streams will be shown in the same  colors

as the multistream text colors.

23.12 Alt-O - Turn Robot On

This  key combination performs the same function as the  Set

ROBOT option in the AMTOR Terminal Menu. The hot key  combi#

nation  however,  is  active in both the  Packet  and  AMTOR

communications modes.

This  function  key also puts the Packet and  AMTOR  Contest

operation into automatic.

23.13 Alt-P - Turn Printer ON/OFF

This  key  combination  performs the same  function  as  the

toggle printer on/off option in the Parameter Menu.

23.14 Alt-Q - Call QRZ

This key combination transmits the sequence 'QRZ QRZ QRZ  DE

YOURCALL'  and  turns the transceiver back to  receive.   In

AMTOR, it sends the sequence in FEC and adds the SELCAL.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 87

23.15 Alt-R - Retry Call/Connect

This  key  combination performs the same task as  the  Retry

choice in the Call Menu.

23.16 Alt-S - Scan Log

This key combination performs the same function as the  Scan

option in the LOG Menu.

23.17 Alt-X - Exit to DOS

This  key combination performs the same task as the Exit  to

Dos  choice  in  the Main Menu. It  exits  the  program  and

returns you to DOS.

23.18 Alt-Y - Zap PacketCluster

This  key combination performs the same task as the  Packet#

Cluster  choice  in the Call Menu. It connects  you  to  the

PacketCluster  and issues a 'SHOW/DX' instruction when  con#

nected.

23.19 Alt-Z - Zap BBS

This  key combination performs the same task as the Zap  BBS

choice in the Call Menu.

23.20 Alt-= - Toggle Miscellaneous Flag

This  key  combination performs the same function as  the  +

option in the parameters menu.

23.21 End -  > +?  [Over]

In  the Packet Communications Mode this command  sends  your

callsign  followed by the > and Line feed  characters.   You

may use this to signal 'OVER' in a packet contact.

In  the  AMTOR Communications Mode this  command  sends  the

'+?'  sequence  of characters which is the  software  'OVER'

signal.

In  the  MORSE and RTTY (Baudot  and  ASCII)  communications

modes, this command lets you transmit the 'KN' sequence  and

returns  you  to the Receive Mode.  For example if  you  are

4X6AA and you are working G3ZCZ then you would use  Function

Key  3  (F3) to transmit the sequence 'G3ZCZ de  4X6AA'  and

this  key to send 'KN' and revert to receive.  You use  this

at the END of a transmission in any mode except Packet.

If  you  have typed something into the 'type  ahead  buffer'

then  press  this key, the contents of the  buffer  will  be

transmitted followed by the turnover command.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 88

23.22 PgUp - Scrollback (Scrolls back incoming text)

This  key lets you repeat the display of up to 255 lines  of

incoming text which has scrolled out of the incoming window.

You may print it or capture it to disk during the repeat.

A  small menu displays your options when showing the  scroll

back information.  You may move the menu window up and  down

with  the left and right arrow keys.  Use the 'ESC'  key  to

terminate the Scrollback Mode.

You can save the contents scrollback buffer displayed on the

screen by turning the "Capture-to-Disk' on, and then writing

text to the window.  Anything written will be captured.   If

the  Capture-to-disk is ON before or during scrollback,  you

will get two copies of the text in the file.

23.23 The Home Key

The  Home Key will clear the window screen that  the  cursor

happens to be in at the time that the key was depressed, and

will then position the cursor at the top left hand corner of

that window.

23.24 The Down Arrow Key

The  Down  Arrow  Key toggles  the  Normal/Reverse  Sideband

command  in  all PK-232 communications modes  except  MORSE.

The sideband is depicted in the status window by the Up  and

Down arrows.  If the Down Arrow is showing, you are set  for

Lower Sideband (LSB) which is the one normally used.

If you have reversed the sidebands a flashing Up Arrow  will

be displayed.

If you set your transceiver to LSB you will be set correctly

for all modes.

For CW operation you may set your rig to the CW position and

ignore the sideband setting.  If you use FSK just set it  so

that it works for you.

23.25 Shift/Baudrate Control Keys

The  following keys are used to control the Shift  (Wide  or

narrow) and the Radio link Baudrate in the BAUDOT and  ASCII

communications  modes.  In the BAUDOT and  ASCII  communica#

tions  modes all baud rates supported by the PK-232  may  be

selected.

23.25.1 RIGHT ARROW speeds up the baud rate.

23.25.2 LEFT ARROW slows down the baud rate.

23.25.3 UP  ARROW toggles the shift from Wide (850  Hz)

to Narrow (170 Hz) and back.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 89

23.25.4 DELETE toggles the USOS command (RTTY only).

In  the  CW  communications mode, the  left  and  right

arrows change the CW speed by 1 wpm.

The  state of each of the above commands are  displayed

in the status window at the top of the screen.  If USOS

is active, 'USOS' will be shown.  The shift is shown as

'W' or 'N' for wide or narrow respectively.

23.26 Control-Z (^Z)

This  key  combination is used in the  Packet  Communication

Mode  to signal the end of a file or message,  when  loading

messages into another LAN-LINK system or into a BBS.  In the

AMTOR  Communications Mode, it is  automatically  translated

into  the sequence ' :EOF: +? ' which can be used to  termi#

nate a message being stored in another LAN-LINK AMTOR  mail#

box.

24.0  LAN-LINK Disk Files

The following disk files are associated with the program.

LAN-LINK.COM

LAN-LINK.SYS

LAN-LINK.RUN

LAN-LINK.LOG

LAN-LINK.DIR

CONFIG.SYS

*.MBX

*.BBS

LAN-LINK.001 .. LAN-LINK.010

QBU-RX.BAT

QBU-TX.BAT

Consider each of them in turn.

24.1  LAN-LINK.EXE

This is the program.  You activate it from DOS by typing the

word  LAN-LINK  followed by ENTER in the  normal  manner  of

invoking DOS commands.

24.2  LAN-LINK.SYS

This  file contains the configuration information  for  both

the  program and the TNC.  YOU MUST EDIT IT before  you  run

the   program   on  the  air  the  first  time.    Use   the

customization  options  in the parameters menu to  make  the

changes.

The contents of first few lines of the default file together

with an item number are shown below.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 90

1.   Callsign

2.   Selcal

3.   CQ Text

4.   Band

5.   Power

6.   CQ Delay (seconds)

7.   CQ Delay (minutes)

8.   :QSP: Message Relay Flag

9.   PC Serial Communications Port Number

10.  TNC Type

11.  PC to TNC Baud Rate

12.  VHF Log Book File

13.  HF Log Book File

14.  Capture to Disk File name

15.  Path Directory File name

16.  Brag Tape File name

17.  BBS Snatch Mail Trigger Word

18.  QTC Snatch Flag

19.  BBS Snatch Flag

20.  Shut Down Mode

21.  PC to TNC Data Bits

22.  PC to TNC Parity Bits

23.  PC to TNC Stop Bits

24.  DCD Flag

25.  QBM Download Subdirectory

26.  BBS Read Request Command

27.  Local BBS Callsign

28.  SAREX Callsign

29.  SAREX Capture to Disk File name

30.  PacketCluster Call

31.  Digipeat Detect Flag

32.  Node Drop Link Flag

33.  Sound Flag

34.  Display Computer Generated Commands Flag

35.  Digipeat PK-232 Date/timestamp in Call Sequence Flag

36.  Keyboard Character Beep value.

37.  :QBU: Receiving Station Batch file (QBU-RX.BAT)

38.  :QBU: Originating Station Batch file (QBU-TX.BAT)

39.  Capture-to-disk Flag

40.  Beacon Control/LAN-LINK handshake Flag

41.  Spare

42.  Start of Scan Words (MUST HAVE AT LEAST ONE)

43.  * end of Scan Words

44.  Start of Common Calls (MUST HAVE AT LEAST ONE)

45.  * end of Common Calls

46.  Start of Custom Colors

47.  Remaining TNC start up Commands.

The  contents of each line are described in the  section  of

the Parameters menu which deals with customizing them.   The

remaining  lines contain commands to the TNC.   For  further

information  about them refer to the TNC  manual.   LAN-LINK

assumes different defaults in some instances to those set in

the TNC.  The SYS file contains only those that are  differ#

ent to the TNC default reset state.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 91

NOTE: If you are not using a TNC1 you MUST set the  HEADERLN

parameter  to  ON.  The packet headers and the text  of  the

packets must show up on your screen on separate lines.   You

must  also have the "Echo as Sent" feature set ON ('EAS  ON'

for PK-232 and 'XM' ON ' for the KAM).

24.3  LAN-LINK.RUN

This file contains incoming data from the TNC recorded while

the  'capture to disk' function was active.  You may  change

the  default name of the 'capture to disk' file in the  LAN-

LINK.SYS file.

24.4  LAN-LINK.LOG

This  file  contains the log entries.  It  will  be  created

automatically  to  initialize  the  logging  function.   The

format  of the logbook file is the same as that  created  by

the  G3ZCZ CONTEST.BAS REV 3 program and is designed  to  be

appended into the G3ZCZ LOGBOOK package presented as part of

the  PC-HAM  software package.  It should be edited  with  a

word  processor in the nondocument mode to delete the  first

line  and  clean it up before the file is  appended  to  the

logging package data files.

24.5  LAN-LINK.DIR

This  is the directory file as described in  Section  6.1.2.

You  use it to store connect, NET/ROM, TheLink,  KA-Node  or

digipeat paths as well as name/callsign relationships.

Examples are

Joe  G3ZCZ

Alon 4Z4ZB V 4X6AA

Milt 4X6AA

LR   4X6LR

hf-il 4x4hf v 4z4zb 4x4il

hf-rj 4x4hf v 4z4zb 4z4rj

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT! K1HTV

w9tnn-2  jrw !% w3eax-7 !& w9tnn-2

You  create this file with your word processor in  its  non-

document mode.  You must leave AT LEAST one space  character

between the first (key) word and the connect path.  When you

type in the key word, the program does not care about  upper

or lower case.

You may also use the 'reMember Path' option in the Call Menu

to make entries in this file.

If  you already have a YAPP.DIR or a PK232COM.DIR file  just

rename it to LAN-LINK.DIR and add any NET/ROM and/or KA-Node

paths.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 92

NOTE:  All path entries in LAN-LINK.DIR must be on a  single

80 column line.

24.6  CONFIG.SYS

You must have a CONFIG.SYS file on the boot disk if you  are

going  to  use the disk logging features.   This  file  must

contain the line

FILES = 20

See the DOS manual for more information about the CONFIG.SYS

file.

24.7  *.MBX

These  are the files associated with the stations  you  have

designated  as LAN store and forward mailboxes.  Other  sta#

tions can only :QRU: with you if you have a  'theircall.MBX'

file  on  your disk.  Thus if for example you  want  to  use

4Z4ZB  as  a  store and forward mailbox, you  will  have  to

create  an ASCII file called 4Z4ZB.MBX using Edit  Menu,  or

the non-document mode of your word processor.  In this  file

you will put a list of the call signs of the stations  4Z4ZB

can  connect to reliably.  If for example you want 4Z4ZB  to

be  able to store messages from you to 4X6AA, 4X6LR,  G3ZCZ,

ON8IK, F0WN,and G8BTB, you would put those call signs in the

file, one call on a line as follows.

4X6AA

4X6LR

G3ZCZ

ON8IK

F0WN

G8BTB

When  the QRU function is invoked either by you manually  or

by  4Z4ZB  connecting  to you and sending  you  the  command

:QRU:, any message files present on your system addressed to

any of those stations (E.G. G3ZCZ.OUT and ON8IK.OUT) will be

transferred  from you to 4Z4ZB just as if you had  tried  to

:QSP:  the  messages manually or as if G3ZCZ or  ON8IK   had

connected  directly to you.  The file names on  your  system

will be changed to *.OLD.

You  must  create  one file for each  designated  store  and

forward station, ie. 4Z4ZB.MBX, G3ZCZ.MBX etc.

24.8  *.BBS

These are BBS AUTO-Upload Command files.

Each file is named for the callsign of the BBS that you wish

to access.  It must have exactly the name as the callsign of

the  BBS.  Thus if the BBS is W9TNN-1 the name of  the  file

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 93

must  be W9TNN-1.BBS, or if the BBS is 4Z4ZB then  the  file

must  be called 4Z4ZB.BBS.  The file must be an ASCII  file.

That  means  the  word processor must be used  in  the  non-

document mode to create it.

This file contains the command sequence that you would  have

typed  at  the  keyboard  if  you  were  accessing  the  BBS

manually.  The ONE difference is that the text of a  message

you are uploading with the S command must be followed by the

character  string :EOF: (on a separate line) as  illustrated

by the example below.

The  contents of A TYPICAL *.BBS file is shown below  as  an

example.

k 1234

d \net\lan-link.doc

s g8btb

Test Message

This is a test message.

:EOF:

sp G3ZCZ @ N4QQ

Greetings

The text of this message is greetings from LAN-LINK.

/ex

lm

r 567

r 593

24.9 LAN-LINK.001 - LAN-LINK.010

These  are 10 different ASCII files with fixed  names.  They

can be transmitted using the Alternate function keys.  Their

contents  can be viewed using the Shift function  keys.   In

all  cases  the function key to use for  a  particular  file

corresponds  to the number in the file suffix. For  example,

use Alt/Shift Function Key 3 (Alt-F3) to transmit/view  file

LAN-LINK.003.  You may also use the Edit Menu to edit/create

the files.

In  case you forget what you put in each of the files,  LAN-

LINK allows you to take a peek into all of them at the  same

time.   Use Control Function key 1 (Control-F1) to  look  at

the first line of all the LAN-LINK.001 - LAN-LINK.010  files

and the Brag Tape.

24.10 QBU-RX.BAT

This  is the command or batch file used in the :QBU:  origi#

nating station.

24.11 QBU-TX.BAT

This is the command or batch file used in the :QBU:  receiv#

ing station.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 94

25.0 Mailbox features

This section describes the mail box and message handling features

built into LAN-LINK.

25.1 Answering Machine

LAN-LINK  contains an AMTOR (Robot beacon CQ mode only)  and

PACKET  Mode   SMART "answering machine" facility.  You  can

leave  messages on your disk (in the same directory  as  the

LAN-LINK.*  files,  if  you  are using  a  hard  disk),  for

different  stations.  When someone connects to you,  if  you

left  a message for him, he (or she or even it as  the  case

may be) and only that station will receive it automatically.

No one else will normally be able to download that message.

25.2  Mail Beacon (Annunciator)

To ensure that people know that you have left a message  for

them a 'MAIL for' list is loaded into your Packet Beacon and

transmitted every 30 minutes (Refer to the BTEXT command  in

the TNC manual) as  ':QTC:' followed by a list of calls.  If

no  mail is pending, or the only message in your  system  is

one   addressed  to  you,  the  beacon   transmissions   are

inhibited.   This  conforms to good  operating  practice  on

crowded channels (at least inhibiting the beacon does).

The mail beacon in the AMTOR Mode is transmitted as part  of

the beacon AUTOCQ message.  It is automatically updated when

a message is transmitted.  If you don't set the beacon  (see

below)  you  will not transmit a mail beacon  text  in  your

AMTOR CQ message.

The  Mail  beacon text is set up either when  you  tell  the

computer  to set it up or when you disconnect a  packet  QSO

from another station.

In this manner you may update the list while the program  is

running,  for  example  by using Sidekick  or  the  included

Editor, to write a message, or preferably the NOTE feature.

Any   station   using   LAN-LINK  can   be   configured   to

automatically  attempt  a connect (QTC-Snatch)  when  seeing

their callsign in someone else's LAN-LINK :QTC: beacon  mail

message list.

If  for some reason the message got lost (e.g.  because  the

link  was  marginal), :QSM: will get you a  repeat  of  that

message.

25.3  How to leave a message

You  can use the note feature or type the message as a  non-

document  file on Wordstar, the included Editor, or  if  you

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 95

use another word processor, you must leave the message as an

ASCII text file.

If  you  use the Note feature, the message will be  given  a

header  as  if it came in from someone else.  if you  use  a

word processor or the included Editor to write the  message,

you will have to write in a header or show somehow that  the

message  is  not  'live'.  You may then  later  time  tag  a

message  header  on  message  files  created  by  the   word

processor.   In this way people will realize that it  is  an

automatic message by seeing the header.  On the other  hand,

you may wish to fool people into thinking that you are home.

You  just  name  the message file as  the  callsign  of  the

station  to whom you wish the message to be sent at  connect

time.  You must also give it the filetype of 'OUT'.

For example a message for G3ZCZ would be stored on the  disk

as  a  file named 'G3ZCZ.OUT', and similarly  a  message  to

4X6AA would be stored on disk as '4X6AA.OUT'.

Once the message has been sent, an entry to that effect will

be made in the LAN-LINK.RUN  data capture file. The  message

file  will  then  be renamed and given  the  file  extension

'.OLD'.  At that time any previously existing mail file with

the  file extension '.OLD' will be deleted.  This stops  the

addressee  receiving the message on successive  connects  if

you  can't delete it for some reason (you forget)  while  at

the same time, the message is still available to you in case

you need it.

You  should use a separate utility program if you  want  the

calls  in the mail beacon sorted.  You may for example  wish

to  sort them alphanumerically or in the order of  the  date

that they were left in your answering machine.

Incoming  messages triggered by the QTC-Snatch will be  left

in a message file '<yourcall>.OUT', which can be scanned  by

using  the Alt-M Key combination. If you have  non  LAN-LINK

friends, train them to trigger the QTC-Snatch by  simulating

a  beacon,  and prefixing and ending their text  with  lines

containing the '>' as the only character.  Better yet,  give

them a copy of LAN-LINK and tell them to register it.

25.4 Remote Beacon Shutdown

I recognize that some stations may abuse the beacon facility

of  this  program.  I have thus built  into  the  program  a

'remote  shutdown' capability.  If someone running  LAN-LINK

has their beacon timer set too often, connect to them in the

AMTOR   or  Packet Communications modes, and  tell  them  to

:QRT:  which will clear their beacon mode and take them  off

the air for a while at least.

In the Packet Communication Mode, the CQ beacon is shut down

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 96

but the 30 minute mail beacon is not touched.  In the  AMTOR

Communication  Mode, the CQ delay time is set  to  infinity.

The  LAN-LINK  Mailbox  will thus still be  active  in  both

modes, but will not call CQ on the air.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 97

APPENDIX 1. ANOMALIES and BUGS

A1.1 Command/Converse Mode anomalies

Since  the Program has no way to check the command/converse  mode

status  of the TNC, you may see various Cmd: and  error  messages

from  the  TNC  when using the menus or  function  key  commands.

Ignore them for now.  If you get an error message, try using  the

function key again.

A1.2 Buffer messages in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ Mode, if the other station drops the  link

while  you are recording the text, BEFORE the callsign  has  been

recognized  as  a valid callsign, LAN-LINK will time  out  before

logging the QSO and restarting the CQ sequence.  If someone  else

calls  in  before the time-out has occurred, the text  from  both

stations  should be captured to disk, but the log entry  may  not

show  both  stations, thus the 'LOG' must be  compared  with  the

'RUN' file for QSO purposes.

LAN-LINK in the Beacon/Mailbox Mode, normally logs the contact at

the  end  of the line in which it recognized the  callsign  as  a

valid callsign.

A1.3 FEC +? responses in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ or Mailbox mode, if another station  sends

a  +? in the FEC Mode, the beacon will attempt to send  the  LAN-

LINK.TXT file.  The PK-232 however is not in the Converse Mode at

that  time and will interpret the text of the file  as  commands.

If  by some chance, the text corresponds to a valid  command  the

PK-232 will execute it and put the PK-232 in an unknown state and

you  may  not notice that it did.  As such I recommend  that  the

first  character  of  each line in the  file  LAN-LINK.TXT  be  a

non  valid command character such as a number or a period (.)  or

another punctuation sign.

A1.4 Using a CGA card with a monochrome monitor

If  you are using a CGA card with a monochrome monitor, the  text

you  type  into the OUTWINDOW will be fainter than  the  incoming

text.   This is because of the colors used.  If you are  using  a

Hercules  compatible  card you will not  notice  any  difference.

Change the colors using the Parameter Menu.

A1.5 The Packet Conference Mode

The  Conference  Mode has been tested on a PK-232, KAM,  a  SANYO

MBC-675  which  has a clock speed of 4.7MHz and other  PC  clones

with faster clock speeds.  If you have problems with intermittent

selection of I/O channels try slowing down your CPU clock.  There

is a software delay loop in the program that is used to allow the

TNC time to switch I/O streams after receipt of the stream change

command.   This  delay  is there because  LAN-LINK  tests  for  a

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 98

connect  on each channel before outputting any text, so  that  it

does not send unaddressed packets on non connected I/O channels.

A1.6 Packet Conference Mode disconnects

A  Packet  conference  Mode disconnect returns  the  TNC  to  the

Command  Mode, even if you remain connected to one or more  other

stations.

A1.7 Strange Events

If  the TNC does not use an interrupt driven serial port you  may

experience problems when using baud rates greater than 1200.  The

PK-232  DOES  NOT LIKE LAN-LINK to use RS-232 baud  rates  faster

than 2400, and sometimes not even that.  Use 1200 until YOU  have

tested and ARE SURE that a faster one will work.

IF YOU HAVE ANY STRANGE PROBLEMs AND ARE USING an RS-232 TERMINAL

BAUD RATE to the TNC WHICH IS GREATER THAN 1200, set the Terminal

Baud Rate to 1200 and see if the problem goes away.

Make sure that your configuration is set the way you think it is.

Verify  it  using the 'display deFault' (and the  Misc.  Flag  if

necessary) option(s) in the Parameter Menu.

A1.8 *** DISCONNECTED

In  the  Packet Communications Mode, when you  are  connected  to

someone and the TNC sends this text string to the computer,  LAN-

LINK thinks you have been disconnected and resets.  This normally

only happens when a true disconnect occurs.  However, if the text

shows up in the middle of a line being sent to you, it WILL  HAVE

THE SAME EFFECT AND THINK THAT YOU HAVE DISCONNECTED if the  DCD-

Flag is not set in the SYS file, and the TNC does not use the DCD

signal on pin 8 of the RS-232 interface.

A1.9 *** CONNECTED

In  the  Packet  Communications Mode,  when  you  are  monitoring

traffic  on  the  LAN someone sends this  text  string,  LAN-LINK

thinks you have been connected if the DCD-Flag is not set in  the

SYS file, and the TNC does not use the DCD signal on pin 8 of the

RS-232 interface.

A1.10 Greek Characters and other garbage in the Incoming Screen.

If  you see Greek characters in the Incoming screen you  probably

have  the wrong parity set in the TNC. first make sure  that  the

RS-232 baud rate is correct.  If it is, take the batteries out of

it and reset it.  If that fails, play with the parity.   LAN-LINK

sets  the serial port as configured in the TNC.SYS file for  each

TNC.   For example, for the PK-232 it is 8 bits, no  parity,  for

the  TNC2 it is 7 bits.  If you still can't get it to  work,  set

the interface to 7 bits in the configuration file.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 99

A1.11 Dumb TNC

If  the  TNC  seems to respond to commands  but  is  not  sending

anything back, you may have one of two problems.  You may have  a

bad RS-232 cable with on wire disconnected.  If the computer  can

operate  the TNC in another communications program, try to  reset

the  TNC, take the batteries out, etc.  You may also have a  not-

so-compatible PC.

APPENDIX 2. Local Area Network Capability

VHF  Packet  radio systems can be considered as part of  a  Local

Area  Network (LAN) in which messages can be left by one  station

in the computer belonging to a second station.  At HF the same is

true,  but the area becomes greater.  The basic problem  here  is

that  people  can only send and receive messages to or  from  you

when you are on-line.  To compensate for this, BBS stations  were

developed which allowed both messages and bulletins to be  stored

by anyone for later retrieval.

LAN-LINK  allows you to store messages in your computer so as  to

use  it  as  an  automatic answering  machine.   By  storing  the

messages in a file called by the name of callsign, with the  file

extension  '.OUT',  anyone connecting to the  computer  will  get

their  message  without  having to request  it.    LAN-LINK  also

extends  that concept to cover the eventuality of you wanting  to

take  your computer off-line for some reason.  You may  now  load

the  message  into any other computer you can  connect  to  using

elements  of  the  Q  code adapted  to  the  High  Level  Network

Communications  Language (NC/L) proposed in my book Software  for

Amateur  Radio, (Book number 1560) published by TAB  Books,  Blue

Ridge Summit, Pa. 17214 U.S.A.

A2.1 LAN Protocol (G3ZCZ Version)

The  protocol  is as follows. When connected to someone  who  has

their  LAN-LINK  configured  as a host, if you want  to  store  a

message  you send the following instruction to the other  station

:QSP:  <callsign>  where  <callsign> is the call of  the  station

that the message is for, not the callsign of the host station  in

whose  computer you are storing the message. [Note use  only  one

space  character  after  the :QSP:].  LAN-LINK  sends  the  :QSP:

message automatically using QSP option in the Message Menu.

For  example if you want to store a message for 4Z4ZB in  4X6AA's

computer  which is configured as a Store and Forward system,  you

would first connect to 4X6AA and then send the command as

:QSP: 4Z4ZB .

Better  still use :QSP: option in the Lan-Link Menu  to  automate

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 100

the sequence.

The computer at 4X6AA will respond either with a statement saying

that  it is ready for you to go ahead, or send a  message  saying

that  it can't comply.  If it is ready you get a  positive  reply

which will take the form :QRV:  <callsign> which if you know  the

Q code, means " I am ready to accept a message for  <callsign>".

At this time you may go ahead and send the message.  If you  type

the message at the keyboard, you may use either a control Z  (^Z)

character or the character sequence :EOF: followed by a  carriage

return  (the  ENTER key) to terminate the message.  If  you  have

first  written the message into a text file you may then send  it

using  the  'Send  File'  option  of  the  Files  Menu  and   the

termination sequence is automatic.

Once  you have completed the message, the other  (host)  computer

will  either reply that the message has been successfully  stored

or give you an error message.

If  the  message  is stored and ready to be sent  next  time  the

addressee  connects  to that computer, you will see  the  message

:QSL: on your screen.  If something went wrong, you will get back

a  negative message taking the form :QNO: followed by  a  number.

The number tells you why the operation failed.

A2.2 Message Format

The  message is stored just as if you had left it in your  system

(except that a header is added identifying the time of  reception

and  the call of the sending station).  It will contain a  header

showing  the  date and time it was received  and  your  callsign.

Should a message for that station already be in the system, yours

will be appended to it.  In the event the your upload is aborted,

the  amount  of text received before the abort occurred  will  be

stored as the message.

When  you  disconnect from the other LAN-LINK host  station,  its

mail beacon will be updated.

Once the message is loaded in the host, it can only be deleted by

the operator of the host station.  When the addressee connects to

the host and receives the message, the file type will change from

'.OUT' to '.OLD'.

A2.3 File Uploading/Downloading

There  comes a time when you want to leave a file on your  system

for someone to download later.  You can do this by using NC/L  to

control the transfer.  You must leave the file in a  subdirectory

defined  in  the LAN-LINK.SYS FILE called  \LAN-LINK\FILES.   You

must first create it if it does not exist.  The station who wants

the message just asks for it using :QBM: <filename>, note they do

not  need to use the full subdirectory path.  They  must  however

leave  a single space character between :QBM: and  the  filename.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 101

In AMTOR the Mailbox must be turned on for remote functions.

For  example, you have the latest ARRL DX bulletin, and you  want

to  pass it on.  You could pass it to selected people by  copying

the  file to individual messages with the filetype  '.OUT',  e.g.

4X6AA.OUT, 4Z4ZB.OUT or even G3ZCZ.OUT if you want me to get  it,

which wastes a lot of disk space.

On  the  other  hand  you could tell people  that  the  file  was

available  for downloading, either in the CTEXT  connect  message

line  which everyone gets when connecting to you  by  configuring

LAN-LINK accordingly, or in individual ('.OUT') messages.

If you include the following in your CTEXT message

"ARRLDX.015 now available"

people will know that to download it, they just have to type  the

line  ':QBM: ARRLDX.015' (one space character between  :QBM:  and

the  file name) to get the file.  LAN-LINK is not designed  as  a

BBS,  however,  if you copy the contents of  the  \LAN-LINK\FILES

subdirectory  into  a  file called DIR (USE DIR >  DIR  in  DOS),

anyone  can  get a list of the files you have stored  in  it  for

downloading by typing :QBM: DIR.  Since they do not get the  real

directory, just what was in it at the time you copied it into the

file  'DIR',  you may keep "hidden" files in  this  subdirectory.

You may thus leave a file for someone to download, tell them that

its  there  by  leaving  them a  message  (which  they  will  get

automatically when they connect) and no one else connecting  will

know that it is there.

:QDB: allows you to upload text or ASCII files.

If  you  use the 'lan-linK' option of the Packet  Terminal  Menu,

LAN-LINK will automate the QDB and QBM features for you.

A2.4 Path Determination to a Dx station

If  you want to establish a digipeat path to a  station  somewhat

out of your direct range, you need to know which of the  stations

that you can connect to can hear that desired DX station.  If you

could  get a call monitored (MH list) from the stations that  you

connect  to,  you  would be able to see if the  station  you  are

connected to has heard your desired DX station.

LAN-LINK  uses  the :QMH: command to request such a  list.   When

LAN-LINK   receives  a :QMH: command, it drops back  to  the  TNC

Command  Mode and issues a 'MH' command to the TNC (except  to  a

TNC1 which does not support it).  It stores the list received  in

a  temporary  buffer, and then returns to the Converse  Mode  and

transmits the list back to the station it is linked to.

By  judicious  use  of :QMH: you can  determine  paths  to  other

stations.   Note however, that just because one station can  hear

another  station,  it  does not mean that it can  work  it.   For

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 102

example,  the station you are connected to may be using  a  power

level  of 1 watt or so, while the station 200 miles away that  it

heard was using 100 watts.  Test the path yourself, or/and  leave

a message asking how reliable the connect path between those  two

stations is.

If  you and a small group use a non standard packet  channel  for

inter-group  communications, you may also use the :QMH:  function

to  find  out who the station you are connected with,  has  heard

recently, when you first sign on the LAN channel.

APPENDIX 3     BASIC NC/L DICTIONARY

Messages can be transferred into any other LAN-LINK computer  you

can connect with using elements of the Q code adapted to the High

Level Network Communications Language (NC/L) proposed in my  book

Software  for Amateur Radio, (Book number 1560) published by  TAB

Books, Blue Ridge Summit, Pa. 17214 U.S.A.

NC/L  command words are summarized in the  following  paragraphs.

There  is  no command word to request your message.   To  receive

your message, you do nothing, you will receive your mail automat#

ically  when  connecting/linking with a  LAN-LINK  station.   You

cannot normally read messages addressed to another person.

A3.1 :QBM:

To download a file, send

:QBM: filename.type

The filename.type is the file you want. For example

:QBM: dir

:QBM: help

:QBM: LAN-LINK.doc

A3.2 :QDB:

To upload files to another LAN-LINK system send

:QDB: filename.type

Files  will be uploaded into the same sub-directory as  LAN-LINK,

and will have a message header inserted at the top of the file to

allow  the  operator  to  know who uploaded  the  file.   If  the

operator  then wants to make the file available for  downloading,

he or she can move it to the \LAN-LINK\FILES sub-directory.   The

'.TYP' following the filename is optional.

If  you attempt to upload a file which has the same name  as  one

that  already  exists on the remote system, you will  receive  an

'error' message.

This command is the reverse of :QBM:.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 103

A3.3 :QMH:

To request a call monitored list ('MH') from the LAN-LINK station

that you are connected with, send

:QMH:

A3.4  :QSM:

To  request a repeat of a message from a station using  LAN-LINK,

send

:QSM:

This  command will be valid as long as the host station  has  not

deleted  the <yourcall>.OLD file on his disk.  If the  file  does

not exist, you will receive a :QNO: 2 'error' response.

A3.5  :QSP:

To leave a message, send

:QSP: callsign

The  callsign is that of the station you wish to leave a  message

for.

A3.6 :QRA:

When  disconnected, to see what stations (using  LAN-LINK  1.53+)

are on the LAN, or which LAN-LINK stations have messages pending,

send

:QRA:

All LAN-LINK stations will reply with a mail beacon within a  few

seconds.

A3.7 :QRT:

To shut down an AMTOR/Packet mode Mailbox/beacon station which is

causing QRM, link or connect to the station and send

:QRT:

A3.8 :QRU:

To  upload messages from one LAN-LINK/PK232COM (1.38+) system  to

your computer, send

:QRU:

You  may  only use the QRU function with stations  designated  as

Store and Forward mailboxes.  Put a list of stations that you can

connect  to reliably in your <callsign>.MBX file and send  it  to

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 104

other  stations in your LAN also using LAN-LINK.  They will  then

be  able to dump their mail messages on you and you on  them.  If

they  do the same and send you their file, then you will be  able

to send messages via them for stations that they can work.

While  QRU  gives you the capability to bulk upload  messages  to

another  station in your local area, when you take  your  machine

off  line, it may also be used to transfer messages  between  two

LANs via well sighted gateway digipeaters.

A3.9  :QNO:

':QNO:' and error message text is a response to a request.

A3.10 :QJG: is a response to :QRU:

It confirms that the QRU sequence is complete.  There are no more

messages pending.

A3.11 :QRV: callsign is a response to :QSP: callsign.

The computer is ready to send the message. End the message with a

control Z (^Z) character, or the sequence :EOF:

A3.12 :QSL: is a response to a command

It  confirms receipt of message to that callsign or that  a  file

has  been successfully uploaded and stored to disk.  It  is  also

used to acknowledge receipt of a ':QRT:' command.

A3.13 :QTC: massage list

If  LAN-LINK receives :QTC: it will respond with the beacon  text

showing the mail queue.

:QTC:  precedes a list of callsigns for whom messages are  stored

up  on a computer.  It is used in Packet Beacon transmissions  or

AMTOR Beacon mode CQ calls.

A3.14 :QBU: External Protocols

This  feature is supplied to allow experimentation with  external

protocols.

PROPOSED EXTENSIONS

A3.14 :QYU:     YAPP format file upload.

A3.15 :QYD:     YAPP format file download.

APPENDIX 4. USING OTHER TNCs

The PACKET Mode capabilities can be used on all TNCs.  There are

minor  differences  in  the command  dialogue  between  different

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 105

TNCs.  The dialogue differences affecting the operation of  LAN-

LINK are built into the program and set when the TNC type is read

from  the  LAN-LINK.SYS file at the program load time.   The  non

packet modes will obviously result in error messages if attempted

on a packet mode only TNC.

The   following  notes  apply  to  different  TNCs  or   digital

communications controllers.

A4.1 TNC2 (MFJ 1270) Version 1.1.2

A4.1.1 Default Commands

Rename  the  LAN-LINK.SYS  file as  PK232.SYS.   Rename  the

TNC2.SYS  file as LAN-LINK.SYS and edit it using  your  word

processor  in its non document mode to customize it to  your

station.

A4.1.2    Program differences

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

Software controlled HF/VHF switching.

Software controlled baudrate switching.

CQ/BEACON Packet terminal mode.

TNC Transmit Buffer flush.

A4.2 TNC1 (HD-4040)

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC.

A4.2.1     Default Commands

Rename  the  LAN-LINK.SYS  file as  PK232.SYS.   Rename  the

TNC1.SYS  file as LAN-LINK.SYS and edit it using  your  word

processor  in its nondocument mode to customize it  to  your

station.

Ignore error messages at start up.

A4.2.2     Program differences

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

Day/Time functions.

'MH' and remote :QMH: functions.

MAIL Packet Terminal Mode.

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

Conference and multiconnect modes.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 106

A4.3 Kaypro KPC-2

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC.

A4.3.1     Default Commands

Rename  the  LAN-LINK.SYS  file as  PK232.SYS.   Rename  the

KPC.SYS  file  as LAN-LINK.SYS and edit it using  your  word

processor  in its nondocument mode to customize it  to  your

station.

A4.3.2     Program differences

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

MAIL Packet Terminal Mode.

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

A4.4 KAM

LAN-LINK switches colors when it sees the /V or /H following  the

callsign  in the packet header, or in the calls monitored  ('MH')

list.  If you are monitoring both HF and VHF simultaneously,  the

colors  will show you which radio you heard which signal on.   If

the  Mail-Snatch or the QTC-Snatch are triggered,  LAN-LINK  will

change  modes  and attempt the connect on the correct  radio  but

will  not  change the band/power information or  the  logbook  to

reflect  that change in the logbook or in the status window  dis#

play.

A4.4.1     Default Commands

Rename  the  LAN-LINK.SYS  file as  PK232.SYS.   Rename  the

KAM.SYS  file  as LAN-LINK.SYS and edit it using  your  word

processor  in its nondocument mode to customize it  to  your

station.

A4.4.2     Program differences

The following functions DO NOT WORK because the TNC does not

support them.

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

The NAVTEX Communications Mode.

The Alt-D key in the MORSE Communications Mode.

The AMSAT-OSCAR Menu does not allow UoSAT option, as the KAM

does not have 1200 baud ASCII capability.

The BAUDOT NAVY MARS Protocol is also not available.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 107

A4.5 PK-232 EPROM Version 30 DEC 1988 and subsequent ones.

Turn   the  'BBS  OFF'  if  you  want  the  Alert  Call,   packet

communications mode header colors and Digipeat Detect features to

work otherwise turn it on.

A4.6 MFJ 1278

LAN-LINK  assumes  that  the Radio 1 port  is  connected  to  the

VHF/UHF packet transceiver, and the Radio 2 port is connected  to

the HF transceiver with the multi mode communications capability.

To transmit CW turn the VOX on.

The MFJ1278 suffers from one major deficiency as far as  LAN-LINK

is concerned.  The Echo-As-Sent only works in the AMTOR  Communi#

cations Mode.  That means that you will not see the contents of a

file  echo in the incoming window as the file is being  transmit#

ted.

A4.6.1     Default Commands

Rename  the  LAN-LINK.SYS  file as  PK232.SYS.   Rename  the

MFJ1278.SYS file as LAN-LINK.SYS and edit it using your word

processor  in its nondocument mode to customize it  to  your

station.

A4.6.2     Program differences

The  following functions DO NOT WORK because either the  TNC

does  not  support them or does not do so  in  a  consistent

manner.

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

The AMSAT-OSCAR Menu does not allow UoSAT option.

The BAUDOT NAVY MARS Protocol is not available.

CW Identification.

The  NAVTEX communications mode.  (MFJ1278 allows,  but  not

set in LAN-LINK).

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 108

APPENDIX 5 UPDATES/REVISION HISTORY

1.51 :QRA:  and  Lan-linK menu added.  LAN-LINK.DIR  scan  during

BBS-Snatch  bug and CW receive bugs fixed. Changes in  AMTOR

Robot text.

1.52 Code  rewritten  changing variable types.  Fixed  BBS-Snatch

time-out  and  :QSP:  menu bugs. Fixed  TNC2  Terminal  mode

display  bug. Typing speed much faster. Added  Multiple  TNC

capability, AMTOR colors, packet communications mode  header

colors,  T, V and U options in Files Menu, Function  Keys  2

and  4,  Alt- A, B, E, F, Q, S, and Z, AMTOR  ARQ  Call/talk

option,  LAN-LINK.001-010 disk files (and  Alt/Shift/Control

functions keys supporting them), AMTOR ROBOT infinite delay,

Satellite  TLM Menu, '*' menu exit key function, and  lastly

'Oops'  in Communications menu.. Number of Scan  words   in#

creased  to  16.  Help screen  expanded.  BBS.BBS  file  now

renamed numerically like incoming mail after 'Zap'.  Changes

in  AMTOR  ROBOT call recognition and  operation  when  link

station drops link in state 15 (PK-232 only).

1.53 TNC2   beacon setup bug fixed.  1.52 QTC Beacon  bug  fixed.

MFJ1278,  Alt-P added. Packet header colors added  for  TNC2

and MFJ1278.  F, X and Z added to Log Menus.  L and X  added

to Parameter Menu.  Time Display and Event/Clock Menu added.

Changes in AMTOR Robot call recognition and timing.

1.54 Customized colors, Alt-N and J hot keys and Edit Menu added.

Alt-D  key  and F8 functions separated.  Changes  in  bottom

line  prompts, Parameter menu, AMTOR Robot call  recognition

and  packet communications mode multiple  connect  handling.

More  parameters  in  LAN-LINK.SYS  and  customization  from

within LAN-LINK added.  SAREX capture bug fixed.

1.55 Changed  to Turbo Pascal 5.5. Initialize calls,  :QDB:  long

file transfer and Sound display bugs fixed. F3 and F8  turns

Robot  off.  Expanded  contest operation.  KAM  AMTOR  Robot

drop-link  added. KN fixed in CW. RTTY and Navy MARS  Baudot

File Transfer Bug.

1.56 RTTY and Navy Mars File transfer bug, Command Echo bug,  and

Disconnect  Capture-to-Disk bug fixed.  Alt-K and  Alt-Y,  I

and   L   options  in  Parameter  Menu,   :QBU:,   Automatic

packet/AMTOR  contest mode, customizable BBS  Bulletin  read

command and PacketCluster added.  J option in Parameter Menu

and  'RY's in RTTY CQ call deleted. Callsigns  converted  to

upper case. CW speed now shown in status window,  left/right

arrows change CW speed by 1 wpm. Sound flag kills ^G bell on

incoming text. /EX or  /ex as well as :EOF: can now be  used

in  *.bbs file column 1. PC to TNC now uses  hardware  hand#

shake  (^S/^Q  deleted).  SAREX Orbiter  menu/functions  and

Metabeacon  interface,  and   U option  in  Parameter  Menu#

changed. Logs split to HF and VHF.

1.57 (Coming Soon) dBASE compatible logbook files.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 109

APPENDIX 6 LAN-LINK 1.56 REGISTRATION FORM

To: Joe Kasser G3ZCZ

P O BOX 3419

SILVER SPRING

MD 20918.

CALL ________________   TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY    ___________________________________________

STATE   _________  POSTCODE  ____________ TELEPHONE ____________

TNC TYPE _________ FIRMWARE REV __________  HOME BBS ___________

DISK SIZE 5.25 _____ 3.5 ____ Other ? _____________

I  enclose  a check for $35.00. Please register me as a  user  of

LAN-LINK.  I am currently  using Version ______  which I obtained

from  _________________________________________.

Please send me the latest version of LAN-LINK or if a more recent

one  does not exist at this time, QSL my registration and add  my

name  to  the  list  to receive a free  update  when  it  becomes

available.

Signature

My favorite operating modes are ______________________________.

Additional  Features I'd like to see in future releases  of  LAN-

LINK.

Keep Conference Bridge Up  __ YAPP Binary File compatibility   __

Expanded Contest Operation __ More AMTOR (explain below)       __

NTS capability             __ Picture Transfer capability      __

Telephone Modem capability __ Logbook file compatible to DBASE __

DX Cluster Menu (Like BBS) __

Others

NOTE:  some  additions  come into LAN-LINK as a  result  of  user

suggestions, so here's your chance to get some input in.

Comments, likes, dislikes, wish list, etc.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 110

APPENDIX 7 LAN-LINK PROBLEM REPORT

Name ______________________________________  CALL _______________

Street __________________________________________________________

City _______________________________  State _____  ZIP __________

Tel. ____________________  LAN-LINK version ____

1.  Describe your computer system.

Manufacturer ____________________ Model ___________TNC __________

Firmware _____

DOS Version  ________________  Clock Frequency _________ MHz.

List peripheral equipment and cards installed in your computer.

Video Herc/EGA/CGA_____  _______________________

_______________________  _______________________

_______________________  _______________________

What  memory  resident programs were installed when  the  problem

occurred?

_______________________  _______________________

_______________________  _______________________

2.   Describe  the problem in detail. What were  the  Misc.  flag

state-sequence numbers when the problem was happening?

3.  Describe the sequence of events or keystrokes that led to the

problem.

4.  Any other information that may help in locating the problem.

Use  additional sheets of paper if necessary.  ATTACH a  copy  of

your  LAN-LINK.SYS  file.

Mail to:  Joe Kasser, P O Box 3419, Silver Spring, MD. 20918.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 111

APPENDIX 8 Other PC Software by Joe Kasser G3ZCZ

8.1 PC-HAM 3.5 BY G3ZCZ

LOGBOOK

Full  blown logging package.  With automatic check  of  logs

for awards such as DXCC.  Allows you to recall any entry  by

call sign within seconds. Indexed displays, QSL'ing, Contest

mode QSL'ing (prints the lot) and lots more. Although  writ#

ten  in  dBASE3  the package  contains  a  compiled  version

(LOGBOOK.EXE),  so  you  don't need dBASE to  run  it.   The

source  code is given to registered users.  It  can  convert

your exported LAN-LINK.LOG file to LAN-LINK.DBF to put  this

package  to  work.  Ideal for DX-peditions or for  DX  robot

users to handle QSLing and log statistics.

CONTEST

Keeps  Dupes in memory, logs QSO's to disk in  format  which

can be processed by the LOGBOOK package.  Needs BASIC.

CQSS

Sweepstakes   game   written  in  BASIC.   Work   the   ARRL

Sweepstakes contest on your computer.  You are located  just

outside  Washington DC.  A propagation model is built in  to

the  program.   This program is REQUIRED  training  for  all

sweepstakes  operators.  Earlier version of the  program  is

described  in detail in 'Software for Amateur Radio' by  Joe

Kasser  G3ZCZ, published by TAB Books, Blue  Ridge   Summit,

PA. 17214.

WHATSON

Predict  HF  Propagation for given days. Contest  mode  with

printout to whole world at hourly intervals. Needs BASIC.

This  software  AND MORE comprises PC-HAM which is  available  as

Shareware  from G3ZCZ.  Send a blank formatted disk, SASE, and  a

QSL card for a copy or save time and money and register as a user

for  $36.50 (half of $73).  Request PC-HAM and specify disk  size

(3.5 or 5.25 inches).

8.2 STARTREK The Computer Program

An  ideal  task  for the beginner to learn a  language  on  is  a

simulation  game  which is written around the computer  that  the

beginner  has  available.  For in that case,  there  is  complete

control  of all inputs and outputs,   This kind of game in  which

the player makes decisions based on the information available  to

him  or  her  available at the time), can  be  made  sufficiently

sophisticated  and complex so as to make writing it  an  adequate

challenge for anybody.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 112

The  techniques  used in writing a good game are  the  same  that

programmers use in professional activities.  Writing a good  game

poses  a  challenge that allows you to develop  good  habits  and

techniques  for  programming  and  also allows  you  to  learn  a

language in an interesting manner.  By taking an orderly approach

to the game design, complex operations may be clearly  understood

and  converted  to  computer  code with the  aid  of  a  language

reference manual, irrespective of the language being used.

This product teaches the techniques for writing such a game using

the  STARTREK game as an example, and the BASIC language  as  the

programming language in which to write the code.

Registration fees.

Single   Copy   $15.00

10 - 50  Copies $12.00 per registered copy.

50 - 100 Copies $10.00 per registered copy.

100 +    Copies  $8.00 per registered copy.

This  is  available  as  Shareware  from  G3ZCZ.   Send  a  blank

formatted disk and an SASE for a copy or save time and money  and

register  as a user by sending a check for the  registration  fee

according  the  schedule posted above.  Request  Startrek  -  The

Computer Program and specify disk size (3.5 or 5.25 inches).

8.3 WHATS-UP 0.42

This version of WHATS-UP is a concept demonstration version of  a

Telemetry   Decoding  and  display  Program  for   the   MicroSat

Spacecraft (OSCARs 16, 17, 18 and 19). It is table driven via the

configuration files to allow maximum flexibility.

This  version  does not do binary telemetry  decoding.   It  only

decodes the ASCII format. It also contains a few cosmetic display

bugs. I expect (but don't promise) to put out Revision 1.00  with

more  features,  (such  as automatic  spacecraft  parameter  file

switching   to   match   spacecraft,   scheduled   events,    TNC

configuration  files, etc.) and proper documentation once  binary

telemetry formats have been standardized and stabilized.

WHATS-UP contains the following features:

* Real time and Playback modes.

* Automatic Capture-to-disk of raw telemetry.

* Extracts predetermined telemetry channel data to a  database or

spreadsheet readable file for further analysis.

* Link quality measurement.

* Capability to print the raw telemetry as it is received.

* Up  to  16 user configurable display pages (screens).  You  set

the position on the page (width of engineering unit field,  and

number of decimal places) that a parameter is displayed  at.

* Wild card page (parameter shows up on all pages).

* Selectable  display of Engineering units or Hex byte  for  each

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 113

display page.

* Display of raw packets (i.e. STATUS)

* Color changes if a parameter value changed between   successive

frames.

* Audio  and  visual alarms if a telemetry value  exceeds,  falls

below or falls outside a preset limit value(s).

* Dumb split screen terminal mode (a la LAN-LINK).

* Customizable colors, PC to TNC baud rate, data parity and  stop

bits.

* Default spacecraft configuration files.

* Time of day clock display (in HH:MM:SS format)

If you would like a copy of WHATS-UP send in a FORMATTED disk and

sase.  If  you have captured any telemetry  to  disk,  especially

daytime  or  southern hemisphere passes, please put them  on  the

disk.  Don't let the data files become too long. I  suggest  that

you rename them every couple of days or so, such as DOVE2-16.RUN,

DOVE2-18.RUN, etc.

Suggestions for improvements and additions are always welcome.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 114

APPENDIX 9 HOW SHAREWARE WORKS:

The  Association of Shareware Professionals (ASP) has established

standards for its members and for any organization which has "ASP

Approved"  status. The ASP wants to make sure that the  shareware

principle  works for you. If you are unable to resolve a  problem

with  an  ASP  member  or  organization  (other   than  technical

support),  the ASP may be able to help. Please write to

The  ASP Ombudsman, P.O. Box 5786, Bellevue, WA  98006, USA.

You  are encouraged to copy the floppy disk and share  it  freely

with  others.  You have the luxury of trying out the  product  at

your own pace and in the comfort of your own home or workplace.

After  you  have used the material for  a  reasonable  evaluation

period  (90  days),  you should either  discontinue  use  of  the

material  or register your copy.  Your support is  important  and

greatly  appreciated.  With it, Shareware authors are  encouraged

to  design and distribute new products. Without it, a great  deal

of high quality, low cost software will cease to be available.

Why pay at all?

*  You receive support from the author.

*  You receive a CURRENT copy of the program.

*  Your input and ideas help shape future products.

*  A sense of pride and ownership in having honestly

participated in the Shareware revolution.

*  You help to keep software prices down by supporting a

distribution method which doesn't depend on expensive

advertising campaigns.

Be  aware of the following restrictions, designed to protect  the

community  of Shareware users and to prevent greedy  people  from

taking unfair advantage of the trust, hard work and good will  of

Shareware authors.

1.   No  price or consideration may be charged for the  material.

However, a distribution cost may be charged for the cost  of

the diskettes, shipping and handling, not to exceed $6.

2.   The  files and programs on the disks may not be modified  or

deleted.

3.   The  material  cannot  be sold as part of  some  other  more

inclusive package.

4.   The material cannot be "rented" or "leased" to others.

5.   The end user must be told clearly in writing on the  outside

of  the package and in all advertising that the  diskette(s)

are "Shareware."

6.   The package must contain a written explanation that the disk

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 115

is   for  evaluation  purposes,  and  that   an   additional

"registration  fee"  is  expected  by  the  author,  if  the

material is used beyond an initial evaluation period.

7.   In the case of distribution via any telecommunications link,

the following must be done:

An error checking protocol must be used.

The individual files must be combined into, and  transferred

in a library or archive format.

8.   Shareware  distribution  is  permitted only  in  the  United

States, Canada, England, and Australia.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 116

INDEX

ACHG, 6, 78

Alarm, 3, 22, 23, 27, 38, 39, 69, 79

Alarms, 113

Alarmwindow, 5, 69

Alert, 2, 5, 7, 12, 20, 22, 27, 29, 51, 57, 69, 70, 84, 85, 107

ALFD, 72

ALIST, 6, 73, 74

ALISTEN, 6, 78, 79

Alt-A, 7, 84

Alt-B, 7, 84

Alt-C, 7, 15, 20, 77, 84

Alt-D, 7, 17, 38, 57, 73, 74, 83, 84, 106, 108

Alt-E, 7, 20, 84

Alt-F, 7, 24, 84, 93

Alt-J, 7, 84

Alt-K, 7, 86, 108

Alt-L, 7, 60, 86

Alt-M, 7, 59, 86, 95

Alt-N, 7, 76, 86, 108

Alt-O, 7, 16, 17, 52, 60, 86

Alt-P, 7, 86, 108

Alt-Q, 7, 17, 86

Alt-R, 7, 87

Alt-S, 7, 60, 68, 87

Alt-X, 7, 14, 15, 87

Alt-Y, 7, 87, 108

Alt-Z, 7, 33, 87

AMSAT, 52

AMSAT-OSCAR, 4, 12, 25, 50, 51, 52, 106, 107

AMTOR, 2, 6, 7, 8, 10, 11, 12, 13, 14, 16, 17, 18, 19, 20, 22,

23, 24, 25, 27, 28, 31, 39, 43, 45, 55, 56, 60, 62, 63, 68,

70, 73, 74, 78, 79, 84, 85, 86, 87, 89, 94, 95, 96, 97, 101,

103, 104, 107, 108, 109

AMTOR-FEC, 18

APPENDIX, 8, 9, 15, 17, 42, 43, 44, 97, 99, 102, 104, 108, 109,

110, 111, 114

ARQ, 2, 6, 7, 22, 27, 28, 74, 78, 79, 84, 85, 108

Arrows, 26, 88, 89, 108

ASCII, 2, 4, 6, 7, 10, 11, 12, 13, 17, 18, 35, 41, 46, 50, 73,

82, 85, 87, 88, 92, 93, 95, 101, 106, 112

Attack, 4, 51

Attempt, 22, 26, 27, 28, 29, 30, 40, 58, 61, 94, 97, 102, 106

AutoCQ, 19, 94

Automatic, 2, 4, 11, 12, 13, 16, 17, 18, 20, 22, 23, 24, 39,

45, 55, 60, 62, 72, 81, 82, 86, 95, 99, 100, 108, 111, 112

Automatically, 11, 12, 14, 21, 28, 33, 41, 45, 58, 63, 76, 77,

89, 91, 94, 99, 101, 102

AUTO-Upload, 92

Backslash, 53

Backspace, 35

Band, 5, 14, 18, 28, 45, 46, 47, 56, 65, 90, 106

Batteries, 15, 49, 98, 99

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 117

Baud, 5, 6, 7, 14, 15, 17, 50, 51, 58, 67, 68, 73, 74, 75, 80,

82, 88, 90, 98, 106, 113

BAUDOT, 2, 6, 10, 11, 13, 18, 19, 51, 72, 73, 80, 85, 87, 88,

106, 107, 108

Baudrate, 7, 18, 88, 105

BBS, 1, 2, 3, 4, 5, 7, 12, 15, 18, 25, 26, 27, 31, 32, 33, 34,

37, 38, 39, 40, 48, 52, 54, 55, 61, 62, 66, 67, 71, 72, 75,

85, 87, 89, 90, 92, 93, 99, 101, 107, 108, 109

BBS's, 34, 62

BBS-Snatch, 108

Beacon, 4, 8, 11, 12, 13, 19, 23, 27, 32, 33, 38, 45, 50, 51,

52, 53, 54, 55, 56, 57, 58, 59, 61, 62, 75, 79, 90, 94, 95,

96, 97, 100, 103, 104, 105, 106, 108

Beacons, 6, 19, 50, 51, 53, 54, 56, 74, 75

Blank, 22, 26, 44, 45, 46, 47, 72, 81, 111, 112

Blink, 22, 52

BottomWindow, 5, 69

BTEXT, 57, 94

Buffer, 2, 4, 7, 8, 11, 17, 23, 24, 40, 49, 56, 66, 77, 84, 85,

87, 88, 97, 101, 105, 106, 107

Bug, 18, 108

Bugs, 8, 97, 108, 112

Bulletin, 18, 62, 72, 101, 108

Bulletins, 12, 62, 72, 75, 99

Call, 2, 3, 4, 5, 7, 12, 13, 15, 16, 19, 20, 21, 22, 23, 25,

26, 27, 28, 30, 31, 32, 35, 36, 37, 38, 39, 46, 47, 51, 52,

53, 56, 60, 61, 62, 63, 64, 65, 69, 70, 74, 75, 77, 78, 79,

80, 81, 82, 84, 85, 86, 87, 90, 91, 92, 96, 99, 100, 101, 103,

107, 108, 109, 110, 111

Caller, 13, 55

Calls, 4, 5, 19, 20, 22, 24, 26, 28, 32, 46, 49, 53, 54, 55,

61, 62, 64, 69, 77, 79, 83, 84, 86, 90, 94, 95, 97, 104, 106,

108

Callsign, 2, 4, 6, 12, 14, 15, 16, 18, 20, 21, 22, 23, 26, 27,

28, 29, 31, 32, 38, 41, 43, 44, 45, 46, 48, 51, 58, 59, 60,

61, 62, 63, 71, 74, 75, 76, 78, 79, 81, 82, 87, 90, 91, 92,

94, 95, 97, 99, 100, 103, 104, 106

Callsigns, 4, 55, 60, 62, 104, 108

Capture, 2, 4, 5, 6, 12, 18, 19, 20, 26, 35, 51, 55, 58, 64,

71, 81, 82, 83, 88, 90, 91, 95, 108

Captured, 16, 88, 97, 113

Capture-to-Disk, 3, 4, 16, 18, 20, 21, 38, 52, 55, 56, 62, 71,

81, 82, 88, 90, 108, 112

CFROM, 66, 83

CGA, 8, 97, 110

CHIRPCOPY, 6, 13, 24, 73, 74

Choice, 10, 26, 27, 35, 39, 40, 42, 44, 48, 49, 50, 55, 66, 73,

74, 78, 80, 82, 83, 86, 87

Choices, 25, 27, 35, 40, 44, 48, 49, 55, 68, 74, 78, 80, 82,

83

CMD, 2, 22, 24, 97

CMSG, 16, 53, 54, 56, 66, 67

Color, 5, 6, 13, 20, 68, 69, 70, 71, 76, 113

Colors, 5, 12, 14, 55, 68, 71, 86, 90, 97, 105, 106, 107, 108,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 118

113

COM, 67, 89, 91, 103

Command, 4, 8, 10, 12, 13, 15, 16, 17, 20, 21, 22, 23, 24, 26,

29, 30, 31, 32, 33, 34, 40, 41, 42, 43, 44, 45, 46, 48, 49,

50, 53, 54, 55, 57, 59, 60, 61, 62, 64, 65, 69, 72, 75, 78,

80, 81, 82, 83, 86, 87, 88, 89, 90, 92, 93, 94, 97, 98, 99,

101, 102, 103, 104, 108

Communications, 1, 2, 5, 6, 10, 11, 16, 17, 18, 19, 20, 23, 24,

25, 27, 28, 30, 31, 32, 33, 38, 40, 50, 51, 55, 56, 57, 58,

63, 66, 67, 68, 69, 70, 72, 73, 74, 78, 80, 82, 83, 84, 85,

86, 87, 88, 89, 90, 95, 98, 99, 102, 105, 106, 107, 108

Computer, 5, 10, 17, 28, 40, 41, 43, 44, 48, 49, 56, 59, 67,

68, 69, 76, 90, 94, 98, 99, 100, 102, 103, 104, 110, 111, 112

CONFIG, 7, 89, 92

Configuration, 2, 10, 11, 15, 16, 17, 30, 38, 42, 46, 49, 52,

63, 71, 72, 74, 76, 89, 98, 112, 113

Connect, 2, 3, 4, 7, 11, 12, 13, 15, 18, 20, 22, 23, 26, 27,

28, 29, 30, 31, 32, 33, 38, 39, 40, 41, 44, 45, 49, 50, 51,

54, 55, 56, 57, 58, 59, 61, 66, 75, 76, 77, 79, 83, 84, 85,

86, 87, 91, 92, 94, 95, 98, 99, 101, 102, 103, 106, 108

Connected, 5, 8, 11, 12, 19, 20, 26, 28, 29, 33, 42, 43, 44,

52, 58, 59, 61, 63, 68, 69, 70, 71, 74, 76, 77, 78, 92, 98,

99, 101, 102, 103, 107

Connectee, 38

CONOK, 54, 66

Control-F, 93

Control-Z, 7, 89

CONVERSE, 2, 8, 10, 23, 24, 42, 44, 50, 59, 77, 78, 79, 97, 101

Copy, 1, 13, 22, 33, 36, 50, 79, 80, 81, 82, 95, 101, 110, 111,

112, 113, 114

CQ, 2, 5, 6, 8, 13, 16, 17, 19, 20, 22, 23, 24, 27, 31, 32, 49,

55, 65, 74, 75, 76, 79, 80, 81, 84, 90, 94, 95, 96, 97, 104,

105, 106, 108

Ctdsk, 55

CTEXT, 4, 5, 16, 49, 50, 53, 54, 56, 66, 101

Ctrl-Z, 7

CW, 6, 7, 68, 73, 78, 79, 80, 81, 82, 83, 85, 88, 89, 107, 108

CWID, 79

DCD, 4, 55, 61, 77, 90, 98

DCDCONN, 77

DCD-Flag, 98

Debug, 6, 72

Default, 5, 8, 9, 15, 16, 17, 18, 21, 22, 24, 28, 40, 42, 45,

46, 47, 48, 49, 52, 53, 54, 57, 59, 62, 63, 64, 65, 67, 68,

77, 83, 89, 90, 91, 98, 105, 106, 107, 113

Defaults, 12, 14, 17, 49, 90

Destination, 28, 29

Diagnostic, 51

Different, 2, 10, 15, 16, 20, 26, 30, 41, 57, 60, 63, 65, 72,

74, 76, 83, 90, 93, 94, 104, 105

Digipeat, 4, 12, 19, 29, 32, 55, 57, 63, 75, 90, 91, 101, 107

Digipeater, 29, 58, 63, 66, 83

Disconnect, 2, 7, 10, 11, 13, 20, 21, 23, 24, 30, 50, 55, 58,

61, 62, 78, 84, 85, 86, 94, 98, 100, 108

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 119

DISCONNECTED, 8, 27, 32, 52, 58, 61, 76, 98, 99, 103

Disconnects, 8, 16, 33, 54, 61, 78, 98

Disk, 1, 2, 4, 5, 6, 7, 12, 13, 16, 18, 19, 20, 21, 26, 33, 34,

35, 36, 38, 40, 42, 45, 46, 47, 51, 52, 58, 63, 64, 71, 81,

82, 88, 89, 90, 91, 92, 94, 95, 97, 101, 103, 104, 108, 109,

111, 112, 113, 114

Diskette, 114

Diskettes, 114

Disks, 114

Display, 3, 12, 20, 22, 23, 32, 38, 39, 41, 44, 50, 52, 56, 57,

59, 61, 65, 68, 71, 73, 74, 75, 76, 85, 86, 88, 90, 98, 106,

108, 112, 113

DOVE, 113

Download, 2, 5, 8, 12, 26, 33, 34, 42, 44, 64, 79, 90, 94, 100,

101, 102, 104

Drop-link, 108

DUPLEX, 10, 11

DX-pedition, 13, 52, 56

DX-peditions, 45, 111

EAS, 91

Echo-As-Sent, 107

Edit, 3, 7, 12, 16, 21, 25, 35, 36, 37, 38, 41, 45, 64, 85, 86,

89, 92, 93, 105, 106, 107, 108

Editor, 12, 17, 21, 35, 36, 45, 47, 52, 94, 95

EGA, 110

Enable, 17

Enabled, 15, 19, 51, 58, 61, 69, 72

End, 7, 11, 17, 21, 26, 31, 34, 36, 37, 39, 45, 47, 52, 53, 58,

59, 60, 62, 87, 89, 90, 97, 104, 114

Enter, 2, 5, 7, 11, 15, 16, 22, 23, 26, 27, 28, 30, 31, 32, 33,

34, 35, 38, 39, 41, 43, 44, 45, 46, 54, 55, 60, 63, 64, 68,

75, 77, 80, 82, 84, 85, 86, 89, 100

EOF, 34, 35, 37, 43, 89, 93, 100, 104, 108

EPROM, 9, 52, 107

Esc, 14, 15, 52, 88

Escape, 25

Event, 3, 12, 23, 25, 30, 32, 38, 39, 40, 45, 50, 56, 61, 63,

86, 100, 108

Favorite, 21, 109

Feature, 11, 15, 21, 23, 27, 28, 30, 31, 32, 33, 39, 43, 45,

48, 49, 50, 51, 52, 59, 60, 62, 65, 71, 72, 79, 80, 81, 82,

91, 94, 95, 104

FEC, 2, 6, 8, 13, 22, 24, 31, 78, 79, 84, 85, 86, 97

File, 3, 5, 6, 8, 11, 12, 13, 14, 15, 16, 21, 22, 24, 25, 26,

28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 40, 41, 42, 43,

44, 45, 46, 47, 48, 49, 51, 52, 54, 55, 56, 57, 58, 59, 60,

63, 64, 65, 71, 72, 76, 77, 78, 79, 80, 81, 82, 88, 89, 90,

91, 92, 93, 94, 95, 97, 98, 99, 100, 101, 102, 103, 104, 105,

106, 107, 108, 109, 110, 111, 112

Files, 2, 3, 4, 7, 11, 14, 15, 16, 17, 19, 21, 24, 25, 26, 33,

35, 36, 38, 40, 41, 42, 45, 47, 48, 52, 56, 57, 58, 64, 72,

89, 91, 92, 93, 94, 95, 100, 101, 102, 108, 112, 113, 114,

115

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 120

FIRMWARE, 109, 110

Flag, 2, 4, 5, 6, 7, 15, 18, 21, 27, 30, 32, 45, 55, 57, 58,

59, 61, 62, 66, 69, 72, 83, 87, 90, 98, 108, 110

Flags, 46, 71

Flash, 20

Flashing, 18, 19, 20, 27, 32, 52, 56, 59, 71, 88

FSK, 88

Function, 2, 10, 11, 12, 13, 15, 17, 20, 23, 24, 25, 26, 31,

32, 33, 38, 41, 47, 49, 52, 55, 57, 60, 62, 73, 74, 77, 78,

79, 81, 82, 84, 86, 87, 91, 92, 93, 97, 102, 103, 108

Functions, 18, 23, 72, 101, 105, 106, 107, 108

Gateway, 104

Global, 36

GMT, 14

Header, 5, 12, 27, 32, 38, 48, 51, 61, 69, 78, 95, 100, 102,

105, 106, 107, 108

HEADERLN, 91

Hercules, 97

HF, 2, 5, 6, 10, 16, 28, 56, 64, 69, 73, 76, 77, 90, 91, 99,

105, 106, 107, 108, 111

HOME, 7, 88, 95, 109, 114

HOST, 43, 99, 100, 103

Hot, 7, 10, 15, 16, 24, 41, 60, 83, 85, 86, 108

Incoming, 3, 7, 8, 12, 13, 15, 16, 21, 22, 23, 24, 26, 32, 33,

35, 37, 48, 52, 54, 57, 58, 68, 69, 70, 71, 72, 77, 81, 82,

83, 88, 91, 95, 97, 98, 107, 108

INDEX, 116

Information, 14, 15, 17, 22, 44, 45, 46, 72, 81, 88, 89, 90,

92, 106, 110, 111

Inhibitor, 33

INPUT, 23, 109, 114

InWindow, 5, 58, 68, 76

KA, 12, 28, 30

KAM, 1, 8, 10, 16, 17, 23, 50, 51, 61, 63, 67, 73, 75, 76, 80,

82, 91, 97, 106, 108

Kaypro, 8, 106

KA-Node, 30, 91

Key, 7, 10, 11, 13, 14, 15, 16, 17, 20, 22, 23, 24, 25, 26, 27,

29, 31, 32, 33, 36, 38, 45, 48, 49, 52, 55, 57, 60, 61, 62,

64, 68, 72, 73, 74, 76, 77, 78, 79, 81, 82, 83, 84, 85, 86,

87, 88, 89, 91, 93, 95, 97, 100, 106, 108

Keyboard, 4, 11, 17, 23, 41, 55, 57, 68, 77, 80, 83, 90, 93,

100

Keys, 2, 7, 10, 11, 12, 17, 20, 24, 25, 41, 60, 83, 85, 88, 93,

108

KM, 34

KN, 60, 87, 108

KPC, 1, 8, 49, 106

LAN, 8, 12, 13, 18, 28, 42, 43, 44, 48, 51, 53, 56, 60, 61, 62,

65, 92, 98, 99, 102, 103, 104

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 121

LANs, 104

LAN-LINK, 1, 2, 3, 5, 6, 7, 9, 10, 11, 12, 14, 15, 16, 17, 18,

19, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34,

35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 49, 51,

52, 53, 54, 55, 56, 57, 58, 59, 61, 62, 63, 64, 65, 66, 67,

71, 73, 76, 77, 78, 79, 81, 82, 83, 86, 89, 90, 91, 92, 93,

94, 95, 96, 97, 98, 99, 100, 101, 102, 103, 104, 105, 106,

107, 108, 109, 110, 111, 113

LETTERS, 6, 76, 78, 79, 81, 82

LF, 72, 80, 81

Line, 14, 16, 18, 23, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35,

36, 37, 41, 45, 47, 52, 57, 59, 60, 61, 62, 63, 65, 67, 68,

69, 72, 76, 77, 78, 80, 81, 82, 87, 90, 91, 92, 93, 97, 98,

101, 104, 108

Link, 4, 11, 16, 18, 20, 23, 26, 28, 29, 30, 42, 51, 53, 55,

56, 58, 60, 65, 73, 77, 78, 85, 88, 90, 94, 97, 103, 108, 112,

115

LM, 34, 93

Local, 4, 8, 12, 13, 28, 29, 30, 32, 34, 43, 62, 90, 99, 104

Log, 3, 4, 5, 7, 11, 12, 20, 25, 34, 37, 44, 45, 46, 47, 56,

60, 64, 68, 85, 86, 87, 89, 90, 91, 97, 108, 111

Logbook, 3, 5, 11, 35, 37, 44, 45, 46, 47, 55, 56, 64, 85, 91,

106, 108, 109, 111

Logpage, 56

LogScanwindow, 5, 68

LogWindow, 5, 68

Log-to-disk, 55

LSB, 88

Mail, 1, 6, 8, 12, 18, 20, 32, 33, 34, 35, 44, 56, 57, 61, 62,

66, 74, 75, 78, 79, 83, 89, 90, 94, 95, 96, 100, 102, 103,

104, 105, 106, 108, 110

Mailbox, 6, 8, 11, 12, 13, 23, 56, 78, 79, 80, 92, 94, 96, 97,

101, 103

Mailboxes, 92, 103

Mail-Snatch, 4, 32, 62, 106

Manual, 10, 14, 17, 22, 24, 28, 29, 31, 33, 50, 57, 76, 77, 83,

90, 92, 94, 112

MARS, 6, 13, 19, 72, 80, 81, 106, 107, 108

MBL, 26, 33, 62

MBX, 7, 44, 75, 83, 89, 92, 103

Menu, 2, 3, 4, 6, 7, 10, 11, 12, 14, 15, 16, 17, 22, 23, 24,

25, 26, 27, 35, 36, 38, 40, 41, 42, 44, 45, 47, 48, 49, 50,

51, 52, 53, 54, 55, 59, 62, 63, 69, 70, 71, 72, 74, 77, 78,

80, 82, 83, 84, 86, 87, 88, 89, 90, 91, 92, 93, 97, 98, 99,

100, 101, 106, 107, 108, 109

Menus, 10, 20, 26, 68, 97, 108

Message, 3, 4, 6, 7, 8, 11, 12, 13, 15, 17, 18, 25, 26, 29, 33,

34, 37, 38, 40, 42, 43, 48, 50, 55, 56, 57, 58, 59, 60, 61,

65, 66, 68, 77, 79, 80, 85, 86, 89, 90, 92, 93, 94, 95, 97,

99, 100, 101, 102, 103, 104

Messages, 3, 4, 8, 12, 19, 33, 34, 37, 42, 43, 44, 48, 52, 57,

58, 59, 60, 62, 72, 75, 77, 83, 89, 92, 94, 95, 97, 99, 101,

102, 103, 104, 105

Metabeacon, 53, 54, 108

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 122

METEOR, 56

MFJ, 1, 8, 9, 10, 23, 50, 51, 63, 105, 107, 108

MH, 22, 55, 69, 101, 103, 105, 106

MHlist, 2

MHWindow, 5, 69

MicroSat, 112

MicroSats, 51

Mode, 2, 3, 4, 5, 6, 8, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19,

20, 21, 22, 23, 24, 25, 27, 28, 30, 31, 32, 33, 35, 36, 37,

38, 40, 42, 44, 45, 47, 49, 50, 51, 52, 54, 55, 56, 57, 58,

59, 60, 63, 65, 66, 70, 71, 72, 73, 74, 75, 76, 77, 78, 79,

80, 81, 82, 83, 84, 85, 86, 87, 88, 89, 90, 91, 92, 93, 94,

95, 96, 97, 98, 101, 103, 104, 105, 106, 107, 108, 111, 113

Modem, 51, 109

Modes, 6, 10, 11, 19, 20, 21, 22, 23, 24, 25, 28, 32, 37, 41,

45, 46, 55, 56, 57, 61, 63, 64, 68, 69, 72, 73, 74, 76, 77,

85, 86, 87, 88, 95, 96, 105, 106, 109, 112

Modification, 50

Modifications, 1

MON, 66

Monitor, 8, 10, 20, 32, 69, 75, 97

Monochrome, 8, 97

Morse, 2, 6, 7, 10, 11, 13, 18, 23, 73, 79, 81, 82, 83, 84, 87,

88, 106

MRPT, 27

Multi, 5, 6, 7, 20, 70, 71, 107

Multiconnect, 12, 24, 28, 50, 70, 71, 76, 77, 78, 105

Multiple, 6, 34, 61, 74, 75, 76, 86, 108

Multistream, 86

Multi-user, 76

MYCALL, 79, 81, 83

NAVTEX, 6, 18, 73, 74, 106, 107

Navy, 6, 13, 19, 72, 80, 81, 106, 107, 108

NET, 12, 28, 29, 30, 32, 58, 91, 93

Nondocument, 91, 105, 106, 107

NTS, 109

Number, 5, 12, 18, 19, 20, 21, 24, 25, 32, 33, 38, 39, 43, 47,

48, 52, 53, 54, 56, 57, 60, 65, 67, 68, 72, 76, 83, 89, 90,

93, 97, 99, 100, 102, 108, 112

Off-line, 33, 43, 99

ON-LINE, 3, 42, 44, 99

Operation, 10, 11, 14, 18, 22, 31, 45, 47, 49, 55, 60, 74, 75,

86, 88, 100, 105, 108, 109

Option, 5, 10, 14, 17, 26, 27, 28, 31, 32, 37, 38, 39, 41, 42,

43, 44, 45, 46, 47, 48, 49, 50, 51, 53, 54, 55, 56, 57, 58,

59, 60, 61, 62, 63, 64, 65, 66, 68, 69, 71, 72, 74, 77, 78,

79, 80, 81, 82, 83, 84, 86, 87, 91, 98, 99, 100, 101, 106,

107, 108

Orbiter, 4, 52, 53, 108

Originating, 78, 90

OSCARs, 112

Outgoing, 5, 13, 15, 21, 23, 24, 33, 37, 40, 48, 49, 56, 57,

68, 69, 70, 72, 84

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 123

OutWindow, 5, 40, 68, 97

Packet, 1, 2, 6, 7, 8, 10, 11, 12, 13, 16, 18, 19, 20, 21, 22,

23, 24, 25, 26, 27, 28, 30, 31, 32, 33, 38, 39, 40, 41, 44,

45, 49, 50, 51, 52, 53, 55, 56, 57, 58, 59, 60, 61, 62, 63,

64, 68, 69, 73, 74, 75, 76, 78, 84, 85, 86, 87, 89, 91, 94,

95, 97, 98, 99, 101, 102, 103, 104, 105, 106, 107, 108

PacketCluster, 4, 31, 63, 85, 87, 90, 108

PacketClusterZap, 19

Packets, 4, 19, 20, 27, 50, 51, 52, 54, 56, 58, 75, 77, 78, 91,

98, 113

Parameter, 4, 17, 25, 27, 55, 63, 65, 67, 70, 71, 80, 86, 91,

97, 98, 108, 112, 113

Parameters, 4, 5, 6, 10, 14, 16, 17, 22, 55, 62, 65, 67, 71,

73, 87, 89, 90, 108

Path, 2, 5, 8, 12, 15, 26, 27, 28, 29, 30, 31, 32, 45, 58, 61,

64, 74, 90, 91, 92, 100, 101, 102

PBBS, 66

PC-HAM, 11, 45, 47, 91, 111

Period, 97, 114, 115

Periodic, 3, 38, 39, 61

PgDn, 35

PgUp, 7, 35, 88

PK, 1, 9, 10, 13, 14, 15, 16, 17, 20, 23, 24, 49, 50, 51, 61,

63, 66, 67, 72, 73, 74, 75, 76, 77, 79, 80, 81, 82, 83, 84,

88, 90, 91, 97, 98, 103, 105, 106, 107, 108

Power, 5, 14, 15, 18, 45, 46, 47, 49, 65, 90, 102, 106

Problem, 9, 98, 99, 110, 114

Problems, 2, 11, 15, 17, 18, 67, 97, 98, 99

Program, 1, 2, 8, 9, 10, 11, 14, 15, 16, 17, 21, 24, 25, 31,

32, 44, 45, 60, 65, 67, 71, 83, 87, 89, 91, 94, 95, 97, 99,

105, 106, 107, 111, 112, 114

Prompt, 5, 24, 26, 27, 31, 49, 52, 54, 62, 63, 69, 72

PromptWindow, 5, 68

QBM, 3, 5, 8, 42, 43, 64, 90, 100, 101, 102

QBU, 3, 5, 42, 64, 65, 90, 93, 104, 108

QBU-RX, 5, 7, 42, 64, 89, 90, 93

QBU-TX, 5, 7, 42, 64, 89, 90, 93

QDB, 3, 8, 42, 43, 101, 102, 108

QJG, 8, 44, 104

QMH, 8, 12, 101, 102, 103, 105

QNO, 8, 42, 43, 59, 100, 103, 104

QRA, 3, 8, 13, 44, 103, 108

QRM, 10, 17, 23, 51, 56, 103

QRT, 5, 8, 19, 51, 56, 66, 83, 95, 103, 104

QRU, 3, 8, 42, 43, 44, 92, 103, 104

QRV, 5, 8, 42, 43, 66, 67, 100, 104

QRZ, 7, 53, 55, 85, 86

QSL, 8, 38, 42, 43, 45, 53, 55, 100, 104, 109, 111

QSM, 8, 94, 103

QSO, 11, 19, 20, 21, 24, 32, 39, 41, 54, 56, 60, 73, 74, 75,

79, 81, 94, 97

QSP, 3, 4, 8, 19, 42, 43, 55, 59, 90, 92, 99, 103, 104, 108

QSY, 56

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 124

QTC, 4, 8, 12, 19, 55, 56, 57, 58, 59, 62, 90, 94, 104, 108

QTC-Snatch, 58, 94, 95, 106

QYD, 8, 104

QYU, 8, 104

Radio, 1, 2, 10, 11, 12, 51, 52, 73, 83, 88, 99, 102, 106, 107,

111

Rate, 5, 6, 7, 14, 15, 17, 67, 74, 75, 79, 80, 82, 88, 90, 98,

113

Receive, 1, 2, 6, 7, 10, 11, 24, 36, 40, 41, 43, 44, 56, 59,

66, 73, 78, 79, 83, 84, 85, 86, 87, 94, 99, 102, 103, 108,

109, 114

Recognition, 20, 23, 63, 108

Recognize, 10, 16, 23, 58, 95

Recognized, 22, 26, 45, 63, 97

Recognizes, 29

Recognizing, 56

Reconfigure, 15

Relay, 4, 55, 59, 90

Remote, 8, 65, 95, 101, 102, 105

Request, 4, 18, 20, 28, 29, 31, 34, 39, 48, 51, 58, 61, 62, 72,

90, 99, 101, 102, 103, 104, 111, 112

Retry, 2, 7, 27, 31, 32, 51, 85, 87

Return, 7, 21, 23, 31, 41, 42, 43, 52, 53, 72, 78, 79, 80, 83,

85, 86, 100

Reverse, 50, 88, 102

Reversed, 50, 88

REVISION, 9, 108, 112

RLI, 26, 33, 62

RM, 34, 62

RN, 34, 62

Robot, 2, 3, 6, 7, 16, 17, 19, 20, 23, 38, 39, 52, 53, 54, 55,

56, 78, 79, 80, 85, 86, 94, 108, 111

RTTY, 4, 6, 13, 19, 23, 50, 51, 68, 73, 79, 81, 87, 89, 108

RXR, 50, 51

RY's, 108

SAREX, 4, 6, 13, 19, 50, 51, 52, 53, 54, 55, 71, 90, 108

Scan, 3, 4, 6, 7, 27, 33, 34, 44, 46, 48, 52, 55, 59, 62, 71,

72, 85, 86, 87, 90, 108

Scanned, 68, 81, 82, 95

Scanning, 29, 34, 48

Scheduler, 12, 39, 40

Screen, 3, 8, 13, 14, 17, 23, 25, 32, 36, 37, 46, 56, 58, 68,

69, 74, 77, 80, 81, 82, 85, 88, 89, 91, 98, 100, 108, 113

Scrollback, 7, 88

ScrollBackWindow, 5, 68

Scrolled, 21, 88

Scrolling, 55

Section, 14, 15, 18, 19, 21, 29, 31, 45, 48, 51, 61, 72, 83,

90, 91, 94

SELCAL, 4, 7, 13, 14, 19, 20, 23, 27, 28, 31, 62, 63, 80, 81,

82, 85, 86, 90

Semiautomatic, 11, 45, 60

Send, 2, 3, 7, 8, 11, 14, 17, 21, 22, 24, 25, 27, 33, 34, 40,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 125

41, 43, 58, 59, 66, 76, 77, 78, 79, 81, 82, 84, 85, 86, 87,

97, 98, 99, 100, 102, 103, 104, 109, 111, 112, 113

Sequence, 2, 11, 14, 15, 17, 18, 19, 21, 22, 23, 24, 27, 28,

29, 31, 33, 34, 40, 42, 43, 44, 52, 55, 56, 61, 62, 63, 66,

68, 74, 79, 80, 81, 82, 86, 87, 89, 90, 93, 97, 100, 104, 110

SERH, 55

SERW, 55

Shake, 108

Shareware, 1, 9, 111, 112, 114, 115

Shift, 2, 6, 7, 18, 25, 50, 51, 74, 76, 80, 81, 82, 88, 89, 93,

108

Shows, 12, 14, 22, 40, 42, 48, 53, 54, 72, 76, 78, 98, 112

Sj, 33

Snatch, 4, 19, 32, 33, 34, 55, 56, 58, 61, 90

SOLO, 5, 6, 19, 66, 67, 74, 75

SP, 34, 35, 37, 93

Speed, 7, 73, 77, 83, 84, 85, 89, 97, 108

STANDBY, 6, 73, 78, 79

State, 10, 15, 18, 26, 27, 32, 50, 53, 59, 61, 65, 66, 71, 77,

83, 86, 89, 90, 97, 108, 109, 110

State-sequence, 6, 72, 110

Station, 2, 6, 8, 10, 11, 12, 13, 15, 16, 18, 19, 20, 21, 22,

23, 24, 26, 27, 28, 29, 30, 31, 32, 37, 42, 43, 44, 48, 50,

51, 54, 56, 57, 58, 60, 64, 65, 71, 72, 74, 75, 76, 77, 78,

79, 90, 92, 93, 94, 95, 97, 99, 100, 101, 102, 103, 104, 105,

106, 107, 108

Status, 2, 4, 14, 18, 22, 23, 27, 32, 46, 49, 50, 51, 52, 53,

55, 56, 57, 59, 60, 65, 68, 71, 72, 74, 76, 77, 78, 80, 81,

82, 88, 89, 97, 106, 108, 113, 114

StatusWindow, 5, 68

Store, 12, 13, 19, 43, 44, 59, 63, 91, 92, 99, 103

Subdirectory, 26, 41, 64, 90, 100, 101

Subject, 33, 37

Sub-directory, 102

Sub-menu, 62, 63, 64, 65, 67, 71

Sub-modes, 19

SYS, 3, 6, 7, 14, 15, 16, 22, 31, 34, 35, 38, 42, 44, 46, 49,

55, 59, 65, 71, 77, 78, 81, 82, 89, 90, 91, 92, 98, 100, 105,

106, 107, 108, 110

SYSOP, 52

Talk, 2, 3, 7, 10, 21, 24, 25, 27, 40, 41, 76, 84, 85, 108

Terminal, 1, 5, 6, 7, 10, 14, 15, 19, 24, 25, 27, 49, 58, 66,

67, 68, 74, 77, 78, 80, 82, 86, 98, 101, 105, 106, 108, 113

Text, 2, 5, 7, 12, 13, 16, 17, 21, 22, 23, 24, 26, 28, 31, 33,

34, 35, 36, 38, 39, 40, 41, 43, 45, 49, 50, 52, 54, 56, 57,

58, 59, 60, 65, 67, 68, 69, 70, 71, 76, 77, 78, 79, 81, 82,

84, 86, 88, 90, 91, 93, 94, 95, 97, 98, 100, 101, 104, 108

Theircall, 92

TheLink, 28, 91

Time, 2, 3, 4, 5, 10, 11, 12, 14, 15, 21, 22, 24, 26, 29, 32,

33, 34, 38, 39, 40, 41, 42, 47, 48, 49, 51, 54, 55, 57, 58,

60, 61, 62, 64, 65, 66, 67, 72, 74, 76, 77, 81, 82, 83, 88,

89, 93, 95, 96, 97, 100, 101, 105, 108, 109, 111, 112, 113

Timer, 22, 53, 54, 95

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 126

Timestamp, 90

Timetag, 21

Time-out, 97, 108

TNC, 1, 2, 4, 5, 7, 8, 10, 11, 12, 14, 15, 16, 17, 22, 23, 24,

25, 26, 27, 29, 30, 31, 36, 40, 41, 42, 49, 50, 52, 53, 54,

55, 57, 61, 63, 66, 67, 68, 72, 73, 75, 76, 77, 78, 81, 82,

83, 84, 85, 89, 90, 91, 94, 97, 98, 99, 101, 105, 106, 107,

108, 109, 110, 112, 113

Traffic, 6, 20, 51, 52, 57, 62, 74, 75, 76, 98

Transfer, 8, 18, 26, 38, 40, 43, 46, 100, 104, 108, 109

Transmit, 2, 10, 11, 17, 21, 23, 24, 27, 31, 40, 41, 44, 56,

57, 60, 78, 87, 93, 94, 105, 106, 107

Transparent, 24, 42, 84

TRFC, 18, 19, 27

Trigger, 4, 13, 33, 58, 62, 90, 95

Try, 15, 23, 28, 29, 31, 33, 46, 59, 63, 72, 77, 97, 99

Turn, 3, 4, 6, 7, 10, 14, 17, 21, 26, 38, 39, 41, 53, 54, 55,

57, 58, 59, 60, 61, 62, 66, 72, 73, 75, 78, 79, 81, 82, 85,

86, 89, 107

Turnoff, 66, 81, 82

TXR, 50, 51

TXT, 5, 16, 21, 37, 40, 56, 64, 79, 97

Type, 2, 10, 11, 14, 15, 16, 23, 26, 27, 28, 29, 30, 31, 34,

35, 39, 40, 42, 43, 44, 47, 48, 49, 50, 55, 56, 73, 74, 76,

77, 78, 79, 80, 82, 83, 84, 86, 87, 90, 91, 94, 97, 100, 101,

102, 105, 109

Type-ahead, 23

Unacknowledged, 50

Unaddressed, 58, 59, 98

Unconnected, 44

UNPROTO, 54, 76

Unshift, 51, 81

Unshift-on-space, 18

UoSAT, 4, 50, 106, 107

Uplink, 29

Upload, 5, 8, 33, 43, 64, 100, 101, 102, 103, 104

Uppercase, 3, 40, 41, 63

USOS, 7, 18, 51, 80, 81, 89

Verbose, 62

Window, 2, 14, 18, 21, 22, 23, 24, 26, 27, 32, 35, 36, 37, 41,

44, 45, 46, 50, 51, 52, 55, 56, 57, 59, 60, 65, 68, 69, 70,

71, 72, 74, 76, 78, 80, 81, 82, 86, 88, 89, 106, 107, 108

Word, 4, 18, 19, 20, 21, 22, 23, 26, 29, 34, 35, 36, 37, 40,

44, 48, 60, 62, 72, 74, 76, 89, 90, 91, 92, 93, 95, 102, 105,

106, 107

Worked, 20, 53, 54, 60, 67

Wpm, 73, 89, 108

YAPP, 2, 8, 15, 91, 104, 109

Yourcall, 30, 31, 34, 48, 57, 58, 86, 95, 103

Zap, 3, 7, 27, 31, 32, 37, 38, 39, 52, 61, 62, 71, 85, 87, 108

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990

LAN-LINK VERSION 1.56 PAGE 127

ZapBBS, 18, 19, 61

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990