Computer Control of ICOM Amateur Gear

Carl Clawson, N7KBV

General Description

Several ICOM products feature a built-in computer

interface connected to a 1/8" phone jack on the rear panel.

However, ICOM gives no information about this interface in

the instruction manuals that I've seen.  The information

exists within ICOM, and my dealer managed to get it for me.

This interface, called the "CI-V" by ICOM, is standard

on the following models:  735, 761, 275, 375, 475, and

R7000.  Earlier models used the CI-IV parallel interface,

which can be connected to CI-V by ICOM's UX-14 converter.

Models using the CI-IV are the 751, 271, 471, 1271, and R71.

The information I got from ICOM was written with the 735 in

mind, but a listing of a BASIC program to control the R7000

was appended.  I believe it will help with the other models,

too.  The control codes and data format should be the same

for all of them.

The computer interface allows you to do such things as:

-    Set and read the frequency and modulation mode

-    Set VFO A, VFO B, or memory mode

-    Select memory channel

-    Store displayed frequency into memory

-    Transfer displayed memory-mode frequency to a VFO

Even without a computer, you can run a cable between

the remote control jacks of two rigs, and whenever the fre-

quency or modulation mode of either is changed, the other

will track it if possible.  (If the rigs have incompatible

frequency coverages, like the 735 and R7000, funny things

can happen.)

The interface is bi-directional, using TTL levels on a

single line for sending and receiving serial ASCII data.

You may need an appropriate hardware interface to convert,

for example, RS232 to TTL.  (I understand that Commodore

computers have TTL inputs and outputs so that no interface

is needed with them.) I used the Motorola MC1488 and MC1489

chips powered by two 9-volt batteries to interface to my

RS232 line.  Appendix 1 lists the pinouts used for this

interface.  If you'd rather buy something, ICOM sells the

- 2 -

model CT-17 level converter for RS232.

The interface uses a "carrier-sense, multiple-access

with collision detection" local area network protocol, so

that multiple rigs can be connected in parallel without dif-

ficulty.  Thus, you can use the same RS232 line and level-

converter interface to control many rigs.  Each rig must

have a unique address, which is set by internal jumpers.

Each model comes factory preset to its own address, which is

04 for the 735 and 08 for the R7000.  Appendix 2 has infor-

mation on the jumper settings.

I will use the words "receive" and "send" to refer to

data transmission.  Thus a "receiver" isn't necessarily an

R7000; it's any device receiving data from the CI-V bus.  I

will use the word "rig" to mean an ICOM product using the

CI-V interface.

Data Format

The rigs send and receive data in variable length pack-

ets, which are formatted as follows:

Byte #    Contents

1         hex 'FE' (i.e. 11111110 binary, 254 decimal)

2         hex 'FE'

3         <RX>

4         <TX>

5         <Code>

6-n       <BCD data of variable length>

n+1       hex 'FD'

The two hexadecimal FE bytes signal the beginning of a

packet, and the FD byte signals the end.  <Code> is the con-

trol code sent by the computer to the rig, which determines

the action that the rig will take.  In some cases, the

receiving rig will include a control code in its response to

the sender; see the section "Control Codes and Responses"

below.  <TX> is the address of the device sending the data,

and <RX> is the address to which the data is being sent.

When a rig responds to a data packet, it addresses that

response to the <TX> in the packet.  Thus, if your computer

requests a rig to report its frequency, it will address that

report to the computer and other rigs will ignore the data.

(Of course, the computer can lie about its address and trick

one rig into talking to another!) Your computer should use

its own unique, non-zero address on the network.  I will

assume in the examples that the computer is at address 02.

There are two control codes that can cause any rig on the

network to respond when sent with <RX>=0; I discuss these

- 3 -

below.

Bytes #6 through #n contain any data required by the

control code, in BCD format with 2 decimal digits per byte.

Frequency data is sent starting with the byte containing the

1-Hz and 10-Hz digits.  These digits are sent even if they

are not used by the rig, so that the data format is the same

for all rigs regardless of their frequency coverage and

resolution.  For example, consider the frequency 25.13244

MHz.  This is broken up into two-digit groups:

25  13  24  40

It is then coded in BCD.  In other words, consider each

digit group to be a hexadecimal number instead of decimal.

The decimal values of these BCD digit groups are 2x16+5=37,

1x16+3=19, 2x16+4=36, and 4x16+0=64.  Now send these groups,

starting with the least significant.  The complete data

packet will be, in hexadecimal,

FE  FE  <RX>  <TX>  <Code>  40  24  13  25  FD

If you have more than one rig on the network you may

occasionally receive a sequence of 5 bytes of hex 'FC'.

This is the "jammer code" used by a rig to indicate that a

collision has occurred.  Each rig, when sending, monitors

the interface.  If it does not receive exactly what it sent,

then a collision occurred, i.e., another rig was sending

data at the same time.  If a rig detects a collision when

sending a packet, it will wait until the network is idle,

then send the jammer code.  A rig that receives this code

will realize that a collision has occurred and ignore the

previously received packet.  Because the ICOM remote control

ports are bi-directional, your computer will receive every-

thing it sends, so you can check for collisions from your

computer, too.

Control Codes and Responses

Most of the following codes are addressed to a specific

rig, which addresses an acknowledgement packet to the <TX>

contained in the control packet.  The first two codes, 00

and 01, can be sent to the "group call address" of 00, in

which case any rig will receive them without sending an ack-

nowledgement.  These two codes are sent in this way by any

rig when its mode or frequency is changed by manual control,

and are received by any rig on the network.  This allows a

number of rigs to track each other in frequency and mode

without computer intervention.  Rigs can be inhibited from

sending and receiving group call packets by an internal

- 4 -

jumper.  See Appendix 2 for more details.  These codes can

also be sent with a specific, non-zero <RX>, in which case

they will be received by the addressed rig even if the group

call function is disabled.

00   Set frequency.  See above for format of frequency data.

See code 05 below for more details.

01   Set modulation mode.  One or two data bytes are

required to indicate the mode desired.

Data      Mode

00        LSB

01        USB

02        AM

03        CW

04        RTTY

05        FM

05 00     SSB (R7000)

02   Report tuning range.  No data required.  The rig will

report its frequency limits in the format

FE FE <RX> <TX> 02 <upper limit> 2D <lower limit> FD

(Hex 2D is the ASCII hyphen.) According to ICOM, some

rigs report the lower limit first.

03   Report frequency.  No data required.  The addressed rig

returns its displayed frequency to the sender in the

format

FE FE <RX> <TX> 03 <Frequency> FD

04   Report modulation mode.  No data required.  The

addressed rig returns its mode to the sender using the

codes listed above.  Rigs with selectable bandwidth

return an additional byte indicating the bandwidth.

The format of the response is

FE FE <RX> <TX> 04 <Mode> <Bandwidth> FD

The bandwidth codes are:

Data      Bandwidth

01        Width 1 (widest)

02        Width 2 (narrower)

03        Width 3 (narrowest)

- 5 -

05   Set frequency.  The data format is given above.  If the

data contains fewer digits than the rig uses, the

digits sent will be changed and the rest will remain

the same.  If the rig receives valid frequency data

within its tuning range, it responds with a packet con-

taining the data "FB":

FE FE <RX> <TX> FB FD

If it didn't like the data, it responds with the data

"FA":

FE FE <RX> <TX> FA FD

These acknowledgement codes are used by all following

commands.

The 735 responds to out-of-range frequency data by

sending the "FA" acknowledgement and: 1) If the fre-

quency it receives is less than 0.1 MHz, it sets its

frequency to 0.1 MHz, 2) If it received 4 bytes of fre-

quency data that is more than 30 MHz, it sets itself to

30 MHz, and 3) If it received more than 4 bytes of fre-

quency data, the data is ignored.

06   Set modulation mode.  If one byte is sent, it sets the

mode per the above table.  If two bytes are sent, the

second is the IF bandwidth.

07   Set VFO status.  If no data is sent, the rig changes

from MEMORY mode to VFO mode.  If data 00 or 01 is

sent, the rig sets VFO A or VFO B respectively.

08   Set memory channel.  If no data is sent, the rig

changes from VFO mode to MEMORY mode.  If BCD channel

data is sent, the rig changes to that memory channel.

09   Store displayed frequency and mode into displayed

memory channel.  No data required.

0A   Write frequency and mode from displayed memory channel

to a VFO.  No data required.

Examples

I will give a few examples of codes and responses for

the 735.  The 735 is at address 04 and the computer is at

02.  First, let's find out what's in memory channel 1 (code

08 to set the channel, and code 03 to read the frequency).

Assume the 735 has the frequency 7.12750 MHz stored in

memory #1.

- 6 -

Computer to 735:

FE FE 04 02 08 01 FD

735 to computer:

FE FE 02 04 FB FD

Computer to 735:

FE FE 04 02 03 FD

735 to computer:

FE FE 02 04 03 00 75 12 07 FD

Now let's change the frequency and mode to 14.02500 MHz USB

(codes 05 and 06).

Computer to 735:

FE FE 04 02 05 00 50 02 14 FD

735 to computer:

FE FE 02 04 FB FD

Computer to 735:

FE FE 04 02 06 01 FD

735 to computer:

FE FE 02 04 FB FD

Now store the changed result back into memory #1.  This

channel is already displayed, so we need only the store com-

mand, code 09.

Computer to 735:

FE FE 04 02 09 FD

735 to computer:

FE FE 02 04 FB FD

- 7 -

Appendix 1 -- RS232 to TTL converter

This is an easily built converter that will run your

ICOM gear from a standard RS232 line.  It can be built in an

hour or two for about $10.  The converter uses two inexpen-

sive, widely-available chips -- the Motorola MC1488 line

driver and MC1489 line receiver.  The only other components

needed are a box, connectors, a power switch, and a couple

.01 or .1 capacitors to bypass the power supply leads.  I

used two 9V batteries for power.  Power for the 1488 can be

+-9 to +-15 volts, and the 1489 requires +5 to +10 volts.

With a suitable DC-DC converter chip and a 78L05 or 78L08,

you could power the circuit off of your 13.8-V supply.  Or,

if you're clever, figure out a way to trickle charge two 9V

nicads off of your RS232 line.

The pinouts are as follows:

For the 1488 -

Pin       Connect to

1         -V

2         ICOM remote jack center conductor

3         RS232 pin 3 (RD)

7         Ground

14        +V

For the 1489 -

Pin       Connect to

1         RS232 pin 2 (TD)

3         ICOM remote jack center conductor

7         Ground

14        +V

Ground pin 7 of the RS232 line and the outer conductor of

the ICOM remote jack, and you're on line.

- 8 -

Appendix 2 -- Jumper Selections

There are jumpers in the ICOM rigs to set the device

address, baud rate, and to enable the group call feature.

You must look on the schematic to find them.  The jumper to

enable the group call feature is called the "transceive" bit

by ICOM, and may be labeled with "TRV" on the schematic.

The others are labeled "DBn" on the 735 and R7000; look for

something similar.  The rigs are factory-set to 1200 baud,

transceive enabled.

I have specific information from ICOM on the 735

jumpers, and for the R7000 I have a likely guess based on

the 735 codes and the schematic.

For the 735 -

The jumpers use lines labeled DB0 through DB5 at con-

nector J22 on the PL board.  Lines DB0 through DB2 set the

device address.  Line DB3 is the transceive enable.  The

baud rate is controlled by DB4 and DB5 according to

DB4  DB5  Baud

0    0    undefined

1    0    9600

0    1    1200

1    1    300

The 735 is set at the factory to address 04.

For the R7000 -

The jumpers use lines DB0 through DB7 at connector J17

on the logic board.  The address is set by DB0 through DB4,

DB5 is the transceive enable, and the baud rate is set by

DB6 and DB7.  The factory address is 08.

If you have any other rig, you can determine its

address by hooking it up to your computer and changing the

frequency or modulation mode manually.  You will receive a

group call packet that contains as its 4th byte the address

of the rig.

.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.oOo.

Thomas Brown, KA2UGQ       BITNET: twb0@lehigh.bitnet

Lehigh University Box 855  ARPA: twb0%lehigh@ibm1.cc.lehigh.edu

Bethlehem, PA  18015       UUCP: ..!att!twb0@lehigh.bitnet

(215) 758-1200             AX.25: ka2ugq@ka2ugq.nj.usa.na (145.01 MHz)

Stress: The confusion caused when one's mind overrides the desire to

choke the life out of someone who desperately deserves it.