LAN-LINK VERSION 1.50  PAGE 1

LAN-LINK Version 1.50 (c) 1988 by G3ZCZ

(C) Copyright 1988 - All Rights Reserved

By Joe Kasser G3ZCZ

POB 3419,

Silver Spring,

Md., 20901, U.S.A.

(301) 593 6136

USA Packet BBS Routing G3ZCZ @ N4QQ

World Packet BBS Routing G3ZCZ @ N4QQ.MD.USA.NA

LAN-LINK  is  a Personal Packet Terminal Program  for  the  TNC1,

TNC2,  KPC-2  and  most  of  all  a  smart  multi  mode   digital

communications controller for the KAM and the PK-232.

LAN-LINK is written and supported by Joe Kasser, G3ZCZ.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 2

TABLE OF CONTENTS

1.0  INTRODUCTION

1.1  PROGRAM HIGHLIGHTS

2.0  Getting Started

2.1 Upgrading from YAPP.

3.0  Things You Should Know

4.0  Function Keys

4.1- Capture to Disk Toggle

4.2  Section intentionally blank.

4.3- Transmit Callsign Sequence

4.4  Section intentionally blank.

4.5- Show Packet MHlist.

4.6  Auto CQ Call.

4.7- Activate Type Ahead Buffer

4.8- Disconnect/Receive.

4.9- Put TNC in CMD mode

4.10- Put TNC in CONVERSE /Transmit mode /AMTOR FEC

5.0  Main menu

6.0 The Call Menu

6.1 Alert Call

6.2 Connect/Call

6.3 Enter Call

6.4 reMember Path

6.5 Call CQ

6.6 Retry

6.7 Show Directory

6.8 Target Call

6.9 aUto CQ

6.10 Zap BBS

7.0  Return to DOS

8.0 Files Menu

8.1  Send Brag Tape

8.2  File Directory

8.3  Erase File

8.4  Rename File

8.5  Send File

9.0 Help Menu

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 3

10.0 Log Menu

10.1 Show last Few Entries

10.2 Log Entry

10.3 Print Log

10.4 Scan Log

11.0 Message Menu

11.1 BBS Command Files

11.2 Current Messages

11.3 Scan Message

11.4 Leave Note

11.5 Old Messages

11.6 QSP LAN-LINK

11.7 QRU LAN-LINK

11.8 Time tag

12.0 TNC Menu

12.1 Set TNC Date/Time

12.2 Flush TNC Buffer

12.3 Initialize TNC

12.4 Show Calls Monitored List

12.5 Show Connect Status

12.6 Change CQ Text

12.7 Change Ctext

13.0 Parameter Menu

13.1 Turn Beacon Mode On/Off

13.2 Clear Connect Count

13.3 Turn Digipeat Detect On/Off

13.4 Display DeFaults

13.5 Set Message Count

13.6 Turn NETROM Drop Out On/Off

13.7 Turn Printer On/Off

13.8 Set CQ delay time

13.9 Turn Relay On/Off

13.10 Turn Sound On/Off

13.11 Turn Contest Mode On/Off

14.0 Q Codes [NC/L]

15.0 Run DOS Command

16.0 CommunicationS Modes

16.1 VHF PACKET

16.2 HF PACKET

16.3 ASCII (RTTY)

16.4 BAUDOT (RTTY)

16.5 MORSE (CW)

16.6 AMTOR (STANDBY RECEIVE)

16.7 AMTOR (ALIST [chirpcopy])

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 4

16.8 SIGNAL

16.9 NAVTEX

17.0 Terminal Mode Menu

17.1 Packet Terminal Menu

17.1.1 Change Packet Baud Rate

17.1.2 CQ/Beacons

17.1.3 Everything

17.1.4 Read the Mail

17.1.5 Just My Station (Solo)

17.1.6 All Traffic

17.1.7 Multiple User Connects

17.1.8 Single User Connects

17.1.9 Conference

17.1.10 Conference Bridge On/Off

17.2 AMTOR Terminal Menu

17.2.1 Force Break [ACHG]

17.2.2 Start Mode B [FEC]

17.2.3 Force LETTERS case

17.2.4 Read the Mail [ALISTEN]

17.2.5 Override ROBOT/Mailbox

17.2.6 Standby Receive [ARQ]

17.2.7 Set ROBOT/Mailbox

17.2.8 CW Identification

17.3 Baudot Terminal Menu

17.3.1 Set Baud Rate

17.3.2 Select Shift

17.3.3 NAVY MARS Message Protocol

17.3.4 USOS Off

17.3.5 SELCAL set/reset

17.3.6 CW Identification

17.4 ASCII Terminal Menu

17.4.1 Set Baud Rate

17.4.2 Select Shift

17.4.3 SELCAL set/reset

17.4.4 CW Identification

17.4 CW Terminal Menu

17.5.1 Lock to Receive Signal

17.5.2 Set Morse Speed

17.5.3 Unlock Morse Speed

18.0 Hot Keys

18.1 Alt C- Call/Connect to

18.2 Alt D- Disconnect or Receive

18.3 Alt L- Edit Log

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 5

18.4 Alt M- Message Scan

18.5 Alt R- Retry Call/Connect

18.6 Alt X- eXit to DOS

18.7 Alt Z- ZAP BBS

18.8  End- > +?  [Over]

18.9 PgUp- Scrollback (Scrolls back incoming text).

18.10 The HOME Key

18.11 The DOWN ARROW key

18.12 Shift/Baudrate Control keys

18.13 Control Z (^Z)

19.0  LAN-LINK disk files

19.1  LAN-LINK.COM

19.2  LAN-LINK.SYS

19.3  LAN-LINK.RUN

19.4  LAN-LINK.LOG

19.5  LAN-LINK.DIR

19.6  CONFIG.SYS

19.7  *.MBX

19.8  *.BBS

20.0 Mailbox features

20.1 Answering Machine.

20.2 Mail Beacon (Annunciator).

20.3 How to leave a message

20.4 Remote Beacon shutdown.

APPENDIX 1. BUGS and Anomalies.

A1.1 Command/Converse mode anomalies.

A1.2 Buffer messages in AMTOR Beacon CQ mode.

A1.3 FEC +? responses in AMTOR Beacon CQ mode.

A1.4 Using a CGA card with a monochrome monitor.

A1.5 The Packet Conference Mode.

A1.6 Packet Conference Mode disconnects.

A1.7 Strange Events.

A1.8 *** DISCONNECTED.

A1.9 *** CONNECTED.

A1.10 Greek Characters on Incoming Screen.

A1.11 Dumb TNC.

APPENDIX 2. Local Area Network Capability

A2.1 LAN Protocol (G3ZCZ Version).

A2.2 Message Format.

A2.3 File Uploading/Downloading.

A2.4 Path Determination to a DX Station.

APPENDIX 3. NC/L Dictionary.

A3.1 :QBM:

A3.2 :QDB:

A3.3 :QMH:

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 6

A3.4 :QSM:

A3.5 :QSP:

A3.6 :QRT:

A3.7 :QRU:

A3.8 :QNO:

A3.9 :QJG:

A3.10 :QRV:

A3.11 :QSL:

A3.12 :QTC:

A3.13 :QYU:

A3.14 :QYD:

APPENDIX 4. USING OTHER TNC'S.

A4.1 TNC2 (MFJ 1270) Version 1.1.2

A4.2 TNC1 (HD-4040)

A4.3 Kaypro KPC-2

A4.4 KAM EPROM Version 2.8.2.

A4.5 PK-232 EPROM Version 30 AUG 1988

APPENDIX 5. Registration Form.

APPENDIX 6. Problem Reporting Form.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 7

1.0  INTRODUCTION

LAN-LINK  is a Personal Digital Communications  Terminal  Program

for a Packet Radio TNC.  The program is designed to optimize  the

configuration  of  the  TNC in each communications  mode  and  to

provide some smart terminal features.  It takes advantage of  the

extra   features  of  the  PK-232  which  is  made  by   Advanced

Electronics  Applications Inc. It is designed to allow anyone  to

use and get the most out of their packet TNC as well as from  the

PK-232 and KAM for Morse, ASCII, BAUDOT, AMTOR as well as  Packet

Radio communications without having to keep the manual handy.

LAN-LINK  is  a  sophisticated program.  In its  basic  state  it

allows  you to use the TNC in an optimal manner.   It  configures

the TNC for you to maximize the communications efficiency in  the

communications  mode  of your choice.  That means,  for  example,

when working Packet on HF you need to program the TNC  parameters

to different values than you would use on VHF to make maximum use

of  the  mode.  One significant difference is the length  of  the

packet itself, for the longer it is, the greater the  probability

of  QRM  destroying  it.  This program  will  adjust  the  packet

parameters for you.

Since  the  computer is now involved, other  features  have  been

added  to  simplify  operation, and several  features  have  been

automated.   All these operations are performed using  menus  and

function keys as documented below.  It will take a while to learn

how  to  use this program in a manner that suits you,  read  this

document and have fun, after all, isn't that one of the  purposes

of Amateur Radio.

1.1  PROGRAM HIGHLIGHTS

Function key and Menu driven.

Automatic logbook entries for Packet and Mailbox/Beacon  mode

AMTOR  Connects,  semi automatic logbook  entries  for  other

modes.

Log  file can be processed by the DBASE  Logbook  Package  of

PC-HAM  by G3ZCZ for indexed listings, tracking of  DXCC  and

other AWARDS, etc.

Contest  operation, sends standard message and  automatically

increments QSO count.

Automatic  optimized configuration of the TNC for  each  com-

munications mode.

All mode Function key  'OVER' feature (End).

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 8

1.1.1     PACKET RADIO

Automatic capture to disk disk of all packet radio  connects.

Automatic indication of the number of Packet connects.

Local   Area   Network  (LAN)  message  store   and   forward

capability.

Capable of automatic connect attempts to download a QTC  from

an other station in the LAN.

Capable  of  automatic connect attempts to a  packet  BBS  to

download  your incoming messages, when your callsign  appears

on the BBS mail beacon annunciator.

Capable  of  automatically requesting Bulletins  on  subjects

that interest you from your local packet BBS.

Digipeat monitoring and capture.

Alert signal to let you know when a predetermined call  shows

up in a packet header on frequency.

Conference mode in multi- connect situations.

Bridge mode in multi- connect situations.

Path determination to Dx station via :QMH:.

Indicator that a specific station designated as the  'target'

call   connected   in  Packet  mode,  or  linked   to   AMTOR

Beacon/Mailbox while you were away.

Automatic  NET/ROM and KA Node path set up from  LAN-LINK.DIR

call/path directory file.

Selective  answering machine and MAILBOX using  NC/L  command

dialog.

Screen  indication  of  connect by  desired  station  (target

call).

Automatic Beacon Mode CQ caller.

Will  call  CQ repetitively and either work the  connect  and

keep  going  after disconnect or signal you when a  reply  is

received.

1.1.2     AMTOR

Automatic  CQ caller.  Will call CQ repetitively  and  signal

you when a reply is received.

Local   Area   Network  (LAN)  message  store   and   forward

capability.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 9

Selective  answering machine and MAILBOX using  NC/L  command

dialog.

Function  key  change from monitoring FEC CQ's  to  QSO's  in

progress (chirpcopy).

Automatic Beacon Mode CQ caller.

1.1.3     MORSE/BAUDOT/ASCII

Automatic  CQ caller.  Will call CQ repetitively  and  signal

you when a reply is received.

RTTY SELCAL.

2.0  Getting Started

MAKE  A  BACK UP COPY OF THIS DISK, BEFORE YOU REMOVE  THE  WRITE

PROTECT TAB.

If  you don't have a PK232 then rename the LAN-LINK.SYS  file  to

PK232.SYS  and  rename your TNC.SYS file  as  LAN-LINK.SYS.   For

example,  if  you  are  using  a  TNC2,  perform  the   following

operations in DOS.

RENAME LAN-LINK.SYS PK232.SYS

RENAME TNC2.SYS LANLINK.SYS

Edit the LAN-LINK.SYS file replacing G8BTB with your callsign and

Selcal information.  If your TNC does not support AMTOR you still

need a dummy entry in that line.

For details of the rest of the customization possibilities in the

'LAN-LIN.SYS' file, read Section 19.2.

2.1 Upgrading from YAPP

LAN-LINK uses the PK232 in a different configuration to  the

way  that YAPP does. In particular, LAN-LINK  uses  software

flow control.

To  facilitate conversion from YAPP to LAN-LINK  a  YAPP.CNF

file   is   provided  in  this  package.   YAPP.CNF   is   a

configuration  file for YAPP Version 2 that  will  configure

the PK232 for YAPP and then back to LAN-LINK.

If you are currently using YAPP, replace your YAPP.CNF  file

with this one and load YAPP.  Then terminate the program  in

the  normal manner by using the Alt-X key.  You should  then

be able to load and run LAN-LINK without any problems.  Each

time  you sub-sequently want to run YAPP, YAPP will  set  up

the  PK232, execute and then reconfigure the PK232 back  for

LAN-LINK.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 10

If you are using a YAPP.DIR file, rename it to LAN-LINK.DIR.

3.0  Things You Should Know

Any  Terminal  Node  Controller  (TNC)  is  a  complex  piece  of

equipment.   It can operate in a Command Mode, in which you  tell

it to do something, or in a Converse Mode in which you are  using

it  to talk to other stations.  Many people confuse the two  when

first  getting on Packet.  If you monitor the channels  you  will

recognize command mode TNC instructions on the air, and when  you

use  the  TNC you will receive the "error" reply  when  you  type

something  thinking  that you are in the Converse  Mode  but  are

really in the Command Mode.  LAN-LINK is designed to make  Packet

operation  simple  using Menus and high level (Function  and  Hot

key) commands.

Morse,  ASCII, BAUDOT communication modes are HALF  DUPLEX.   One

station  is transmitting at any time.  You thus have to  use  the

Transmit/Converse  and Receive/Disconnect function keys  to  turn

the transmitter on and off respectively.

AMTOR is also HALF DUPLEX but has some of the attributes of  full

duplex.   In  this  mode  you  control  who  is  talking  through

software,  only one station can send data at a time, but you  can

type  characters  in to the buffer and they will  be  transmitted

automatically  when the other station turns the QSO over to  you.

Once   you  have  linked  with  someone,  you  do  not  use   the

Receive/Disconnect function UNTIL THE QSO IS OVER.  Both stations

are  communicating automatically thanks to the TNC.  If you  want

the  other operator to respond type the +? key sequence  (or  use

the 'END' key).

Packet  Radio communications  on the other hand is a full  duplex

mode  of communications.  Once you have connected  with  someone,

you  do not use the Receive/Disconnect function UNTIL THE QSO  IS

OVER.   Both stations are communicating automatically  thanks  to

the  TNC.  If you want the other operator to respond type  the  >

key followed by the 'Enter' key, better yet, use the 'end' key.

Note  that  Morse,  AMTOR, ASCII and BAUDOT  are  character  mode

communications  in  that each character you type is sent  as  you

type them.  In Packet Communications, the TNC stores them up  and

sends out a burst or packet when you type the 'Enter' key or  you

type  enough characters to fill up a packet.  Studies have  shown

that the human attention span when sitting at the keyboard is  of

the  order  of 2 seconds. If you are in a  keyboard  to  keyboard

packet  radio connect, remember this and don't forget to use  the

'Enter'  frequently,  especially before you stop to  think  about

what to send next.

LAN-LINK  has  the following functions built in to it in  a  menu

driven format.

Morse Code  20 wpm (10 wpm for the KAM).

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 11

BAUDOT  with default of 45 WPM narrow shift (LSB).  You  may  set

other  baud rates directly, or use the left and right arrow  keys

to change the baud rate (see below).

ASCII  with  default 110 BAUDS narrow shift (LSB).  You  may  set

other  baud rates directly, or use the left and right arrow  keys

to change the baud rate (see below).

Packet VHF or HF baud rates and shifts (LSB).

AMTOR 100 BAUDS narrow shift (LSB).

All communications between the computer and the TNC normally  use

the  8  BIT  NO  PARITY  and  1  stop  bit  RS-232  Serial   Port

configuration. You CAN THUS SEND FOREIGN LANGUAGE CHARACTERS FROM

THE KEYBOARD OR FILES.  If you want to limit the character set to

the  7  bit ASCII set, set the TNC parameters  accordingly.   The

baud rate defaults to 1200, but you may set it to 2400, 4800 9600

or even 19200 if your CPU clock is fast enough.

Access  to  the  TNC Command mode is provided in  case  the  user

wishes to override any of these defaults.

IF YOU USE THIS PROGRAM, YOU STILL MUST READ THE TNC MANUAL.

Note   that   AMTOR  and  PACKET  are  full   duplex   modes   of

communications.  When you connect with someone, there is no  need

to use the transmit and receive function keys.  In fact you  will

have  problems  if you do.  Do not disconnect or break  the  link

until the QSO is over.

WARNING  ....   THIS PROGRAM OPERATES IN AN OPEN LOOP  MODE.   IT

REMEMBERS WHAT INSTRUCTIONS OR COMMANDS IT GAVE THE TNC.  IF  FOR

SOME REASON, THE TNC IS NOT SYNCHRONIZED TO THE PROGRAM YOU  WILL

NOT GET CORRECT DISPLAYS ON THE SCREEN.

4.0  Function Keys

Function  keys  are  used to perform  operations  common  to  all

communications modes, or to allow fast performance of a  function

without  having  to  go through several  layers  of  menus.   The

function keys listed below are active.

1- Capture to Disk Toggle.

3- Transmit Callsign Sequence.

5- Show Packet 'MH'list.

6- Auto CQ Call.

7- Activate Type Ahead Buffer.

8- Disconnect or Receive.

9- Put TNC in CMD mode.

10- Put TNC in CONVERSE /Transmit mode /AMTOR FEC.

End- > +?  [Over].

PgUp- Scrollback (Scrolls back incoming text).

ESC- Activates the Main Menu.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 12

The  action  of  each of the function keys is  described  in  the

following sections.

4.1- Capture to Disk Toggle

This  key toggles the "capture to disk" command.   When  the

"capture  to disk" is active, all incoming  text  (including

'Echo as sent' outgoing text) is stored on a disk (floppy or

hard)  in a file called 'LAN-LINK.RUN'.  If this  file  does

not  exist  the first time that LAN-LINK is  loaded,  it  is

created.   From that time on, successive activations  append

new data to the contents of the file.

Each time the command is activated a datestamp (or  timetag)

will be written to the file.

When  a  Packet station connects "capture to disk"  will  be

activated  automatically.   Packet  connects  will  thus  be

recorded in their entirety automatically unless you turn off

the "Capture to disk" during the course of the QSO. "Capture

to disk" will be stopped at disconnect time unless you  have

commanded it on manually.

You  may later edit the file at your convenience  with  your

favorite word processor.

LAN-LINK will automatically close the "capture to disk" file

when you terminate the program and return to DOS.

4.2  Section intentionally blank.

4.3- Transmit Callsign Sequence

This command lets you transmit the following sequence <other

call) DE <your call>.  For example if you are 4X6AA and  you

are  working  G3ZCZ  then this command  would  transmit  the

sequence  G3ZCZ  de 4X6AA.  You use this at the start  of  a

transmission in any mode except Packet.

4.4  Section intentionally blank.

4.5- Show Packet 'MH'list

This  command  lets  you see what calls the  TNC  has  heard

recently on the Packet channels.  You can see up to 18, with

the  data and time they were last logged by the TNC.   Refer

to the 'MH' command in the TNC manual for the particulars of

the display.

The  MH list shows up in a special window and freezes  until

you depress any key.  While the list is frozen, LAN-LINK  is

not monitoring received data, so don't freeze it too long.

If  the MH list is blank, or information comes in  from  the

TNC  between the time you entered the command and  the  time

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 13

the  list is sent back,  subsequent text from the  TNC  will

show up in the MH window until the next 'Cmd:' appears or 18

lines of text have been received.

This command does not work on the TNC1.

4.6  Auto CQ Call.

This command will send a CQ sequence, wait about a minute or

so  and then repeat the CQ sequence every minute or so until

it receives your call back from someone else.  When it  does

so, it will alert you with a Connect Alarm signal and  reset

to the normal or listening mode.

When   the  Automatic  CQ  sequence  is  in  operation,   an

indication  will  be  shown  in  the  status  window.    The

indicator  will be steady while the transmitter  is  sending

the CQ text, and will blink while the receiver is  listening

for a reply.

The time delay between calls is set using 'Q' command in the

Parameters Menu (Set CQ delay time).

The  automatic  CQ  command will only work in  the  TNC  non

Packet  modes if the 'Echo as Sent' mode is set to 'ON'  per

the LAN-LINK.SYS file.  In other words, you must be able  to

see  the CQ text slowly echo in the incoming window for  the

command to work.  The reason for this is that the "K K K"  (

or  "+?" in AMTOR) sequence is used to arm the  software  to

look for a reply

In  AMTOR  you  will get the connect  alarm  only  when  the

replying station transmits the '+?' character combination.

The check for the detection of your callsign and the connect

alarm  sequence only occur after a line feed character  have

been received.

Under  most conditions in AMTOR, many in RTTY, and  some  in

MORSE,  LAN-LINK will recognize the callsign of the  station

answering  your CQ and display it in the status window.   In

the  event  that it fails completely, the  sequence  '--?--'

will  show up in the status window.  Since  the  recognition

algorithm use just look for the first word following the 'DE

space'  sequence  and assumes that it has the  callsign,  be

careful  to  check it when operating under  QRM  conditions.

You can override the automatic call selection with the Enter

Callsign command in the Call Menu.

The automatic CQ sequence is canceled when an incoming  call

is detected (only if the beacon or robot/mailbox mode is not

selected), if you push the disconnect command key, or if you

change communications modes.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 14

4.7- Activate Type Ahead Buffer

The use of this key sets LAN-LINK to the "Type Ahead"  mode.

Any  characters  then typed at the keyboard will  be  placed

into  the TNC transmit buffer.  The contents of  the  buffer

will  be transmitted over the air when the transmit  command

is invoked [FK10].

This  feature  allows you to 'type-ahead' as you  can  enter

text  into the transmit buffer while viewing incoming  words

in  the  INPUT window in the screen.  Any  characters  typed

from  the keyboard will be echoed in the OUTGOING window  on

the screen.

THIS  command  IS NOT ACTIVE IN THE AMTOR or  PACKET  modes.

Packet  is full duplex, while in AMTOR the TNC  buffers  the

data until you receive the next +? sequence.

4.8- Disconnect, Disengage or Receive

This  command  key  returns  the  station  to  the   receive

communications  (TNC command) mode.  It may also be used  to

abort  the automatic CQ sequence.  If you use it to abort  a

CQ  sequence while the transmission is in progress, you  may

also  want  to flush the PK-232  transmit buffer  the  Flush

command in the TNC Menu.

In a multi connect situation the program will prompt you  to

tell it which IO channel to disconnect.

In  AMTOR  this  command key toggles  LAN-LINK  between  the

Chirpcopy  mode  (listening  to  a  QSO  in  progress)   and

monitoring FEC transmissions (CQ calls).

In MORSE this key also disengages the receive speed lock  in

the PK-232 to unlock the PK-232 from the last signal copied.

It lets you lock onto another signal faster.

4.9- Put TNC in CMD mode

This  command puts the TNC into the command mode.   At  this

time, the program is transparent and acts as a dumb terminal

(except  for  placing  incoming and  outgoing  text  in  the

relevant windows).  you use this mode for overriding any  of

the TNC default conditions.  (SEE I TOLD YOU THAT YOU HAD TO

READ THE TNC MANUAL).

4.10- Put TNC in CONVERSE /Transmit mode /AMTOR FEC

This  command places the TNC into the transmit  or  converse

mode.  The transmitter is turned on and the contents of  the

transmit   buffer  are  transmitted.   In  the  non   packet

communications modes, the outgoing characters will be echoed

back  from the TNC and displayed in the INCOMING  window  on

the CRT as they are transmitted on the air.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 15

In the AMTOR modes this key puts the TNC in the FEC mode and

turns the transmitter on.

5.0  Main menu

When  you  touch the ESCape key, the Main Menu  appears  on  your

screen  and provides a number choices which are described in  the

following sections.

C Call Menu

D Return to DOS

F Files Menu

H Help Menu

L Log Menu

M Message Menu

N TNC Menu

P Parameter Menu

Q Q Codes [NC/L]

R Run DOS Command

S communicationS Mode Menu

T Terminal Mode Menu

X eXit LAN-LINK

Type in the single letter to make your choice.

6.0 The Call Menu

The Call Menu provides the following choices.

A Alert Call

C Connect/Call

E Enter Call

M reMember Path

Q Call CQ

R Retry

S Show Directory

T Target Call

U aUto CQ

Z Zap BBS

Type in the single letter to make your choice.

6.1 Alert Call

The Alert call feature is used in the Packet  communications

mode  when disconnected and the terminal set for  'TRFC'  or

'CQ/BEACON'.  LAN-LINK will scan the packet headers received

from  the  TNC, and, when it sees a  packet  originated  (or

digipeated if the MRPT parameter in the TNC is set to 'ON'),

by  the station whose call you have entered as  the  'Alert'

call,  it  will sound an alarm at the  console.    The  line

containing the packet header will also be displayed in light

green.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 16

This  option toggles the Alert call feature on and off.   If

it  is  off, you are prompted for the call to  be  monitored

for, if it is on, it is turned off.  The toggle is set  when

you  enter  a  callsign, and cleared when  you  depress  the

'Enter' key instead of a callsign.

The  Alert call will be shown flashing in the status  window

when in the disconnected state.

NOTE: If the terminal is set for 'CQ/BEACON' the Alert  will

only  sound if the specified station transmits CQ or  BEACON

packets.

6.2 Connect/Call

This  option  works in almost the same manner  in  both  the

Packet  and Non Packet communications modes as described  in

the following sections.

6.2.1 Non Packet Modes

You  use  this option to call a station heard on  the  band.

You may also use it to call 'QRZ' or even 'TEST'.

In  AMTOR the link request will be sent in the ARQ  mode  to

the SELCAL of the desired station.

6.2.2 Packet Mode

You use this option to attempt a connect with another packet

station.   In  a multi connect situation  the  LAN-LINK will

automatically select a free IO channel to try the connect.

6.2.2.1 Names, Handles and Paths

If  you so desire, you can create a directory file  (default

name  is  LAN-LINK.DIR)  and  keep  a  list  of  names   and

callsigns.   The computer will look up a name and  call  the

station by its callsign.  For example if you put

Joe G3ZCZ

as a line in the file, when you tell the computer to connect

to Joe, it will try to connect to G3ZCZ.  If you enter

4X4HF 4X4HF V 4X1AA, 4Z4ZB, 4X4IL

you only need to type '4X4HF' to set up the correct  connect

path.

6.2.2.2 NET/ROM and The Link.

You  can  use  this  feature  to  automate  a  NET/ROM  path

connection.  In the manual NET/ROM path setting up mode, you

would  first connect to your local node, then when  you  are

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 17

connected  with  it,  connect to the next node,  and  so  on

through any intermediate nodes within the whole path  before

you finally connect with your destination station.  You  may

have digipeaters in any of the paths.

For example if you wish to connect to K1HTV who is in  range

of  the SSCT node, and you are in Washington DC,  you  might

first have to connect to your local NET/ROM node (ELK) via a

digipeater, then each of the intermediate nodes, SNJ3,  NNJ,

WMA before you could try to connect with K1HTV.

Consider the manual sequence to make the connection.   K1HTV

is  the station the contact is being attempted with, and  he

is  located  within  range of the SCCT  NET/ROM  node.   The

station attempting the connect is located in the  Washington

DC area one digipeat away from the ELK NET/ROM node.

The  connection to the local node is performed  first.   the

operator has to type ' C ELK v WB4APR-5' to make that  link.

When  the  connect with the node is achieved,  the  operator

must  type 'C SNJ3' to connect to the next node.  Each  time

the  connection  is  made, the operator  must  type  in  the

command to make the next one, until finally the last node is

reached and a connect request can be made to the destination

station  (either  direct or via  a  digipeater).   Sometimes

there may be an intermediate non NET/ROM connection  between

two stations somewhere in the path.

This procedure is tedious and time consuming, because it may

take  several  seconds to complete each stage of  the  link.

LAN-LINK automates this sequence by scanning the line in the

LAN-LINK.DIR  file associated with the callsign entered  and

transmitting each section of the line when it recognizes the

'Connected  to ' reply from each NET/ROM node in  the  path.

When it finally gets to the destination it sounds a chime to

alert you that the connect has been achieved.

The LAN-LINK.DIR file entry for this path is as follows.

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT! K1HTV

where the '!' separates each stage of the link.

The key word is K1HTV.  LAN-LINK then scans the line to  see

if  there is a '!' in it.  If there is it knows that  it  is

dealing  with  a  NET/ROM path.  It then tells  the  TNC  to

attempt a connect with the characters following the key word

up  to  the  first '!'.  In this case it tells  the  TNC  to

connect  to  ELK via WB4APR-5. When the TNC sends  back  the

connected message, LAN-LINK tells the TNC to try to  connect

to  the next node in the path, namely in this example  SNJ3.

The sequence continues each time a connection is made  until

the link is complete.

To set up a directory entry just use the format shown above.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 18

Place  a '!' sign after the uplink path and  different  node

segments. Do not place one after the downlink path.

Depending  on  the  configuration,  LAN-LINK  will  issue  a

'disconnect' command to the TNC in the event the NET/ROM  or

KA-Node connect attempt fails.  This feature is designed for

known paths not for general path determination.

6.2.2.3 KA-Nodes

You  can  connect  or crossconnect via  KA-Nodes  using  the

directory file.  You must still use the ! separators as well

as two new symbols %  and &.  Use % for 'C' and & for 'X' in

KA-Node links..

For  example,  given that a path to W9TNN-2 is  first  to  a

local  KA node [JRW] then to a second node on the  same  vhf

frequency  [W3EAX-7]  and lastly via a cross connect  on  40

Meters  to W9TNN2-, the line in the LAN-LINK.DIR file  would

be as follows.

w9tnn-2  jrw !% w3eax-7 !& w9tnn-2

You should be able to mix NET/ROM and KA-Node paths as in

n7dva      ! w3eax-7 !& ww7zz !& w7zzz !% n7dva v nn7nn

The different symbols are used for the KA node to tell  LAN-

LINK which replies to look for as well as what to prefix the

call with.

Depending  on  the  configuration,  LAN-LINK  will  issue  a

'disconnect' command to the TNC in the event the NET/ROM  or

KA-Node connect attempt fails.  This feature is designed for

known paths not for general path determination.

6.2.2.4 Loop Backs

In the Packet communications mode, if you want to loop  back

through someone else enter the '/' character followed by the

call  of  the station you want to loop  back  through.   For

example if I (G3ZCZ) want to loop back through 4X6AA, I just

need  to  enter  /4X6AA as the call to connect  to  and  the

LAN-LINK  will tell the TNC to try to connect to  G3ZCZ  via

4X6AA,  namely  issue the command 'C G3ZCZ V 4X6AA'  to  the

TNC.

6.3 Enter Call

This  option lets you enter the call of any station  calling

you into the program. If you type the enter key in  response

to the prompt, you can later use the F3 (function key 3)  to

transmit the sequence "de <yourcall>". You may also use  the

Retry feature to call that station in the non AMTOR modes.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 19

In  AMTOR,  you use the feature to enter  the  callsign  and

SELCAL  of the station you are listening to and may want  to

call.

6.4 reMember Path

You may use this option to put an entry in the  LAN-LINK.SYS

file.

6.5 Call CQ

You  use  this  command  key to  call  CQ.   In  the  Packet

Communications  mode, the TNC will transmit a line  of  text

along with the CQ packet.  The line of text is set up in the

LAN-LINK.SYS file described in section 19.

In  the  AMTOR  communications mode,  your  SELCAL  will  be

transmitted along with the CQ call in the FEC mode.

6.6 Retry

This  option  allows you to retry a connect  that  for  some

reason did not go through the first time.  It save wear  and

tear  on  your fingers particularly if the  desired  connect

path is via a number of digipeat relays or through a  number

of NET/ROM NODES.

In the non packet modes, it allows you to retransmit a  call

to another station without having to re-enter the call.

If you have previously used Enter feature to put a  callsign

in the program when you hear that station, you can then  use

the Retry command to call him later when the QSO in progress

terminates.

6.7 Show Call Directory

This  option displays the contents of the LAN-LINK.SYS  call

directory file.

6.8 Target Call

The Target call feature is used in the Packet communications

mode when disconnected. LAN-LINK  will monitor the  stations

connecting  to  you. When it sees a connect by  the  station

which you have entered as the 'Target' call, it will set the

flashing Connect Count display to show a 'happy face'.

This option toggles the Target call feature on and off.   If

it is off, you are prompted for the call to be monitored, if

it  is  on, it is turned off.  The toggle is  set  when  you

enter  a callsign, and cleared when you depress the  'Enter'

key instead of a callsign.

The Target call will be shown flashing in the status  window

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 20

when in the disconnected state.

6.9 aUto CQ

This option is the same as function key 6.

6.10 Zap BBS

This  key  initiates  a  BBS  mail  download  sequence.   It

initiates the same sequence as the BBS_Mail_Snatch.  You use

this key if you connect to a BBS before you hear its beacon,

or  if you have disabled the BBS_Mail_Snatch feature.   This

key  must be used ONLY when connected to the BBS and  BEFORE

the first '>' is received.  Note: it is that first '>'  that

triggers  the sequence.  If you have already  received  that

first '>', send the BBS any command, and the sequence should

begin when the BBS signals that it has completed  processing

that command.

The sequence performed by the BBS_Mail_Snatch is as follows:

RM        To read your messages, (Configurable)

L         To let you know what is new on the BBS,

LM        To list ALL your messages on the BBS,

B         To log you off the BBS.

LAN-LINK will wait for the '>' as the last character of  the

line of text being received from the BBS, before issuing the

next command in the sequence.  Your messages will be  placed

in a message file <yourcall.OUT> as if someone else had left

a message for you.

When  you use this command you will be asked if you want  to

Upload  or download. If you want to download, just push  the

'Enter'  key.   A Download (Mail from the BBS to  you)  will

automatically  be  followed by an Upload if the  *.BBS  file

exists.

The 'RM' may be customized in the LAN-LINK.SYS file to  'RN'

or 'VM' depending on the software in your local BBS and your

preference as shown in section 19.

During  the  time  that the BBS is  responding  to  the  'L'

command  with a list of new messages, LAN-LINK  is  scanning

the  messages to see if it finds one containing a Scan  Word

(See  Section  19).  If it does it will queue a  request  to

download that message.

7.0  Return to DOS

When  you exit the program, the TNC is put into the  VHF  default

mode  in which it rejects calls from any other  stations.   (Read

about  the 'CFROM' command in the TNC manual).  you can  thus  if

you  wish leave it powered, and if you also leave the  VHF  radio

powered  up, you will be active as a digipeater, but at the  same

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 21

time  anyone  trying  to connect with you  will  receive  a  busy

signal.   The MBX command (Reading the Mail) is also  cleared  at

this time.

You may customize the TNC state at shut down as shown in  section

19.

8.0 Files Menu

The Files Menu provides the following choices.

B Send Brag Tape

D File Directory

E Erase File

R Rename File

S Send File

Type in the single letter to make your choice.

During  the file transfer, the outgoing text will appear  in  the

OUTwindow.  If the text is long enough and fills the TNC  buffer,

you will see it stop, and then continue when the TNC is ready  to

accept  more text, the message sequence will resume.   NOTE  that

^S/^Q software flow control is used.

When you transmit a file (B or S) you are returned to the receive

mode  after the file is sent except in the Packet  communications

mode.  In the Packet Communications mode, when the file has  been

loaded into the TNC the computer will add the ^Z character to the

outgoing   text  to  signal  to  the  recipient  that  the   file

transmission is over.

The computer will signal you (using the bell character, when  the

file has been loaded into the TNC,  Note that is not the same  as

the  time that the TNC has transmitted the file, because the  TNC

incorporates a character buffer.

8.1  Send Brag Tape

This  command  allows you to transmit a standard  text  file

historically  known as a "Brag Tape".  The default  name  of

the brag tape file is LAN-LINK.TXT, and it should be created

with a word processor in the non-document mode.

8.2  File Directory

This command shows you the directory of the disk drive.

8.3  Erase File

This command allows you to erase or delete a file.

8.4  Rename File

This  command  allows you to rename a file.  You  could  for

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 22

example  rename a document file to a "callsign.OUT" file  so

that  someone will receive it automatically next  time  they

connect.

8.5  Send File

This  command allows you to transmit an ASCII file  via  the

PK-232.

9.0 Help Menu

This option will activate the HELP display. Help displays a brief

summary of the function and hot keys.  This window will remain on

display until a character is typed at the keyboard.

10.0 Log Menu

The Log Menu provides the following choices.

F Show last Few Entries

L Log Entry

P Print Log

S Scan Log

Type in the single letter to make your choice.

10.1 Show last Few Entries

This  command displays the last few log entries made  during

the  active  session. If no QSO's have been made  since  the

program was loaded the display window will be blank.  Up  to

5 log entries will be displayed in the window.  You use this

command for a quick look at which stations have connected to

you most recently in your absence.

10.2 Log Entry

LAN-LINK  performs automatic logbook entries for Packet  and

AMTOR  Beacon  mode  Connects, and  semi  automatic  logbook

entries for other modes.  The Log file is compatible to that

of  the CONTEST program in the G3ZCZ PC-HAM package and  can

be  processed  by  the DBASE  Logbook  Package  for  indexed

listings,  tracking of DXCC and other AWARDS, etc.  This  is

ideal  for special event stations and DX-peditions, for  the

whole QSL process and further statistical analyses of  their

operation has.

For  this  feature to work, the file  LAN-LINK.LOG  must  be

present  on the disk.  If it isn't, it will be  created  for

you. After one logging operation has occurred, edit out  the

initial  text.   You  must perform  this  editing  operation

before  appending  the contents of the logbook file  to  the

logbook  database of the PC-HAM LOGBOOK Package.   Edit  the

log  before appending it to the DBASE package, as there  may

be some null characters at the end of the file which may  be

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 23

recognized as a blank entry by DBASE.

It  is  also advisable not to let the logbook file  get  too

big.

When  an  AMTOR beacon mode or packet  connect  occurs,  the

connect  is automatically logged.  In the Packet  mode,  the

connect  path  is  placed in the  comments  section  of  the

logbook entry.  In the non packet modes, the log data  entry

is  set up for you with default 599 values for  the  reports

exchanged.

You invoke this option to enter the data into the log or set

the  band and power for subsequent automatic logging.   When

you invoke the command you will be presented with a  typical

menu in the menu window and a current entry line in the  log

window.

You  should choose the character associated with each  item.

For  example,  if  you want to change the  Band,  enter  the

letter  'B', to change the Comments, enter the  letter  'O'.

Enter  the text when prompted.  You will then  be  presented

with an updated entry.

To  enter  the data into the logbook, use the 'L'  key.   To

save  the  data  but  not  make a  log  entry  use  the  '*'

character,  for  example you would do that if  you  want  to

change the band or power information.

You  may use the 'qSl sent' entry to tag the fact  that  you

have written out a QSL card.

The  band and power information are set up from the  default

values stored in the configuration file LAN-LINK.SYS.   They

are  displayed in the status window.  If you are  using  the

logging  command,  you  should  change  them  when  changing

bands/powers  and  exit  the logging command  with  the  '*'

character  so  that  you don't put a blank  entry  into  the

logbook.

Note that you can also use this command to enter the odd SSB

contact  into  the  logbook  file.   If  you  do,  use   the

characters  'SSB'  or 'FM' for those voice modes  to  remain

compatible with the rest of the DBASE logbook package.

NOTE   Do  not use the comma character ',' in  the  comments

field.   If you examine the logbook file you will  see  that

all  items  are separated by comma's.  If you  use  a  comma

character in the comments you will screw up the transfer  of

data into the logbook package.

The comments field in the logbook is restricted to a maximum

of  10 characters.  If you try and enter more than  10,  the

surplus will be ignored.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 24

10.3 Print Log

This command prints the contents of the LAN-LINK.LOG file in

a formatted manner to the printer. It requests the  callsign

of  the  log  entry to be displayed. If you  want  to  print

entries  for particular calls or parts of call, enter  those

call  or parts of calls.  Valid entries are G, G3, G3Z  etc.

If  you  want  to print every entry in  the  log  enter  the

asterisk '*' character.

10.4 Scan Log

This command displays the contents of the LAN-LINK.LOG  file

in  a  formatted  manner on the  screen.   It  requests  the

callsign  of the log entry to be displayed. If you  want  to

see  entries  for particular calls or parts of  call,  enter

those  call or parts of calls. Valid entries are G, G3,  G3Z

etc.  If  you want to see every entry in the log  enter  the

asterisk '*' character.

11.0 Message Menu

The Message Menu provides the following choices.

B BBS Command Files

C Current Messages

M scan Message

N leave Note

O Old Messages

P QSP LAN-LINK

U QRU LAN-LINK

T Time tag

Type in the single letter to make your choice.

11.1 BBS Command Files

This option shows you a list of the BBS command files on the

default directory.  In other words, all files with the  file

extension of '.BBS'.

11.2 Current Messages

This  option shows you a list of the outgoing messages  that

have not been sent. In other words, all files with the  file

extension  of '.OUT'.  See Section 20 for a  description  of

outgoing  messages  in your "Smart  Packet  Radio  Answering

Machine".

11.3 Scan Message

This  feature allows you to scan messages in  the  computer.

After  depressing the key you will be prompted for  INcoming

or  OUTgoing  (I/O)  messages.   If  you  request   Incoming

messages  you  will  see  the  messages  addressed  to  your

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 25

callsign.   After scanning your messages,  the file will  be

renamed  to  yourcall.n. where n is an  incrementing  number

from 1 to 999.

11.4 Leave Note

In  all  modes  you can leave a  brief  note  (message)  for

another station.  When you invoke this command the header is

written  to the message file, then as you enter the text  it

is stored on the disk in the usual format.

WHILE  WRITING YOUR NOTE, LAN-LINK can't receive  characters

from  the  TNC. All other features ARE INHIBITED  OR  LOCKED

OUT.  THUS IF SOMEONE CONNECTS TO YOU, THE TNC WILL KNOW THE

BUT LAN-LINK WILL NOT.  BE WARNED !!!

After you have left the note and received the 'QSL'  message

from LAN-LINK, if you are in the Packet communications mode,

push  function  key  8 Alt-D to load  the  callsign  of  the

recipient into the beacon.

When  you  leave a note, leave it for the root call  of  the

connectee, for example to leave a note for G3ZCZ/W3 leave it

for G3ZCZ, similarly to leave a note for  WB8WKA-1 leave  it

for  WB8WKA.  LAN-LINK is smart enough to extract  the  root

call from any callsign it links/connects with.

11.5 Old Messages

This feature shows you a list of the messages that have been

sent.  In other words, all files with the file extension  of

'.OLD'.

11.6 QSP LAN-LINK

LAN-LINK contains store and forward routines for messages in

a Local Area Network (LAN) as described in Appendix 2.

When connected to another LAN-LINK station, you may use  the

NC/L :QSP: command to leave a message at that station.  This

QSP  command  automates that feature for you.  It  lets  you

just  enter the callsign of the station you wish to leave  a

message  for.   It will then send the  text  sequence  :QSP:

<callsign>.   The  other LAN-LINK HOST station  will  either

reply  :QNO: followed by a number or :QRV: <callsign>.   The

'QNO'  message means that the other computer  cannot  accept

the message, the :QRV: message means that it is waiting  for

you to send it.

If  you get the :QRV: reply, go ahead and type the  message,

or  send the text file containing the message.  If you  type

the  message,  use a control Z (^Z) character  and  carriage

return  to terminate the message.  (In AMTOR, LAN-LINK  will

translate the ^Z to :EOF: +? to terminate the message).  You

will  then  receive either a :QSL: from  the  other  station

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 26

meaning that the message is accepted, or a :QNO: followed by

a  number  which  means that something went  wrong  and  the

message  was  not  stored.  QNO  numbers  are  described  in

Appendix 3.

11.7 QRU LAN-LINK

The  NC/L  QRU  command allows you to  upload  a  number  of

messages  to  the other station.  You use it when  you  have

messages  pending on your system and want to pass them to  a

third  computer for storage in the LAN, while you take  your

computer off-line.

You  can only use this option with a station which you  have

previously  designated  as  a store and  forward  mail  box,

namely  one for which you have set up a <callsign>.MBX  file

(see  section  19).   This command may be triggered  by  you

using this choice or by the connecting station sending you a

:QRU: command.

When the sequence is over, the computer sending the messages

will transmit the NC/L command word :QJG: to signal that the

sequence is complete.  Should you receive it before you  get

any messages, then there weren't any pending.

If  you  want  to download messages  from  another  LAN-LINK

system,  connect to it and type the sequence :QRU:  followed

by the 'Enter' character.  That system will then respond  to

you as if the operator there had initiated a QRU command.

Appendix 2 contains more information on the LAN capabilities

of LAN-LINK.

11.8 Time Tag

This option is used to place a date time header on a message

file  previously prepared by a word processor.  The name  of

the file must be in the format 'CALLSIGN.OUT' (For  example,

G3ZCZ.OUT).   It requests the callsign of the message  file.

It then opens a new file, writes the header to the new  file

and copies the contents of the original '*.OUT' file to  the

new one.  When all is done, the original filename is  change

to '*.BAK' and the new one becomes '*.OUT'.

12.0 TNC Menu

The TNC Menu provides the following choices.

D Set TNC Date/Time

F Flush TNC Buffer

I Initialize TNC

M calls Monitored List

S Show Connect Status

Q Change CQ Text

X Change Ctext

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 27

Type in the single letter to make your choice.

12.1 Set TNC Date/Time

This  feature puts the TNC into the command mode  and  loads

the date into it from the computer time of day clock.

This command does not work on the TNC1.

12.2 Flush TNC Buffer

This  option  is used to flush the program's  outgoing  text

buffer and that of the PK-232.

12.3 Initialize TNC

This feature sets up the defaults for the TNC from the  file

LAN-LINK.SYS.   If  you have put batteries in the  TNC,  you

should normally only have to run this command when you first

power up the TNC or change the batteries.

It  will also set the date and time within the TNC from  the

computer  clock, and, when it has loaded  the  configuration

file,  may prompt you to set the TNC Terminal mode.  If  you

are  using a TNC1 or a TNC2, it will not prompt you  because

it assumes that they only operate on VHF packet. If you  are

using a KPC-2, you may select 'H' or 'V', and of course,  if

you have a PK-232, you may chose any of the options.

12.4 Show Calls Monitored List

This feature performs the same operation as Function key  5.

It is equivalent to the TNC 'MH' command.

12.5 Show Connect Status

When you are in the Packet communications mode, this command

lets you see the state of the connection.  It will show  you

if  there  are any unacknowledged packets en  route  to  the

other  station.   If  you  have been  sending  text,  it  is

advisable  to  use this command before  you  disconnect,  to

ensure that all the data has been received and  acknowledged

by the other party.  You may use this command in either  the

TNC Command or the TNC Converse mode.

In a multi connect situation, it will show you the status on

all the IO channels.

You  may  also use this command to reset the  status  window

display  after  a relink or any other event  which  was  not

reflected in the status window.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 28

12.6 Change CQ Text

This  feature  allows you to change the contents of  the  CQ

text message string. This is the string that is  transmitted

along  with the CQ call.  The default value is  loaded  from

the LAN-LINK.SYS file.

12.7 Change Ctext

This  feature allows you to change the message that will  be

received by another packet station when it connects to  you.

Refer to the 'CTEXT' command in the TNC manual.

13.0 Parameter Menu

The Parameter Menu provides the following choices.

B Turn Beacon Mode On/Off

C Clear Connect Count

D Turn Digipeat Detect On/Off

F Display DeFaults

M Set Message Count

N Turn NETROM Drop Out On/Off

P Turn Printer On/Off

Q Set CQ delay time

R Turn Relay On/Off

S Turn Sound On/Off

T Turn Contest Mode On/Off

Type in the single letter to make your choice.

13.1 Turn Beacon Mode On/Off

This command toggles the beacon option for the automatic  CQ

caller  in the AMTOR and Packet communications modes.   When

it  is active a [B] will be displayed in the status  window.

In  this mode, the Automatic CQ sequence will  resume  after

the  AMTOR link is dropped or a Packet  Communications  mode

disconnect.

In  the  beacon mode, the last few logbook entries  will  be

displayed in the logpage window which overlays the  outgoing

text window at the bottom of the screen.

In   the  Packet  mode,  the  CTEXT  message  is  the   only

transmission  from you that the other station will  receive,

so make it good and don't forget to set 'CMSG ON'.

In  the AMTOR Mailbox/Beacon mode, the contents of the  LAN-

LINK.TXT file will be transmitted to the station who  called

you.   The capture-to-disk will then be turned on to  record

one  transmission from that station.  The QSO will  then  be

logged  to  the  logbook file.  Don't forget  to  check  the

entries  in  the LAN-LINK.LOG and text in  the  LAN-LINK.RUN

files in case the computer made a mistake in recognizing the

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 29

call sign of the other station.

THE  BEACON MODE IS DESIGNED TO ENCOURAGE PEOPLE TO QSY  OFF

THE  CROWDED PACKET HF CHANNELS. DO NOT USE IT ON A  CROWDED

HF CHANNEL OR ON AN ACTIVE VHF CHANNEL.  IF YOU DO, YOU WILL

SOON FIND THAT OTHER STATIONS HAVE SET THEIR TNC'S TO REJECT

YOUR  PACKETS, AND YOU WILL NOT WORK ANYBODY, YOU WILL  JUST

BE A SOURCE OF QRM AND WILL BE KNOWN AS SUCH.

THE  MODE IS ALSO DESIGNED FOR ROBOT BEACONS FOR THE  HIGHER

FREQUENCIES.  FOR EXAMPLE IF YOU LEAVE IT RUNNING ON 6 OR 10

METERS YOU NEVER KNOW WHEN A SHORT OPENING WILL ALLOW A  QSO

TO  SOME  PART OF THE WORLD,  A DX CONTACT  YOU  WOULD  HAVE

OTHERWISE  MISSED.    DO NOT LEAVE IT RUNNING ON  20  METERS

WHEN  THE  BAND  IS  OPEN.   FURTHER  APPLICATIONS  ARE   IN

METEORSCATTER AND OTHER "EXOTIC" MODES OF COMMUNICATIONS.

If you are using this mode in a special event station or DX-

pedition  you may type characters into the transmit  buffer,

while the other station is sending to you.  When he, she  or

even  it  turns  it back to you,  you  will  transmit  those

characters before the sign off sequence.

In both the Packet and AMTOR communications modes, the robot

may  be shut down by stations connecting to you and  telling

you to QRT.

13.2 Clear Connect Count

You  use  this option to reset the PACKET and  AMTOR  beacon

mode  "connect  has taken place indicator"  and  QTC  Snatch

count.   These  are  flashing [C] and [Q]  displays  in  the

status  window.  Whenever someone else connects or links  up

to  you  the [C] display will be  incremented.   The  number

shown with the 'C' is the number of connects that have taken

place since the Indicator was last reset.  If a 'happy face'

is  flashing  along  with the connect  count,  your  "target

station"  was  amongst those connecting. The  [Q]  indicator

alerts  you  to the fact that LAN-LINK tried to  connect  to

another  station  after  recognizing your  call  in  a  mail

beacon.   Both  are  reset  by  this  option.   If  you  are

monitoring  Packet traffic on a channel, you may not  notice

that a connect has previously taken place.  These  indicator

alert  you to check the contents of the Incoming  mail  file

('yourcall.OUT') for any incoming messages.

If the 'Q' count is much greater than the 'C' count you  may

be  copying a beacon from a station that you cannot  connect

with directly.

13.3 Turn Digipeat Detect On/Off

This option toggles the digipeat display command on and off.

If  the Digipeat Display is active, any  signals  digipeated

through  you will be stored to the LAN-LINK.RUN file  and  a

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 30

'beep'  will  be sounded for each packet  being  digipeated.

The status display window will also contain a [D].

13.4 Display DeFaults

This feature lets you display how LAN-LINK was configured by

the  LAN-LINK.SYS file.  It also displays the state  of  any

flags  you have set/reset if they are not already  shown  in

the Status window.  YOU SHOULD USE IT AFTER EDITING THE LAN-

LINK.SYS FILE TO MAKE SURE THAT EVERYTHING IS CORRECT.

13.5 Set Message Count

As  an  alternative to function key 8 or Alt-D,  using  this

option  will  also load a QTC 'Mail' list into  your  Packet

communications   mode   Beacon   and   initiate   a   beacon

transmission every 30 minutes (Refer to the BTEXT command in

the  TNC manual).  If no messages are present, or  the  only

message   is   to  you  (incoming   or   snatched),   beacon

transmissions   are  inhibited.   This  conforms   to   good

operating practice on crowded Packet channels.

13.6 Turn NETROM Drop Out On/Off

When  connecting via NETROM, the normal way of operating  is

to  disconnect  if  the  connect  attempt  fails.   You  may

configure the NETROM connect attempt flag to stay  connected

at the node where the failure occurred.  Use the normal mode

when  trying  to get through on a known path, and  the  stay

alive mode when exploring NETROM circuits.

13.7 Turn Printer On/Off

This  option  toggles  the printer on  and  off.   When  the

printer  is turned on, any incoming data will be printed  in

real  time.  If you intend to print files received  at  1200

bauds  in  the  packet communications   mode,  use  a  print

spooler or you may loose data.  The printer is automatically

turned off in the packet mode when a disconnect occurs.

13.8 Set CQ delay time

This  option allows you to change the time that the  program

waits for a response after a CQ call until the following one

is  transmitted.  The default value is loaded from the  LAN-

LINK.SYS file.  You may want to experiment since the  actual

time  will  depend on the clock frequency of  the  computer.

Make  it a small number such as 4 or 6 when operating  AMTOR

or  a  DX-pedition, and make it a larger number such  as  30

when  operating  on crowded packet  channels.   The  maximum

allowed value is 255.

13.9 Turn Relay On/Off

This  option  toggles the Store and  forward  Message  Relay

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 31

Capability of your system on and off.  If the Message  Relay

Capability is active (toggled on), any one who connects  can

store a message in your system using the ':QSP:  <callsign>'

NC/L command. If the Message Relay Capability is not  active

anyone  who  attempts to ':QSP: a message'  will  receive  a

':QNO:  1' "error" message.  The status display window  will

show a [R] when the Message Relay Capability is active.  The

default  state  for the Message Relay Capability  is  loaded

from the LAN-LINK.SYS file.

When the Store and forward capability is active, the  suffix

'R' will be added to the

'[ZCZ] *** LAN-LINK 1.50'

message  received  by anyone connecting to  your  system  to

notify  them that they can store messages.  In  this  manner

they  don't  have  to try to leave a message  and  then  get

':QNO:  1'  error  message to find out  if  the  feature  is

toggled off.

13.10 Turn Sound On/Off

This  option toggles the sound on and off.  If the sound  is

off,  a flashing indicator will be displayed in  the  Status

Window.   When the sound is toggled on again, a  chime  will

sound to verify the audio.

13.11 Turn Contest Mode On/Off

This option first toggles the contest mode.  If the  contest

mode  is active, the word 'TEST' and the number of the  next

QSO will be displayed in the status window. When the contest

mode is toggled on, this command then prompts you to set the

starting  QSO  number  and  write in  the  non  changing  or

standard  message  that will be transmitted as  part  of  an

exchange of messages during a contest.

If  you set the QSO number to '1' it will be transmitted  in

each  exchange  and  then incremented each time  you  log  a

contact.   If you set it to '0', it will not be  transmitted

nor  incremented.  Use the '0' option for contests which  do

not require serial numbers.

The contest feature is toggled off the next time you  select

this  command, or when you change the PK-232  Communications

mode (for example, switch from Morse to BAUDOT).

Note  that  while  this does give you  some  semi  automatic

contest  operation  by  using  command  keys,  there  is  no

checking of callsigns for duplicates built into the program.

In  a contest, you enter the callsigns  and call  the  other

station  in  the normal manner, then use the  'End'  key  to

transmit  the exchange (QSO number and standard  text).

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 32

14.0 Q Codes [NC/L]

This  option  displays  help information about  the  use  of  the

Network Communications Language (NC/L) "Q" codes.

15.0 Run DOS Command

This option allows you to enter a DOS shell.  Make sure that  you

restore  the  system  to the directory  which  contains  LAN-LINK

before  you exit from the shell.  If you don't, LAN-LINK will  be

searching  the  wrong  directory for  files,  and  display  ERROR

messages.

You may use  the DOS shell while in QSO when you have loaded  the

transfer buffers as well as while monitoring activity.

16.0 CommunicationS Modes

This menu allows you to change the communications mode of the PK-

232.   If  your TNC hardware cannot support  those  modes,  don't

bother to try them.  This is software not hardware, and in  spite

of  what  people  tell you, software can't  do  everything.   The

following communications modes are available.

V VHF PACKET

H HF PACKET

A AMTOR (STANDBY RECEIVE)

B BAUDOT (RTTY)

C MORSE (CW)

M AMTOR (ALIST [chirpcopy])

S ASCII (RTTY)

G SIGNAL

N NAVTEX

Type  in the single letter to make your choice.  Don't forget  to

select the correct Radio port on the PK-232.

16.1 VHF PACKET

This  is  the  1200  Baud VHF  Packet  mode.  LAN-LINK  will

optimize the TNC link parameters for this mode.

16.2 HF PACKET

This is the 300 Baud HF Packet mode. LAN-LINK will  optimize

the TNC link parameters for this mode.

16.3 ASCII (RTTY)

This is the ASCII RTTY mode. LAN-LINK will configure the TNC

parameters for this mode at 110 bauds.

16.4 BAUDOT (RTTY)

This  is the BAUDOT RTTY mode. LAN-LINK will  configure  the

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 33

TNC parameters for this mode at 45 bauds.

16.5 MORSE (CW)

This  is  the  CW  mode. LAN-LINK  will  configure  the  TNC

parameters for this mode at 20 wpm (10 wpm for the KAM).

In this mode the ALT-D key will disengage the receive  speed

lock  inside the PK-232 to facilitate locking  onto  another

signal.

16.6 AMTOR (STANDBY RECEIVE)

When  you select the AMTOR standby mode you are placed  into

the  'standby  receive' mode.  If you are in  AMTOR  standby

mode,  you  will  not  be able to  display  signals  of  the

chirpcopy  mode  (which means that you will not be  able  to

display a QSO in progress).

The  Alt-D key or function key 8 toggles between  the  AMTOR

ARQ and the ALIST communications modes.

16.7 AMTOR (ALIST [chirpcopy])

If  you  want to read the mail on an existing AMTOR  QSO  in

progress, select the AMTOR ALIST or chirpcopy mode.

The  Alt-D key or function key 8 toggles between  the  AMTOR

ARQ and the ALIST communications modes.

16.8 SIGNAL

Use  the 'G' option to select the 'SIGNAL' mode.   Read  the

PK-232  documentation carefully to understand how this  mode

works.  Once the PK-232 has identified the digital signal it

is  tuned  to,  use the Communications Mode  Select  menu  a

second  time  to select the relevant mode and then  set  the

baud rates and shift to display the data if necessary.

16.9 NAVTEX

Use the 'N' option to select the 'NAVTEX mode.  Read the PK-

232   documentation  carefully to understand how  this  mode

works.

17.0 Terminal Mode Menu

This menu allows you to set the terminal configuration mode.  The

Terminal  Menu  provides  different choices  depending  on  which

communications mode you have selected.

17.1 Packet Terminal Menu

The  Packet Terminal mode of operation will be shown in  the

Status window at the top of the screen.  If you are  reading

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 34

the mail on a station, the callsign of that station will  be

shown  next  to the word "MAIL".  If you  are  connected  to

someone,  the call (and path) will be shown prefixed by  the

"*>" sequence.

In  the Packet Communications mode, you are  presented  with

the following choices.

B Change Packet Baud Rate

C CQ/Beacons

E Everything

M Read the Mail

S Just My Station (Solo)

T All Traffic

0 Multiple User Connects

1 Single User Connects

F Conference

R Conference Bridge On/Off

Type in the single letter to make your choice.

17.1.1 Change Packet Baud Rate

In  the PACKET mode you may change the RF baudrate   between

300, 600, 1200 and 2400 bauds.

17.1.2 CQ/Beacons

In  this  mode, you will see CQ and BEACON  packets  on  the

channel.  (This corresponds to 'MONITOR 1'on the PK232).

This mode only works on the PK-232 and the KAM.

17.1.3 Everything

You can set the TRAFFIC mode to 'Everything'. It corresponds

to 'MONITOR 6' on the PK-232.

17.1.4 Read the Mail

You  can  read the mail on a BBS or other  station  and  get

bulletins without connecting to that station yourself.   You

use  this mode to do so.  you will be prompted for the  call

whose  packets you wish to monitor.  Remember to  enter  the

call of the station sending the packets, not the call of the

one  receiving them.  (This corresponds to 'MONITOR  0'  and

'MBX' callsign). This mode only works on the PK-232.

You  can also use this command to monitor both  sides  (read

the mail) on a QSO on a busy packet channel.

You may want to turn the Digipeat Data Display mode off when

reading the mail.

If  you are not using a PK-232 you will see the  headers  as

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 35

well as the traffic.

17.1.5 Just My Station (Solo)

In  this mode, you will only see messages addressed to  you.

You  will only get messages from people who connect to  you.

(This corresponds to 'MONITOR 0').

17.1.6 All Traffic

In this mode you will see all of the data packet traffic  on

channel.   You  can use this mode to check that the  TNC  is

working. (This corresponds to 'MONITOR 4' or 'MONITOR ON').

17.1.7 Single User Connects

This is the normal mode one on one mode of operation ('USERS

1').

17.1.8 Multiple User Connects

When you are in a multiple connect mode, an asterisk will be

shown in the status window next to the word 'PACKET'.

LAN-LINK supports two multiple connect modes, individual and

conference.

An  extract of an actual 'off the air' CRT display  (showing

received  text  only) in a multi user mode is  shown  below.

Note  each line of text as received by the TNC  is  prefixed

(by  the PK232) with the IO channel number and  callsign  of

the station its connected to.

1:4X4IL: *** CONNECTED to 4X4IL via 4X4IC, 4X6JP

0:4X4IC: joe do u read?

2:4Z4ZB: *** CONNECTED to 4Z4ZB

1:4X4IL: Look for me on PR via 4X6JP 73s and Shalom.

2:4Z4ZB: GE JOE I WILL DIS THEN U CAN GO VIA THE REP....>

1: *** DISCONNECTED: 4X4IL via 4X4IC, 4X6JP

0:4X4IC: UNPROTO  CQ

2:4Z4ZB: GO AHEAD BOYS...DE ALON..>

0:4X4IC:

2: *** DISCONNECTED: 4Z4ZB

This  is the normal Multi Connect Mode as described  in  the

TNC  manual.  Here you  are connected to up to  10  stations

and will send different traffic to each of them.  Each  time

you wish to send something to a particular station, you must

select  the  IO channel the station is connected  on  before

typing the text or sending the file.

The stream change character for all TNCs in the VHF multiple

connect  modes is '|' ($7C). In the KAM, it is'\' on HF  and

'|' on VHF.  Its the same key as now but one is in the shift

case.   Be  careful  on  HF as the  '\'  shows  up  in  file

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 36

directories and confuses the computer.

You select the channel by typing the '|' character  followed

by (in the TNC) the channel number.  For example if you  are

connected  to  G8BTB  on IO channel 0 and  to  G3ZCZ  on  IO

channel 1, then to talk to G8BTB you would type '|0'  before

any text, and similarly to send something to G3ZCZ you would

type a '|1' before the text you're going to send me.  If you

are  not using a PK232, read the TNC manual to determine  if

the IO streams are identified by numbers or by letters.  The

TNC2 standard configuration uses letters.

17.1.9 Conference

In  the conference mode on the other hand,  everything  that

you  type  at the keyboard is automatically  transmitted  to

each  station that you are connected with.  Thus if you  are

linked  to two stations each line will be packeted twice  by

the  TNC.  You don't have to worry about sending  the  wrong

thing to the wrong person, as they will all get the stuff.

You  enter  this  mode by selecting the 'F'  option  in  the

Terminal Menu.  You terminate the mode by selecting the  '1'

option.

You  connect in the usual manner using Alt-C or the  Connect

option in the Call Menu.  You MUST WAIT FOR A CONNECTION  TO

BE  ESTABLISHED OR TIME OUT before attempting to connect  to

the  next  station.  If you do not, you will  get  an  error

message back from the TNC.

In  the conference mode, as the same text is transmitted  on

all IO channels, the throughput will be reduced.  Slow  down

your  typing speed or use short packets.  Should  you  check

the link status or call monitored list, you may not be  able

to  get  back into the converse mode if you  have  too  many

packets outstanding. If that happens, wait a while and  then

try  to set the converse mode with command key 10.  You  may

have to try it a few times, depending on how good your links

are.

DON'T  TRY  TO SEND A FILE IN THE CONFERENCE MODE,  ITS  NOT

DESIGNED FOR IT. KEEP THE PACKETS SHORT.

BE ALSO WARNED THAT IF YOU USE THE CONFERENCE (BRIDGE)  MODE

ON  BAD LINKS (SUCH AS CROWDED HF CHANNELS AND  OTHERS  WITH

LOTS  OF RETIRES), THE KEYBOARD MAY SEEM TO LOCK  UP,  WHILE

LAN-LINK  WAITS  FOR SPACE IN THE TNC BUFFER TO  OUTPUT  THE

TEXT.

The conference mode will only work in the PK-232 if  DCDCONN

is  set to ON.  So if you are using a PK-232, LAN-LINK  will

do it for you when you enter the conference terminal mode in

case you forgot or/and haven't set the LAN-LINK.SYS file  to

include the line 'DCD ON', At the same time set LAN-LINK.SYS

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 37

default  states to show the calls of other stations on  each

incoming  line  in the MULTI connect modes.  (Read  the  TNC

manual for how to do this).

The conference mode will only work in the TNC2 if DCDCONN is

jumpered so that Pin 8 of the RS-232 Interface changes state

when you connect to someone else.  If you get error messages

and see the characters '|J' on your screen, then the  jumper

probably isn't set.

The conference mode does not work on the TNC1.

In  the  conference  mode,  if  any  one  of  the   stations

disconnects,  the TNC will return to the command  mode,  and

you  must  use command key 10 to return the program  to  the

converse mode before you type another character of text.

17.1.10 Conference Bridge On/Off

This option toggles the conference bridge on and off. In the

Bridge  Multi  Connect  Mode you are  acting  as  a  central

switch.   You  can link together up to 10  stations  in  one

multi way connection.  In this mode any packets arriving  on

any  steam  are echoed to all other streams  that  you  have

'connects' on except the one that they came in on.  Anything

you type will be sent to all connected streams.

In  this  manner, all stations will not see their  own  text

echoed  back.  Each line of text output by LAN-LINK in  this

mode  will contain the callsign of the  station  originating

the  packet. Your packets will contain your call as  entered

in  line 1 of the LAN-LINK.SYS file.  Thus for  example,  if

your  call was KB8RP/4X and normally shows up in the  packet

header as KB8RP here it will show in the data as KB8RP/4X.

Just remember that the bridge puts out a lot of packets,  so

do multi way connects on a quiet frequency.  You must be  in

the  conference  mode  to  turn  the  bridge  on,  and   any

disconnect  will take the bridge down.  Should that  happen,

toggle  it on again.  A bridge will be shown in  the  status

window whenever the conference bridge is up.

17.2 AMTOR Terminal Menu

In the AMTOR Communications mode, you are presented with the

following choices.

C Force Break [ACHG]

F Start Mode B [FEC]

L Force LETTERS case

M Read the Mail [ALISTEN]

O Override ROBOT/Mailbox

S Standby Receive [ARQ]

R Set ROBOT/Mailbox

W CW Identification

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 38

Type in the single letter to make your choice.

17.2.1 Force Break [ACHG]

This  option forces a changeover or 'break in'.  If for  any

reason  you want to interrupt the other station you  send  a

'BREAK' signal in software this way.

17.2.2 Start Mode B [FEC]

This option places the terminal into the FEC transmit  mode.

This  puts  you  on  the air,  anything  you  type  will  be

transmitted.  Function key 10 does the same thing.

17.2.3 Force LETTERS case

This  option  lets  you force the  PK-232  receiver  to  the

letters  case  if for some reason you appear to  be  in  the

numbers case.  It will return you to the converse mode.

17.2.4 Read the Mail [ALISTEN]

This option lets you read the Mail on a QSO.  Unfortunately,

when  you tune in a chirpy AMTOR QSO in the ARQ  or  standby

mode you won't get any copy on the QSO in progress.  You can

however read the mail on one by entering the ALISTEN mode.

17.2.5 Override ROBOT/Mailbox

This option lets you override the ROBOT BEACON and chat to a

station  that  has linked to the ROBOT.  Use  it  after  the

message in LAN-LINK.TXT has been sent, namely when the other

station is sending a message to you.

17.2.6 Standby Receive [ARQ]

This option returns you to the ARQ standby mode. You must be

in this mode to receive signals from another station who  is

calling  you.   When  you  tune for  CQ  calls,  they  sound

somewhat like fast RTTY with a ringing sound similar to  the

connect  alarm  used by LAN-LINK (but at  a  faster  ringing

rate).

17.2.7 Set ROBOT/Mailbox

This  option lets you turn on the Mailbox during  a  regular

AMTOR  QSO so that the other station can leave a message  or

download  a  file remotely. You must only use  it  when  the

other station is sending text.

17.2.8 CW Identification

This  option is used to send a CW identification.  The  CWID

is sent in the FEC mode.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 39

The  identification  consists  of  the  sequence  'DE  <your

callsign>' where your call sign is that entered into the PK-

232 as  'MYCALL'.

You  use  this  feature  if your  license  requires  you  to

identify in morse code as well as in the operating mode.

17.3 Baudot Terminal Menu

In  the Baudot Communications mode, you are  presented  with

the following choices.

B Set Baud Rate

F Select Shift

N NAVY MARS Message Protocol

U USOS Off

S SELCAL set/reset

W CW Identification

Type in the single letter to make your choice.

17.3.1 Set Baud Rate

This feature allows you to set a desired communications baud

rate  directly.  You will be prompted to enter  the  desired

rate.  Use this command to avoid having to shift through too

many intermediate states, if for example, you want to make a

direct change from 45 to 100 bauds.

17.3.2 Select Shift

This  feature lets you select Wide (850 Hz) or  narrow  (170

Hz) shift directly.  if you have a KAM, you may also  select

the Commercial Shift (425 Hz).

The  shift is displayed in the status window at the  top  of

the screen. It is shown as 'W', 'N' or 'C' for wide,  narrow

or commercial respectively.

17.3.3 NAVY MARS Message Protocol

The Navy Mars option toggles the Navy MARS mode on and  off.

This  feature is only present on the PK-232.  When  On,  the

feature provides the following protocol.

17.3.3.1    Any  Carriage  Return  or Enter  (CR)  from  the

keyboard is sent as  CR, CR, (Line Feed) LF.

17.3.3.2    In  a file transmission lines are terminated  by

the same CR,CR,LF sequence.

17.3.3.3    In a file transmission, blank lines are sent  as

a LF (without a CR).

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 40

17.3.3.4    In  a  file  transmission, a plus  sign  '+'  is

replaced   by   a  sequence  of   12   "letters"

characters.   There can be more than one +  sign

on a line.

17.3.3.5    In  a file transmission, the sequence 'NNNN'  is

followed   by   a  sequence  of   12   "letters"

characters.   There  can  only  be  one   'NNNN'

sequence on a line.

When the NAVY MARS feature is active a [N] will be displayed

in the status window.

17.3.4 USOS On/Off

This  option toggles the Unshift on space (USOS) feature  on

and  off.   When  the USOS is on, the  TNC  reverts  to  the

letters shift each time a 'space' character is received.  It

helps when displaying noisy copy.

If USOS is active, 'USOS' will be shown in the status window

at the top of the screen.

17.3.5 SELCAL set/reset

This  command  key toggles the SELCAL feature  on  and  off.

When the SELCAL is activated, as shown in the status window,

the incoming text from the TNC is scanned.  When either  the

letters  "ZCZC" or 'your callsign' (as entered in line 1  of

the LAN-LINK.SYS file) is received the "capture-to-disk"  is

turned  on.  All subsequent text will be stored in the  LAN-

LINK.RUN  file  until the letters "NNNN" are  received.   At

that time the "capture-to-disk" is turned off.

You  may  use this feature when monitoring  commercial  RTTY

broadcasts  to  record news and other information,  or  when

calling CQ to start recording all incoming text and keep the

QSO on disk.

You  may  use  function  key 1  turn  the  capture  to  disk

permanently  on  to override the automatic  turnoff  if  you

wish.

17.3.6 CW Identification

This  option is used to send a CW identification.   You  use

this if your license requires you to identify in morse  code

as well as in the operating mode.

The  identification  consists  of  the  sequence  'DE  <your

callsign>' where your call sign is that entered into the PK-

232 as  'MYCALL'.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 41

17.4 ASCII Terminal Menu

In the ASCII Communications mode, you are presented with the

following choices.

B Set Baud Rate

F Select Shift

S SELCAL set/reset

W CW Identification

Type in the single letter to make your choice.

17.4.1 Set Baud Rate

This feature allows you to set a desired communications baud

rate  directly.  You will be prompted to enter  the  desired

rate.  Use this command to avoid having to shift through too

many intermediate states, if for example, you want to make a

direct change from 110 to 1200 bauds.

17.4.2 Select Shift

This  feature lets you select Wide (850 Hz) or  narrow  (170

Hz) shift directly.  if you have a KAM, you may also  select

the Commercial Shift (425 Hz).

The  shift is displayed in the status window at the  top  of

the screen. It is shown as 'W', 'N' or 'C' for wide,  narrow

or commercial respectively.

17.4.3 SELCAL set/reset

This  command  key toggles the SELCAL feature  on  and  off.

When the SELCAL is activated, as shown in the status window,

the incoming text from the TNC is scanned.  When either  the

letters  "ZCZC" or 'your callsign' (as entered in line 1  of

the  LAN-LINK.SYS file) is received the  capture-to-disk  is

turned  on.  All subsequent text will be stored in the  LAN-

LINK.RUN  disk file until the letters "NNNN"  are  received.

At that time the capture-to-disk is turned off.

You  may  use  function  key 1  turn  the  capture  to  disk

permanently  on  to override the automatic  turnoff  if  you

wish.

17.4.4 CW Identification

This  option is used to send a CW identification.   You  use

this if your license requires you to identify in morse  code

as well as in the operating mode.

The  identification  consists  of  the  sequence  'DE  <your

callsign>' where your call sign is that entered into the PK-

232 as  'MYCALL'.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 42

17.5 CW Menu

In  the CW Communications mode, you are presented  with  the

following choices.

L Lock to Receive Signal

S Set Morse Speed

U Unlock Morse Speed

Type in the single letter to make your choice.

17.5.1 Lock to Receive Signal

This option locks the PK-232 to the speed of the incoming CW

signal.

17.5.2 Set Morse Speed

This option lets you set the CW speed.

17.5.3 Unlock Morse Speed

This option unlocks the PK-232 from the incoming CW  signal.

It  performs the same task as the Alt-D key.  It is used  to

speed  up capture of a new signal or to break the lock  when

the PK-232 appears to have locked up at the wrong speed.

18.0 Hot Keys

LAN-LINK  also  contains a number of "Hot" keys to allow  you  to

perform common operations with a minimum of key strokes.  A  list

of what is available is shown below, followed by a description of

what happens when you push each key.

Alt C- Call/Connect to

Alt D- Disconnect or Receive

Alt L- Edit Log

Alt M- Message Scan

Alt R- Retry Call/Connect

Alt X- Exit to DOS

Alt Z- Zap BBS

18.1 Alt C- Call/Connect to

This  key performs the same task as the Call option  in  the

Call Menu.  The Alt C key has been provided for those people

whose  fingers  have learnt YAPP and don't wish  to  be  re-

educated.

18.2 Alt D- Disconnect or Receive

This key performs the same task as function key 8.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 43

18.3 Alt L- Edit Log

This  key performs the same task as the Edit Log  choice  in

the Log Menu.

18.4 Alt M- Message Scan

This  key performs the same task as the Message Scan  choice

in the Message Menu.

18.5 Alt R- Retry Call/Connect

This  key performs the same task as the Retry choice in  the

Call Menu.

18.6 Alt X- eXit to DOS

This key performs the same task as the Exit to Dos choice in

the Main Menu. It exits the program and returns you to DOS.

18.7 Alt Z- ZAP BBS

This key performs the same task as the Zap BBS choice in the

Call Menu.

18.8  End- > +?  [Over]

In  the Packet communications mode this command  sends  your

callsign  followed by the > and Line feed  characters.   You

may use this to signal 'OVER' in a packet contact.

In  the  AMTOR  communications mode  this  command  sends  a

callsign   sequence  followed  by  the  '+?'   sequence   of

characters which are the software 'OVER' signal.

In  the  MORSE and RTTY (Baudot  and  ASCII)  communications

modes, this command lets you transmit the following sequence

<other  call) DE <your call> and returns you to  the  PK-232

Command  Mode.   For example if you are 4X6AA  and  you  are

working G3ZCZ then this command would transmit the  sequence

'G3ZCZ de 4X6AA'.  You use this at the END of a transmission

in any mode except Packet.

If  you  have typed something into the 'type  ahead  buffer'

then touch this key, the contents of the buffer will will be

transmitted followed by the callsign sequence.

18.9 PgUp- Scrollback (Scrolls back incoming text).

This key lets you repeat the display of incoming text  which

has  scrolled out of the incoming window.  you may print  it

or capture it to disk during the repeat.

A  small menu displays your options when showing the  scroll

back information.  You may move the menu window up and  down

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 44

with  the left and right arrow keys.  Use the 'ESC'  key  to

terminate the scrollback mode.

You can save the contents scrollback buffer displayed on the

screen by turning the "capture to disk' on, and then writing

text to the window.  Anything written will be captured.

18.10 The HOME Key

The  home key will clear the window screen that  the  cursor

happens to be in at the time that the key was depressed, and

will then position the cursor at the top left hand corner of

that window.

18.11 The DOWN ARROW key

The  DOWN  ARROW  key toggles  the  Normal/Reverse  Sideband

command in all PK232 communications modes except MORSE.  The

sideband is depicted in the status window by the up and down

arrows.  If the down arrow is showing, you are set for Lower

Sideband (LSB) which is the normally used one.

If you have reversed the sidebands a flashing up arrow  will

be displayed.

If you set your transceiver to LSB you will be set correctly

for all modes.

For CW operation you may set your rig to the CW position and

ignore the sideband setting.  If you use FSK just set it  so

that it works for you.

18.12 Shift/Baudrate Control keys

The  following keys are used to control the Shift  (Wide  or

narrow) and the Radio link baudrate in the BAUDOT and  ASCII

communications    modes.    In   the   BAUDOT   and    ASCII

communications modes all baud rates supported by the  PK-232

may be selected.

18.12.1 RIGHT ARROW speeds up the baud rate.

18.12.2 LEFT ARROW slows down the baud rate.

18.12.3 UP  ARROW toggles the shift from Wide (850  Hz)

to Narrow (170 Hz) and back.

18.12.4 DELETE toggles the USOS command (RTTY only).

The  state of each of the above commands are  displayed

in the status window at the top of the screen.  If USOS

is active, 'USOS' will be shown.  The shift is shown as

'W' or 'N' for wide or narrow respectively.

18.13 Control Z (^Z)

This key is used in the Packet communications mode to signal

the  end  of a file or message, when loading  messages  into

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 45

another  LAN-LINK  system  or  into a  BBS.   In  the  AMTOR

communications mode, it is automatically translated into the

sequence  ' :EOF: +? ' which can be used as a fast  break-in

changeover or to terminate a message being stored in another

LAN-LINK AMTOR mailbox.

19.0  LAN-LINK disk files

The following disk files are associated with the program.

19.1  LAN-LINK.EXE

This is the program.  You activate it from DOS by typing the

word  LAN-LINK  followed by ENTER in the  normal  manner  of

invoking DOS commands.

19.2  LAN-LINK.SYS

This  file contains the configuration information  for  both

the  program and the TNC.  YOU MUST EDIT IT before  you  run

the  program  on  the air the first  time.   Use  your  word

processor  in  the NON-Document mode.    There  are  several

things which you can customize according to your preference.

YOU  CAN USE OPTION 'F' IN THE PARAMETERS MENU TO CHECK  THE

DEFAULTS.

The  first  few lines of the default file together  with  an

item number are shown below.

1.   G8BTB

2.   GBTB

3.   CQ, This is G8BTB/W3 (Silver Spring) Md. calling.

4.   2

5.   10

6.   4

7.   1

8.   1

9.   KAM

10.  1200

11.  LAN-LINK.LOG

12.  LAN-LINK.RUN

13.  LAN-LINK.DIR

14.  LAN-LINK.TXT

15.  Mail

16.  1

17.  1

18.  0

19.  8

20.  0

21.  1

22.  1

23.  \LAN-LINK\FILES\

24.  RM

25.  AMSAT

ARRL

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 46

26.  * end of Scan Words, start of defaults

Consider the function of each line in turn.

19.2.1 Full callsign.

The  first line  contains your callsign.  Please do not  use

G8BTB.  Replace G8BTB  with yours.  You may use / characters

if you wish.  This is the call sign that will be used in the

non- Packet modes.  You may thus use "portable  designators"

such  as G3ZCZ/A, G3ZCZ/W3 or G3ZCZ/4X, which you cannot  do

in Packet.

This is the call that will be monitored for in the  Digipeat

monitoring  mode.  In this mode which is toggled on and  off

by the Alt F key, if someone uses you as a digipeater  while

you  are  not connected to anyone else, the  event  will  be

logged  to disk.  This mode will only work if your  callsign

as  stored in this line is a valid (uppercase  only)  packet

callsign.

For  example, in the PK-232, if the callsign is  G3ZCZ  then

the  sequence >G3ZCZ> will be recognized as a  digipeat  and

the  event logged.  Thus 4X6AA*>G3ZCZ>4Z4ZB would be  logged

as a digipeat event.

If you have a / character in your callsign, and use the base

call  for  packet identifying the location in the  CQ  text,

then  the  function  will work if you  place  the  "portable

location" as a suffix.  For example, if the call is G3ZCZ/4X

and  the packet identification is G3ZCZ, then  the  digipeat

monitor  will work. If the call was entered as 4X/G3ZCZ  the

digipeat monitor would check for 4X and would not appear  to

work.

Store  your callsign using UPPERCASE characters if you  want

the DIGIPEAT DETECT option to work (IE. G3ZCZ).  If you  use

lower  case (ie. g3zcz), recognition will never take  place.

The TNC automatically converts case to upper case in the non

packet communications modes.

If  you  are  using an SSID (e.g.  G3ZCZ-8)  in  the  packet

communications  mode,  you  should  enter  the  whole   call

INCLUDING the SSID.

19.2.2 The SELCAL.

The  second  line contains the AMTOR SELCAL  configured  for

your  callsign.  You use it again later to tell  the  PK-232

what the SELCAL is.  This line just tells it to LAN-LINK.

If you are not using a PK-232 or KAM, the line must still be

there.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 47

19.2.3 The CQ Text.

The third line contains the line of text transmitted by  the

PK-232 in a CQ call (in any mode except for MORSE). Think of

this message as a CQTEXT line.  Edit this line as well.

19.2.4 The Default Band.

The fourth line contains the band information.  The  default

value  is 2 for 2 Meters.  Use the Meter value for the  band

to keep the display correct.  It is displayed in the  status

window.  If you normally operate on another band change  the

default  value  in the file to reflect your  custom  default

state.

19.2.5 The Default Transmitter Power.

The fifth line contains the value for the transmitter power.

The  default value is 10 for 10 watts.  It is  displayed  in

the  status  window.  If your transmitter uses  a  different

power level change the default value in the file to  reflect

your custom default state.

19.2.6 The CQ Delay.

The sixth line contains a number which tells the program how

long to wait for a reply before transmitting a CQ call  when

in the beacon (automatic) mode.  A value of 4 corresponds to

about 2 minutes.  This value of time will vary depending  on

the  clock  speed  of  your  computer.   You  may  have   to

experiment with different values.  A value of 0 inhibits the

beacon mode.  The maximum value is 255.

19.2.7 The Message Relay Flag.

The  seventh  line  configures the  LAN  store  and  forward

capacity  of  your system also known as  the  Message  Relay

Capability.   If  it is a 0 your computer  will  not  accept

messages for forwarding.  If it is a 1, it will accept them.

See Appendix 2 for details about LANs.

Remember,  if  you  configure your  computer  to  store  and

forward  messages,  you have a moral obligation  to  deliver

them.

The state of this feature shows up as an [R] in your  status

display  and in the connect message which is transmitted  to

the other station at connect time.

19.2.8 The Communications Port

This  line  contains the default Communications  port.   The

default  value is 1. You may set it at any value  between  1

and 4.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 48

If  you only have one com port, set the 'SYS' file  to  com1

even  if  your card is configured as  COM2.   LAN-LINK  will

ALWAYS assume COM1 for a single card.

19.2.9 The TNC type.

This  line tells the program which TNC you are  using.   The

PK-232 command set is slightly different to those of TNC2 in

the packet mode and is very different to those of the KAM in

the  non packet modes.   Thus the command dialog has  to  be

changed  somewhat to make the 'SOLO, TRAFFIC, CQ/BEACON  and

MAIL' terminal modes work correctly for different TNC's.

Valid TNC's are PK232

KAM

KPC

TNC1

TNC2          (don't type the '.').

The  TNC2  used to test LAN-LINK was an MFJ  1270.  The  KPC

includes  software changeover from HF/VHF.  Most  TAPR  TNC2

clones should work with the TNC2 setting.

19.2.10 The Terminal Baud Rate.

This  line tells LAN-LINK what baud rate the TNC is set  to.

This is the baudrate on the RS-232 line between the TNC  and

the computer.  Valid baud rates for LAN-LINK are 300,  1200,

2400, 4800, 9600 and 19,200.  If you specify something  else

LAN-LINK will default to 1200.

If the TNC does not use an interrupt driven serial port  you

may  experience problems when using baud rates greater  than

1200.  The PK-232 does not like baud rates faster than 2400,

and  sometimes  not even that. The PK232 seems to  drop  the

first  character  of some lines at 4800  bauds.    Use  1200

until  you have tested and are sure that a faster  one  will

work.

This program has been tested on a TNC2 using a terminal baud

rate  of  9600 bauds and worked correctly.   it  also  works

correctly on a KAM at 2400 Bauds.

19.2.11 Logbook File.

This  is  the name of the file to be used as  your  logbook.

The default name is LAN-LINK.LOG.

19.2.12 Capture to Disk File

This  is the name of the file used to capture incoming  text

to disk. The default name is LAN-LINK.RUN.

19.2.13 Call Directory File.

This  is the name of the file used as your  call  directory.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 49

The default name is LAN-LINK.DIR.

19.2.14 Brag Tape.

This  is  the name of the file used as  your  bragtape.  The

default name is LAN-LINK.TXT.

19.2.15 Mail_Snatch Trigger Word.

You can set the mail snatch trigger in the BBS Mail  beacon.

Most BBS's use word 'Mail' and a string of calls to indicate

the presence of messages.  If your BBS uses something  else,

you may customize LAN-LINK on this line to tell it what word

your BBS uses.

Note  that  if your BBS uses 'QTC' you may  end  up  calling

yourself  if you disconnect while traffic is flowing on  the

LAN.

19.2.16  The QTC _Snatch or Automatic Connect Attempt Flag.

If  the  value  entered  on  this line  is  a  1,  then  the

QTC_Snatch  Flag is enabled.  if the value on the line is  a

0, then the flag is disabled.

If  the  QTC_Snatch  flag is  enabled,  LAN-LINK  scans  the

contents of packets appearing in the Inwindow on the  screen

while the terminal is disconnected.  Should it recognize the

three text strings 'LAN-LINK',  ':QTC:', and 'your callsign'

in  the contents of a packet, it will then issue  a  connect

request to the callsign of the LAN-LINK station that origin-

ated  the beacon transmission.  If the connect is made,  the

other station will send you the message automatically  which

LAN-LINK will "capture to disk".  The connect will time  out

and terminate a few minutes later.

The  normal  connect  request will be  made  DIRECT  to  the

station.  If you expect messages from stations you can  only

connect  with via a digipeater, place the path in your  LAN-

LINK.DIR  directory file.  It should be there anyhow if  you

connect  to  the  station regularly.  For  example,  if  you

expect  messages  from  4Z4ZB  whom you  can  only  work  if

digipeated via 4X1AA then place an entry like

4Z4ZB   4Z4ZB V 4X1AA

in the LAN-LINK.DIR  file.

The  QTC_Snatch  leaves  your messages  in  a  message  file

<yourcall.OUT>.   It  is  triggered by the first  >  in  the

initial  connect response of LAN-LINK 1.50R> (note the  '>')

and  terminated by the 'over' at the end of the message  ie.

G3ZCZ/W8>  (note the '>'). If you train stations  not  using

LAN-LINK  who  want to leave you messages  to  trigger  your

QTC_Snatch, by transmitting the following unaddressed packet

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 50

'LAN-LINK :QTC: YOURCALL',

your  system will connect to them.  (For example to leave  a

message for me, they would send

'LAN-LINK :QTC: G3ZCZ '

as  an  unaddressed  packet  [in  converse  mode  when   not

connected, or in their beacon text].)

If they then send you a '>' on a line by itself, followed by

the  message itself, and terminated by an other '>'  as  the

last  character  of the line, the message will end up  in  a

message  file rather than in the LAN-LINK.RUN file  and  you

can  scan  it  fast using the Alt M key.   It  goes  without

saying  that the message itself should not contain  the  '>'

character.   If  it does,by the way, the first part  of  the

message  will  be in the message file, while  the  remainder

will be in the LAN-LINK.RUN file.

19.2.17 The BBS_MAIL_Snatch flag.

This   line   controls   the   BBS_Mail_Snatch.    If    the

BBS_Mail_Snatch  is  enabled,  when a  BBS  sends  a  packet

announcing 'Mail', if your callsign is in the first line  of

that  message, LAN-LINK will issue a connect request  (again

via the path in LAN-LINK.DIR) to that BBS.  When  connected,

it will act as if you have initiated a ZapBBS command.   The

Zap counter will be incremented each time a  BBS_Mail_Snatch

takes place.

Set this line as follows,  0 = Disabled, 1 = Enabled.

19.2.18 Flag for Shut Down.

You  may customize the shut down sequence of the  TNC.   You

have  the  choice  of QRT, QRV and QRV  +  CTEXT  Toggle  as

described in the following sections.

Set  this line as follows, 0 = QRT, 1 = QRV 2 = QRV  +  CMSG

toggle..

19.2.18.1 QRT

Turns the TNC into a VHF Digipeater and turns CONOK OFF

(CFROM NONE for the PK-232).  In this condition the TNC

will reject connect requests.

19.2.18.2 QRV

Allows you to stay alive on turnoff. It will send

MON OFF (MON 0 for the PK-232)

^S

to  the  TNC at shutdown without  touching  CONOK/CFROM

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 51

{PK-232}, and

^Q

at  turn  on.  You will then  have  to  initialize  the

terminal  and  communications mode after  reading  your

mail.   Connects should then be logged at the time  you

read the mail, not at the time they took place.

19.2.18.3 QRV + CTEXT TOGGLE

This  is the same as the QRV state but CMSG is  toggled

on/off.  When stations connect to you while LAN-LINK is

active, they will not receive the CText message.   When

other  stations  connect to you while LAN-LINK  is  not

active, they will receive the CText message

19.2.19 TNC Data Bits

You  can  set  the number of data bits on  the  RS-232  line

between the computer and TNC to 7 or 8.  Use 7 with a TNC  2

if you have problems with greek characters.

Set this line to either 7 or 8.

19.2.20 TNC Parity

Use  this  line  to set the Parity bit on  the  RS-232  line

between the computer and TNC as described below.

0 = None, 1 = Odd, 2 = Even, 3 = Mark, 4 = Space.

19.2.21 Number of TNC Stop Bits

You  can  set  the number of stop bits on  the  RS-232  line

between  the computer and the TNC to 1 or 2.  Use 1  at  all

terminal (not packet) baud rates except 300.

Set this line to 1 or 2.

19.2.22 DCD Status Bit.

If you are using a KAM or a PK-232 then this line should  be

set to 1.  If you are using another TNC, if you can set  the

TNC so that pin 8 on the RS-232 interface changes state when

connected, do so and set this line to 1.  If you can't, then

set this line to 0.  If this line is 0, the multiple connect

modes  will  not work so don't attempt to invoke  them.   If

this  line  is  o,  the  words  '***  CONNECTED'  and   '***

DISCONNECTED'  when  seen  in the  LAN  may  cause  spurious

apparent connects and disconnects in LAN-LINK.

Set this line to 0 = not connected, 1 = connected.

19.2.23 QBM Download Directory.

This is the directory the :QBM: command will search to  find

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 52

a  file  to  be  downloaded.  The  default  value  is  \LAN-

LINK\FILES.  Set them up using DOS.  If you are using a hard

disk, put LAN-LINK in a LAN-LINK subdirectory.

19.2.24 Read Request Command.

This is the command LAN-LINK issues to the BBS during a  Zap

sequence to request your mail.  The default value is RM  for

Read  Mine.  On a WA7MBL BBS you may change it to VM  for  a

Verbose Read of the full packet headers.  On a W0RLI BBS you

may change it to RN to only request new messages.  If you do

change it, use the 'F' option in the Parameters Menu to make

sure  it  is  correct.  If it isn't  correct,  the  Zap  BBS

feature will not work,

19.2.25 Scan Words.

In  the ZAP BBS feature you can now ask the program to  scan

the  lines read from the BBS and if it sees any of the  scan

words in the line, it will queue a request for that bulletin

by  number.  You can only set 10 such Scan words (should  be

enough) and request up to 63 messages/bulletins at a time.

NOTE  that  words that show up in the BBS  prompt  line  are

ILLEGAL.  For example if your BBS prompts you with

N4QQ BBS>

then 'N4QQ' and 'BBS' should not be used as Scan Words.

19.2.26 * marks end of list    ****************************

This line marks the end of the list of Scan words.

19.2.27 The Remaining lines.

The  remaining  lines  contain commands  to  the  TNC.   For

further  information  about them refer to  the  TNC  manual.

LAN-LINK  assumes  different defaults in some  instances  to

those set in the TNC.  The SYS file contains only those that

are different to the TNC default reset state.

NOTE: If you are not using a TNC1 you MUST set the  HEADERLN

parameter  to  ON.  The packet headers and the text  of  the

packets must show up on your screen on separate lines.   You

must  also have the "Echo as Sent" feature set ON ('EAS  ON'

for PK-232 and 'XM' ON ' for the KAM).

19.3  LAN-LINK.RUN.

This file contains incoming data from the TNC recorded while

the  'capture to disk' function was active.  You may  change

the  default name of the 'capture to disk' file in the  LAN-

LINK.SYS file.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 53

19.4  LAN-LINK.LOG.

This  file  contains the log entries.  It  will  be  created

automatically  to  initialize  the  logging  function.   The

format  of the logbook file is the same as that  created  by

the  G3ZCZ CONTEST.BAS REV 2 program and is designed  to  be

appended  into the G3ZCZ LOGBOOK DATABASE package  presented

as part of the PC-HAM software package.  It should be edited

with a word processor in the non document mode to delete the

first  line and clean it up before the file is  appended  to

the DBASE logging package data files.

19.5  LAN-LINK.DIR.

This  is the directory file as described in  Section  6.1.2.

You  use it to store connect, NET/ROM, KA-Node  or  digipeat

paths as well as name/callsign relationships.

Examples are

Joe  G3ZCZ

Alon 4Z4ZB V 4X6AA

Milt 4X6AA

LR   4X6LR

hf-il 4x4hf v 4z4zb 4x4il

hf-rj 4x4hf v 4z4zb 4z4rj

You  create  this file with your word processor in  its  non

document mode.  You must leave AT LEAST one space  character

between the first (key) word and the connect path.  When you

type in the key word, the program does not care about  upper

or lower case.  You may also use the 'reMember Path'  option

in the Call Menu to make entries in this file.

If  you already have a YAPP.DIR file just rename it to  LAN-

LINK.DIR and add the NET/ROM and KA-Node paths.

NOTE:  All path entries in LAN-LINK.DIR must be on a  single

80 column line.

19.6  CONFIG.SYS.

You must have a CONFIG.SYS file on the boot disk if you  are

going  to  use the disk logging features.   This  file  must

contain the line

FILES = 20

See the DOS manual for more information about the CONFIG.SYS

file.

19.7  *.MBX.

These  are the files associated with the stations  you  have

designated as LAN store and forward mailboxes.  Thus if  for

example  you  want  to  use 4Z4ZB as  a  store  and  forward

mailbox,  you  will  have to create  an  ASCII  file  called

4Z4ZB.MBX   using  the  non-document  mode  of   your   word

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 54

processor.   In  this file you will put a list of  the  call

signs of the stations 4Z4ZB can connect to reliably.  If for

example you want 4Z4ZB to be able to store messages from you

to 4X6AA, 4X6LR, G3ZCZ, ON8IK, F0WN,and G8BTB, you would put

those call signs in the file, one call on a line as follows.

4X6AA

4X6LR

G3ZCZ

ON8IK

F0WN

G8BTB

When  the QRU function is invoked either by you manually  or

by 4Z4ZB connecting to you and sending you the command :QRU:

,  any messages present on your system addressed to  any  of

those  stations  (E.G.  G3ZCZ.OUT  and  ON8IK.OUT)  will  be

transferred  from you to 4Z4ZB just as if you had  tried  to

QSP  the  messages  manually or as if G3ZCZ  or  ON8IK   had

connected  directly to you.  The file names will be  changed

to *.OLD.

You  must  create  one file for each  designated  store  and

forward station, ie. 4Z4ZB.MBX, G3ZCZ.MBX etc.

19.8  *.BBS.

These are BBS AUTO-Upload Command files.

Each file is named for the callsign of the BBS that you wish

to access.  It must have exactly the name as the callsign of

the  BBS.  Thus if the BBS is W9TTN-1 the name of  the  file

must  be W9TNN-1.BBS, or if the BBS is 4Z4ZB then  the  file

must  be called 4Z4ZB.BBS.  The file must be an ASCII  file.

That  means  the  word processor must be used  in  the  non-

document mode to create it.

This file contains the command sequence that you would  have

typed  at  the  keyboard  if  you  were  accessing  the  BBS

manually.  The ONE difference is that the text of a  message

(you  are uploading with the S command must be  followed  by

the   character  string  :EOF:  (on  a  separate  line)   as

illustrated by the example below.

The BBS 'S' command (including its variations such as  'SP')

is the only multiple line command allowed.  You can thus put

all  the single line commands such as H, ?, L, and R in  the

command file.

A  'multiple  command'  is defined in  this  instance  as  a

command which allows you to enter multiple lines before  the

BBS  will  send back a '>' character (IE one  that  the  BBS

tells you to type in text and terminate it with the  Control

Z character).

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 55

The  contents of A TYPICAL *.BBS file are shown below as  an

example.

k 1234

s g8btb

Test Message

This is a test message.

:EOF:

sp G3ZCZ @ N4QQ

Greetings

The text of this message is greetings from LAN-LINK.

:EOF:

lm

r 567

r 593

NOTE: the 'K 1234' command was used to delete a message that

had  already  been read. If you use 'KM' as you  may  delete

mail BEFORE reading it.   The outgoing mail was placed  next

in  the  file  in case of an abnormal  disconnect.   The  lm

command  was used to record the message numbers, and to  see

what  old messages were still present on the BBS.  Lastly  a

list  of messages to be read was included (the  numbers  had

been recorded in the last BBS_Mail_Snatch).  There were also

no blank lines in the file.

Each  message  ended with the :EOF:  character  string,  but

there was no need for any special end of BBS file character.

In the event of a BBS Mail_Snatch, if the *.BBS file  exists

for  that BBS, the file will be processed and  the  contents

sent.   Thus  if you don't get a chance to  send  the  mail,

before  LAN-LINK  downloads the mail from the BBS,  it  will

send it for you.

When the BBS has accepted the commands the name of the  file

is changed from *.BBS to *.OLD ( IE. the file name  KE8X.BBS

is changed to KE8X.OLD).

In the real world however, message formats are non standard,

and  can  contain  any set  of  characters.   The  algorithm

developed  here  transmits the next line in the  *.BBS  file

when LAN-LINK see's a 'trigger' character_string unless  one

of the 'inhibitor' character_strings is also present on  the

line of the message text received by LAN-LINK.

These  'trigger'  character_strings  are '>'  (as  the  last

character  of the line of text ), 'Enter', 'Subject',  'Sj:'

and 'Msg:'.  The 'inhibitor' character_strings are 'R:', '<'

and 'Message'(R: as in routing headers and < and Message  as

in Message-Id: <message number> in message headers).

This should cover most simulated manual BBS message  reading

(WA7MBL and W0RLI/VE3GYQ) and avoid false triggering due  to

'>'s showing up in message and routing headers.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 56

NOTE than when LAN-LINK finishes processing the *.BBS  file,

it  will  send  a 'B' command to the BBS  to  terminate  the

session.  This feature is designed to minimize the time  you

spend  connected  to  the BBS by allowing you  to  scan  the

message list when automatically receiving your mail, prepare

your  replies and download requests off-line and then  later

read  the  incoming messages or files without tying  up  the

BBS.  It thus positively disconnects you from the BBS.

NOTE:  If you upload the 'KM' command from your *.BBS  file,

you  may  delete messages before reading  them.   Do  delete

'read'  mail  from your local BBS.  Don't clutter  his  disk

with your read mail. The 'LM' command is built into LAN-LINK

to remind you of what you have out there.

20.0 Mailbox features

This section describes the mail box and message handling features

built into LAN-LINK.

20.1 Answering Machine.

LAN-LINK  contains an AMTOR (Robot beacon CQ mode only)  and

PACKET  mode   SMART "answering machine" facility.  You  can

leave  messages on your disk (in the same directory  as  the

LAN-LINK.*  files,  if  you  are using  a  hard  disk),  for

different  stations.  When someone connects to you,  if  you

left  a message for him, he (or she or even it as  the  case

may be) and only that station will receive it automatically.

No one else will normally be able to download that message.

20.2  Mail Beacon (Annunciator).

To ensure that people know that you have left a message  for

them a 'MAIL for' list is loaded into your Packet Beacon and

transmitted every 30 minutes (Refer to the BTEXT command  in

the TNC manual) as  ':QTC:' followed by a list of calls.  If

no  mail is pending, or the only message in your  system  is

one   addressed  to  you,  the  beacon   transmissions   are

inhibited.   This  conforms to good  operating  practice  on

crowded channels (at least inhibiting the beacon does).

The mail beacon in the AMTOR mode is transmitted as part  of

the beacon autocq message.  It is automatically updated when

a message is transmitted.  If you don't set the beacon  (see

below)  you  will not transmit a mail beacon  text  in  your

AMTOR CQ message.

The  Mail  beacon text is set up either when  you  tell  the

computer  to set it up or when you disconnect a  packet  QSO

from another station.

In this manner you may update the list while the program  is

running,  for example by using Sidekick to write a  message,

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 57

or preferably the NOTE feature.

Any   station   using   LAN-LINK  can   be   configured   to

automatically  attempt  a connect (QTC_Snatch)  when  seeing

their callsign in someone else's LAN-LINK :QTC: beacon  mail

message list.

If  for some reason the message got lost (e.g.  because  the

link  was  marginal), :QSM: will get you a  repeat  of  that

message.

20.3  How to leave a message.

You  can use the note feature or type the message as a  non-

document  file  on  Wordstar, or if  you  use  another  word

processor, you must leave the message as an ASCII text file.

If  you  use the Note feature, the message will be  given  a

header  as  if it came in from someone else.  if you  use  a

word processor to write the message, you will have to  write

in a header or show somehow that the message is not  'live'.

You  may  then later time tag a message  header  on  message

files  created  by the word processor.  In this  way  people

will  realize that it is an automatic message by seeing  the

header.  On the other hand, you may wish to fool people into

thinking that you are home.

You  just  name  the message file as  the  callsign  of  the

station  to whom you wish the message to be sent at  connect

time.  You must also give it the filetype of 'OUT'.

For example a message for G3ZCZ would be stored on the  disk

as  a  file named 'G3ZCZ.OUT', and similarly  a  message  to

4X6AA would be stored on disk as '4X6AA.OUT'.

Once the message has been sent, an entry to that effect will

be made in the LAN-LINK.RUN  data capture file. The  message

file  will  then  be renamed and given  the  file  extension

'.OLD'.  At that time any previously existing mail file with

the  file extension '.OLD' will be deleted.  This stops  the

addressee  receiving the message on successive  connects  if

you  can't delete it for some reason (you forget)  while  at

the same time, the message is still available to you in case

you need it.

You  should use a separate utility program if you  want  the

calls  in the mail beacon sorted.  You may for example  wish

to  sort them alphanumerically or in the order of  the  date

that they were left in your answering machine.

Incoming  messages triggered by the QTC_Snatch will be  left

in  a message file ( <yourcall.OUT> ), which can be  scanned

by  using the Alternate 'M' function key.  If you  have  non

LAN-LINK  friends, train them to trigger the  QTC_Snatch  by

simulating  a beacon, and pre-fixing and ending  their  text

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 58

with lines containing the '>' as the only character.  Better

yet, give them a copy of LAN-LINK and tell them to  register

it.

20.4 Remote Beacon shutdown.

I recognize that some stations may abuse the beacon facility

of  this  program.  I have thus built  into  the  program  a

'remote  shutdown' capability.  If someone running  LAN-LINK

has their beacon timer set too often, connect to them in the

AMTOR   or  Packet Communications modes, and  tell  them  to

:QRT:  which will clear their beacon mode and take them  off

the air for a while at least.

In  the  Packet communications mode, the CQ beacon  is  shut

down  but the 30 minute mail beacon is not touched.  In  the

AMTOR communications mode, the CQ delay time is set to  255.

The  Mailbox  will thus still be active in both  modes,  but

will not beacon on the air at the frequency it was set at.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 59

APPENDIX 1. ANOMALIES (and BUGS)

A1.1 Since  the Program has no way to check the  command/converse

mode  status of the TNC, you may see various Cmd: and  error

messages  from the TNC when using the menus or function  key

commands.   Ignore  them  for  now.  If  you  get  an  error

message, try using the function key again.

A1.2 In the AMTOR Beacon CQ mode, if the other station drops  the

link  while you are recording the text, BEFORE the  callsign

has been recognized as a valid callsign, LAN-LINK will  time

out  before logging the QSO and restarting the CQ  sequence.

If  someone else calls in before the time-out has  occurred,

the text from both stations should be captured to disk,  but

the  log  entry may not show both stations, thus  the  'LOG'

must be compared with the 'RUN' file for QSO purposes.

LAN-LINK  in  the  Beacon/Mailbox mode,  normally  logs  the

contact  at the end of the line in which it  recognized  the

callsign as a valid callsign.

A1.3 In  the AMTOR Beacon CQ or Mailbox mode, if another  station

sends a +? in the FEC mode, the beacon will attempt to  send

the  LAN-LINK.TXT  file.  The PK-232 however is not  in  the

converse  mode at that time and will interpret the  text  of

the  file  as  commands.   If  by  some  chance,  the   text

corresponds  to a valid command the PK-232 will  execute  it

and  put  the  PK-232 in an unknown state and  you  may  not

notice  that  it did.  As such I recommend  that  the  first

character  of  each line in the file LAN-LINK.TXT be  a  non

valid command character such as a number or a period (.)  or

another punctuation sign.

A1.4 If  you are using a CGA card with a monochrome monitor,  the

text  you type into the OUTWINDOW will be fainter  than  the

incoming  text.  This is because it is set to  light  green.

If  you  are using a Hercules compatible card you  will  not

notice any difference.

A1.5 The  conference  mode has been tested on a  PK-232,  KAM,  a

SANYO MBC-675 which has a clock speed of 4.7MHz and other PC

clones with faster clock speeds.  If you have problems  with

intermittent selection of IO channels try slowing down  your

CPU  clock.  There is a software delay loop in  the  program

that  is  used to allow the TNC time to  switch  IO  streams

after  receipt of the stream change command.  This delay  is

there  because LAN-LINK tests for a connect on each  channel

before  outputting  any  text,  so that  it  does  not  send

unaddressed packets on non connected IO channels.

A1.6 A  Packet conference Mode disconnect returns the TNC to  the

command  mode, even if you remain connected to one  or  more

other stations.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 60

A1.7 Strange Events.

If the TNC does not use an interrupt driven serial port  you

may  experience problems when using baud rates greater  than

1200.  The PK-232 DOES NOT LIKE LAN-LINK to use RS-232  baud

rates  faster than 2400, and sometimes not even  that.   Use

1200  until YOU have tested and ARE SURE that a  faster  one

will work.

IF  YOU  HAVE ANY STRANGE PROBLEMs AND ARE USING  an  RS-232

TERMINAL  BAUD RATE to the TNC WHICH IS GREATER  THAN  1200,

set  the Terminal Baud Rate to 1200 and see if  the  problem

goes away.

A1.8 *** DISCONNECTED

In the Packet communications mode, when you are connected to

someone and the TNC sends this text string to the  computer,

LAN-LINK thinks you have been disconnected and resets.  This

normally  only  happens  when  a  true  disconnect   occurs.

However, if the text shows up in the middle of a line  being

sent to you, it WILL HAVE THE SAME EFFECT AND THINK THAT YOU

HAVE DISCONECTED if the DCD_Flag is not set in the SYS file,

and the TNC does not use the DCD signal on pin 8 of the  RS-

232 interface.

A1.9 *** CONNECTED

In  the Packet communications mode, when you are  monitoring

traffic on the LAN someone sends this text string,  LAN-LINK

thinks you have been connected if the DCD_Flag is not set in

the SYS file, and the TNC does not use the DCD signal on pin

8 of the RS-232 interface.

A1.10 Greek Characters and other garbage in the Incoming Screen.

If  you  see  greek characters in the  Incoming  screen  you

probably  have the wrong parity set in the TNC.  first  make

sure  that the RS-232 baud rate is correct.  If it is,  take

the  batteries out of it and reset it.  If that fails,  play

with  the parity.  LAN-LINK sets the serial port to 8  bits,

no  parity.   If  you still can't get it to  work,  set  the

interface to 7 bits in the SYS file.

A1.11 Dumb TNC

If  the TNC seems to respond to commands but is not  sending

anything  back, you may have one of two problems.   You  may

have  a bad RS-232 cable with on wire disconnected.  If  the

computer  can  operate  the TNC  in  another  communications

program, try to reset the TNC, take the batteries out,  etc.

You may also have a not-so-compatible PC.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 61

APPENDIX 2. Local Area Network Capability

VHF  Packet  radio systems can be considered as part of  a  Local

Area  Network (LAN) in which messages can be left by one  station

in the computer belonging to a second station.  At HF the same is

true,  but the area becomes greater.  The basic problem  here  is

that  people  can only send and receive messages to or  from  you

when you are on-line.  To compensate for this, BBS stations  were

developed which allowed both messages and bulletins to be  stored

by anyone for later retrieval.

LAN-LINK  allows you to store messages in your computer so as  to

use  it  as  an  automatic answering  machine.   By  storing  the

messages in a file called by the name of callsign, with the  file

extension  '.OUT',  anyone connecting to the  computer  will  get

their  message  without  having to request  it.    LAN-LINK  also

extends  that concept to cover the eventuality of you wanting  to

take  your computer off-line for some reason.  You may  now  load

the  message  into any other computer you can  connect  to  using

elements  of  the  Q  code adapted  to  the  High  Level  Network

Communications  Language (NC/L) proposed in my book Software  for

Amateur  Radio, (Book number 1560) published by TAB  Books,  Blue

Ridge Summit, Pa. 17214 U.S.A.

A2.1 LAN Protocol (G3ZCZ Version)

The  protocol  is as follows. When connected to someone  who  has

their  LAN-LINK  configured  as a host, if you want  to  store  a

message  you send the following instruction to the other  station

:QSP:  <callsign>  where  <callsign> is the call of  the  station

that the message is for, not the callsign of the host station  in

whose  computer you are storing the message. [Note use  only  one

space  character  after  the :QSP:].  LAN-LINK  sends  the  :QSP:

message automatically using QSP option in the Message Menu.

For  example if you want to store a message for 4Z4ZB in  4X6AA's

computer  which is configured as a Store and Forward system,  you

would first connect to 4X6AA and then send the command as

:QSP: 4Z4ZB .

Better  still use QSP option in the Message Menu to automate  the

sequence.

The computer at 4X6AA will respond either with a statement saying

that  it is ready for you to go ahead, or send a  message  saying

that  it can't comply.  If it is ready you get a  positive  reply

which will take the form :QRV:  <callsign> which if you know  the

Q code, means " I am ready to accept a message for  <callsign>".

At this time you may go ahead and send the message.  If you  type

the message at the keyboard, you may use either a control Z  (^Z)

character or the character sequence :EOF: followed by a  carriage

return  (the  ENTER key) to terminate the message.  If  you  have

first  written the message into a text file you may then send  it

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 62

using  the  'Send  File'  option  of  the  Files  Menu  and   the

termination sequence is automatic.

Once  you have completed the message, the other  (host)  computer

will  either reply that the message has been successfully  stored

or give you an error message.

If  the  message  is stored and ready to be sent  next  time  the

addressee  connects  to that computer, you will see  the  message

:QSL: on your screen.  If something went wrong, you will get back

a  negative message taking the form :QNO: followed by  a  number.

The number tells you why the operation failed.

A2.2 Message Format

The  message is stored just as if you had left it in your  system

(except that a header is added identifying the time of  reception

and  the call of the sending station).  It will contain a  header

showing  the  date and time it was received  and  your  callsign.

Should a message for that station already be in the system, yours

will be appended to it.  In the event the your upload is aborted,

the  amount  of text received before the abort occurred  will  be

stored as the message.

When  you  disconnect from the other LAN-LINK host  station,  its

mail beacon will be updated.

Once the message is loaded in the host, it can only be deleted by

the operator of the host station.  When the addressee connects to

the host and receives the message, the file type will change from

'.OUT' to '.OLD'.

A2.3 File Uploading/Downloading.

There  comes a time when you want to leave a file on your  system

for someone to download later.  You can do this by using NC/L  to

control the transfer.  You must leave the file in a  subdirectory

defined  in  the LAN-LINK.SYS FILE called  \LAN-LINK\FILES.   You

must first create it if it does not exist.  The station who wants

the message just asks for it using :QBM: <filename>, note they do

not  need to use the full subdirectory path.  They  must  however

leave  a single space character between :QBM: and  the  filename.

In AMTOR the Mailbox must be turned on for remote functions.

For  example, you have the latest ARRL DX bulletin, and you  want

to  pass it on.  You could pass it to selected people by  copying

the  file to individual messages with the filetype  '.OUT',  e.g.

4X6AA.OUT, 4Z4ZB.OUT or even G3ZCZ.OUT if you want me to get  it,

which wastes a lot of disk space.

On  the  other  hand  you could tell people  that  the  file  was

available  for downloading, either in the CTEXT  connect  message

line  which everyone gets when connecting to you  by  configuring

LAN-LINK accordingly, or in individual ('.OUT') messages.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 63

If you include the following in your CTEXT message

"ARRLDX.015 now available"

people will know that to download it, they just have to type  the

line  ':QBM: ARRLDX.015' (one space character between  :QBM:  and

the  file name) to get the file.  LAN-LINK is not designed  as  a

BBS,  however,  if you copy the contents of  the  \LAN-LINK\FILES

subdirectory  into  a  file called DIR (USE DIR >  DIR  in  DOS),

anyone  can  get a list of the files you have stored  in  it  for

downloading by typing :QBM: DIR.  Since they do not get the  real

directory, just what was in it at the time you copied it into the

file  'DIR',  you may keep "hidden" files in  this  subdirectory.

You may thus leave a file for someone to download, tell them that

its  there  by  leaving  them a  message  (which  they  will  get

automatically when they connect) and no one else connecting  will

know that it is there.

:QDB: allows you to upload text or ASCII files.

A2.4 Path Determination to a Dx station

If  you want to establish a digipeat path to a  station  somewhat

out of your direct range, you need to know which of the  stations

that you can connect to can hear that desired DX station.  If you

could  get a call monitored (MH list) from the stations that  you

connect  to,  you  would be able to see if the  station  you  are

connected to has heard your desired DX station.

LAN-LINK  uses  the :QMH: command to request such a  list.   When

LAN-LINK   receives  a :QMH: command, it drops back  to  the  TNC

command  mode and issues a 'MH' command to the TNC (except  to  a

TNC1 which does not support it).  It stores the list received  in

a  temporary  buffer, and then returns to the converse  mode  and

transmits the list back to the station it is linked to.

By  judicious  use  of :QMH: you can  determine  paths  to  other

stations.   Note however, that just because one station can  hear

another  station,  it  does not mean that it can  work  it.   For

example,  the station you are connected to may be using  a  power

level  of 1 watt or so, while the station 200 miles away that  it

heard was using 100 watts.  Test the path yourself, or/and  leave

a message asking how reliable the connect path between those  two

stations is.

If  you and a small group use a non standard packet  channel  for

inter-group  communications, you may also use the :QMH:  function

to  find  out who the station you are connected with,  has  heard

recently, when you first sign on the LAN channel.

APPENDIX 3     BASIC NC/L DICTIONARY

Messages can be transferred into any other LAN-LINK computer  you

can connect with using elements of the Q code adapted to the High

Level Network Communications Language (NC/L) proposed in my  book

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 64

Software  for Amateur Radio, (Book number 1560) published by  TAB

Books, Blue Ridge Summit, Pa. 17214 U.S.A.

NC/L  command words are summarized in the  following  paragraphs.

There  is  no command word to request your message.   To  receive

your  message,  you  do do nothing, you will  receive  your  mail

automatically  when connecting/linking with a  LAN-LINK  station.

You cannot normally read messages addressed to another person.

A3.1 :QBM:

To download a file, send

:QBM: filename.type

The filename.type is the file you want. For example

:QBM: dir

:QBM: help

:QBM: LAN-LINK.doc

A3.2 :QDB:

To upload files to another LAN-LINK system send

:QDB: filename.typ

Files  will be uploaded into the same sub-directory as  LAN-

LINK, and will have a message header inserted at the top  of

the  file  to allow the operator to know  who  uploaded  the

file.  If the operator then wants to make the file available

for  downloading,  he  or  she can  move  it  to  the  \LAN-

LINK\FILES sub-directory.  The '.TYP' following the filename

is optional.

If  you attempt to upload a file which has the same name  as

one  that  already  exists on the remote  system,  you  will

receive a :QNO: 14 'error' message.

This command is the reverse of :QBM:.

A3.3 :QMH:

To  request a call monitored list ('MH') from  the  LAN-LINK

station that you are connected with, send

:QMH:

A3.4  :QSM:

To  request a repeat of a message from a station using  LAN-

LINK, send

:QSM:

This  command will be valid as long as the host station  has

not  deleted  the <yourcall>.OLD file on his disk.   If  the

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 65

file  does  not exist, you will receive a  :QNO:  2  'error'

response.

A3.5  :QSP:

To leave a message, send

:QSP: callsign

the  callsign  is that of the station you wish  to  leave  a

message for.

A3.6 :QRT:

To  shut  down an AMTOR/Packet mode  Mailbox/beacon  station

which  is  causing QRM, link or connect to the  station  and

send

:QRT:

A3.7 :QRU:

To upload messages from one LAN-LINK/PK232COM (1.38+) system

to your computer, send

:QRU:

You  may only use the QRU function with stations  designated

as Store and Forward mailboxes.  Put a list of stations that

you can connect to reliably in your <callsign>.MBX file  and

send  it to other stations in your LAN also using  LAN-LINK.

They  will then be able to dump their mail messages  on  you

and  you  on them. If they do the same and  send  you  their

file,  then you will be able to send messages via  them  for

stations that they can work.

While  QRU gives you the capability to bulk upload  messages

to  another station in your local area, when you  take  your

machine  off line, it may also be used to transfer  messages

between two LANs via well sighted gateway digipeaters.

A3.8  :QNO:

'NO' or error number is a response to a request.

The  following  error numbers are  associated  with  message

store and forward operations.

?    What you typed was not an NC/L command.

1    Computer not configured as Store and Forward system.

2    Requested ASCII file/ message (:QBM:) does not exist.

3    You made an error in the name of the callsign for  whom

the  message  is  intended  (It  must  be  at  least  3

characters long).

4    File creation error in host system.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 66

5    Error occurred during reception and storage of message.

Could  be  that the computer ran out of  space  on  the

disk,  or  something  else went wrong  in  storing  the

message.

6   :QRU:  You  are not authorized as a  store  and  forward

mailbox.

7   :QRU: Error in opening <callsign>.MBX file.

8   :QRU: Error in closing <callsign>.MBX file.

9   :QRU: Sequence  Error in callsign of message to go.  The

bad callsign will be shown after the error number.

14  :QDB:  File  Name In Use Error.  The name you  gave  the

file you want to upload already exists on that  system.

Try a different name.

90   NC/L defined function not implemented in this release.

99   LAN-LINK compatible program, but requested function has

not been implemented.

A3.9  :QJG: is a response to :QRU:

It confirms that the QRU sequence is complete.  There are no

more messages pending.

A3.10 :QRV: callsign is a response to :QSP: callsign.

The  computer  is  ready you to send the  message.  End  the

message  with  a control Z (^Z) character, or  the  sequence

:EOF: .

A3.11 :QSL: is a response to a command.

It  confirms receipt of message to that callsign or  that  a

file has been successfully uploaded and stored to disk.   It

is also used to acknowledge receipt of a ':QRT:' command.

A3.12 :QTC: massage list.

If  LAN-LINK receives :QTC: it will respond with the  beacon

text showing the mail queue.

:QTC:  precedes  a list of callsigns for whom  messages  are

stored up on a computer.  It is used in Packet Beacon  tran-

smissions or AMTOR Beacon mode CQ calls.

PROPOSED EXTENSIONS

A3.13 :QYU:     YAPP format file upload.

A3.14 :QYD:     YAPP format file download.

APPENDIX 4. USING OTHER TNC'S.

LAN-LINK  is  designed for the PK-232.  However the  PACKET  mode

capabilities  can  be  used  on other  TNC's.   There  are  minor

differences  in the command dialog between different TNC's.   The

dialog differences affecting the operation of LAN-LINK are  built

into the program and set when the TNC type is read from the  LAN-

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 67

LINK.SYS  file  at the program load time.  The non  packet  modes

will obviously result in error messages if attempted on a  packet

mode TNC.

The   following  notes  apply  to  different  TNC's  or   digital

communications controllers.

A4.1 TNC2 (MFJ 1270) Version 1.1.2

A4.1.1     Default Commands.

Rename  the LAN-LINK.SYS file as PK232.SYS.  Rename the  TNC2.SYS

file as LAN-LINK.SYS and edit it using your word processor in its

non document mode to customize it to your station.

A4.1.2     Program differences.

The  following  functions DO NOT WORK because the  TNC  does  not

support them.

All NON PACKET modes.

Software controlled HF/VHF switching.

Software controlled baudrate switching.

CQ/BEACON Packet terminal mode.

TNC Transmit Buffer flush.

A4.2 TNC1 (HD-4040)

A4.2.1     Default Commands.

Rename  the LAN-LINK.SYS file as PK232.SYS.  Rename the  TNC1.SYS

file as LAN-LINK.SYS and edit it using your word processor in its

non document mode to customize it to your station.

A4.2.2     Program differences.

The  following  functions DO NOT WORK because the  TNC  does  not

support them.

All NON PACKET modes.

Day/Time functions.

'MH' and remote :QMH: functions.

MAIL Packet terminal mode.

CQ/BEACON Packet terminal mode.

TNC Transmit Buffer flush.

Conference and multi-connect modes.

A4.3 Kaypro KPC-2

A4.3.1     Default Commands.

Rename  the LAN-LINK.SYS file as PK232.SYS.  Rename  the  KPC.SYS

file as LAN-LINK.SYS and edit it using your word processor in its

non document mode to customize it to your station.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 68

A4.3.2     Program differences.

The  following  functions DO NOT WORK because the  TNC  does  not

support them.

All NON PACKET modes.

MAIL Packet terminal mode.

CQ/BEACON Packet terminal mode.

TNC Transmit Buffer flush.

A4.4 KAM  EPROM Version 2.8.2.

A4.4.1     Default Commands.

Rename  the LAN-LINK.SYS file as PK232.SYS.  Rename  the  KAM.SYS

file as LAN-LINK.SYS and edit it using your word processor in its

non document mode to customize it to your station.

A4.4.2     Program differences.

The  following  functions DO NOT WORK because the  TNC  does  not

support them.

TNC Transmit Buffer flush.

The SIGNAL communications mode.

The NAVTEX communications mode.

The Alt-D key in the MORSE Communications mode.

The BAUDOT NAVY MARS Protocol is also not available.

A4.5 PK-232 EPROM Version 30 AUG 1988

Turn the 'BBS OFF' if you want the Alert Call and Digipeat Detect

features to work otherwise turn it on.

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 69

APPENDIX 6 LAN-LINK REGISTRATION FORM.

To: Joe Kasser G3ZCZ

P O BOX 3419

SILVER SPRING

MD 20904.

CALL ________________   TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY    ___________________________________________

STATE   _________  POSTCODE  ______________________

TNC TYPE _____________________

I  enclose  a check for $35.00. Please register me as a  user  of

LAN-LINK.  I am currently  using Version ________________   which

I obtained from  _________________________________________.

Please send me the latest version of LAN-LINK or if a more recent

one  does not exist at this time, QSL my registration and add  my

name  to  the  list  to receive a free  update  when  it  becomes

available.

Signature

Comments, likes, dislikes, wish list, etc.

My favourite operating modes are ______________________________.

Additional  Features I'd like to see in future releases of  LANK-

LINK.

Keep Conference Bridge Up       __________

YAPP Binary File compatibility  __________

Export Logs to dBASE 3          __________

Expanded Contest Operation      __________

More AMTOR                      __________

Others

_________________________________________________________________

LAN-LINK.DOC (c) G3ZCZ 1988

LAN-LINK VERSION 1.50  PAGE 70

APPENDIX 7 LAN-LINK PROBLEM REPORT

Name ______________________________________  CALL _______________

Street __________________________________________________________

City _______________________________  State _____  ZIP __________

Tel ____________________  LAN-LINK version _____________________

1.  Describe your computer system.

Manufacturer ____________________ Model ___________TNC __________

DOS Version  ________________  Clock Frequency _________ MHz.

List peripheral equipment and cards installed in your computer.

_______________________  _______________________

_______________________  _______________________

_______________________  _______________________

What  memory  resident programs were installed when  the  problem

occurred?

_______________________  _______________________

_______________________  _______________________

2.  Describe the problem in detail.

3.  Describe the sequence of events or keystrokes that led to the

problem.

4.  Any other information that may help in locating the problem.

Use additional sheets of paper if necessary.  ATTACH copy of your

LAN-LINK.SYS  file.  Mail to:  Joe Kasser, P O Box  3419,  Silver

Spring, MD. 20901.

LAN-LINK.DOC (c) G3ZCZ 1988

#@######