REALISTIC PRO-2005 SCANNER MODIFICATIONS

THE 2005 SCANNER IS EASILY MODIFIED TO INCLUDE ALL THE

800 Mhz FREQUENCIES. THE NEW COVERAGE WILL COVER 25-520,

760-1300 Mhz.THE MODIFICATION REQUIRES THE TOP COVER TO

BE REMOVED, THAT'S JUST THE TWO SCREWS IN THE BACK.  THE

SPEAKER WILL UNPLUG FROM THE PC-BOARD,SO IT WON'T BE IN

THE WAY.  THE TWO DIODES ARE MOUNTED ON THE BACK OF BOARD

THREE, THAT'S THE ONE THE LCD AND KEYPAD ARE ON. THE

DIODES NUMBERS ARE D502 AND D503. THEY ARE MOUNTED DIRECTLY

BEHIND THE 3 BUTTON ON THE KEYPAD. TANDY WAS NICE ENOUGH TO

STICK THEM IN AN OPENING THAT GIVES ENOUGH ROOM FOR

DESOLDERING OR A PAIR OF SNIPS.  THIS STATEMENT IS FOR YOUR

INFORMATION ONLY IN CASE SOMEDAY THE FCC LEGALIZES THIS,

THIS MODIFICATION WILL MAKE THE SCANNER OPERATE AT MAXIMUM

FREQUENCY CAPACITY. PLEASE OBSERVE FCC RULES AND

REGULATIONS REGARDING THE COMMUNICATIONS PRIVACY ACT.                               HAVE FUN!!