Conductivities and Dielectric Constants for Common Types of Earth

Surface Type			Dielectric	Conductivity	Quality

Constant	mS/m

--------------------------	----------	------------	---------

Salt water			81		5000.0		Excellent

Fresh water			80		1.0

Pastoral, low hills, rich	20		30.3		Very good

soil (Dallas, TX to

Lincoln, NE)

Pastoral, low hills, rich	14		10.0

soil (OH and IL)

Flat country, marshy,		12		7.5

densely wooded (LA near

Mississippi River)

Pastoral, medium hills and	13		6.0

forestation, (MD, PA, NY,

exclusive of mountains

and coastline)

Pastoral, medium hills and	13		5.0		Average

forestation, heavy clay

soil (central VA)

Rocky soil, steep hills		12-14		2.0		Poor

(mountainous areas)

Sandy, dry, flat, coastal	10		2.0

Cities, industrial areas	5		1.0		Very poor

Cities, heavy industrial	3		0.1		Extremely poor

From QST July 1987