First the Sales Pitch:

HandyCODE AR

Microsystems Software, Inc.  announces a software TSR product for

MS-DOS called HandyCODE AR

HandyCODE AR provides a path to IBM PC,  Pc-Jr,  XT, AT, PS/2 and

compatibles (including '386 PCs) by interpreting Morse code input

and converting in into keyboard data.   The standard  Morse  code

assignments  have  been extended to include the extra keys on the

PC's keyboard,  plus defined and user-defined  commands.    These

commands are provided by a powerful macro facility.   HandyCODE's

macros are shortcuts for  frequently  used  keystrokes,  such  as

loading  your application programs or inserting a frequently used

sentence or a list of names into your word processor.   With  the

macro  capability  any  series  of  keystrokes  can be automated.

HandyCODE provides a comprehensive practice code utility and  on-

screen help facility.  The user can configure iambic, dot memory,

letter/word  spacing,  and  select  speeds from 1 to 99 words per

minute.  HandyCODE has built-in 1-key, keyer,  or bug mode opera-

tion.    You  can configure the software for either Auto-Space or

Manual-Space (-...-) modes.   HandyCODE is compatible  with  most

popular  software and networks.   HandyCODE can help you learn or

improve your CW skills while you run  virtually  any  popular  PC

software.    HandyCODE takes approx.  30k of resident memory when

configured for "on-line" HELP and 4k of macro memory.

Using some ingenuity,  you could connect a PC to your rig and  do

off-the-air copy or keyboard-to-CW control, or even set up an un-

attended CW bulletin board.   The standard interface is through a

provided 25-pin connector to the PC's LPT port.    The  LPT  port

also provides 8 TTL compatible outputs.   Bit 7 is activated con-

current with the speaker tone and therefore can be  used  to  key

your  transmitter.    The remaining 7 bits are controlled by your

software and may be used for rotators, XMT/RCV, lights, etc.  For

off-the-air copy,  you need to convert the  BFO  tone  from  your

receiver into a TTL signal and "OR" this with your paddle or key.

If you don't require the external "RIG" control,  you may connect

your keyer paddle via the COM: (25 or 9) or  GAME  ports.    Just

tell us at time of order and we will supply the proper connector.

MSI also includes a software program called PLAYFILE which allows

you to play any ASCII file in "perfect" morse.

HandyCODE  is  available with connector for $149.  or $89 without

"rig" control capabilities.   A rehabilitation  version  is  also

available  for $295 which provides speech output capability.   To

order call (508) 626-8511 or write MSI, 600 Worcester Rd.,  Suite

B-2,  Framingham,  MA 01701.   Master CARD and VISA accepted.  30

day money-back guarantee!

That wasn't so bad was it?

Now the Demo:

The HCODE.ARC file contains three files:

-  HCD.EXE    HandyCODE demo Program- 30 Minute time limit before unload

-  MOVL.EXE   HandyCODE Configuration program; DO NOT EXECUTE.

-  HCD.DOC    You are already reading this file so we guess you found it

1.  Copy Both .EXE files into any directory (MAX 1 off of root; EX: C:\HCODE)

2.  Type HCD/C

3.  Configure HandyCODE as desired.  NOTE:  you cannot select other than the

KEYBOARD for paddle control on this demo version.

4.  To Reconfigure just type HCD when HandyCODE is already in memory and the

Configuration screen will appear.

5.  At any time when the program is operating, you may key the hm couplet

(....--) to view the full page help screen with morse code extended

assignments.

6.  Give us a call at (508) 626-8511 if you have any questions.

Note that our BBS also has the latest versions of any demo Software and

a great selection of Shareware.