P A R A F I L . E X E

A PARAGON MEMORY CHANNEL FILE UTILITY

C A U T I O N !

THE PROGRAM ASSUMES THAT THE RADIO IS ON.

DO NOT LEAVE THE RADIO IN ANY WAIT STATE.

Use the ESCAPE key to EXIT a function when uncertain.

This program can be used as a stand along program for

memory  management or as a complete  utility  for the

radio. If you do not have a serial board installed DO

NOT try to Connect to the radio.

RECORD DATA:  Each PMF file can contain 62 memories. Each  memory

record is 24 bytes or 1,488 bytes for each file.

I N S T A L L A T I O N

-------------------------------

Type  PARAFIL <ENTER>. On startup you will be asked for  the

following four entries for the PARAGON.USR file:

Callsign:  Enter your call.  The  letters A-Z, numerals 0-9,

and  the slash '/' are to be used in  the  normal

convention.

Fast Delay:  .477 is the default value.

Voice Delay:  8.1 is the default value.

Verify: "Y":  Verify enabled or ON. The program will prompt the

the  user either before or after an entry.  It is

suggested  that  the  default value be used until

you are accustomed to the program.

"N":  Verify disabled, almost all of the  prompts  will

be OFF.  The  ERASE all  memory  query  is always

ON.  Follow  the prompts. The  main  screen  will

show the state of the above.

If you give the program to someone else, or later find  that

the file needs to be changed, just delete it and run the program.

This file is used for two programs, PARAFIL and PAR-XL.

The program assumes that the PARAGON.USR file along with any

.PMF memory files are in the current directory and/or drive.

Page1

* PARAFIL MAIN MENU *

�������������������������������������������������ͻ

�  C)onnect to PARAGON        O)pen a PFM file    �

�  A)dd to memory file        E)dit memory file   �

�  L)oad PMF file             P)rint memory file  �

�  S)ave PARAGON memories     Q)uit    Verify     �

�������������������������������������������������ͼ

CONNECT:

Opens  the  serial port (COM1:), extracts,  and  prints  the

current data in the status frame.

OPEN:

Enter the file name with no extension. If no name is entered

the   default  name  PARAGON.PMF is used. When a file  is  opened

any previous file loaded will be closed.

ADD:

The  add screen will appear along with the next record  num-

ber.  No duplicate channel numbers are allowed.  Please  remember

that  the  channel number takes  precedence  over  the  frequency

when using the scan/memory tune function/s. All but  the   alpha-

numeric  tag  has to be entered. When  entering the  frequency  a

decimal  need  not  be entered for a whole number  and  only  one

decimal  is  allowed, formatting is automatic. The  frequency  is

also retained for the next record as a reminder. The default  for

MODE is  USB, and the crystal FILTER is 6.0.  See EDIT below.

When the record is correct "F10= Confirm  ESC= Exit" will be

appear. On pressing F10 you will be asked to either S)ave, E)dit,

or Q)uit. If saved the record will be written, form cleared,  and

ready  for  the next entry. Upon exiting, if  any  records  where

added, an indexing will take place.

EDIT:

The  program  uses only the over-type  mode  while  editing.

Please  use the below editing commands. If you are in the  middle

of a edit and you wish to exit, put the cursor on the first field

and press F10 or ESC.

CURSOR MOVEMENT COMMANDS:

[PgUp] - Move to the first field

[PgDn] - Move to last field

[DnAr],[Tab],[Enter],[^M],[^X] - Move down a field

[UpAr],[^E] - Move up a field

[^S],[LfAr] - Move a character left

[^D],[RtAr] - Move a character right

[^F],[End] - Move to end of field

[^A],[Home] - Move to beginning of field

DATA INSERT/DELETE COMMANDS:

[Del],[BSP],[^G],[^H] - delete a single character

[^Y] - delete entire line

[^T] - delete to end of line

[Ins] - insert space at cursor position

If  a field was changed while editing a check mark  will  be

placed  at the side of the field. If a mistake was made  and  you

wish to restore the field to the original entry do the following:

1. Go to the field to be restored.

2. Press ^Y to clear it.

3. Press <ENTER> and the original entry will be shown.

This feature is not available for the alpha tag. (more)

Page 2

After  you edit a record  you will be  asked for an  OK,  if

so, then it will be rewritten. If the channel number was changed,

and  no duplicate was found, you will again be asked for  an  OK.

Just an added precaution. Follow the prompts. See ADD above.

Note: No entry deletion is available. You can use the PAR-XL

program,  or manually erase a memory channel/s and then save  the

new version. Experiment.

LOAD:

Used  to load a .PMF file into the PARAGON. When a  file  is

loaded  into memory only the channels in the file are  used.  Any

memory channel in memory of a different number are unaffected.

PRINT Console / Printer:

Console:  Listing is by channel number only.

Printer:  You can list by records,  frequency,  or by memory

channel number.

SAVE:

Enter the name of the memory file or us the default name. If

a  file  exists with the same name you be  notified.  All  memory

channels  are read, checked, and if in use written to the  file.

The format is the same as Sto-Enter. Patience OM.

QUIT:

You guessed it!

ERASE:

The erase all PARAGON memory function is activated by press-

ing ^Z. All the memory channels will be cleared from the PARAGON,

whether in use or not. If you use the PAR-XL program  after doing

a complete erasure, DO NOT USE THE MEMORY TUNE MODE. Enter two or

memories before using this function. The scratch-pad channel (62)

will remain in memory, if entered.

A  PARAGON  memory file is indexed/sorted  twice  under  the

following  conditions: when it is loaded, if any  additional  re-

cord/s where added, and after an editing session.

Timing:

If  an  hangup  occurs try changing  the  fast/voice values.

DO NOT LEAVE THE  PARAGON  IN A WAIT STATE.  Complete any  manual

operations before using any software function. Run the program on

disk called 'TIMDEF' to check against your machine.

Printer:

The program uses the following codes;

ASCII code:   SI (Select condensed)

Decimal:   15

ASCII code:  DC2 (Cancel condensed)

Decimal:   18

ASCII code:   FF (Form feed)

Decimal:   12

Comments:

Please address any comments or ideas to:

J. F. Masur, Route 2, Box 855, Perkins, OK 74059

Signed "Joe" KD5LQ                                        1/27/89