PICPRO

For use with the Kantronics FAX mode

PACKET PICTURE TRANSFERS and WEFAX

by

Buck Rogers  K4ABT

January 3, 1988

THIS IS WHAT "PICPRO" IS ALL ABOUT

This program is a small version of the larger and much more

versatile program "PACFILE" !  We hope this program will introduce

you to a few of the many features which are built into the larger

"PACFILE". Some of the more advanced features of PACFILE are:

Color Changeable Picture Control

Automatic Packet Picture and File transfers

WeFax "ALT W" control

Function Key Control

ASCII Files from disk transfers

Binary Files Transfer

Programmable Routing Buffers

Automatic Configuration of TNC

10 line Type ahead buffer

Connect Alarm

Connect Message Taker

Pop Up Windows

On Line Clock

Receive and Transmit to and from Disk

32 Kbyte Capture and Save buffer

Display Files

Delete Files

PacFile is the most powerful packet terminal program for the

serious packet radio operator to date.  Please INCLUDE YOUR

CALLSIGN when ordering.  PacFile is available from:

Kantronics Inc.

1202 23 rd Street

Lawrence, Kansas 66046

Price: $29.95.

Using the Kantronics WEFAX features to receive a weather fax

picture with your communications receiver, is accomplished in

this manner:

Boot PICPRO.EXE...

To set the terminal parameters press F3.

After the communications parameters are set to the proper

baudrate, parity to None, Databits to 8, and 1 stopbit.

It is important to define the correct comport ie; com 1...etc.

While holding the CONTROL key down, press the F10 key, this will

save the PICPRO configuration to disk. The next time you boot the

"PICPRO.EXE", you will not have to reconfigure the system.  PICPRO

will do it for you.  You may need to make some changes in the KAM

or KPC commands.  You may use the following settings as a guide:

MON       ON

8BIT      ON

AUTOLF    OFF

AX25      ON

ECHO      OFF

PACL      0

PAR       4

HE        OFF

MAXFRAME  7 (IF DIRECT CONNECT, NO DIGIS..OTHERWISE USE 4)

USERS     1

MAXUSERS  1       *** (ALL software flow controls off, PACFILE

and PicPro use RTS/CTS hardware

handshaking, pins 2, 3, 4, 5 and 7 if

DB-25 connector and RS-232 C standard).

XFLO      OFF

TRFLO     OFF

TXFLO     OFF

FLO       OFF

XOFF      00

XON       00

CSTAMP    OFF

MSTAMP    OFF

MCON      OFF

MCOM      OFF

MAXUSERS  0/1 (KAM or KPC-4, spell "MAXUSERS", do not

abbreviate) USERS 1/1    (KAM or KPC-4)

SCREENL   0   (It MUST be set to ZERO (0) )

U NONE

You can now use PICPRO to receive the WEFAX pictures.

Be sure you have the KAM or KPC audio input, connected to the

audio output of a stable communications receiver or transceiver.

At the moment PICPRO is in the terminal mode. You can now move

into the WEFAX or PICTURE mode by performing a CONTROL A.

You should continue to see the CMD: prompt on your screen. If not

you should reset the KAM or KPC, making sure the ABaud settings

agree, between the KAM/KPC and the PC. Confirm the correct

comport.

Once you have the WEFAX station tuned in, at the cmd: prompt, type

WEFAX 1280 1. Start the scan of the chart by pressing F1.

You can restart the FAX picture or resync it by pressing the F1

key, and as many times as you like.

Once in the FAX mode and while you are receiving a weather fax

picture, you can change the overscan rate by using the page up

and page down keys to vary the sweep, ie; wefax 1280 1 is what

you enter when you press F1, but you may want to set the complete

picture to your screen with less resolution. To do so just press

the page-up key five (5) times. With practice, you will soon have

the fax picture appearing exactly as you would like to print.

Save the weatherfax picture in this manner:

After the weather fax picture is complete, press F8 to place

it into the RAM buffer. You can look at the picture in the buffer

using ALT D. If it is a picture or chart, you wish to save, press

ALT S and give it a name, (we use an extension of PIC

for pictures and FAX for weatherfax) then press <enter>.

If not, ESC <enter> to go to the packet CMD: mode. Control A and

F1 will again take you to the FAX mode you were in, or the ESC key

will return you to the PICPRO terminal mode.

If you like to look at pictures in colors other than the received

colors, just look at this touch of genius Bob has supplied in

PICPRO. While the picture is displayed on your screen, use the up,

and down arrows and watch the miriade of colors as you manipulate

the arrow keys.

When you first pulled the directory of the PICPRO disk, you saw

a program called, appropriately "VF.EXE". Place this program

on the "picture disk" with the pictures you are filing. At any

time you wish to look at or view the pictures which you have

saved, just boot the VF.EXE program and at the prompt for

filename, give it the picture title you want to see and enter.

Press ESC to view more pictures with VF.EXE.  Shift F10 will send

the picture to some printers.

Another program on the disk is SCD.EXE. When you find a picture

in a program or file you would like to send or save for future

packet transfer, simply load it. It will advise you how to

proceed if a picture appears on screen you wish to save.

This "screen dump" program will save as many pictures as you can

look at while loaded or until the disk is full. Each successive

picture title will be incremented by one, (SCREEN.000...SCREEN.001

etc..) until the screen dump/save utility SCD.EXE is removed from

the "background" of the PC.

If the VF.EXE does not print your displayed picture with Shift

F10, then use the following method to send the picture to your dot

matrix printer.

Most MS/DOS disks contain a program called "GRAPHICS.COM".

This is the vehicle which allows a "SCREEN DUMP" or hardcopy of

the viewed screen. It must be loaded prior to booting PICPRO.EXE.

Once it is resident in upper memory, it can be called at any time

to do a printout of the screen by envoking the "print screen"

command.  On some PC clones, this is the "Shift / Print" keys.

Here is the command summary of the PICPRO.EXE commands.

Control A    = Go to PIC / FAX mode

* remember these commands are activated by first entering the

PIC/FAX mode

WEFAX 1280 1 = enters the wefax receive mode.

Pageup/Pagedown sets number of overscan lines.

F1           = restarts or resync chart.

F8           = captures the FAX chart to the buffer.

ALT S        = saves the buffer to disk, enter title/ext.

F10          = escape from scan mode while receiving FAX.

ALT L        = loads a picture or chart for viewing or

transmitting.

ALT D        = Display picture or chart while in the FAX mode.

Use arrow keys to change colors.

ESC          = While in FAX mode, will return to terminal mode.

The following commands are the "terminal mode" commands !

ALT F1       = Automated picture send/receive mode. (Same program

needed at each end)

ALT D        = Change default drive/directory.

ALT G        = Change screen colors, saved with Cntl F10.

F10          = Look at current drive/directory.

F1           = Send control C  (03 hex) to KPC/KAM.

F2           = Send CONVERSE command to KAM/KPC.

F3           = Set terminal/KPC baudrate, etc..saved with Cntl F10

Control F10  = Saves your system configuration to the PICPRO disk.

For more detailed information, you may wish to consult the C Q

Magazine article beginning on page 22 of the April 1988 issue.

Good Packeting, de BucK4ABT

100 Pictures and Schematics in the PICPRO and PACFILE format are

available, 6 disks / $25.00.  Single disks / $5.00 approximately

20 pictures for $5.00 each from:

Buck Rogers  K4ABT

506 Pheasant Ridge Drive

Warner Robins, Georgia  31088