*.* Structures Unlimited *.*

Presents

D3RUN

Copyright (C) 1991, Structures Unlimited, Brookfield, WI

All rights reserved

INTRODUCTION

============

This limited version of D3RUN is offered to current and future users

of Finite  Element  Analysis,  free  of  charge.    The program has been

restricted to a maximum of 50 nodes for trial use and BBS distribution.

To obtain the  full  3D  Frame Analysis program with over 6000 available

degrees of freedom, a  users manual, and future updates, simply complete

the registration form included.

HELPFUL HINTS

=============

1.  General

All input must be unit consistant.

2.  Member Definition

A.  Z reference node locates the XZ-plane of the element.

B.  Pin codes:  enter one (1) to release the degree of freedom.

IPC1 - ABC

A - axial release

B - Y rotation end one (1)

C - Z rotation end one (1)

IPC2 - ABC

A - X rotation release (torsion)

B - Y rotation end two (2)

C - Z rotation end two (2)

C.  Rigid links are identified with a beam property of 99.

D.  Rigid ended elements require an "A" and "B" dimension which are

the length of the rigid section for end one (1) and end two (2),

respectively.

3.  Restraints

A.  The restraint code, numbers 1-6, are entered to selectively restrain

degrees of freedom.

B.  Rotational values for input are in radians.

4.  Load Input

Nodal and gravity loads are input in the global coordinate system, while

member loads are input in the local (member) coordinate system.

5.  Printing Output

If  an  80 column printer  is  being  used,  set  your  printer  for

condensed mode since the standard output is wider than 80 columns.

6.  Plotting

When  plotting a sectional view, enter  the  node  number  and  axis

reference to define the view direction.  For  example, a "2Z" would

plot the members viewed from node 2 and looking in the Z direction.

For further information and needed help please fax us at

1-414-784-7523.

!!!!!!  good luck !!!!!!

==============================================================================

[                           REGISTRATION FORM                                ]

[                                 D3RUN                                      ]

[                                                                            ]

[ Date:  ____/____/____                     TO:  Structures Unlimited        ]

[                                                P.O. Box 547                ]

[                                                Brookfield, WI 53008-0547   ]

[                                                                            ]

[  FROM:                                                                     ]

[        Name: __________________________________________________            ]

[                                                                            ]

[     Company:  _________________________________________________            ]

[                                                                            ]

[     Address: __________________________________________________            ]

[                                                                            ]

[        City: __________________________ State: ___ Zip: _______            ]

[                                                                            ]

[   Area Code: (______) Phone: __________________________________            ]

[                                                                            ]

[   Comments:  __________________________________________________            ]

[                                                                            ]

[              __________________________________________________            ]

[                                                                            ]

[              __________________________________________________            ]

[                                                                            ]

[                                                                            ]

[        ___ Enclosed is $350.00 for a single registration of D3RUN,         ]

[            and a users manual.                                             ]

[                                                                            ]

==============================================================================