___________________________________________

C O M S E T

Modem / Uart Setup Program

Version 9.0..release date 4/5/92

from Hank Volpe Copyright 1987,1988,1989,1990,1991,1992

BBS Phone: 410-256-3631

Voice Phone: 410-256-5767

___________________________________________

Introduction

Comset is a utility that is designed to setup a communications port

and a AT (Hayes) Compatible smartmodem. Many modems, especially

internals, do not have a method to reset the modem when it is

activated, nor does an adequate method exist to setup the serial port

for use.

Comset will setup your serial port to any speed from 300 to 56000 bps

and then check to see if your modem is on-line. If it isn't, Comset

will perform a hardware reset to "wake" it up. Next a software reset

is performed by sending the command ATZ. If the reply OK is received,

you are informed that the modem is fine. If OK is not received, Comset

will attempt 3 times to reset the modem before informing you of an

error. If you have an external modem and it is off, Comset will abort

after setting up the serial port to the desired baud rate.

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 2

What's new in Version 9.0 ?

---------------------------

You can always download the latest version of Comset, plus get user

support on the Modem Doctor BBS (410-256-3631) 24 hrs a day. All

users, registered or shareware, are always welcome. Additionally,

you can find Comset on most national BBS systems.

As with most shareware products, comset continues to evolve due to good

feedback from users like yourself. The major improvements in Version 9

are in speed and code size. Modem setups are faster, and Uart only

setups happen without a visible delay. Also the Uart only setup mode

allows you to raise control lines DTR and RTS without the need for

an active modem or serial device. This could be a help to those

looking for serial cable problems, or setting up devices like serial

printers. Also, A new test was added to confirm that the baud rate

register actually set to the desired baud rate. It's one thing to tell

a chip to set, another that it does!

Finally, the top speed is now 57.6k. Acutally it always was,

but National Semiconductor, the makers of the 16450,16550,and 16550AFN

chips refer to this speed as 56000. Well, so did I. Technically, the

baud clock on your serial port could have a 2.86% error at 56000, which

means if it was a positive error, your port speed would be 57600 baud.

Remember though, it could also be a minus error. This is why I and

National Semiconductor followed the conservative name approach and

picked 56000...but since no one else did, and to avoid confusion,

Comset 9 adopts the new nomenclature.

Some of the features of Comset include the following ;

Level 1 options...Shareware Version

** User Selected baud rates from 300 to 57600 bps.

** User Selected Comm port assignments [1 to 4].

** Modem Status register testing.

- Comset tests the registers for DTR/RTS and Carrier Detect

** Redirectable output..can be sent to a file for logging.

or used over any serial link.

** RTS/CTS DTR/DSR Handshake testing.

** Ability to setup the UART alone for dumb modems or

other serial devices up to 57.6K baud

** New "s" switch allows you to swap any comm pointers in memory

** New "p" switch allows you to see your comm address assignments

** Log in of Comm ports 3 & 4 to DOS for use by other programs

such as high level languages or applications.

** Ability to set a uart up and raise control lines without

the need for an active serial device or modem.

Registered users have access to these special features;

** Batch file parameter passing and error reporting.

** Ability to send custom Hayes command strings to the modem

using a simple ASCII text file

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 3

I. USING COMSET

----------------

The usage syntax for Comset is simple.

COMSET [speed] [port] U

or

COMSET [speed] [port]

or

COMSET h

or

COMSET i

or

COMSET p

or

COMSET s  [port] [port]

[speed] and [port] are user options

u tells Comset to set the UART only

h invokes runtime help for the user

i shows you revision and program information

p peeks at your comm port memory address assignments

s swaps the memory address assignments between the

first port you entered and the second.

Comset is easy to use. Typing COMSET from the dos prompt will

automatically attempt to set up serial port Com1 to 2400 bps and

attempt to wake up its modem. Selecting a different speed involves

only adding the speed after the word Comset...ie COMSET 1200 sets

serial port COM1 to 1200 bps instead of the default 2400. Trailing

zeros are not necessary, so you could also type COMSET 12 to set up

serial port COM1 to 1200 bps. If you wish to use a different serial

port, add that after the speed...ie COMSET 24 2 will set serial port

Com2 to 2400 bps. If you accidentally enter an invalid baud rate, the

default of 2400 bps will be used. If you accidentally enter an invalid

range of comm ports, the default of COM1 will be used. Defaults are

always used if the user does not supply speed and port. Typing  a U

after any option instructs Comset to setup the  serial port only and

not to look for a modem.  This is handy for setting up  serial

printers or other dumb devices to speeds  higher than 9600 baud.

Mode.com from dos will do the same, but not at  57600 baud, and not

on all machines!.

Typing Comset p will display all of the comm port addresses logged

into Dos. This can be handy when you are trying to configure a serial

card for proper addresses.

Typing Comset s will swap the address assignments in memory of com1

and com2. If you entered comset s 1 4, then the address assignments

will be swapped between com1 and com4 instead of com1 and com2. Any

valid port from 1 to 4 can be used.

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 4

Typing COMSET h will bring up a runtime summary of features and other

helpful information. Ideally, Comset should be run out of your

autoexec.bat file when you  first turn on your computer. Internal

modem users can appreciate the fact that probably 2 or 3 times out of

a 100 their internal modem does not "wake-up" properly when the

computer is first turned on, and this can hang-up some communications

programs. Also, BBS sysops sometimes experience problems with modems

when stopping the BBS and trying to use the modem to call out.  This

is why Comset was written!

II. SERIAL PORT INFORMATION

---------------------------

Comset supports all the industry standard Serial Port address

assignments for modems.  Almost all PC-modems allow you to use COM1-4,

however only specific software  can use COM3 or COM4 on most machines.

Comset will not only use COM3 and COM4 itself, but it will log those

ports into DOS for use by other programs such as high-level languages

and applications (something that under normal conditions can't be done

with an IBM-PC or most clones without special software). Using the new

"s" feature, you can also swap COM3 to COM1's address for those

programs that cannot access a COM3 or COM4. Using the "p" feature, you

can find out what serial port assignments your computer is using.

III ERROR REPORTING INFORMATION

-------------------------------

Comset checks your serial port in three distinct steps;

1) Set the Serial Port computer UART to the desired baud rate and

raise the DTR and RTS lines. If a serial port is not detected, Comset

aborts and displays the message that there is no serial port with that

COMM designation your system.

2) Check that DSR and CTS are returning from the modem, if not report

an error and stop testing.

3 a)If steps 1 & 2 are successful, wake up the modem by sending the

command string "ATZ". If OK is received, report all is  Normal and

leave the RTS and DTR lines active.

b)If the modem did not wake up, issue a more advanced hardware and

software command string to reset and wake up the modem. Three attempts

are made..if it fails, the RTS and DTR lines are turned off. This

could happen

if you tell Comset to test a serial port that does not have a

compatible modem attached. In that case, a "failure" of this kind

would be "normal" for your system.

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 5

You can redirect the output of Comset to a file if you wish to

maintain a log of each initialization. To do this, just use the

command string

COMSET [speed] [port] >> comset.log

Speed and port are optional. The >> symbol tells dos to make a file

called comset.log and append it each time Comset is run.

IV FULL FEATURE USE AND ERROR REPORTING

---------------------------------------

Registered Users of Comset receive some advanced features, some of

which were described above in previous sections. An additional feature

of the registered version is that of error-code reporting. Error code

reporting simply stated passes "result-codes" from Comset to its

calling program. This allows you to write some advanced batch-file

procedures or to take specific actions.

Running the registered copy, additional information is displayed

informing you of the Error code received. Also that error code can be

interpreted by Dos Batch files using the ERRORLEVEL function. Under

normal conditions, a result code of 0 is returned to the calling

program. If something wrong is detected, an error code of 99 is

returned to the calling program.

A simple use of this can be to start a communications program if the

modem is ok, but if not, default to some other action...an example

batch-file program looks like this;

echo off

comset 24 1

IF ERRORLEVEL 99 goto stop

echo passed

goto exit

:stop

echo failed!

:exit

Other uses are left up to your imagination...you could flash up help

screens to the user like "HEY DUMMY...TURN ON THE MODEM!!!" or

something similar. Combined with redirecting the output to a logging

file, you could trace a history of failures or other actions.

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 6

Also,  new  in the registered version of Comset is the ability to send

custom setup strings to your modem from a simple text file. To use

this feature, all you need to do is the following;

1) Make a file with valid AT commands using edlin or any ASCII

text editor and name the file Comset.cmd

2) Place this file in the same directory that you are running

comset out of. If you are running Comset from a path, then

you must have a Comset.cmd file in the directory you are

currently in for this feature to work. The reason for this

is  so  that  you  could have multiple  comset.cmd files

in different directories for different uses.

When  Comset starts,  it will look for Comset.cmd in  the  current

default directory,  and if it finds it,  it will send that command

string to the modem instead of the ATZ command. An example of such a

command set is the following;

From dos, type edlin comset.cmd

when edlin starts, type I for insert.

Next, type AT M0 H1 then return

Next, type control-Z

Next, type e and the file will save.

The next time you run comset, it will find the file comset.cmd and

tell the modem to shut off the speaker and go off-hook.

Any valid AT command set for your brand of modem can be sent.  The

only restriction is that all these commands fit on one line and  the

first two letters must be AT.

You  can  also redirect the output of this function to a  file  as

explained above. Users of Modems can use this to capture the settings

of their modem and save it in text form for future reference.

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 7

V. REGISTRATION INFORMATION

--------------------------

You can freely make copies and distribute the shareware version of

Comset over BBS systems or  public domain libraries as long as no fee

for Comset is charged  (except duplication costs) and as long as

Comset.exe is distributed along with this file Comset.doc. These may

be archived together for use on a bbs system. Under no circumstances

can the registered version of Comset be distributed. Each version

displays "shareware" or "registered" when it is run from the dos

prompt.

Comset is copyrighted by Hank Volpe and is not public domain software.

Shareware version users are granted a limited license for its use. No

warranties for Comset are stated, expressed or implied.

Shareware authors continually provide computer users with low priced

software compared to the traditional retail approach. If you register

Comset, you will be able to use all the features ( Com1-4 support,

auto-log in of Com3 and Com4 for Dos applications, advanced error

reporting and batch file support).

Registration is simple...and price is low. Comset is only $10, which

includes the latest version and free maintenance updates. To register,

send $10 along with your name and address to;

Hank Volpe

PO Box 43214

Baltimore MD 21236

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 8

VI. ERROR MESSAGES

THERE IS NO COMx IN YOUR SYSTEM -

Comset was instructed by the user to test a non-existent comm

port. This could happen automatically if you don't have a COM1

in your system. If this is the case, use the line

COMSET [speed] [port] to select your specific needs.

MODEM NOT RESPONDING PROPERLY -

The modem under test did not set up even after 3 extensive

hardware and software attempts to do so. Either you have a non-

compatible or non-intelligent modem, or you are using the wrong

opening baud rate. Comset's defaults are 2400 bps and COM1. If

you only  have a 1200 or 300 bps modem, use the line

COMSET [speed] [port] and use a lower speed. Valid speeds are

300 bps,  1200 bps , 2400 bps [default], 4800 bps, 9600 bps,

19200 bps, 38400 bps, and 57600 bps. Trailing zeros are not

necessary..ie 576 is 57600 bps. Any other values default

automatically to 2400 bps.

Comset Ver 8.0 Documentation  Copyright 1987-1991 Hank Volpe

Page 7

MODEM APPEARS TO BE OFF OR DISCONNECTED -

The hardware handshaking pairs RTS/CTS or DTR/DSR are not

communicating properly. This should NEVER happen on internal

modems unless the CTS signal is not enabled. Internal Modems

usually keep this signal at a logic 1, so if you see this error,

check your internal modem for proper strapping of the CTS and DSR

signals.

If you have an external modem,and the power for the modem is on,

you will see this error message if you do not have the CTS signal

at a logic 1 or you have a defective or inadequate Serial Cable.

Most external modems have Dip-switches on the back for setting

the CTS signal. Normally the CTS follows the Serial Ports RTS and

the modem  DSR (sometimes called MR) follows the Serial Ports

DTR.

In the case of an external modem you will see this error message

every time the modem has not been turned on. This is telling you

that your serial port hardware is fine, just that your modem is

off.

Comset Version 9.0 Documentation  copyright (c) 1992 by Hank Volpe

Page 9

VII COMSET PROGRAM HISTORY

--------------------------

Version 1 - Simple Assy program fixed at 300 bps. Two versions

COM1 and COM2.

Version 2 - Baud rate raised to 1200 bps.

Version 3 - Baud rate raised to 2400 bps.

Version 4 - COM1 / COM2 support in one program.

Version 5 - Major rewrite, added interrupt support, hardware

and software testing, COM 1-4 support, Multiple

baud rate support, error checking and reporting,

and support for high speed modems to 19200 bps.

Version 6 - This version adds the U parameter to allow for testing

and setup of dumb modems and other serial devices.

The registered version adds the ability to send a

custom command string from a file named comset.cmd

to the modem in place of the normal ATZ command

Version 7 - This version adds baud rates to 38.4K,improved Uart

communications module.

Version 8 - This version adds speeds to 56K baud, improved

Uart communications module, addition of the

comm port swap and comm address features

Version 9 - Improves speed of routines, adds the ability for

uart only setups not to require an active device,

changes the top speed description to 57.6k, and

adds a test to verify the baud rate register set

to the desired baud rate

Current Software release is Version 9.0