WRITING ROMABLE CODE IN TURBO C:

Updated 2/2/89 for Turbo C 2.0 by the author.

I recently completed a consulting project to design a

special-purpose controller using an embedded 8088 with firmware

in ROM. To create the firmware, I developed a scheme for

generating ROMable code from Turbo C.

The scheme utilizes the following:

1. Rules for writing C code.

2. ROMIZE.C, a program to post-process the Turbo C Compiler

assembler output.

3. INSJUMP.C, a program to patch the ROM binary code to

insert a restart jump instruction.

4. BINTOHEX.C, a program to produce Intel-format files for

downloading ROM images to an EPROM programmer.

5. C0ROMX.ASM, a replacement for C0.ASM, the startup code.

In addition, the scheme requires the Turboc C command-line

compiler, TCC.EXE; an assembler, Microsoft Version 3.0 or later,

or equivalent (I use Microsoft V5.0); TLINK or MS-Link; and

EXE2BIN (more about EXE2BIN later).

The scheme involves the following steps:

1. C code is written following a few rules.

2. TCC is used to translate the C code to assembly.

3. ROMIZE is used to perform certain modifications to the

assembly code.

4. The modified assembly code is assembled into a .OBJ file.

5. The .OBJ file is linked with the C0ROMX.OBJ startup code

and with other .OBJ files (if needed; these may be from C code

that has gone through the four above steps, or routines

originally coded in assembly).  This produces a .EXE file.

6. EXE2BIN converts the .EXE into a binary ROM image.

7. INSJUMP inserts the reset jump into the ROM image.

8. If necessary, the binary ROM image is converted into

whatever format the particular EPROM programmer requires. (Mine

is a plug-in card for the PC which takes binary format directly,

so I don't have to do this step.)

All these steps can be run in a .BAT or MAKE file, making the

conversion of the original C code to ROM code almost automatic.

RULES AND ASSUMPTIONS:

The rules and assumptions for writing ROMable code using this

scheme are as follows:

1. The Compact memory model (small code, large data) is used.

This results in a maximum of 64K of code and initialized static

variables (i.e. constants) but up to 1 MB of Data space.

2. Initialized static variables are considered constants, and

may not be changed during program execution because they are

stored in ROM.

3. Variables defined as "extern" are considered constants.

Such variables would be used for font tables, for example.  True

"variables" (which change during program execution) may not be

defined in one C module and declared as "extern" in another.  The

only means of inter-module variable passing is through function

parameters and return values (a good programming practice

anyway). IT IS OK to define variables in a C module and reference

these as EXTRN's in assembler-coded modules, however.

4. Most of the Runtime Library routines may NOT be used,

because they have been compiled with the wrong segment groupings

for the ROM environment.  Many of the library routines are not

usable, anyway, because they pertain to the DOS environment and

use system facilities that are not available in the ROM

environment.  Library routines that are needed will have to be

recreated, either in C or assembly.  You may want to order the

Turbo C Runtime Library from Borland as a starting point. I

ordered the Runtime Library several months into the project that

stimulated the creation of this system, but I didn't really need

it.

I was able to compile and debug over 90% of my code (which

was about 3,500 lines of C) using the Integrated Environment on a

PC, then re-compile (with #DEFINES and #IFDEFs to accommodate the

environmental differences) for ROM with little further debugging.

SEGMENT USAGE IN TURBO C:

Turbo C assigns uninitialized static variables to a segment

called _BSS and initialized static variables (i.e., constants) to

a segment called _DATA. These two segments are associated via a

GROUP directive, and assumed, under normal circumstances, to be

jointly addressable by the DS register.  Instructions are

generated in the _TEXT segment, and assumed addressable by the CS

register.

In the ROM environment, the constants in the _DATA segment

and the code in the _TEXT segment are both located in the ROM and

addressed by CS.  Only BSS is addressed by DS.  Therefore to

convert the code generated by the compiler, ROMIZE is used as

follows:

WHAT ROMIZE DOES:

ROMIZE makes two passes through the assembly source code

generated by the command line compiler.  In the first pass, it

reads the code and accumulates a table of all symbols defined in

the _DATA segment.  There are only a few such symbols, since

Turbo C places all string constants in a single symbol "s@", and

refers to individual strings as "s@+n".  Likewise, all

non-string constants are part of a single symbol called "d@" and

referred to by their displacements.  There are however, a large

number of references to these symbols in the code.

In its second pass, ROMIZE re-reads the source code and

writes it out with modifications.  The modifications are:

1. The command that groups _BSS and DATA together into DGROUP

is changed so that only _BSS is in DGROUP.

2. A new GROUP command is inserted creating CGROUP as _TEXT

and _DATA.

3. The ASSUME statement is changed from

ASSUME cs:_TEXT,ds:DGROUP

to:

ASSUME cs:CGROUP,ds:DGROUP

4. In any line of code with a reference to a symbol in the

_DATA segment, the group reference is changed from DGROUP to

CGROUP, or a CGROUP reference is added if no group name is

present.

5. Preceding a line of code with a reference to a symbol in

the _DATA segment may be a "mov x,ds" or a "push ds"instruction.

This is changed to a "mov x,cs" or "push cs".

6. Following a line of code with a reference to a symbol in

the _DATA segment may be a "push ds" instruction. This is changed

to a "push cs".

In order to do the looking backward and forward, ROMIZE

maintains a three-line pipeline, reading new lines into the head

of the pipe and writing lines from the end.

The various segment naming options of the compiler do not

produce the desired changes for the ROM environment. I would be

happy to pursue this subject further with anyone who is curious

about it.

LINKING:

You can use either TLINK or MS-LINK. The command line will,

of course be different for each. The "Warning - no stack segment"

will be produced by either. This is normal.

You must read the link map carefully to verify that your code

fit in the size of ROM you use. Consider the following partial

link map:

LINK : warning L4021: no stack segment

Start  Stop   Length Name                   Class

00000H 05A74H 05A75H _TEXT                  CODE

05A80H 07DF7H 02378H _DATA                  DATA

07DF8H 0A530H 02739H _BSS                   BSS

0A531H 0A531H 00000H _BSSEND                BSSEND

Origin   Group

0000:0   CGROUP

07DF:0   DGROUP

The .bin file that came out of EXE2BIN for the module that

produced this link map was A531 (hex) bytes long, however, this

module does fit in 32K. The key indicator is the sum of the

lengths of _TEXT and _DATA, and consequently, the starting

address of DGROUP (_BSS). Here it is 7DF8, about 500 bytes

under 32K.

MORE ABOUT EXE2BIN:

EXE2BIN is needed to convert the .EXE file into a binary

image that can be transmitted to an EPROM programmer. It may need

to perform "segment fixups" (segment relocation), if your program

includes constructs like the following:

static char *menu[] = {"Preview   ","Add Before","Add After "};

This is a convenient way to generate arrays of strings for

such things as menus.  This generates an array of pointers whose

segment parts must be relocated by EXE2BIN to the segment address

of the ROM.  Therefore, when the EXE2BIN step is run, you will be

prompted to enter a segment address (F800 hex in the attached

example).

For those users of versions of DOS (such as 3.3) that do not

include EXE2BIN, there are several alternatives: Use EXE2BIN from

DOS 2.0 (I think 2.1 will work also), or use EXE2BIN from DOS 3.0

or 3.1 and patch it so it does not test for DOS version. The test

is at the beginning of these programs and is easily defeated.

Except for the version check, I know of no DOS version

dependencies in these programs. (I did all my development under

DOS 3.2, however,  which still includes EXE2BIN).

The EXE2COM program available on CompuServe and many bulletin

boards may work in place of EXE2BIN (I have not tested it),

EXCEPT when segment relocation is required (i.e., you use the

above-described construct).  It does not do segment relocation.

The fact that EXE2BIN stops and prompts you for a segment

relocation factor is an annoyance.  You can use the PC Magazine

KEY-FAKE.COM (see PCMagnet) to supply the required value in a

batch or MAKE file. If KEY-FAKE is used in a MAKE file, you must

have invoked KEY-FAKE at least once (without arguments is fine)

before running MAKE. KEY-FAKE installs as a TSR the first time

it is invoked, and apparently, Turbo MAKE doesn't like programs

that do this, so if you invoke KEY-FAKE in a MAKE file

without it already being resident Turbo MAKE terminates with an

error 768.

MORE HINTS:

If you declare an array of strings as mentioned in the above

discussion, the "static" storage class is not necessary if the

array is defined outside of any function.  If such an array is

defined as local to (within) a function, then be certain to add

"static", otherwise, Turbo C generates code to copy the array

into the function's local variable space (the stack). This is

unnecessary if the array is indeed a constant.

Although you won't be using many (any) library functions, you

may still want to include a number of standard header files.

The library functions you recreate ought to use the same

definitions as the library versions, anyway.

"dos.h" is an especially useful item.  Although chances are

you won't be using any of the file I/O functions defined in it,

it includes some macros and in-line functions that are EXTREMELY

useful in the ROM environment such as MK_FP(), inport(),

outport(), FP_SEG(), FP_OFF(), peek() and poke().

FLOATING POINT LIMITATIONS:

If you need to use floating point arithmetic, you may have a

problem. If your target hardware includes a numeric co-processor

then you will have to order the Runtime Library and modify the

8x87 support routines to run in the ROM environment. If you don't

have an 8x87 and still want to use floating point, you can't

because the Runtime Library does not include all of the floating

point emulator source code.

SAMPLE PROGRAMS:

A simple C program which reads a value from an input port

and writes a selected string to an output port is included. A

Turbo Make file is provided to create the ROM from this code,

which illustrates all of the necessary processing steps.

STARTUP CODE -- C0ROMX.ASM:

This code is simple, compared to the regular Turbo C startup

code.  About all that is required is to set up the DS, SS, and SP

registers.  The particular values for these will depend upon the

amount and address space of your RAM.  The code has an optional

sequence to initialize static variables to 0, as is "normal" in

C.  I did not use such initialization because my particular

application had battery-backed-up static RAM. Note that

C0ROMX.ASM does not have a starting address in its END statement.

The actual code startup is supplied by a JMP inserted by INSJUMP.

Note also that the linking step specifies C0ROMX as the first

object module. That guarantees it will start at relative address

0000 in the ROM.

INSJUMP.C:

The 8088 family processors start up after a RESET by

executing code beginning at FFFF:0. INSJUMP inserts an inter-

segment jump to relative location 0 at the ROM location that

corresponds to FFFF:0.  INSJUMP can process 32K (i.e. 27256) or

64K (27512) EPROMs. It is used as follows:

INSJUMP filename [64K]

If the second parameter "64K" is not present, INSJUMP assumes it

is processing a 32K ROM. The JMP instruction is to F800:0, and

it is inserted at location 7FF0 in the ROM. For a 64K ROM, the

instruction is a JMP F000:0 inserted at ROM location FFF0.

The .bin file generated by EXE2BIN may be less than 32K

for a 32K ROM, or less than 64K, for a 64K ROM. INSJUMP

extends the file out to 32K or 64K with FF's. Erased EPROMs

contain all FF's, so unused locations will not have to be

programmed, which will shorten programming time on most

programmers.

As supplied, INSJUMP also calculates a checksum and places

it in the last two ROM bytes. My product has a self-test that

includes ROM checksum on power-up. Other users may want to put

additional data, such as a part number, in the ROM. This can be

done with INSJUMP.

WRITING ASSEMBLY-LANGUAGE FUNCTIONS:

The only differences in writing assembly-language functions

for the ROM environment versus DOS are segment groupings. The

Turbo C User's guide suggests compiling a small C module to

assembler as a starting point.  For the ROM environment do the

same thing, then process the assembler through ROMIZE.

If your assembler functions have only a code segment called

_TEXT, with no other segment definitions, and if they reference

only local variables and parameters stored on the stack, then you

can assemble them just once and use the identical .OBJ modules in

both ROM and DOS versions of your main C program.

BINTOHEX.C:

This program converts a binary ROM image to Intel Hex format.

It is has not been tested.  If you need some other format, such

as Data I/O, contact me.

This has been a simple treatment of a complicated subject.

I have tried to include as much information as I could think of,

but there are undoubtedly a lot of pitfalls I haven't covered.

Anyone who wishes to pursue this subject further is urged to

contact me:

Bob Haas

P. O. Box 1455

Tualatin, OR   97062-9557

(503) 692-1593

CompuServe User # 70357,3530