T A B L E   O F   C O N T E N T S

���������������������������������

Installation . . . . . . . . . . . . . . . . . . . . . .    1

DW6000 Main Menu . . . . . . . . . . . . . . . . . . . .    2

Receive Patches From DW6000  . . . . . . . . . . . . . .    3

Upload Patches To DW6000 . . . . . . . . . . . . . . . .    5

Save Patches To PC . . . . . . . . . . . . . . . . . . .    7

Load Patches From PC . . . . . . . . . . . . . . . . . .   12

Copy / Swap Patches  . . . . . . . . . . . . . . . . . .   14

Edit A Patch . . . . . . . . . . . . . . . . . . . . . .   15

Patch Parameter Screen . . . . . . . . . . . . . . . . .   16

Sample Help Screen . . . . . . . . . . . . . . . . . . .   18

Editing Keys Help Screen . . . . . . . . . . . . . . . .   19

Print Patches  . . . . . . . . . . . . . . . . . . . . .   20

Print Blank Charts . . . . . . . . . . . . . . . . . . .   21

Quit DW6000  . . . . . . . . . . . . . . . . . . . . . .   22

DW6000 / Author Information  . . . . . . . . . . . . . .   23

Samples Follow:

Patch #22 Chart

Blank Patch Chart

����������������������������������������������������������������������

This documentation (c) Copyright Tracy Barber, 1991.  The

DW6000 synthesizer is a product manufactured by Korg (Keio

Electronic Laboratory Corporation).  References to their Owner's

Manual are from the 1985 manual.  Tracy Barber is not associated

with Korg.

I N S T A L L A T I O N

�����������������������

Installation of the DW6000 editor / librarian is quite easy.

FIRST, MAKE A BACKUP COPY OF YOUR DISK!  Replacement disks, if they

are needed, will be charged shipping and handling!

If you have a hard drive, get to the drive that you want the

program to be installed on.

Example:  C:\>

Type PROMPT $P$G to get the subdirectory information on the

screen if you haven't done it at boot time.

Next, if you want to make a subdirectory, type MD DW6000 and

press <Enter>.

Example:  C:\DW6000>

Finally, place this disk in your floppy drive, then copy your

files from that drive.

Examples:  COPY A:*.*     -OR-

COPY B:*.*

That's all there is to it.  No funny stuff!

����������������������������������������������������������������������

This program will work quite nicely on a floppy drive, although

it may run a bit slower.  If you have only a 360k drive, you might

want to copy the documentation onto another disk and keep the program

files on your working copy.  Refer to your DOS manual when in need of

COPY commands or BACKUP.  This program is not "bound" to any drive.

����������������������������������������������������������������������

This program does require a CGA, EGA, VGA or better monitor that

can display 4 colors at any one time (CGA mode) in order to display

the envelope graphs on the screen.

READ THE READ.ME DOCUMENTATION FOUND WITH YOUR PROGRAM!

Page   1

D W 6 0 0 0   M a i n   M e n u

�������������������������������

������������������������������������������������������Ŀ

�       Welcome To The DW6000 Editor / Librarian       ��

�                                                      ��

�       DW6000 (c) Copyright 1991 By Tracy Barber      ��

���������������������������������������������������������

��������������������������������������������������������

�������������������͸

�                   ��

�      Receive      ��

�      Upload       ��

�      Save         ��

�      Load         ��

�      Move         ��

�      Edit         ��

�      Print        ��

�      Charts       ��

�      Info         ��

�      Quit         ��

�                   ��

�������������������;�

���������������������

[F1]=Help           Select DW6000 Function

����������������������������������������������������������������������

The Main Menu is the first screen you will see when you run the

DW6000 program.  This is the "control panel" for working with the

program.

The <Up> and <Down> arrow keys will move the highlight over the

options in the menu.  You can also type the highlighted letter of

each option to move the highlight over that option.  The option will

be selected when you press <Enter>.

At any time, if you are unsure of what the option does, press

the <F1> function key and a help screen will display information

pertaining to that option.  The Info option will only work when you

press <F1>.  <Enter> has no effect on Info.

Page   2

R E C E I V E   P A T C H E S   F R O M   D W 6 0 0 0

�����������������������������������������������������

��������������<<< Receive Patches From DW6000 To P.C. >>>�����������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                               Ready                                �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

���������[ F1-Help ]  [ F10-Receive ]  [ All Other Keys Exit ]������Ľ

����������������������������������������������������������������������

The Receive option will allow you to download a full bank of 64

patches from the DW6000.  The patches will be stored in RAM and will

not be written to disk until you select Save from the Main Menu.  At

the present time, you can only have one bank of patches in memory at

one time.

Pressing the <F10> function key will initiate the download from

the DW6000.  You will see the current patch being stored as the

program proceeds.  All other keys will exit to the Main Menu.

��>  Make sure your midi cables are connected properly.  Midi Out on

the computer is hooked to Midi In on the DW6000.  Midi In on the

computer is hooked to Midi Out of the DW6000.

��>  If you are using a midi patcher, make sure that it is

correctly set to go to the DW6000.

��>  To receive the bank, you need not set any parameters on the

DW6000.

Page   3

R E C E I V E   P A T C H E S   F R O M   D W 6 0 0 0

�����������������������������������������������������

��������������<<< Receive Patches From DW6000 To P.C. >>>�����������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                         Receiving Patches...                       �

�                                                                    �

�                               Patch 88                             �

�                                                                    �

�                        All Patches Received.                       �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                      Hit Any Key To Continue...                    �

���������[ F1-Help ]  [ F10-Receive ]  [ All Other Keys Exit ]������Ľ

����������������������������������������������������������������������

If the download from the DW6000 was successful, you should see

this displayed on your screen.  At this point, hit any key to return

to the Main Menu.

At this point, the patches have no names.  They were given all

generic names, like Patch  11, Patch  33, etc.  Remember if you do

a download, the names from the DW6000 will all be generic.  This means

you will have to work with a disk based file to keep your names in

a meaningful order.  The DW6000 doesn't recognize patch names.  They

were included for your sanity...

If you received an error message, please check your midi cables /

patcher / etc. for possible problems.  When you correct the problem,

you may return here to download.

Page   4

U P L O A D   P A T C H E S   T O   D W 6 0 0 0

�����������������������������������������������

��������������<<< Upload Patches From P.C. To DW6000 >>>������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                               Ready                                �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

���������[ F1-Help ]  [ F10-Upload ]  [ All Other Keys Exit ]�������Ľ

The Upload option will allow you to send patches back to the

DW6000, a complete bank at a time.

��>  The Write Enable switch on the back of the DW6000 must be pushed

to the right to allow the current patches to be overwritten.

��>  Parameter 83 (Omni Mode) must be set to 1.  This way, there will

be no channel conflicts and ensure efficient uploading.

��>  Make sure your midi cables are connected properly.  Midi Out on

the computer is hooked to Midi In on the DW6000.  Midi In on the

computer is hooked to Midi Out of the DW6000.

��>  If you are using a midi patcher, make sure that it is

correctly set to go to the DW6000.

Pressing the <F10> function key will initiate the upload to the

DW6000.  You will see the current patch being uploaded as the program

proceeds.  All other keys will exit to the Main Menu.

Page   5

U P L O A D   P A T C H E S   T O   D W 6 0 0 0

�����������������������������������������������

��������������<<< Upload Patches From P.C. To DW6000 >>>������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                         Uploading Patches...                       �

�                                                                    �

�                               Patch 88                             �

�                                                                    �

�                        All Patches Uploaded.                       �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                           Hit Any Key To Continue...               �

���������[ F1-Help ]  [ F10-Upload ]  [ All Other Keys Exit ]�������Ľ

����������������������������������������������������������������������

If the upload to the DW6000 was successful, you should see this

displayed on your screen.  At this point, hit any key to return to the

Main Menu.  You may then restore Parameter 83 and push the Write

Enable switch on the back of the DW6000 to the left to protect the

patches you just uploaded.

If you received an error message, please check your midi cables /

patcher / etc. for possible problems.  When you correct the problem,

you may return here to upload.

��>  If this is the Demo version, this option will be disabled.  You

will not be able to upload.

Page   6

S A V E   P A T C H E S   T O   P C

�����������������������������������

���������������������<<< Save Patches To P.C. >>>�������������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�             Save Current Bank To The P.C.?  [Y,n]:                 �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

������������[ F1 - Help ]  [ Y - Save ]  [ N / ESC - Exit ]���������Ľ

����������������������������������������������������������������������

The Save option will allow you to save the current bank of RAM

patches to either a hard drive or a floppy drive, depending on how

you load the program.

��>  The program works only with the "default" drive.

��>  If you load the program on drive C:, you will only be able to

see drive C: files.  If you load the program from a floppy disk,

you will only be able to see the files on that diskette.

You will be asked if you want to save the current bank.  Enter a

<Y> here to save.  Type in a <N> or press the <ESC> key to stop this

option.

Page   7

S A V E   P A T C H E S   T O   P C

�����������������������������������

���������������������<<< Save Patches To P.C. >>>�������������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                        New File?  [Y,n]: Yes                       �

�                                                                    �

�                           [TEST.DW6      ]                         �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

������������[ F1 - Help ]  [ Y - Save ]  [ N / ESC - Exit ]���������Ľ

����������������������������������������������������������������������

If you answer <Y>, this is a new file, you will then have to

type in that file name.  The default file extension should always be

.DW6 when saving files.  This is the file extension that the Load

option will use when retrieving save files.

��>  Adhere to DOS file naming conventions when typing in your file

name.  If you type in an incorrectly specified file name, DOS

and this program may not be able to retrieve it later on.

Page   8

S A V E   P A T C H E S   T O   P C

�����������������������������������

���������������������<<< Save Patches To P.C. >>>�������������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�       ���������������������D:\DW6000\*.DW6������������������Ŀ     �

�       �    ..\         .\          TEST.DW6    TRACY.DW6     ��    �

�       �                                                      ��    �

�       �                                                      ��    �

�       �                                                      ��    �

�       �                                                      ��    �

�       �                                                      ��    �

�       �                                                      ��    �

�       �                                                      ��    �

�       �                                                      ��    �

�       ���������������������������������������������������������    �

�        ��������������������������������������������������������    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

������������[ F1 - Help ]  [ Y - Save ]  [ N / ESC - Exit ]���������Ľ

����������������������������������������������������������������������

If you answer <N> to New File?, you will be shown a file pick list

like the one above.

You may:

��>  Select one of the files displayed to overwrite.

��>  Select one the parent path displayed.  (..\)  This will take you

to the directory above the current one you are in.  With this,

you could go to another directory and then save a file by over-

writing one in that directory.

��>  Press the <ESC> key to stop this option.

If you wish to save the file, select the file with the arrow keys

and then press <Enter> to save it.

Page   9

S A V E   P A T C H E S   T O   P C

�����������������������������������

���������������������<<< Save Patches To P.C. >>>�������������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                              File Name:                            �

�                              TEST.DW6                              �

�                                                                    �

�                     Are You Sure ? [Y,n]: Yes                      �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

������������[ F1 - Help ]  [ Y - Save ]  [ N / ESC - Exit ]���������Ľ

����������������������������������������������������������������������

No matter how you save a file, you will be prompted like above

to verify that you want to save the file.   Answer <Y> to save it,

<N> or <ESC> will stop this option.

Page  10

S A V E   P A T C H E S   T O   P C

�����������������������������������

���������������������<<< Save Patches To P.C. >>>�������������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                Saving The Current Patches To The P.C.              �

�                                                                    �

�    Data -> ................................................... 64  �

�                                                                    �

�                     The File Has Been Saved.                       �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                      Hit Any Key To Continue...                    �

������������[ F1 - Help ]  [ Y - Save ]  [ N / ESC - Exit ]���������Ľ

����������������������������������������������������������������������

At this point, the program will take over and write out all 64

patches to the disk file you requested.

If there were any errors writing the file, you will be displayed

a prompt letting you know.  The most common errors are lack of disk

space and bad disks.

Page  11

L O A D   P A T C H E S   F R O M   P C

���������������������������������������

��������������������<<< Load Patches From P.C. >>>������������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�      ���������������������D:\DW6000\*.DW6������������������Ŀ      �

�      �    ..\         .\          TEST.DW6    TRACY.DW6     ��     �

�      �                                                      ��     �

�      �                                                      ��     �

�      �                                                      ��     �

�      �                                                      ��     �

�      �                                                      ��     �

�      �                                                      ��     �

�      �                                                      ��     �

�      �                                                      ��     �

�      ���������������������������������������������������������     �

�      ���������������������������������������������������������     �

�                                                                    �

�                                                                    �

�������������[ F1 - Help ]  [ Y - Load ]  [ N / ESC - Exit ]��������Ľ

����������������������������������������������������������������������

The Load option will allow you to load a disk file containing 1

bank of 64 DW6000 patches.  You will be shown a file pick list like

the one above.  These patches will be stored in RAM only.

You may:

��>  Select one of the files displayed to load.

��>  Select one the parent path displayed.  (..\)  This will take you

to the directory above the current one you are in.  With this,

you could go to another directory and then load a file in that

directory.

��>  Press the <ESC> key to stop this option.

If you wish to load the file, select the file with the arrow keys

and then press <Enter> to save it.

Page  12

L O A D   P A T C H E S   F R O M   P C

���������������������������������������

��������������������<<< Load Patches From P.C. >>>������������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                        Loading Patches From:                       �

�                         D:\DW6000\TRACY.DW6                        �

�                                                                    �

�                               Patch 64                             �

�                                                                    �

�                      The File Has Been Loaded.                     �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                       Hit Any Key To Continue...                   �

�������������[ F1 - Help ]  [ Y - Load ]  [ N / ESC - Exit ]��������Ľ

����������������������������������������������������������������������

After you have selected a file, your screen should display the

patch numbers as they are being loaded into memory.  If the load was

successful, hit any key and you will return to the Main Menu.

If there were any errors loading the file, you will be told of

this and then returned to the Main Menu.  Possible problems are

corrupt files, corrupt disks.

Page  13

C O P Y / S W A P   P A T C H E S

���������������������������������

�������������<<< Copy And Swap Patches In Current Bank >>>����������ķ

�                                                                    �

�                                                                    �

�11) Patch  11      31) Patch  31     51) Patch  51     71) Patch  71�

�12) Patch  12      32) Patch  32     52) Patch  52     72) Patch  72�

�13) Patch  13      33) Patch  33     53) Patch  53     73) Patch  73�

�14) Patch  14      34) Patch  34     54) Patch  54     74) Patch  74�

�15) Patch  15      35) Patch  35     55) Patch  55     75) Patch  75�

�16) Patch  16      36) Patch  36     56) Patch  56     76) Patch  76�

�17) Patch  17      37) Patch  37     57) Patch  57     77) Patch  77�

�18) Patch  18      38) Patch  38     58) Patch  58     78) Patch  78�

�21) Patch  21      41) Patch  41     61) Patch  61     81) Patch  81�

�22) Patch  22      42) Patch  42     62) Patch  62     82) Patch  82�

�23) Patch  23      43) Patch  43     63) Patch  63     83) Patch  83�

�24) Patch  24      44) Patch  44     64) Patch  64     84) Patch  84�

�25) Patch  25      45) Patch  45     65) Patch  65     85) Patch  85�

�26) Patch  26      46) Patch  46     66) Patch  66     86) Patch  86�

�27) Patch  27      47) Patch  47     67) Patch  67     87) Patch  87�

�28) Patch  28      48) Patch  48     68) Patch  68     88) Patch  88�

�                                                                    �

�                                                                    �

�          'C' - Copy    'S' - Swap    All Others - Exit: [ ]        �

�                                                                    �

�                                                                    �

��������[ F1 - Help ]  [ C - Copy ]  [ S - Swap ]  [ Esc - Exit ]���Ľ

����������������������������������������������������������������������

The Move option will allow you to copy patches from one location

to another or to move patches by exchanging their locations.

<C> will copy over an existing patch.

Example:

Copy Patch From  [22]  To  [33]  !Overwrites!

<S> will swap locations of 2 patches.

Example:

Swap Patch From  [11]  To  [67]  !Exchanges!

Any other action will exit back to the Main Menu.

If you attempt to copy a patch to itself or to try to swap the

same patch locations, you will be told you can't do that.  Try again.

Page  14

E D I T   A   P A T C H

�����������������������

�������������������<<< Current Patch Bank Names >>>�����������������ķ

�                                                                    �

�                                                                    �

�11) Patch  11      31) Patch  31     51) Patch  51     71) Patch  71�

�12) Patch  12      32) Patch  32     52) Patch  52     72) Patch  72�

�13) Patch  13      33) Patch  33     53) Patch  53     73) Patch  73�

�14) Patch  14      34) Patch  34     54) Patch  54     74) Patch  74�

�15) Patch  15      35) Patch  35     55) Patch  55     75) Patch  75�

�16) Patch  16      36) Patch  36     56) Patch  56     76) Patch  76�

�17) Patch  17      37) Patch  37     57) Patch  57     77) Patch  77�

�18) Patch  18      38) Patch  38     58) Patch  58     78) Patch  78�

�21) Patch  21      41) Patch  41     61) Patch  61     81) Patch  81�

�22) Patch  22      42) Patch  42     62) Patch  62     82) Patch  82�

�23) Patch  23      43) Patch  43     63) Patch  63     83) Patch  83�

�24) Patch  24      44) Patch  44     64) Patch  64     84) Patch  84�

�25) Patch  25      45) Patch  45     65) Patch  65     85) Patch  85�

�26) Patch  26      46) Patch  46     66) Patch  66     86) Patch  86�

�27) Patch  27      47) Patch  47     67) Patch  67     87) Patch  87�

�28) Patch  28      48) Patch  48     68) Patch  68     88) Patch  88�

�                                                                    �

�                                                                    �

�                         Patch Number: [66]                         �

�                                                                    �

��[ F1 - Help ]  [ Enter The Patch Number You Want ]  [ Esc - Exit ]Ľ

����������������������������������������������������������������������

Most of the work you will do with this program will happen with

the Edit option.  This will allow you to change any parameter of the

DW6000 synthesizer.

The first step is to select a patch number to work with.  If you

select a patch that doesn't exist, you will be told of this.  You have

to remember that the DW6000's patch locations are as follows:

11 - 18, 21 - 28, 31 - 38, 41 - 48, 51 - 58, 61 - 68, 71 - 78,

81 - 88  ONLY!

Page  15

P A T C H   P A R A M E T E R   S C R E E N

�������������������������������������������

������������������<<< DW6000 Patch Parameter Screen >>>�����������������ķ

�                                                                        �

�                             <VCF> Options         <VCA> Envelope  0-31 �

�      Patch Name         31> Cutoff     0-63 10   51> Attack           0�

�      ~~~~~~~~~~         32> Resonance        0   52> Decay           31�

�     [Patch  66 ]        33> Kybd Track  0-2  1   53> Break Point     28�

�                         34> Polarity    1-2  1   54> Slope           28�

�    <Oscillator 1>       35> Intensity       19   55> Sustain          0�

�11> Octave      1-3  1   36> Chorus      0-1  1   56> Release          7�

�12> Waveform    1-8  1                                                  �

�13> Level      0-31 31      ��������������ͻ       <MG> Modulation 0-31 �

�                            � Korg DW 6000 �      61> Frequency Speed 31�

�                            ��������������ͼ      62> Delay Time       0�

�                                                  63> Osc (Vibrato)    2�

�    <Oscillator 2>        <VCF> Envelope  0-31    64> VCF (Wah-Wah)    1�

�21> Octave      1-3  1   41> Attack           6                         �

�22> Waveform    1-8  1   42> Decay           17     <Various Options>   �

�23> Level      0-31  0   43> Break Point     15   71> Pitch Bend 0-12  2�

�24> Interval    1-5  1   44> Slope           20   72> VCF Bend    0-1  0�

�25> Detune      0-6  3   45> Sustain         18   73> Portamento 0-31  0�

�26> Noise      0-31  0   46> Release          8   <-> Key Assign  1-3  1�

�                                                                        �

�                                                                        �

�                                                                        �

�[ F1-Help ] [ F2-Keys ] [ F5-Graph ] [ F9-Hear ] [F10-Save] [ Esc-Exit]Ľ

����������������������������������������������������������������������

This is the data entry screen for your patches.  This is the only

place where you can adjust any of the patch data.

Here's a breakdown of the function keys:

<F1> - Context sensitive help on ANY of the fields displayed on

the screen

<F2> - This will display the allowed keystrokes on this screen

<F5> - This will display your VCF and VCA envelopes if you have

a CGA or better color monitor.  If you don't have a color

monitor, the program will sneak a peek to find out what

type of monitor you have.  If incompatible, you will not

be able to display graphs.  The graphs are displayed in

CGA mode only.  The graph is updated every time the <F5>

function key is pressed.  This will rewrite the RAM

memory location with your work.

Page  16

P A T C H   P A R A M E T E R   S C R E E N

�������������������������������������������

<F9> - If you have your DW6000 hooked up to an amp or headphones

you will be able to listen to the sound you are working

on.  The program will display "Audition" in the center of

the screen.  A Middle "C" will play.  When you've had

enough, hit any kit to stop the DW6000 from playing.  The

sound is updated every time you press the <F9> function

key.  Patch 88 is temporarily overwritten and then placed

back correctly when audition is over.  This will rewrite

the RAM memory location with your work.

<F10> - This will rewrite the RAM memory location with your work

in progress.  This is a way to make sure your changes have

been kept.  This DOESN'T write the patch to disk.  You

have to Save the bank in order to do that.

<ESC> - This will stop this option and exit back to the Main Menu.

��>  If you enter values for fields that are invalid, an error

message will be displayed.  You will the need to change

that value before you can proceed.

����������������������������������������������������������������������

For further information pertaining to the parameters found on

this data entry screen, please refer to your DW6000 user's manual

or check out the context sensitive help for each field.  This will

get you through, but you really need to understand the parameters

themselves to get any production from the DW6000 synth.

Page  17

P A T C H   P A R A M E T E R   S C R E E N

�������������������������������������������

�������������������<<< DW6000 Patch Parameter Screen >>>����������������ķ

�                                                                        �

�                             <VCF> Options         <VCA> Envelope  0-31 �

�    ����������������������������[ Help ]��������������������������Ŀ   0�

�    �                         VCF Resonance                        �  31�

�    �                         -------------                        �  28�

�    �                                                              �  28�

�    � This parameter emphasizes the harmonics near the cutoff      �   0�

�11> � frequency, producing a characteristic peaky or bandpass      �   7�

�12> � type of sound.  The higher the value, the higher the         �    �

�13> � resonance peak and the more obvious the effect.  At or       �-31 �

�    � near the maximum value, 31, the VCFs go into self-oscill-    �d 31�

�    � ation producing a pure sine wave, which can be used as an    �   0�

�    � additional sound source for special effects.  The pitch of   �   2�

�    � the VCF tone is affected by the Cut Off, Keyboard Tracking,  �   1�

�21> � EG Int, and MG VCF parameters.                               �    �

�22> �                                                              �>   �

�23> �        No Effect 0 --> Self-oscillation (very peaky)         �2  2�

�24> �                                                              �1  0�

�25> �                                                     ESC=Exit �1  0�

�26> �                                                              �3  1�

�    ����������������������������������������������������������������    �

�                                                                        �

�                                                                        �

�[ F1-Help ] [ F2-Keys ] [ F5-Graph ] [ F9-Hear ] [F10-Save] [ Esc-Exit]Ľ

����������������������������������������������������������������������

This is an example of a context sensitive help screen pertaining

the the Filter Resonance field.  While in the help screen,  you may

be able to page up / page down or press <ESC> to exit back to the

field you were working on.

Page  18

P A T C H   P A R A M E T E R   S C R E E N

�������������������������������������������

����������������������<<< Editing Keys For Data Entry >>>������������������ķ

�                                                                           �

�                              Cursor Movement                              �

�                              ---------------                              �

�                  Ctrl-LeftArrow  - word left                              �

�                  Ctrl-RightArrow - word right                             �

�                  Home            - beginning of field                     �

�                  End             - end of line/field                      �

�                  Ctrl-Home       - beginning of first field               �

�                  Ctrl-End        - end of last field                      �

�                  Tab             - next field                             �

�                  Shift-Tab       - previous field                         �

�                                                                           �

�        Inserting and Deleting                   Other Editing Keys        �

�        ----------------------                   ------------------        �

�Ins             - toggle insert mode     Enter      - process current field�

�Del             - delete character       Ctrl-Enter - process all fields   �

�BackSpace       - delete character left  Ctrl-R     - restore field        �

�Ctrl-BackSpace  - delete word left       Esc        - abort data entry     �

�Ctrl-T          - delete word right                                        �

�Ctrl-U          - delete to end of field                                   �

�Ctrl-Y          - delete to end of form                                    �

�                                                                           �

�                                                                           �

���������������������������[ Hit Any Key To Exit ]�������������������������Ľ

����������������������������������������������������������������������

These are the keystroke combinations to maneuver around on the

data entry screen.

When you are ready to exit, press any key and you will then be

returned to the field you were working on.

Page  19

P R I N T   P A T C H E S

�������������������������

��������������<<< Print Patches From The Current Bank >>>�����������ķ

�                                                                    �

�                                                                    �

�11) Patch  67      31) Patch  31     51) Patch  51     71) Patch  71�

�12) Patch  12      32) Patch  32     52) Patch  52     72) Patch  72�

�13) Patch  13      33) Patch  22     53) Patch  53     73) Patch  73�

�14) Patch  14      34) Patch  34     54) Patch  54     74) Patch  74�

�15) Patch  15      35) Patch  35     55) Patch  55     75) Patch  75�

�16) Patch  16      36) Patch  36     56) Patch  56     76) Patch  76�

�17) Patch  17      37) Patch  37     57) Patch  57     77) Patch  77�

�18) Patch  18      38) Patch  38     58) Patch  58     78) Patch  78�

�21) Patch  21      41) Patch  41     61) Patch  61     81) Patch  81�

�22) Patch  22      42) Patch  42     62) Patch  62     82) Patch  82�

�23) Patch  23      43) Patch  43     63) Patch  63     83) Patch  83�

�24) Patch  24      44) Patch  44     64) Patch  64     84) Patch  84�

�25) Patch  25      45) Patch  45     65) Patch  65     85) Patch  85�

�26) Patch  26      46) Patch  46     66) Patch  66     86) Patch  86�

�27) Patch  27      47) Patch  47     67) Patch  11     87) Patch  87�

�28) Patch  28      48) Patch  48     68) Patch  68     88) Patch  88�

�                                                                    �

�                                                                    �

�                                                                    �

�                  Print Patches From  [22]  To  [33]                �

�                                                                    �

���������������������[ F1 - Help ]   [ Esc - Exit ]�����������������Ľ

����������������������������������������������������������������������

The Print option will allow you to print patches from the current

bank you have in memory.  You will have to enter a beginning patch

number and an ending patch number.  If you only want to print 1 patch

then enter the same number twice.

You will be prompted to set up your printer.  It doesn't matter

if your are printing to a parallel or serial printer.  It is up to

you though to redirect your output to the serial COM port you are to

use.  The program will not check for printer readiness either.

Once your printer is set up, hit any key and the patches will

then start to print.

��>  The patch charts print on a full page, one patch per page.

��>  You don't need to set up special fonts for the patch charts.

They do not print special ASCII characters, only the "-", "|",

and "*" characters for borders.

Page  20

P R I N T   B L A N K   C H A R T S

�����������������������������������

�������������������<<< Print Blank Patch Charts >>>�����������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�           Enter The Number Of Charts to Print (1 - 99): [10]       �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

���������������������[ F1 - Help ]   [ Esc - Exit ]�����������������Ľ

����������������������������������������������������������������������

The Charts option will allow you to print empty patch pages so

you can fill them in with parameters of your choice.  This could be

handy if someone has no computer but has some DW6000 patches to work

with.  It could be an archival system for someone who has no computer.

You will be prompted to set up your printer.  It doesn't matter

if your are printing to a parallel or serial printer.  It is up to

you though to redirect your output to the serial COM port you are to

use.  The program will not check for printer readiness either.

Once your printer is set up, hit any key and the patches will

then start to print.

��>  The blank patch charts print on a full page, one per page.

��>  You don't need to set up special fonts for the patch charts.

They do not print special ASCII characters, only the "-", "|",

and "*" characters for borders.

Page  21

Q U I T   D W 6 0 0 0

���������������������

����������������<<< Quit DW6000 Editor / Librarian >>>��������������ķ

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                    Quit The Program? [Y,n]:                        �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�                                                                    �

�������������[ F1 - Help ]  [ Y - Quit ]  [ N / ESC - Exit ]��������Ľ

����������������������������������������������������������������������

The Quit option will exit from the program.  You WILL NOT be

allowed to save your work in progress from here.  Please save the

current RAM bank your are working on before you quit.

<Y> will exit to the DOS prompt.

<N> will return to the Main Menu.

Page  22

D W 6 0 0 0   I N F O R M A T I O N

�����������������������������������

The Info option will just pop up a help screen and let you know

what the program is, how to contact me, and display credits to the

companies that helped make the DW6000 program possible.

And here we go!

����������������������������������������������������������������������

If you find this program useful and wish to purchase the full

featured version (if this the Demo version), a registration fee of

$25.00 (US) is required for the full featured version of the program.

An additional 7% New York State sales tax is needed as well, for a

total of $26.75.  (Oh well...) The registered program will allow you

to save to the DW6000.  This will also allow you to receive a free

upgrade, when the next one is written, scheduled for April 1992.  Any

comments on this program, possible enhancements, or just plain good

ideas should be made by writing to me or by calling this phone number

--- (518) 438-3656.  Be kind to my answering machine, it doesn't like

rowdy callers...

����������������������������������������������������������������������

E - M A I L :

�������������

You may also post E-Mail on GEnie under the name T.BARBER and I

will reply to all inquiries.

����������������������������������������������������������������������

C R E D I T S :

���������������

Basic Input /          Quick C Version 2.0

Output Routines:       (c) Microsoft Corp.  1987-1989

MIDI Hints And         Jim Conger 'C Programming For MIDI'

Suggestions:           (c) M & T Books  1988

Telephone: 1-800-533-4372

Telephone: 1-800-356-2002

Windows And Wonders!   CXL Library Version 5.2

(c) Innovative Data Concepts  1990

Telephone: 1-800-926-4551

����������������������������������������������������������������������

Page  23