QPV/386 video driver files

~~~~~~~~~~~~~~~~~~~~~~~~~~

If you have a video card which is not supported by QPV/386 (i.e. there is no

*.DRV file for your card), you can write a driver yourself.  You have to know

how the bank switching is done with your card, and you must know how to

program in assembler.

First, write a TASM source file named XXXX.ASM (instead of 'XXXX', choose

something descriptive for your VGA card).  It has to look like the

following:

----------------------------------------------

|         .286                               |

| Code    Segment Para 'Code'                |

|         Assume  cs:Code                    |

|         Org 100h                           |

|                                            |

| Procs   dw      Bank,Init,Exit,0           |

|                                            |

| Bank:   ...                                |

|         ...                                |

|         retf                               |

|                                            |

| Init:   ...                                |

|         ...                                |

|         retf                               |

|                                            |

| Exit:   ...                                |

|         ...                                |

|         retf                               |

|                                            |

| Code    Ends                               |

|         End Procs                          |

----------------------------------------------

If you want to use 386 instructions, don't forget to add USE16 to the

Segment statement.

IMPORTANT:  All routines (Bank, Init, and Exit) must end with 'retf'

(far return).  You may use the memory from [cs:0] up to [cs:0ffh] for

variables.  If you need more memory, you can reserve some using DB,

DW etc.  Note: you must initialize the memory!  It is NOT allowed to

use 'DB ?' or 'DW ?'!  Don't use the data segment, the contents of DS

are undefined and must not be changed.  Example:

Tmp     dw      0,1,2,3

...

mov     ax,cs:[Tmp]

The Init part is called after QPV/386 has switched into graphics mode.

The values for AX and BX which were used to switch to the current mode

are given in AX and BX, respectively.

In CX the number of bytes per line is stored.  You may modify it if

necessary.  If you don't want to modify it, you must save CX on the stack

if you're using it for other purposes.  The default value for CX is

taken from the CFG file; it's usually calculated like this:

16 color modes:  width/8 (for a single plane)

256 color modes:  width

32K color modes:  width*2

16M color modes:  width*3  or  width*4

This routine may modify any registers without saving them, exept CS, DS,

SS, SP, BP and CX (see above).  Most cards don't need an initialization

(because it is done by the BIOS at the mode switch), in this case 'retf'

is the only instruction.

The Exit part is called when the mode is not needed anymore, i.e. before

switching to another mode, or before switching back to text mode.

You must save CS, DS, SS, SP and BP if they're modified.

Again, most cards don't need any exit code, so there's usually only a

'retf'.

The Bank part is called every time a bank switch is necessary.

The bank number is given in AL.  The routine may modify AL and DX without

saving them.  If other registers are used, they must be saved on the

stack.

The bank switching routine needn't be very fast, since it's called only

when it's really necessary (not very often).

To make the driver file:

TASM /M XXXX.ASM

TLINK /T XXXX.OBJ

REN XXXX.COM XXXX.DRV

Then copy the driver file to the directory which holds QPV/386.EXE.

Finally you have to write a config file.  It's best to copy one of the

other config files and make the necessary changes.  The first line

should go like 'XXXX BGR' or 'XXXX RGB'.

For more information on the configuration files, please read CFG.DOC.