Using PkGOLD or KaGOLD with

with Multi-Mode controllers.

Introduction

Operating the PK-232, HK-232, or KAM in modes other than Packet

requires some special attention to parameters.  We recommend careful

reading of the manual or using the Online Help system prior to using

any operating mode (including packet) to reduce the possibility of

misunderstanding and frustration.

AMTOR Enhancements and Improvements!!!

In AMTOR, the [F7] means "ARQ selcal" and just like in packet mode. If

you put a SELCAL on the command entry line and hit [F7] the TNC

will be commanded to initiate an ARQ session. If there is no SELCAL

on the command line, [F7] brings up the familiar "Pick List" of

Quick Connect stations. Now, selecting one will initiate the ARQ

session with the associated SELCAL for that station.

In addition to simplifying routine ARQ sessions, you have the

advantage of seeing the CALLSIGN of the remote station, as well

as the Name if it was entered with [Alt-N].

The [F7] ARQ connect coupled with the QSL and Names

function, and the [ALT-F7] disconnect in AMTOR, you should find

that operating AMTOR ARQ should be a breeze.

To send CQ, simply start typing (or assign a CQ message to

an [Alt-0] to [Alt-9] key. Then use [ESC] to key down and start the

CQ in FEC.  During the transmission, use [ESC] to queue up an

Unkey command, and after sending the CQ, your station will return

to AMTOR receive, and be off the air.

There are a few more comments about AMTOR below, so read on.

Please explore this mode for HF operation.  Recent experiences by

several amateurs have demonstrated that AMTOR is very reliable when

HF packet fails.  For lower powered stations, AMTOR may be the HF

digital mode you will come to use most.

Switching Modes

To change to a non-packet mode, all that is absolutely required is to

type the mode name on the command/text entry line, and hit the CMD key [F10].

This will indeed shift the TNC into another operating mode.

Alternatively, use [F10] without a parameter name on the command line.

This will bring up a "pick list" of .TNC files. Highlight the TNC filename

corresponding to the desired mode and hit enter. You may use the TNC file

editor (in the ALT-S menu area) or any ASCII text editor to modify or

create TNC files with the parameters needed for each mode.

Parameters should be checked in the other modes to assure that the TNC

will operate as expected.  Some of these parameters will be suggested in

the text that follows.

PkGOLD and KaGOLD software works well in these other modes, but it can

be crippled by improperly set parameters.  If you are having difficulty with

non-packet modes, you should check the relevant parameters.  As an

alternative, if all else fails, RESET the TNC as follows:

AEA:

1.    Enter Direct Terminal mode using [Alt-F3].

2.    At the "cmd:" prompt, type "RESET" if using the AEA TNC,

or RESTORE D if using the Kantronics TNC, followed by the [Enter]

key. This will cause the TNC to clear all buffers, and

select default parameters. If this doesn't work, turn off

the TNC and turn it on again, and go to step 3.

3.    After the reset command, the tnc is waiting for you to

depress one or more asterisks (*) in order to "autobaud"

and establish a communications link with the computer.

Autobaud success is evidenced by the opening ROM

message naming the TNC, and ultimately presenting you

with another "cmd:" prompt.

4.    Leave the Direct Terminal Mode using [Alt-F3].

At this point, your callsign is entered as MYCALL, the

TNC is placed into HOST mode, and the Network View

screen will be presented.

5.    Hit the command key [F10] and select RESET to load

the RESET.tnc parameters, then use [F10] to select

STARTUP to load the STARTUP parameters.

Basic Operation:

The program requires characters to be "Echoed as Sent" to detect

proper operation of outgoing text. The appropriate parameter is

set by PkGOLD (EAS ON), and KaGOLD (XMITECHO ON).

All of the non-packet modes supported by PkGOLD and KaGOLD use

essentially the same keystrokes. This makes it easy to remember what to

do when using these other modes.  All modes use WORDOUT which means you can

edit a word that is misspelled as long as you do not hit a [space] key.

After the [space] key, the text is either sent immediately, or queued

up for transmission, and is no longer available for editing.

The [ESC] key is the transmit/receive TOGGLE.  This is an important

key to remember in non-packet modes. It operates by placing a toggle

in the outgoing text buffer to unkey (or in ARQ mode, to send the

changeover characters).

You type ahead, and what you type will go into a transmit buffer

and get sent when possible. If your station is not keyed down and

transmitting, the text is just queued up for sending.  If you have

already keyed your transmitter (hit the [ESC] key to transmit) the

text is sent and echoed to your screen as it is being transmitted.

AMTOR:    ARQ: In this mode, the [ESC] key is used as the changeover

character. The program inserts the changeover (+?) sequence

into the outgoing text buffer at the next position in

the outgoing text buffer.  In this way, you can "queue" up

a changeover by typing (or sending a file of) text and then

depressing the [ESC] key.

To END an ARQ session cleanly, type "R" (for Receive)

on the text entry line, and hit [F10] or use the

[Alt-F7] key to initiate a "disconnect" similar to

packet.

FEC:      The [ESC] key is used to Key down/up in this mode. It is

not a changeover sequence but rather, an indication that

the transmitter should key or unkey.  When AMTOR is showing

as the mode, and you type a few words or lines, then hit

[ESC] to key down, your text will be transmitted in FEC

mode.

Others:   In RTTY BAUDOT, ASCII, MORSE the meaning of the [ESC] toggle

is shown on the screen as a <<K>> (key down) or <<U>> (key

up pending) in the status line.  In AMTOR, the meaning

is similar, but there is no status indication. You will

have to use the [ESC] key carefully. When you hit it,

the changeover (or unkey in the case of FEC) is inserted

but not shown explicitly.

We have found that DIDDLE ON causes problems in ASCII mode

on some version of the AEA ROM. If you experience problems

in ASCII, set DIDDLE OFF.

Making contacts:

BAUDOT:   Enter the Baudot mode using the BAUDOT.TNC file or the

RTTY      by just typing BAUDOT [F10] for AEA users, or RTTY [F10]

for KAM users.

Set DIDDLE ON if you want to make it easy for other

stations to tune in your signal. DIDDLE will send

mark/space tones without actually affecting the remote

receiver/printer. The PK232 sets DIDDLE OFF by default,

meaning your station simply transmits a steady SPACE

tone when no characters are being transmitted.

Note: DIDDLE is not available on early AEA ROM releases.

Set the baud rate and shift using the "Fn" keys

according to the suggestions in the AEA manual.

Use (1) the "Brag" file facility or (2) the File transfer

facility for TEXT file transmission or (3) the Alt-N key

assignment, or (4) the cut/paste buffer editor as a

source for your CQ message or simply type your CQ

"on the fly" as you're keyed down.

Key down using [ESC], send your message or file, then

hit [ESC] again. The "unkey" command is queued up and

will take your system off the air when the buffer

is empty.

When another station sends text, you will see it on

your screen. You can "type ahead" any responses as you

think of them. They will not be sent until you hit the

[ESC] key to toggle the transmitter to the ON state.

When you have finished typing your transmission, hit the

[ESC] key. The text you are sending will finish transmitting

and your station will then unkey.

Usage of the [Alt-n] messages is a great idea for BAUDOT

and other modes as a place to store short lines, like

THANKS FOR NICE QSO, 73 73 73   SK DE WA4EGT WA4EGT KKKK

AMTOR:    You can send a CQ using FEC. When the AMTOR mode is shown,

and you are not "connected" to another station (not in ARQ)

you can type a CQ message, hit the [ESC] key to transmit

and your station will automatically go into FEC mode to send

your CQ.  Hit [ESC] a second time to unkey (you will see no

special indication that an "unkey" is in progress, but

it will occur after your text is transmitted).

If you include your SELCAL in your CQ, another station can

use ARQ to "connect" to your station.

To get to another station, type ARQ followed by the SELCAL

and hit [F10] key.

Use the [ESC] key to switch from Transmit to Receive.

To "disconnect" and turn off your transmitter, type

R [F10] or AMTOR [F10] (the former is preferred).

MORSE:    Set the morse speed and send a CQ. Hit [ESC] and your

station will key down, and start sending the CQ. Hit [ESC]

again and your station will unkey after sending the text.

SIAM:     If you hear a signal and want to "lock" on to it, use

(AEA)     the SIAM key [Alt-F5]. (only on AEA)

The SIAM key toggles to an "OK" key while the TNC analyzes

signals. If you agree with the analysis, a depression of

the SIAM key (now the OK key) will "lock into" that mode,

baud rate, etc.  If there is no analysis yet, hitting the

OK key will produce an error response, trying to tell you

that no mode has been guessed, so OK is premature.

Be sure to hit the CTRL key by itself when in NON-Packet modes. It will

display a line of commands that can be useful in your non-packet operation.

For example, the ARQ key can be used if you want to type in just the SELCAL

and then hit the ARQ function key combination. Or, you can type ARQ SELCAL

and hit [F10].  The function keys are "short cuts" for your use.

Using the Callsign/Name log in NON-Packet modes:

Some other keys of interest: The [Alt-C] key in all modes will allow

you to enter a callsign for the session.  For non-packet modes, since

callsigns are not necessarily "known" to the TNC, you have to enter them

manually.  Using [Alt-C] you can "force" a callsign to anything. At that

time, the program will lookup information it may have about that callsign

including the name of the remote operator.

If a callsign is showing, you may use the [Alt-N] key to add/edit the

name of the remote operator. This is helpful for future reference. You

can also edit the "QSL card" information using [Alt-E] once the callsign

is shown.

The [Alt-P] key shows the results of "previous modes" such as packet.

This can be useful if you wish to "cut/paste" text that you might have

received from a packet BBS into a RTTY contact.  When you leave a

non-packet mode (or the direct terminal mode) all text is sent to the

previous connects screen.  In this way, you can change modes and have

access to text received/sent in other modes.  You could receive an

ARRL bulletin, or a NAVTEX bulletin, switch to another mode, select

the text and send it using the new mode.

Summary:

To change modes, we suggest that you use TNC files rather than

just typing in the mode name and hitting [F10].  This will assure

that relevant parameters are properly set.

Differences exist between HF and VHF operation. You might create

a VHFAMTOR.TNC and HFAMTOR.TNC file, or VHFBAUDO.TNC and HFBAUDOT.TNC

files, etc., depending you what you do routinely.  This way, all that

you do is hit [F10], and pick the right TNC file from the "pick list"

presented to you. Look over the TNC files we've provided.

For CQ messages, you can edit the [Alt-N] key assignments and/or

setup .BRG files in the packet directory with your CQ message. You can

also use the TextOps function (F5) to edit a CQ message in the "cut/paste"

editor buffer. This message can be sent numerous times by simply

hitting [F5] and selecting "copy to current session."  Then hit [ESC]

to start transmitting, and when the text begins going out, hit [ESC]

again to queue up the unkey command.

A note about FAX

The only mode NOT supported by PkGOLD or KaGOLD is FAX, due in part

to the availability of more appropriate hardware and software for this mode.

The Black/White discrimination offered in the TNC does not support

the multi-level resolution inherent in many FAX transmissions (e.g. photographs).

We have tested other programs that offer 16-level resolution, using both

dithering and color substitution for the various levels, and have found the

resulting "pictures" to be excellent.

InterFlex Systems offers PC-HF Facsimile, an advanced version of the

program offered by AEA, at the same price. It comes with a demodulator, and

complete software, and will run on any IBM compatible machine. If you

are interested in high quality FAX software, contact InterFlex Systems

for the latest. It allows cropping images, generating GIF and PCX files,

unattended FAX reception, and more.