SCSIUtil - an utility for doing SCSI low level operations

SCSIUtil (c) 1991,92,93 by Gary Duncan and Heiko Rath

Gary Duncan

Philips PTS

23 Lakeside Dr

Tally-Ho Technology Park

Burwood East Vic 3151

Australia

EMail: gduncan@philips.oz.au

The Software Brewery

Heiko Rath

Raiffeisenstr.10a

D-6108 Weiterstadt    (after 1.July.1993: 64331 Weiterstadt)

Germany

EMail: hr@brewhr.swb.de

Phone: +49 (6150) 2607  (CET!)

BECAUSE THE PROGRAM IS LICENSED FREE OF CHARGE, THERE IS NO WARRANTY

FOR THE PROGRAM, TO THE EXTENT PERMITTED BY APPLICABLE LAW.  EXCEPT

WHEN OTHERWISE STATED IN WRITING THE COPYRIGHT HOLDERS AND/OR OTHER

PARTIES PROVIDE THE PROGRAM "AS IS" WITHOUT WARRANTY OF ANY KIND,

EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE.  THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE

PROGRAM IS WITH YOU.  SHOULD THE PROGRAM PROVE DEFECTIVE, YOU ASSUME

THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

IN NO EVENT UNLESS REQUIRED BY APPLICABLE LAW OR AGREED TO IN WRITING

WILL ANY COPYRIGHT HOLDER, OR ANY OTHER PARTY WHO MAY MODIFY AND/OR

REDISTRIBUTE THE PROGRAM AS PERMITTED ABOVE, BE LIABLE TO YOU FOR

DAMAGES, INCLUDING ANY GENERAL, SPECIAL, INCIDENTAL OR CONSEQUENTIAL

DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THE PROGRAM

(INCLUDING BUT NOT LIMITED TO LOSS OF DATA OR DATA BEING RENDERED

INACCURATE OR LOSSES SUSTAINED BY YOU OR THIRD PARTIES OR A FAILURE

OF THE PROGRAM TO OPERATE WITH ANY OTHER PROGRAMS), EVEN IF SUCH

HOLDER OR OTHER PARTY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES.

The source is provided, therefore you can make changes yourself. The

only thing I'm asking is that if you do make changes, please get in

touch with me (hr@brewhr.swb.de), so that we can coordinate any

further development.

What is SCSIUtil?

It is a little utility to do some low level stuff with SCSI devices.

As I currently don't have the time to work on it (I'm working on my

thesis) I release it to the public as it is. It is far from being

perfect, but it is useful to me. In it's current incarnation it has

the following features:

- read the capacity of a SCSI device

- read 16 bit raw digital audio off an audio CD (works with Sony CDU

561 & 8003) (left/right channel and stereo reading supported)

- read 16 bit raw digital audio off an audio CD and convert to 8 bit

(again left/right channel and stereo reading supported)

- eject/insert a medium via software control

- display the Inquiry data of a drive

- read & interpret a CD-ROM data block address header

- start/stop the motor on a device

- read pages via MODE SENSE

- play audio on a CD-ROM drive (tested with Sony CDU 561 = Apple CD-300)

- read sectors

- seek to a specified sector

- display table of contents (TOC) of a CD

- read subchannel information of a CD (tested with Sony CDU 561)

Why SCSIUtil?

I needed some software to explore my new CD-ROM drive.

Here is what SCSIUtil displays, if you invoke it without parameters:

SCSIutil V1.815beta [Jun 19 1993 : 21:10:56] - written by Gary Duncan

additional features by Heiko Rath, hr@brewhr.swb.de

Usage: SCSIutil [-dscsi_dev] <scsi_id> <command>

-c[r]               : Read capacity [raw]

-d<l|r|s> sec blks  : Read 16 bit raw digital audio(start sector/# blocks)(2)

-D<l|r> sec blks    : Read  8 bit raw digital audio(left or right channel)(3)

-e <0|1>            : Change medium (0=eject, 1=load)

-i[r]               : Inquiry [raw]

-h[r] blk           : Read CD-ROM data block address header

-m <0|1>            : Stop/Start motor {0=stop, 1=start}

-o[r] contr page    : Mode sense (contr = 0-3))

-p st si et ei      : Play audio CD track (1-99), index (1-99)

-r[t] sec_no        : Read sectors [use trackdisk.device]

-s sec_no           : Seek to sector (5)

-t[r|l]             : Display TOC of an audio CD [raw|long]

-u[r] chan fmt track: Read CD sub-channel information [raw] (6)

Note 1: usually scsi_id = (BOARD * 100) + (LUN * 10) + SCSI_TARGET_ID

2: with 's' returns LRLRLR pairs of stereo audio, 2352 bytes per block

3: converted to 8 bit audio (-d and -D work with Sony CDU 561 & 8003)

4: contr 0: current, 1: changeable, 2: default, 3: saved values

5: to park heads, try sec_no of -1

6: Q-channel = 64, fmt: 0=Sub-Q Channel data,1=current CD-ROM pos.,

2=Media Catalog Number (UPC/Bar Code),3=Track ISRC

Documentation on the options of SCSIUtil:

-c[r]               : Read capacity [raw]

This will issue a READ CAPACITY to the device to get the

highest sector available, the sector size and the capacity

(= sector size * max sector). With the 'r' option SCSIUtil

will not try to interpret the returned data.

BTW, the Apple CD-300 returns different sector sizes,

depending on the last read size (that means sometimes it is

96, 2048 or 2352).

-d<l|r|s> sec blks  : Read 16 bit raw digital audio(start sector/#blocks)(*2*)

This will read data directly from the specified CD-ROM drive,

interpreting the information as raw digital input. A value for

the sector to start reading from is required as well as the

number of sectors to read. The data will be in 16-bit format

e.g.  SCSIutil 2 -ds 2 100 Reads the first 100 blocks starting

from sector 2 as Stereo 16 bit data. One sector corresponds to

1/75 of a second.  The data will be output to stdout.  This

works with the Sony CDU-541/561 (Apple uses the CDU-561 in

their Apple CD-300).

The digital audio data sent to stdout looks like this:

struct cdda_sector {

struct cdda_sample {

char left_channel_lsb;

char left_channel_msb;

char right_channel_lsb;

char right_channel_msb;

} samples[588];

};

With mono, either the left or right channel is skipped.

BEWARE: sometimes I get SCSI bus lockups while using this

command. I'm pretty sure that it's not SCSIUtil's fault, but

either a problem of the WD SCSI chip 33C93A 00-04-PROTO in my

Amiga 3000 or a firmware bug in the Apple CD-300 (my firmware

is release 1.8d). The lockups occur only if I read more than

about 150 sectors.

-D<l|r> sec blks    : Read  8 bit raw digital audio(left or right channel)(*3*)

Does basically the same as the '-d' option, only divides every

16 bit word by 256 before sending it to stdout, which makes

for a cheap (and suboptimal) conversion to 8 bit. The warning

about SCSI bus lockups applies also to this command.

-e <0|1>            : Change medium (0=eject, 1=load)

By using this command you can eject a removable medium via

software, e.g. SCSIutil 2 -e 0 ejects the CD-ROM currently in

the drive with SCSI id 2.

-i[r]               : Inquiry [raw]

This command sends the INQUIRY command to the specified

device. This can be used to get all kinds of information about

the device. With the 'r' parameter SCSIUtil will not try to

interpret the information returned.

-h[r] blk           : Read CD-ROM data block address header

This will read the address header information of a particular

CD-ROM block, e.g. SCSIutil 2 -h 100 reads block 100 from SCSI

device 2 and returns the header information.

-m <0|1>            : Stop/Start motor {0=stop, 1=start}

Start or stop the device motor of the specified device.

SCSIutil 2 -m 0 stops a CD that is playing in SCSI device 2.

-o[r] contr page    : Mode sense (contr = 0-3)) (*4*)

Reads MODE SENSE control pages from a SCSI device. The contr

parameter decides what pages to return:

contr 0: current values

1: changeable values (not interpreted correctly by SCSIUtil)

2: default values

3: saved values

The page value must be in the range of 0-63. Page 63 usually

returns all the available MODE SENSE pages.  With the 'r'

option SCSIUtil will not interpret the returned data.

-p st si et ei      : Play audio CD track (1-99), index (1-99)

Allows the playing of a series of audio tracks. Can use index

marks as well e.g. SCSIutil 2 -p 1 1 8 1 plays an Audio CD in

SCSI device 5 from Track 1 to Track 8.

Track and Index should be in the range of 1-99 (according to

the ANSI SCSI 2 specs). The Apple CD-300 accepts an index of 0

as play whole track, therefore I allowed the selection of 0 as

an index, even though the online help states otherwise. To

play a whole CD type: scsiutil 2 -p 1 0 99 0. To play only

track 3: scsiutil 2 -p 3 1 3 1.

-r[t] sec_no        : Read sectors [use trackdisk.device]

Read data sector off a SCSI device. In case of an audio CD use

the -d or -D commands to read digital audio data.

-s sec_no           : Seek to sector (*5*)

Seek the head of a device to the specified sector.

-t[r|l]             : Display TOC of an audio CD [raw|long]

Reads the table of contents of a CD. The TOC can contain

entries for data/audio tracks.  With the 'r' option SCSIUtil

won't interpret the returned data. With the 'l' option

SCSIUtil will display the TOC in a longer form.

-u[r] chan fmt track: Read CD sub-channel information [raw] (*6*)

Reads information from the specified sub-channel of a CD.

Chan = 64 = Q-Sub-channel

Fmt 0: read Q-sub-channel

Doesn't work with the Apple CD-300 drive

Fmt 1: read current position data

Track must be 0

Fmt 2: read media catalog number (UPC/Bar Code)

Track must be 0

Fmt 3: read track international standard recording code (ISRC)

Track must be a valid track

With the 'r' option SCSIUtil doesn't try to interpret the

returned data.

The future:

There are several things that I'd like to implement, take a look at

the TODO file. If you have a need for some additional features you

can always try to talk me into implementing them (hint: I can be

bribed ;-)

Acknowledgements:

Fred Fish for his work for the Amiga community

Gary Duncan (gduncan@philips.oz.au) for having written the original

version and releasing the source to the public

Dave Hassal (dave@ainfo.equinox.gen.nz) for bugging me with questions

and pushing me into finishing SCSIUtil.

Ralph Babel for some hints/tips/tricks.

Franz-Josef Reichert for his Jukebox.

Free Software Foundation, Inc. for the no warranty text and use at

your own risk texts. Also thanks for some high quality SW.

My fellow Software Brewers (Christian Balzer & Armin Sparr) for

support.