Memory Usage by FORTH         910418  J.Bos Brantford Canada

a3=bp  base pointer

a4=ip  inner interpreter pointer

a5=sp  stack pointer (parameter stack = user stack = forth stack = stack )

a7=rp  return stack pointer ( return stack = system stack = Amiga stack )

d5=os  outside register, high word must be set to zero 0000

d6=w   work register, high word must be set to zero 0000

: next,  bp ) jmp, ; ( next, jumps to what FORTH thinks is address 0000 )

addr  contents      label

0000   3c1c        >next<      move.w (a4)+,d6      ip )+ w move,

0002   3a33  6800              move.w 0(a3,d6),d5   0 w bp di.l) os move,

0006   4ef3  5800              jmp    0(a3,d5)      0 os bp di.l) jmp,

000a   0000  0000  incomingSP

000e   4928 <-     init-forth    initial pointer to top in FORTH voc.

0010   f736 <      init-user     sp0     parameter stack address

0012   f796 <                    r0      return stack address, not used

0014   f736 <                    tib     terminal input buffer address

0016   001f <                    width   name field width in bytes

0018   0001 <                    warning error warning mode

001a   0000 <      init-fence

001c   0000 <      init-dp       initial directory pointer

001e   0000 <      init-voc-link initial vocabulary link

0020   137c <      <-find>

0022   0710 <      <?terminal>

0024   1446 <      <abort>

0026   145e <      <block>

0028   14fe <      <cr>

002a   077c <      <exit>

002c   152e <      <expect>

002e   1654 <      <interpret>

0030   06ec <      <key>

0032   16ba <      <load>

0034   1722 <      <number>

0036   32b6 <      <page>

0038   17b6 <      <r/w>

003a   0736 <      <type>

003c   1848 <      <vocabulary79>

003e   1904 <      <word>

0040   f796        up      (user pointer )

0042   f796        rpp     (return stack pointer, not used)

----------------

0044   88 execbase     start of dictionary (first word)

|         |

|         |

|         |

|      dictionary

|         |

|         |

|         |

|         v

|               <--  last entry in dictionary

|

|        pad  is located 64 bytes above the latest

|         |                  entry in the dictionary

|         v

|

|

|

|

|

|                   <-- sp@  top of parameter stack

|         ^

|         |

|           parameter stack

|

f736 ---------------- s0  bottom of empty parameter stack

|           tib      starts at tib @ = F736

|         |

|         v

|                    60 hex bytes

|                    available for tib

|

|         ^

|         |

|           return stack  ( not used, system stack is used )

| -----------------

f796   6520     0     init-r0 = f796  bottom return stack

f798   2020     2         rpp ??  return pointer (not used)

f79a   3362     4     forth directory pointer

--------------------------------------

f79c   f736 >   6     sp0         |

f79e   f796 >   8     r0          |

f7a0   f736 >  10     tib         |

f7a2     1f >  12     width       |

f7a4      1 >  14     warning     |

f7a6   2dd2 >  16     fence       |

f7a8   40d8 >  18     dp          |

f7aa   3c74 >  20     voc-link    |

f7ac   137c >  22     '-find      |

f7ae    710 >  24     '?terminal  |

f7b0   1446 >  26     'abort      |

f7b2   145e >  28     'block      |

f7b4   14fe >  30     'cr         |

f7b6    77c >  32     'emit       |

f7b8   152e >  34     'expect     |   24 words are copied and saved

f7ba   1654 >  36     'interpret  |      for use at the next session

f7bc    6ec >  38     'key        |      by 'freeze'

f7be   16ba >  40     'load       |

f7c0   1722 >  42     'number     |

f7c2   17a8 >  44     'page       |

f7c4   17b6 >  46     'r/w        |

f7c6    736 >  48     'type       |

f7c8   1848 >  50     'vocabulary |

f7ca   1904 >  52     'word       |

--------------------------------------

f7cc     11    54     >in             52 hex bytes = 41 user variables

f7ce     10    56     base

f7d0      0    58     blk

f7d2   23be    60     context

f7d4   f734    62     csp

f7d6   23be    64     current

f7d8   ffff    66     dpl

f7da      0    68     fld

f7dc   411a    70     hld

f7de      0    72     offset

f7e0     2b    74     out

f7e2   8747    76     r#

f7e4   4658    78     scr

f7e6      0    80     state

f7e8   7fff    82     first     start of buf1    end of user variables

|

|

|            404 hex bytes for block buffer buf1

|

|

fbec                  starting address of buf2

|

|

|            404 hex bytes for block buffer buf2

|

|

fff0                  limit     ( end of memory )