KISS Proms for use with TheNode.

Several PROM images are supplied for for use with TNC2 (or clones), and one

for use with the TNC220.

KISS is as released with the TCPIP package. I have used this code, and

it seems to work, but it does have loopholes in its buffer management. As

I have experienced buffering problems with other KISS mode TNCs with TheNode,

i've done a version which will reset if it runs out. This is a bit drastic,

but should keep the system going. (Higher level software will retry the

discarded messages). If it improves things, I'll refine it to discard the

oldest. The new eprom image is in the file JKISS.

220KISS is a version of JKISS, modified to run on the TNC220. This version

only supports the VHF port (port 2) at 1200 baud,and the aync link to the PC

is fixed at 2400. The DCD led is driven by software, but is controlled by the

DCD signal from the modem (ie DCD processing is the same as with the TNC2 -

the SOFTDCD mode of the 220 is not implemented). Other versions are possible

if there is sufficient interest - the main problem is that the KISS command

set would have to be extended to include commands for port and speed switching.

Note the software is now set up to run with the clock speed jumper in the

'Low' speed position - several people have had problems running in the 'Fast'

mode.

BPQKISS - A Multidropped KISS system. (TNC2 and clones only)

I have implemented a system to allow more than one KISS-like TNC to connect

to a single Async port. This is primarily for those running machines with

little expansion capability, but can also enable the TNCs and transceivers

to be located remotely from the PC with a simple 3 wire link. This could be

useful on the lower frequencies, where QRM from the PC blocks the receiver

( I have real problems running a 50meg RX near the PC).  A simple checksum

is also added to each packet, to reduce the risk of corruption if running on

long leads (or even over a modem link).

The system uses polling to prevent contention on the link. Each TNC must

run the BPQKISS program, and each must have a different 'address' byte

patched in at location 20hex. In the PORTS section of BPQCFG.TXT, PROTOCOL

must be set to KISS, KISSOPTIONS to POLLED,CHECKSUM,ACKMODE and CHANNEL

set to correspond to the address in the PROM -

CHANNEL		Address (in byte 20h of PROM)

A                00h

B                10h

C                20h

D                30h      etc

In theory you can have up to 16, but in practice the maximum will depend

on the power of the PC and the speed of the radio ports.

Wiring.

PC		TNC 1		TNC 2

GROUND	7 ---------------7----------------7-----------  etc

TXD	2 ---------------2----------------2-----------  etc

RXD	3 --------------------------------------------  etc

|		  |

-		  -

^		  ^

|		  |

3		  3

-

^   is a diode (1N914 or similar)

With some TNCs and serial cards, a pulldown resistor may be required

from pin 3 on the PC (10k to -12v is suggested). Thanks to G3ZFJ for this

information.

The protocol used for this multidropped option was changed from version 3.59a

onwards to be compatible with similar software produced by KANTRONICS for their

range of TNCs. The new version is called BPQKISS, and replaces the old JKISSP.

For those of you unfamiliar with KISS TNCs, the STA led indicates frames

being received from the PC and the CON led frames being sent to the PC. On

powerup, some LEDS should flash about 3 times - which ones depends on the

version and the RAM size in the TNC.

CWID

I have added CWID to my JKISS and BPQKISS EPROMs. The Callsign to be sent

is patched into the EPROM image with the program PATCHID, which takes two

parameters, the required callsign, and the file to patch (either JKISS or

BPQKISS). Note that the specified file is overwritten, so I suggest you

make a copy of the original first.

PATCHID G8BPQ JKISS    - Note call must be in upper case

The CW patttern which will be sent is displayed on the screen - please

check it, just in case my translate table is wrong!

The ID is sent after one minute, then at 29 minute intervals, with a dot

length of 60ms. If my calculations are correct, this equates to 20 WPM

The ID is send in AFSK (assuming a normal AFSK modem), but because of the

hardware design it is not possible to control which tone corresponds to

mark or space - it depends on what was sent just before the ID starts.

If you are using a modem with a scrambler (eg G3RUH), then the system

wont work - the two tones will sound the same. I'd like to know if a

CWID facility for RUH modems would be useful - if there is a significant

demand I'll see if I can find a solution. A simple on-off keying may be

possible, but would depend on the PTT characteristics of the TX. If you

have a better idea, please let me know!

The normal SLOTTIME/PERSISTENCE code is used to minimise collisions with

other stations (hence the interval of 29 mins, allowing a bit of time for

congestion, without going over the statutory 30 mins interval).

John Wiseman, G8BPQ

7 March 1991

##################