TheNet X-1J Release 2

Configuration instructions

1. Introduction

This  document  describes the build process for  creating  a ROM image for TheNet X-1J release 2. This process differs from the previous versions of TheNet-X in that it is delivered as two files rather than three. This is in response to a number of requests for a simpler process. In addition the patcher has been considerably changed and utilities for hex conversion are included. A routine to print out passwords in a 'user friendly' format is included together with a utility to change the help text in THENET2.X1J.

BEWARE ! The files carry the same names as TheNet X-1J ( original release ). Be sure to use the correct set of files !.

2.   Files.

The ROM image comes as two files,

THENET1.X1J

THENET2.X1J

These two files are loaded into memory as described below.

Before loading them however, both should be configured as described in section 3.

In addition, the following files are also provided :

PATCH.EXE

INTEL.EXE

MOTOROLA.EXE

SETHELP.EXE

PRINTPWD.EXE

INTEL.C

MOTOROLA.C

SETHELP.C

PRINTPWD.C

PATCH.EXE is the windowing patch utility for the ROM images. INTEL.EXE and MOTOROLA.EXE are utilities that are designed to convert binary files into hex notation, in the Intel Intellec and Motorola S formats. SETHELP changes the help screen and PRINTPWD does the pretty printing by extracting the password from a parameter save file created by PATCH.EXE.

The ROM image consists of two halves, one for the lower half of a 512K EPROM, and one for the upper half. The files are loaded as shown :

FILENAME           Load starting at hex

=========================================

THENET1.X1J        0000

THENET2.X1J        8000

No information on how to load the files into a programmer is presented as all are different. Typical scenarios are however given in section 5.

3. Configuration

Each of the two halves of the ROM image contains two different parts, a common set of drivers & interrupt routines and part of the functionality of the node. Part 1 contains the level 2, 3 and 4 software. Part 2 contains the switch. Each must be patched in an identical way to reflect the desired operation as each part contains an identical section at the start of the file for configuration data. This patching may be done manually or it may be done with the patcher.

The first part of the ROM images is identical to TheNet 1.01 in its   configuration.  These  parameters  are   followed   by additional ones for the extended version as shown :

HEX       FIELD

LOCN      SIZE      DESCRIPTION

===========================================================

003B      6 BYTES   CALLSIGN OF THE NODE

0041      BYTE      SSID OF THE NODE CALLSIGN

0042      6 BYTES   ALIAS OF THE NODE

004A      WORD      MIN AUTO UPDATE QUALITY

004C      WORD      HDLC DEFAULT ROUTE QUALITY

004E      WORD      DEFAULT RS232 ROUTE QUALITY

0050      WORD      INITIAL NODE LIFETIME

0052      WORD      MIN LIFETIME TO BROADCAST

0054      WORD      BROADCAST INTERVAL IN SECONDS

0056      WORD      LEVEL 3 TIME-TO-LIVE INITIALISER

0058      WORD      LEVEL 4 TIMEOUT IN SECONDS

005A      WORD      LEVEL 4 RETRIES

005E      WORD      LEVEL 4 ACK DELAY IN SECONDS

0060      WORD      LEVEL 4 WINDOW SIZE

0062      WORD      NUMBER OF BUFFERED FRAMES PER CONNECTION

0064      WORD      NO ACTIVITY TIMEOUT IN SECONDS

0066      WORD      PERSISTENCE

0068      WORD      SLOT TIME IN TENS OF MILLISECONDS

006A      WORD      LEVEL 2 INITIAL T1 COUNTER IN SECONDS

006C      WORD      LEVEL 2 WINDOW SIZE

006E      WORD      LEVEL 2 RETRIES

0070      WORD      LEVEL 2 INITIAL T2 COUNTER

0072      WORD      LEVEL 2 INITIAL T3 COUNTER

0074      WORD      LEVEL 2 DIGIPEAT ENABLE FLAG

0 = DISABLED, 1 = ENABLED

0076      WORD      CALLSIGN VALIDATION, 0 = OFF, 1 = ON

0078      WORD      BEACON MODE, 0 = OFF, 1 = AFTER TRAFFIC,

2 = ALWAYS

007A      WORD      CQ ENABLE FLAG, 0 = DISABLED, 1 = ENABLED

007C      BYTE      FULL DUPLEX FLAG, 0=SIMPLEX, 1 = DUPLEX

007D      BYTE      SEND FLAGS IF NO DATA NEEDED, 1 = YES

007E      BYTE      COMMAND LEAD-IN CHARACTER ( ESCAPE )

007F      BYTE      TRANSMIT KEY-UP DELAY, 10's OF MILLISECS

0080      80 BYTES  DEFAULT PASSWORD

00D0      BYTE      NULL BYTE AT END OF PASSWORD

00D1      80 BYTES  INFORMATION MESSAGE

0121      BYTE      NULL AT END OF INFORMATION STRING

0122      WORD      CW REPEAT RATE IN SECONDS. 0 DISABLES

0124      BYTE      CW BIT SPEED IN 10's OF MILLISECONDS

6 = 20 WPM

0125      BYTE      DEFAULT HOST MODE. 0 = NORMAL

1 = HARDWARE HANDSHAKE CONNECT CONTROL

0126      BYTE      CROSSLINK PROTOCOL MODE CONTROL

0 = TheNet NORMAL CROSSLINK PROTOCOL

1 = USE KISS INSTEAD OF CROSSLINK

2 = AS PER 1, ALSO COPY NON NODE PKTS

3 = AS PER 2 BUT COPY ALL PACKETS

0127      BYTE      MHEARD LIST LENGTH. 0 = OFF, MAX = 100

0128      BYTE      NODE BROADCAST CONTROL. 0 = NO BCAST

1 = HDLC PORT ONLY, 2 = RS232 PORT ONLY

3 = BOTH PORTS

0129      WORD      RS232 NODE BROADCAST INTERVAL ( SECS )

0 DISABLES

012B      BYTE      NODE BROADCAST ALGORITHM CONTROL.

BIT 0 = HDLC, BIT 1 = RS232

WHEN BIT SET, IMPLEMENT VARIANT ALGO.

012C      8 BYTES   OPTIONAL BEACON DIGI LIST, NULL TERM.

0134      WORD      DEFAULT BEACON INTERVAL IN SECONDS

0136      BYTE      CONNECT REDIRECTION, 0=HOST 1=BBS 2=DXC

0137      BYTE      HASH NODE CONTROL. Each bit controls whether

nodes whose alias starts with a '#' are included in node broadcasts on a specific port. Bit 0 determines port 0 ( the radio ), bit 1 controls the RS232 port. If a bit is set, hash nodes are not broadcast.

0138      4 BYTES   THIS IS THE NODE'S AMPRNET ADDRESS.

It is a numeric address of 4 bytes. Each byte corresponds to one byte of the address, for example if the address is 44.131.16.31, then the data stored at each of the bytes 138, 139, 13A and 13B respectively will be 1F, 10, 83 and 2C. Contact your local co-ordinator for an address.

013C      4 BYTES   THIS IS THE AMPRNET ADDRESS USED BY THE NODE TO

RECOGNISE BROADCASTS. The data is stored in the same way as for the node's address ( as shown above ). A typical address would be 44.131.0.0 for the UK.

0140      BYTE      IP PORT MODE CONTROL.

This byte controls the default modes used for AX.25 frames on each port. It is bit mapped, with bit 0 controlling the radio port and bit 1 controlling the RS232 port. If a bit is set, the default mode for that channel is datagram ( UI frame ), if not  it is virtual circuit.

0141      BYTE      IP INITIAL TIME TO LIVE

0142      BYTE      IP ENABLE FLAG.

If zero, the IP router is disabled.

If not zero the IP router is operational.

0143      BYTE      HELP MESSAGES CONTROL BYTE.

Each bit enables or disables a different type of help message as follows :

Bit 0 - 'trying to connect' message

Bit 1 - sysop sees all commands in help

Bit 2 - give a 'good-bye' message to users

Bit 3 - enables the connect text message

Bit 4 - show nodes as alias:callsign

Bit 5 - pass 8 bit data in TALK if set

Bit 6 - Make node alias handling case sensitive

Bit 7 - Enable TexNet interface handler

0144      WORD	  MTU_IP0

This is the MTU for port 0 Level 2 AX.25 ( the radio port ) for the IP router.

0146      WORD	  MTU_IP1

This is the MTU for port 1 Level 2 AX.25 ( the RS232 port ) for the IP router.

0148      WORD       MTU_IPN

This is the MTU for the Net/Rom subnetwork layer. It should NOT exceed 236 for compatibility with Net/Rom

014A      WORD	  MTU_I_MAX

This is the maximum number of data bytes, plus one, that will be accepted in a received L2 AX.25 packet. Above this will cause an error.

014C      WORD	  MTU_L2_MAX

This is the absolute limit on the number of bytes in a received AX.25 packet, counting the data, control, and address information. It is set to 328 usually ( 256 data bytes, 2 control and 70 address ).

014E      BYTE	  Auto reconnect to node on remote dis if 1

014F      BYTE	  NoSlime -

Bit 0, if set, stops 'slime trails' being displayed in the nodes table. Bit 1, if set, stops slime trails being accepted by the node.

0150      BYTE       Bit 0 if set bars digi L2 connects to the node.

Bit 1 if set bars digi downlinks from the node.

0151      BYTE       DEVIATION METER DEFAULT.

This is the default value for the RX Deviation meter.

0152      WORD       METER FLAGS. Each bit controls a meter function

0154      BYTE       S METER SIGNAL MINIMUM This parameter is the

ADC noise floor reading ( 0 to 255 )

0155      BYTE       S METER MULTIPLIER This is used to scale

readings to give the S1-S9 style display

0156      BYTE       DBM MULTIPLIER This is used to scale readings

to give the dBm style display

0157      BYTE       DBM NOISE FLOOR This is the reading that

corresponds to no signal on calibration

0158      BYTE       This is the voltmeter channel 1 scaling factor

0159      BYTE       This is the voltmeter channel 2 scaling factor

015A      BYTE       This is the voltmeter channel 1 offset value

015B      BYTE       This is the voltmeter channel 2 offset value

The patch utility will not assist in changing the help text. That text is positioned at the end of THENET2.X1J. It is a null terminated string of characters. Newlines are represented by the value 0xd ( decimal 13 ). It can be as long or as short as you like, but don't forget that it causes the node to be a source of data and if very long could crash the node. ( Not likely in this version given the space available!). The easiest way to change it is to use the SETHELP utility.

Any problems, give me a ring !

4. The PATCH utility

The patch utility is designed to help configure the two ROM images in a manner that is not as user hostile as hand crafting a binary image. It is invoked as follows :

PATCH   [  file1 file2 ]

If no parameters are given, it will look for files THENET1.X1J and THENET2.X1J in the current directory. It will stop if it cannot load them. If the images are renamed, they may be given as parameters. If this is done, both files must be given, with file1 corresponding to part 1 and file2 corresponding to part 2. The program is menu driven, with extensive help provided on the operation of the program and each parameter. It also tries to make sure that only valid data is entered.

The program may also be instructed to load and save textual representations of the parameters. These consist of ASCII files, with one parameter per line. Each parameter consists of the name of the parameter, and equals sign, and the value for that parameter. The values are mainly numeric, with the obvious exceptions of things like the callsign, alias, password, info message etc. To get an example of the format, use the patcher to dump a file and look at it. The idea of this is not simply to load and dump whole images, but to load partial configurations such as passwords & info messages only or parameters only. The file may be edited to remove or add lines as desired. Note that each parameter MUST only occupy one line. For the information message, whitespace before the first printable character is ignored by the program, and if a newline is to be included, it is denoted by the sequence \m ( i.e. backslash followed by the letter m ). Similarly, to include the backslash character itself, a double backslash must be entered, i.e. \\.

5. Programming examples

There are two utilities included to facilitate conversion to hex for use in programming eproms. The source of both is also included if anyone wants a different file type. The programs have been compiled with Turbo C++.

Each has the same method of invocation,

INTEL infile outfile [ address ]

or

MOTOROLA infile outfile [ address ]

These create INTELLEC or S1 type records respectively. Each reads an input binary file and outputs a hex version. The starting address assumed for the file will be 0000 unless specified otherwise in the command line.

5.1 Intel format, loading as two halves

1. Use the patch program to create the desired image.

2. execute :

INTEL TheNet1.x1j part1

INTEL TheNet2.x1j part2

3. load part1 into the programmer and program the lower half of the ROM.

load part2 into the upper half.

5.2 Motorola format, loading as one image

1. Use the patch program to create the desired image.

2. Execute :

MOTOROLA TheNet1.x1j part1

MOTOROLA TheNet2.x1j part2 8000

COPY part1+part2 romimage

3. Edit romimage with a text editor to remove the intermediate end of file ( S0... ) marker.

4. Load romimage into the EPROM in one go & program it.

6. Acknowledgements

Intel and Intellec are trademarks of the Intel corporation

Motorola is a trade mark of the Motorola company.

My thanks to KH6ILT for the bug fixes to MOTOROLA.C and to John Bednar WB3ESS for the SETHELP utility ( originally named HELPX1J ).