THENET X1-J QUICK REFERENCE GUIDE

==================================================================

SWITCH COMMANDS

ACL       [{CALLSIGN + VALUE} | {CALLSIGN - VALUE} |

{ * VALUE } | { & value }]

ADC

ADC1      [ * | Units_for_Voltmeter_channel_1 ] (see note 6)

ADC2      [ * | Units_for_voltmeter_channel_2 ] (see note 6)

ALIAS     [ * | NEW_ALIAS ]

ARP       [ipaddr [{ - ptcl } | [+ [P] ptcl callsign [ DG | VC ]]]

AUDIT     [ NUMBER_FROM_0_TO_255 ]

BBS       [ CALLSIGN | * | ? ]

BBSALIAS  [ * | NEW_ALIAS ]

BTEXT     [ * | BEACON_MESSAGE_TEXT ] (see note 6)

BYE

CALIBRATE [PERIOD_VALUE_FROM_1_TO_60 [TOGGLE_VALUE_1_TO_PERIOD ] ]

CLOSEDOWN A

CONNECT   [ CALLSIGN [ [V] DIGILIST ... ]

CQ        [ MESSAGE_FOR_CQ_PACKET ]

CTEXT     [ * | CONNECT_MESSAGE_TEXT ] (see note 6)

DXCLUSTER [ CALLSIGN | * | ? ]

DXCALIAS  [ * | NEW_ALIAS ]

HELP

HOST      [ CALLSIGN | * | ? ]

HOSTALIAS [ * | NEW_ALIAS ]

INFO      [ SYSOP_SET_MESSAGE ] (see note 6)

IPADDRESS [ NEW_IP_ADDRESS ]

IPBROADCAST [ NEW_IP_ADDRESS ]

IPROUTE   [ipaddr [/bits ] [ - | { + port [ipgateway [metric ]]]]

IPSTATS   [ { NEWPARAM | * } { NEWPARAM | * } ..... ] (see note 5)

LINKS

MANAGER

METER     [ { new_value | * } ... ] (see note 5)

MHEARD    [ NUMBER_FROM_1_TO_100 ]

MODE      [ { NEWPARAM | * } { NEWPARAM | * } ..... ] (see note 5)

MTU       [ list_of_parameters ] (see note 5)

NODES     [* [*] | NODECALL {+|-} IDENT QUAL COUNT PORT NEIGHBOUR

[DIGIS]]

PARMS     [ { NEWPARAM | * } { NEWPARAM | * } ..... ] (see note 5)

QUIT

RESET     [ ANY_CHARACTER ]

ROUTES    [ PORT NODECALL [ DIGILIST ... ] { + | - } PATHQUALITY ]

STATS

SYSOP

TALK      [ STRING ]

UI        DEST   STRING_OF_TEXT_TO_BE_SENT_IN_UI_FRAME

USERS

------------------------------------------------------------------

Note 1	Any command may be enabled or disabled by the use of the '+' or '-' modifier, as shown below :

ANY_COMMAND   [ + | - | THAT_COMMANDS_PARAMETERS ]

Commands may be caused to be displayed or not in the prompt screen if the '-' or '+' is followed by 'D' ( or 'd' )

Note 2 IP addresses are of the form nnn.nnn.nnn.nnn

where nnn is a number 0..255

Note 3 IProute port parameter is 0 for radio or 1 for rs232

Note 4 ARP ptcl parameter is AX.25 or Net/Rom (may be abbreviated)

Note 5 The commands will accept the 'old' syntax of '* * * value...' or the new offset & value syntax of '/ parameter_number value'

HOST 'ESCAPE' COMMANDS

<escape> C

<escape> D

<escape> P  [ NEW_PASSWORD ]

==================================================================

UNDERSTANDING ACL VALUES              UNDERSTANDING AUDIT BITS

------------------------------------------------------------------

Bit   Function                   | Bit    Function

================================ |================================

0  Bar all incoming L2 connects |  0 Issue L1 stats every 10 mins

1  Bar outgoing L2 downlinks    |  1 Audit L2 connects & discs

2  Ignore nodes broadcasts      |  2 unused

3  Bar gatewaying at level 3    |  3 Audit L4 connects & discs

4  Bar incoming L4 connects     |  4 Audit L7 use of sysop cmd

5  Bar outgoing L4 connects     |  5 Audit all L7 switch commands

6  ignore SSID in searching     |  6 Issue CPU stats every 10mins

7  unused                       |  7 unused

==================================================================

PARMS PARAMETERS

Number Min    Max  Function

-----------------------------------------------------------

1      1    400  Maximum number of destination nodes

2      0    255  Minimum quality for auto update

3      0    255  HDLC ( radio, port 0 ) default quality

4      0    255  RS232 ( crosslink, port 1 ) default quality

5      0    255  Initial value for obsolescence counter

6      1    255  Minimum obsolescence for node broadcast

7      0  65535  Auto update broadcast interval (seconds)

8      0    255  Level 3 ( network ) Time To Live Initialiser

9      5    600  Level 4 ( transport ) timeout (seconds)

10      1    127  Level 4 ( transport ) retries

11      1     60  Level 4 ( transport ) ack delay (seconds)

12      1   1000  Level 4 ( transport ) busy delay (seconds)

13      1    127  Level 4 ( transport ) window size (frames)

14      1    127  Level 4 ( transport ) congestion ctrl threshold

15      0  65535  Level 7 ( switch ) inactivity timeout (seconds)

16      0    255  Persistance for transmit delay

17      0    127  Persistance slottime delay (10's of msecs)

18      1     15  Level 2 ( link ) T1 timeout, ie FRACK (seconds)

19      1      7  Level 2 ( link ) window size (packets)

20      0    127  Level 2 ( link ) retries

21      0   6000  Level 2 ( link ) T2 timeout (10's of msecs)

22      0  65535  Level 2 ( link ) T3 timeout (10's of msecs)

23      0      1  Level 2 ( link ) digipeat enable flag

24      0      1  Callsign validation flag

25      0      2  Node beacon ctrl (0=off, 1=if active, 2=always)

26      0      1  CQ broadcasts enable flag

==================================================================

MODE PARAMETERS

Number Min    Max  Function

-----------------------------------------------------------

1      0      1  Hardware handshake host control mode flag

2      0   3600  CWID repeat period ( seconds )

3      4     10  CWID speed ( 10's msecs per dot )

4      0      3  Nodes broadcast channel enable flags where

0=none, 1=HDLC only, 2=RS232 only, 3=Both ports

5      0      3  Crosslink protocol selection

0=crosslink, 1=KISS,

2=KISS+selective copy, 3=KISS+allcopy

6      0    255  TX keyup delay ( 10's of milliseconds )

7      0      1  Full duplex enable flag

8      0  65535  RS232 ( port 1 ) node broadcast period ( secs )

9      0      3  Node broadcast algorithm control flags

0=off, 2=RS232 port, 1 & 3 not normally used

10    600   3600  Beacon interval ( seconds )

11      0      2  Connect redirection to BBS flag

12      0    255  Help messages flags, 8 bit TALK, case & TEXNET

13      0      3  Hash node broadcast disable (one bit per port)

14      0      1  Enable extra aliases monitoring if set

15      0      1  Enable auto reconnect to node after remote dis

16      0      3  Slime trail control. Each bit controls...

Bit 0 if set hides slime trails in nodes list

Bit 1 if set causes slime trails to be ignored

17      0      3  Digipeat control. Each bit controls ...

Bit 0 set makes node to refuse digi'd L2 uplink

Bit 1 set, node refuses to allow digi downlinks

==================================================================

IPSTAT PARAMETERS

Number Min    Max  Function  ( Those marked '*' are not used )

-----------------------------------------------------------

1      0      3  ip L2 AX.25 Modes ( 1 bit per port, 1=DG )

2      0      1  ip Forwarding, 1=enable router, 0=disable

3      2    255  ip Default TTL

4      0      0  ip In Receives

5      0      0  ip In Header Errors

6 *    0      0  ip In Address Errors

7      0      0  ip Forwarded Datagrams

8      0      0  ip In Unknown Protocols

9 *    0      0  ip In Discards ( TTL exceeded )

10 *    0      0  ip In Delivers

11      0      0  ip Output Requests

12 *    0      0  ip Output Discards

13      0      0  ip Output No Routes

14 *    1     30  ip Reasm Timeout

15 *    0      0  ip Reasm Requireds

16 *    0      0  ip Reasm OKs

17 *    0      0  ip Reasm Fails

18      0      0  ip Frag OKs

19 *    0      0  ip Frag Fails

20      0      0  ip Frag Creates

=================================================================

MTU PARAMETERS

Number Min    Max  Function

-----------------------------------------------------------

1      64   1024  IP Port 0 ( Radio Port ) Level 2 AX.25 MTU

2      64   1024  IP Port 1 ( RS232 Port ) Level 2 AX.25 MTU

3      64    236  IP Net/Rom port MTU

4     257   1025  Level 2 Max data bytes in a packet before error

5     328   1096  Level 2 Total max packet size in bytes

=================================================================

METER PARAMETERS

Number Min    Max  Function

-----------------------------------------------------------

1       0    511  The meter mode flags

2       0    255  The deviation meter scaling factor

3       0    255  The signal strength meter noise floor value

4       0    255  The S meter display format multiplier

5       0    255  The dBm meter display format multiplier

6       0    255  The dBm noise floor value

7       0    255  The voltmeter channel 1 multiplier

8       0    255  The voltmeter channel 2 multiplier

9       0    255  Voltmeter Channel 1 offset value

10       0    255  Voltmeter Channel 2 offset value

==================================================================

THE METER MODE FLAGS ( METER parameter 1 )

BIT       If set, then ....

-----------------------------------------------------------

0        The deviation meter is enabled

1        The signal strength meter is enabled

2        The signal strength is shown as S points rather than dBm

3        ADC channel 3 is enabled ( voltmeter channel 1 )

4        ADC channel 4 is enabled ( voltmeter channel 2 )

5        Voltmeter channel 1 divisor is 1000 rather than 100

6        Voltmeter channel 2 divisor is 1000 rather than 100

7        Voltmeter channel 1 display resolution is 0.1 not 1.0

8        Voltmeter channel 2 display resolution is 0.1 not 1.0

==================================================================

HELP MESSAGES FLAGS ( MODE Parameter 12 )

BIT       FUNCTION

-----------------------------------------------------------

0         If set, the 'please wait, trying xxxx' operates

1         If set, all commands appear in help for sysop

2         If set, the 'goodbye' message is given

3         If set, a welcome message is enabled ( CTEXT )

4         If set, nodes are shown as 'alias:callsign'

5         If set, TALK data is passed as 8 bit data

6         If set, node aliases are deemed to be case sensitive

7         If set, enables the TexNet "*** LINKED to" interface