Tack

The Manual

Introduction:

This manual describes the operation of the Tack data analysis

program.  The current revision of this program is 3.20.  This

software and manual are copyright 1993 through 1989 by George B.

Grant, Somerville, MA  02144.

Compatibilities:

Tack runs on IBM PC compatible computer systems.  It operates

entirely in graphics mode.  As such, it requires a compatible

graphics display adapter.  Tack supports all major display adapters

(Hercules, CGA, EGA, VGA) and has drivers for several other

graphics display adapters including Super VGA. Tack works with a

Microsoft compatible mouse.

Tack is a data analysis program.  It can read data files

created by Patchit, Ninja or those written in the pClamp version

5.1 file format.

Starting Up:

Tack runs entirely in graphics (not text) mode.  In order for

it to run, a graphics driver for the graphics adapter that is

installed in the computer must be available in the directory in

which the Tack executable file is.  These drivers have the file

extension ".bgi".  The drivers required for the following screen

types are as follows:

Display Adapter                         Display Driver

Hercules (Hercules Graphics Adapter)         herc.bgi

CGA (Color Graphics Adapter)            cga.bgi

EGA (Enhanced Graphics Adapter)              egavga.bgi

VGA (Video Graphics Adapter)            egavga.bgi

Super VGA                          svga16m.bgi

(Supports several common SVGA cards and the VESA standard

SVGA interface)

IBM 8514 Graphics Adapter                    ibm8514.bgi

AT&T 400-Line Graphics Adapter               att400.bgi

3270 PC Graphics Adapter                pc3270

By default, Tack will automatically try to detect the proper

graphics driver for the computer.  If the proper driver is not

found by Tack (in the same directory as Tack) when it is started it

will end with an error message.  Otherwise the program will start

and Tack's main screen will appear.

The Screen:

Tack's main screen is divided into three areas.  The menu bar

is on the top of the screen, it allows the user to access all of

the Tack's commands.  The right-most portion of the menu bar shows

the amount of free conventional (real mode) memory available for

Tack's use.  The lower part of the screen shows the status line.

This line displays information about the current status of the

program (which data file, channel and record is currently

selected).  The data display area fills the screen between the menu

bar and the status line.

General:

Tack is a data analysis program.  It can load multiple data

files.  Each file will be loaded into its own "buffer".  New

buffers can be created by the copy buffer operation or by the

result of adding, subtracting or averaging existing buffers.  A

buffer, like a file, can contain multiple channels of data and

multiple records (a record is equivalent to a Clampex "episode").

Most of Tack's functions work on individual buffers, and these

commonly default to the current buffer.  The current buffer is

indicated on the status line at the bottom of the screen.  Buffers

are named with the letters of the alphabet and the current buffer

can be changed (when no menus are active) by typing the letter of

that buffer.  The current buffer can be changed to the

next/previous buffer by typing the up/down arrow key.  Similarly,

the current channel (in the current buffer) can be changed (when no

menus are active) by typing the number of the channel.  Note that

channels are numbered from 0 (i.e. 0 is the first channel, 1 is the

second).  The current channel can be changed to the next/previous

channel by typing the right/left arrow key.  If the buffer only has

one channel of data then you won't be able to switch channels,

obviously.  The current record works slightly differently.  When

you select it (e.g., by Display:Record) it is both selected and

displayed.  The next/previous record can be selected (and

displayed) by typing the plus/minus key.

In many of Tacks' functions, you will need to select a range

of timing within a data record.  For these cases you have the

option of numerically selecting the timing (by entering the times

at the prompt) or by cursor selecting the timing.  If you choose to

select the timing with cursor selection, then the screen will be

redrawn and the data record will be shown.  You can use the mouse

or cursor keys to move the cursor.  The home and end keys move the

cursor to the beginning and end of the data trace respectively.

The left and right arrow keys move the cursor to the one data point

to the left and right on the data trace respectively.  Holding down

the shift key while typing the left and right arrow keys causes the

cursor to move by the step indicated by the "step" value shown on

the top line of the screen.  This step value can be changed by

typing the plus key, which doubles this value (it eventually rolls

over).  Holding down the CTRL key while typing the left and right

arrow keys causes the cursor to move by 1/20th of the width of the

displayed data.  The cursor can be moved under mouse control by

holding down the left mouse button and moving the mouse to the left

and right.  NOTE that if there are more data points in the

displayed data trace than there are points on the screen, then the

cursor (while under mouse control) will not move in increments of

the data points.  For the finest control of timing you will need to

use the cursor keys in this case.  Typing the "ESC" key allows you

to cancel the timing selection operation.  Typing the Enter key, or

the right mouse button accepts the timing for this cursor.  During

the cursor selection operation, the value (both real and 12-bit) of

the data and the time will be shown on lower line of the screen.

Help:

On-line help is available throughout Tack.  Just type the F1

key and a relevant help message will pop up.  The help file

"Tack.hlp" must be in the same directory as Tack and the graphics

driver files.

Accessing the Menus:

The menu bar has a series of pull down menus.  A menu (e.g.

"File") can be accessed in a number of ways.  1) A menu can be

pulled down by clicking on the desired menu entry with the mouse.

2)  A menu can be pulled down by holding down the ALT key while the

selection letter of the desired menu is typed (the selection letter

is usually the first letter of the word and is shown underlined and

in a different color from the rest of that word (unless the current

graphics display only supports two colors, or the color

configuration has been messed with)).  For example, to pull down

the File menu, type ALT-F.  3)  A menu can be pulled down by first

activating the menu bar with the F10 or "/" key.  The file menu

will drop down, but not be selected.  The selection letter of any

of the main menu items can then be typed in order to access that

particular menu.

Certain functions on the individual menus can be accessed

directly, without pulling down the menus, by typing a hot-key (the

hot-keys are shown to the left of a menu entry, on its menu).  For

instance, the F8 key will cause Tack to load a data file if pressed

when the menu bar is not activated.

The Menus:

File Menu:

Load File:  Selecting this option allows the user to load a new

data file into Tack.  If there are data files in the current

directory (files with the extension ".dat"), a selection box will

appear listing the available data files.  A data file may be cursor

selected from this box (with the mouse or keyboard) or else the

name of a file may be typed in (do not type the ".dat" extension).

To change directories, type the F2 key, or click the left mouse

button in the upper, directory, window and type the pathname of the

desired directory.  The list of data files will be updated for this

new directory.

Save File:  Selecting this option allows the user to save the

contents of the currently selected buffer to a new data file.  You

will be prompted to specify the name of the data file (do not type

the ".dat" extension).  If a file of this name already exists, Tack

will warn you and give you the option of overwriting that file or

else of choosing a new filename.  You will then be asked if you

wish to keep the original file's time stamp.  If you specify that

you want to do that thing, then the file creation date and time

will be set to that of the original file.  If you specify not

(don't keep original file time stamp), then the creation date and

time will be set to the current date and time.  This option is very

useful if you like to sort your files by date.

Export:  Selecting this option allows the user to export data from

Tack in a variety of numerical formats.  The export menu has eleven

options:

Buffer:  Selecting this item allows the user to specify the buffer

whose data will be exported.  The buffer selection box lists the

letter corresponding to the buffer and the name of the file that

was originally loaded into the buffer (unless the buffer contains

the result of a combination (addition, subtraction, etc.) of data

from two or more buffers, in which case the name is "untitled").

Channel:  Selecting this item allows the user to select the channel

of data in the specified buffer that will be exported.  Only one

channel of data may be exported in a single operation.  If you need

to export data on multiple channels, you will need to perform the

export operation separate times, creating multiple files.

Records:  Selecting this item allows the user to select which

records of data in the selected buffer, on the selected channel are

to be exported.

Time:  Selecting this item allows the user to select the timing of

the data to be exported.  The user has the option of cursor

selecting the timing from the data traces or else of entering the

times numerically.

Format:  Selecting this item allows the user to specify the export

format for the data.  Two formats are available, Ascii and

Asystant+.  The Ascii format is a text file containing the data as

numbers separated by a user-specified character (the comma ","

character is typically used).  The Asystant+ format writes a binary

data file that can be read by the Asystant+ data analysis program.

X Values:  Selecting this item allows the user to select what

values will be written in the data file for the X axis (the

right-most column of data).  This option is not available for the

Asystant+ export format.  Three X axis options are available:

Time:  The time (in milliseconds) of the data point.

Point #:  The numerical point (beginning from 0) in the data at

which the point occurs.

None:  No value is entered for the X axis.

Y Values:  Selecting this item allows the user to select the format

of the values that are to be exported from the data.  The normal

format is "Real Values", which causes the values to be the true

value of the data (this takes into account the gain).  The other

format is "12 Bit" in which case the values written are the 12-bit

values (-2048 -> 2047) recorded by the data acquisition board.

Sample:  Selecting this item allows the user to select whether or

not to undersample the data in the exported file.  The data can

either not be undersampled or else undersampled by a factor as much

as 20 (every twentieth data point is written to the data file).

This feature is useful if your exported data files are too large.

Go!:  Selecting this item causes the export operation to begin.

You will be prompted for the name of the file to which the data

will be exported.  If a file of this name already exists, Tack will

warn you and give you the option of overwriting the file or else of

choosing a new name.

Quit:  Selecting this item cancels this operation and the data is

not exported.

DOS Shell:  When this is selected the user temporarily exits to DOS

to perform simple commands.  In order to allow this, Tack "spawns"

a copy of the DOS command interpreter.  Note that Tack is still

loaded in memory so there will be less DOS memory available to

execute a command.  Type "EXIT" at the DOS command line ("C:\>") to

return to Tack.

Plot:  When this option is selected Tack plots (or prints) data

traces to the currently selected printer/plotter using the

currently displayed data as its guide.  If a printer/plotter has

not been selected then Tack will nag you to select one (see

Config:Printer).

If Tack has been configured to print to file, you will be

prompted for the name of the file to write.  You will then be

prompted to hit any key to begin printing, you may abort the

process here with the ESC key or the right mouse button.

Otherwise, Tack will print/plot.  As mentioned, Tack uses what is

on the screen as its guide for printing.  All displayed buffers

will be printed at their current scale factor.  If scale bars are

displayed (see Display:Scale Bars), then they will be printed.

Note that Tack will only print once per buffer.  That means that if

you have displayed data records from a single buffer more than once

on the screen (for instance using different magnifications) then

only one will be printed/plotted.

About:  When this option is selected Tack will display informative

things about itself, these can be selected from the submenu:

Data:  Selecting this item causes information to be displayed about

the data stored in one of Tack's buffers.  If more than one buffer

is loaded, then you will be prompted for the buffer to display

information about.  A menu appears with the following items:

A/D Info:  General information about the data and how it was

sampled.

Gain List::  Displays the gains for all of the channels and records

of data in the buffer.

D/A Info:  Displays information about the Output waveform used

during data acquisition.

Comments:  Displays any comments recorded with the data file.

Quit:  Closes the menu.

Memory:  Selecting this item causes a box to appear citing

information about the memory available and in use by Tack.  There

are three types of memory used by Tack.

Conventional Memory:  This is the memory used by all DOS real mode

programs  it is limited to a maximum of 640 Kilobytes (KB).  Tack

will show you the total amount and the amount available.

XMS Memory:  This is memory above the 1 Megabyte boundary that can

be accessed by Tack if an XMS driver (such as Microsoft's

Himem.sys) is loaded.  XMS memory allows Tack to store a great deal

of data without filling up the DOS (conventional) memory area.  If

XMS is available Tack will store data in it.  The handle to and the

amount of XMS memory used by each buffer are shown.

Disk Memory: This shows the total amount and available amount of

disk space on the current disk.

Tack:  Selecting this item causes a box to appear citing

information about Tack (it's current revision level), a copyright

notice and information about its author (me!).

Quit Program:  Selecting this item allows the user to exit the

program.

Buffer Menu:

Copy Buffer:  Selecting this option allows the user to copy the

data in one of Tack's buffers to another buffer.  The copy buffer

menu has four entries:

Source Buffer:  This is the buffer from which the data is to be

copied.  If only one buffer is loaded, then this is automatically

set to that buffer.  Otherwise, selecting this item brings up a

selection box from which the user can choose the buffer to copy

from.

Dest Buffer:  The buffer to which the data is to be copied.

Selecting this item brings up a selection box from which the user

can select the buffer to copy to.  The user can either overwrite

existing data in an already loaded buffer or else copy into a new

buffer.

Go!:  Selecting this item causes the buffer to be copied.

Quit:  Selecting this item cancels this operation and the buffer is

not copied.

Delete:  Selecting this option allows the user to delete various

things from Tack's memory (not disk).  The delete menu has five

entries:

Buffer:  Selecting this allows the user to delete the contents of

a single buffer.  Select the buffer you wish to delete from the

selection box that appears.  Once a buffer is deleted, all buffers

above it are shifted down.  So, if you have four buffers loaded

(A-D) and delete buffer B, then you will end up with three buffers

(A-C), where buffer B now contains what was in C previously and

buffer C contains what was in D previously.

All Buffers:  Selecting this allows the user to delete all data

that is loaded in Tack. The contents of all buffers are deleted.

Channel:  Selecting this allows the user to delete a single channel

of data from the current buffer.  If there is only one channel of

data in the current buffer, then the entire buffer will be deleted.

The current buffer is shown on the bottom of the screen on the

status line.

Records:  Selecting this allows the user to delete a range of data

records from the current buffer.  If you select all of the records

in the buffer, then the entire buffer will be deleted.  The current

buffer is shown on the bottom of the screen on the status line.  If

the current buffer contains multiple channels of data, then the

selected range of records will be deleted from all channels in the

buffer.

Quit:  Selecting this item exits from this menu.

Modify:  Selecting this option allows the user to modify various

aspects of data that is loaded in Tack's memory (not disk).  The

modify menu has five entries:

Sampling Int:  Selecting this allows the user to change the

sampling interval of the data in the current buffer.  The value for

the sampling interval entered here must be larger than the existing

sampling interval, and must be an integer multiple of the old

value.  Tack will enforce this.

NOTE: It is not good practice to change the sampling interval

of data after it has been recorded.  This feature was implemented

as a means of reducing the number of data points for generating

HPGL plots (as some programs to which HPGL data is imported cannot

handle HPGL plots with large numbers of points).

Timing:  Selecting this allows the user to change the timing of the

data in the current buffer.  The new timing range can be entered

either numerically or graphically (by cursor selecting the timing

in the data trace with the mouse and cursor keys).

Gain:  Selecting this item allows the user to change the gain for

the data in the current buffer.

NOTE:  The gain of the data is not likely to change after the

data has been recorded.  This feature was implemented in order to

allow you to correct for stupid errors.

Data:  Selecting this allows the user to change the values of the

data loaded into Tacks' buffers.  The modify data menu has five

options:

Buffer:  This allows the user to select the buffer whose data will

be modified.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data will be modified.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, will be modified.

Operation:  This allows the user to select the operation to perform

on the data.  Five operators are available, these allow the user to

combine the selected data with a user-specified factor:

* Multiply:  This multiplies the selected data by a user-specified

factor.  The user will be queried as to whether to scale the gain

of the data accordingly.  You would typically answer yes.

/ Divide:  This divides the selected data by a user-specified

factor.  The user will be queried as to whether to scale the gain

of the data accordingly.  You would typically answer yes.

+ Add:  This adds to the data a user-specified factor.  Note that

in this case, the user-specified factor is 12-bit, i.e., -2048 to

2047.

- Subtract:  This subtracts from the selected data a user-specified

factor.  Note that in this case, the user-specified factor is

12-bit, i.e., -2048 to 2047.

Separate Recs:  This separates (staggers) the data records from

each other by a user-specified factor.  Note that in this case, the

user-specified factor is 12-bit, i.e., -2048 to 2047.

Go:  Selecting this causes the selected operation to be performed

on the selected data.

Quit:  Selecting this closes the menu.  The selected operation is

not performed on the selected data.

Quit:  Selecting this item exits this menu.

Add:  Selecting this option allows the user to add the data in two

of Tacks' buffers together.  The resultant data may be stored in

another buffer or overwrite one of the original buffers.  Between

the two buffers, channel 0 is added to channel 0, channel 1 is

added to channel 1, etc., and record 1 is added to record 1, record

2 is added to record 2 etc.  If the buffers have unequal numbers of

records or channels, then Tack will use the least common

denominator between the two buffers.

NOTE:  Tack does not check that the gains for the two buffers

are the same.  You should ensure that the data you wish to add

together were recorded at the same gains!  Or else you must scale

the data to compensate for the different gains (use

Buffer:Modify:Data).

The add menu has five entries:

X Adder:  This is one of the two buffers to add together.

Selecting this brings up a selection box from which the user can

select this buffer.

Y Addend:  This is the buffer which is to be added to the adder

buffer (above).  Selecting this brings up a selection box from

which the user can select this buffer.

Z = X+Y Dest:  This is the buffer to which the result of the

addition operation is to be written.  Selecting this brings up a

selection box from which the user can select the destination

buffer.  The user can either overwrite existing data in loaded

buffers or else copy the buffer's contents to a new buffer.

Go!:  Selecting this causes the buffer addition to occur.

Quit:  Selecting this cancels this operation, the buffer addition

does not occur.

Subtract:  Selecting this option allows the user to subtract the

data in one of Tacks' buffers from the data in another buffer.  The

resultant data may be stored in another buffer or overwrite one of

the original buffers.  Between the two buffers, channel 0 is

subtracted from channel 0, channel 1 is subtracted from channel 1,

etc., and record 1 is subtracted from record 1, record 2 is

subtracted from record 2 etc.  If the buffers have unequal numbers

of records or channels, then Tack will use the least common

denominator between the two buffers.

NOTE:  Tack does not check that the gains for the two buffers

are the same.  You should ensure that the data you wish to subtract

were recorded at the same gains!  Or else you must scale the data

to compensate for the different gains (use Buffer:Modify:Data).

The subtract menu has five entries:

X Subtractor:  This is the buffer from which the subtrahend buffer

is to be subtracted.  Selecting this brings up a selection box from

which the user can select this buffer.

Y Subtrahend:  This is the buffer which is to be subtracted from

the subtractor buffer (above).  Selecting this brings up a

selection box from which the user can select this buffer.

Z = X-Y Dest:  This is the buffer to which the result of the

subtraction operation is to be written.  Selecting this brings up

a selection box from which the user can select the destination

buffer.  The user can either overwrite existing data in loaded

buffers or else copy the buffer's contents to a new buffer.

Go!:  Selecting this causes the buffer subtraction to occur.

Quit:  Selecting this cancels the operation and the buffer

subtraction does not occur.

Average:  Selecting this option allows the user to average the data

in two of Tacks' buffers.  The resultant data may be stored in

another buffer or overwrite one of the original buffers.  Between

the two buffers, channel 0 is averaged with channel 0, channel 1 is

averaged with channel 1, etc., record 1 is averaged with record 1,

record 2 is averaged with record 2 etc.  If the buffers have

unequal numbers of records or channels, then Tack will use the

least common denominator between the two buffers.

NOTE:  Tack does not check that the gains for the two buffers

are the same.  You should ensure that the data you wish to average

were recorded at the same gains!  Or else you must scale the data

to compensate for the different gains (use Buffer:Modify:Data).

The average menu has five entries:

X Buffer 1:  This is one of the buffers to average.  Only two

buffers may be averaged together.  Selecting this item brings up a

selection box from which the user can select this buffer.

Y Buffer 2:  This is the other buffer to average.  Only two buffers

may be averaged together.  Selecting this item brings up a

selection box from which the user can select this buffer.

Z Destination:  This is the buffer to which the result of the

averaging operation is to be written.  Selecting this item brings

up a selection box from which the user can select the destination

buffer.  The user can either overwrite existing data in loaded

buffers or else copy the buffer's contents to a new buffer.

Go!:  Selecting this causes the buffers to be averaged.

Quit:  Selecting this cancels this operation, the buffers are not

averaged.

Edit Menu:

Normalize:  Selecting this option allows the user to subtract

baseline data from data.  The normalize menu has six entries:

Buffer:  This allows the user to select the buffer whose data will

be normalized.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data will be normalized.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, will be normalized.

Timing:  This allows the user to select the timing of the baseline

segment of the data.  The user may select the baseline timing

either graphically or numerically.

Go!:  Selecting this item causes the selected data records to be

normalized.

Quit:  Selecting this item cancels this operation, the selected

data records are not normalized.

Leak Subtract:  Selecting this option allows the user to subtract

a "leak" current (time-invariant current, that scales linearly with

voltage) from the data.  This function is implemented for data

records that represent current.  The leak subtract menu has eight

entries:

Buffer:  This allows the user to select the buffer whose data will

be leak subtracted.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data will be leak subtracted.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, will be leak subtracted.

Input Res:  This allows the user to specify the resistance that

will be used to calculate the leak current.  The user can enter the

input resistance (in megaohms) or else can choose to calculate the

input resistance from the data.  If you choose to calculate the

input resistance, when you select go (see below), you will be

transferred to function that will calculate this (see Measure:Input

Resistance, below).

V (mV):  This allows the user to specify the voltage range for the

range of records that you wish to leak subtract.  If more than one

record is specified, you will be prompted for a voltage and an

increment.  The voltage, should be the voltage for the starting

record, the increment specifies how much the voltage is to be

incremented with each successive record.  The starting voltage

should be relative to the holding potential.

Timing:  This allows the user to select the range of timing in the

data records that will be leak subtracted.  This timing may be

selected either graphically or numerically.

Go!:  Selecting this item causes the selected data records to be

leak subtracted.

Quit:  Selecting this item cancels this operation, the selected

data records are not leak subtracted.

Average Recs:  Selecting this option allows the user to overwrite

a data record in a buffer with the average of a range of data

records from that buffer.  The average recs menu has six entries:

Buffer:  This allows the user to select the buffer whose data

records will be averaged.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data records will be averaged.

Records to Average:  This allows the user to select which data

records, on the selected channel, in the selected buffer, will be

averaged.

Overwrite Record:  This allows the user to select the data record

that will be overwritten with the average of the records specified

above.

Go!:  Selecting this item causes the selected data records to be

averaged.

Quit:  Selecting this item cancels this operation, the selected

data records are not averaged.

Scale 'n Sub:  Selecting this option allows the user to subtract

from a range of data records a scaled copy of the data in a given

record.  The algorithm used is as follows:

record#i = record#i - factor * model_rec,

where

factor = (i - zero_level) / (model_rec - zero_level).

The normalize menu has six entries:

Buffer:  This allows the user to select the buffer whose data

records will be scaled and subtracted.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data will be scaled and subtracted.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, will have a scaled copy

of the model record (see below) subtracted from them.

Model Rec:  This allows the user to specify which data record to

scale and subtract from the other specified records in this buffer.

Zero Level:  This allows the user to specify the record at which

the data traces would change sign.  This does not have to be an

integer value, you may use a fractional value if needed.  This

value is used in calculating the scaling factor (see above).

Go!:  Selecting this item causes the scale 'n sub operation to

occur.

Quit:  Selecting this item cancels this operation, the scale 'n sub

operation does not occur.

Smooth:  Selecting this option allows the user to smooth (low-pass

filter) the data.  The smooth menu has eight entries:

Buffer:  This allows the user to select the buffer whose data will

be smoothed.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data will be smoothed.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, will be smoothed.

Timing:  This allows the user to select the range of timing in the

selected data records, over which the data will be smoothed.  The

user may select this timing either graphically or numerically.

Smooth By:  Selecting this allows the user to select the smoothing

method that will be used.  Three smoothing methods are available,

a Gaussian filter, a moving average (rolling window) and a FFT

(fast fourier transform)-based algorithm.  The Gaussian Filter

lowpass filters the data with a user-specified -3db cutoff freq.

This uses the algorithm from "Single Channel Recording" ed. B.

Sakmann & E. Neher, Chap 11.  The moving average algorithm

successively takes the local average of a number of data points

around some central data point and then replaces the central data

point with the average.  The FFT algorithm, performs a FFT on the

data, and then low pass filters it and then reverse FFTs it.  This

uses an algorithm from Numerical Recipes in C.

Cutoff Freq/Window Size/Degree:  Selecting this allows the user to

specify the parameter that determines the amount of smoothing that

will happen to the data.  This menu entry has a different name

depending on what smoothing algorithm is selected.  Cutoff freq -

this specifies the -3db cutoff frequency for the Gaussian Filter.

Window Size - this specifies the width of the window over which the

data will be locally averaged for the Moving Average algorithm.

Degree - this specifies the point in frequency space where the

smoothing begins for the FFT algorithm.  For the latter two

algorithms, higher values give more smoothing.

Go!:  Selecting this item causes the selected data records to be

smoothed.

Quit:  Selecting this item cancels this operation, the selected

data records are not smoothed.

Display Menu:

Buffer:  Selecting this item allows the user to select the current

buffer.  If more then one buffer is loaded, a selection box will

appear from which the user can select a buffer.  The current buffer

is indicated on the status line at the bottom of the display.

Buffers may also be selected with the up or down cursor keys or by

typing the letter of the buffer (these only work no menu is

currently selected).

Channel:  Selecting this item allows the user to select the current

channel of data in the current buffer.  The current channel is

shown on the status line on the bottom of the display.

Record:  Selecting this item allows the user to select the current

data record in the current buffer on the current channel.  The

current record is then displayed on the screen.  This differs from

the Display:Buffer and Display:Channel in that in addition to

selecting the record, the data is also displayed on the screen.

The current record is shown on the status line on the bottom of the

display.

Clear Screen:  Selecting this option clears the data display area.

All Records:  Selecting this option causes all of the records in

the currently selected buffer on the currently selected channel to

be displayed in the data display area.  The current record is set

to the last record in the buffer.

Zoom:  Selecting this option allows the user to zoom in on (or

reset the zoom of) the currently displayed data records.  The zoom

menu has four options.

Dynamic:  This allows the user to select the scale factor, offset

factor and range of timing used for zooming in on the data, in a

dynamic fashion.  The user marquee-selects the part of the data

traces to zoom in on.  All displayed data traces are redrawn to

zoom in on the selected area.

Select the area in the data traces that you want to zoom

in on.  Use the mouse or keyboard:  Marquee-select a region using

the mouse (click and drag).  Once you release the mouse button you

will zoom in.  Or: Use the cursor keys to move the box.  Hold the

shift key down while using the cursor keys in order to resize the

box.  For both of these ops, the amount of box movement for each

keystroke is controlled by the step factor, which can be increased

or decreased using the + and - keys.  Hit return when done, or else

cancel with ESC.

Manual:  Selecting this allows you to set the scale factor, offset

factor and timing range used for scaling manually.  You will select

these values for a single buffer at a time and any displayed data

for this buffer will be redrawn at the new zoom factors.  There are

six entries on the manual zoom menu.

Buffer:  This allows you to select the buffer whose data records

you will be setting the scale, offset and timing ranges for.

Scale by:  This allows you to enter manually the scale factor at

which records are displayed (magnified by).  When you enter a value

for this parameter, all displayed records will redraw at this scale

factor.

Offset:  This allows you to enter manually the offset factor by

which records are vertically offset when displayed.  0 would cause

no offset, 1.0 would cause the displayed records to move to the top

of the screen, -1.0 to the bottom of the screen etc. If a number is

entered for this parameter, all displayed records will redraw using

this offset factor.

Timing:  This allows you to enter manually the range of timing over

which records in the currently selected buffer will be displayed.

Reset:  This causes the scale factor, offset factor and timing

range to be reset for the currently selected buffer.  All displayed

records for the currently selected buffer are redrawn.

Quit:  Selecting this exits this menu.

Reset:  This causes the scale factor, offset factor and timing

range to be reset for all displayed data in ALL buffers.  All

displayed records are redrawn.

Quit:  Selecting this exits this menu.

Scale Bars:  This allows the user to display scale bars for your

data.  The scale bars are displayed for the data in the currently

selected (and displayed) record, on the selected channel in the

selected buffer.  The scale bar menu has five entries.

Height:  This allows the user to enter the height, or amplitude of

the scale bar.

Width:  Enter the width (in msec) of the scale bar.

Label?:  This allows you to select whether or not to label the

scale bars.

Go!:  Selecting this draws and allows you to control the placement

of the scale bars.  The screen clears and you can move and change

the orientation of the scale bars.  Use the cursor keys or mouse to

move the scale bars around the screen.  The "+" key changes the

cursor key movement increment.  Hitting The space bar rotates the

scale bars.  Hit Enter when done, or ESC to cancel.

Quit:  Selecting this cancels the operation, the scale bars are not

drawn.

Measure Menu:

Dynamic Scan:  Selecting this allows the user to take measurements

from the current record, on the current channel in the current

buffer.  Using the cursor, you can measure values and times from

the data.  The lower line of the screen indicates the value of the

data at the time where the cursor is.  Use the cursor keys or the

mouse to move the cursor (vertical scan bar).  For cursor movement,

the + key changes the movement increment.  Ctrl-Cursor Keys make

bigger jumps. The Enter key (or right mouse button) drops the 1st

cursor and starts a second cursor.  The ESC key aborts the dynamic

scan operation.  If two cursors are in place then hitting the F2

key will calculate the mean of the data over the timing range,

hitting the F3 key will calculate the minimum and maximum values of

the data over the timing range, hitting the F4 key will calculate

the area under the curve for the timing range.  Hitting the F5 key

fits a line (mx+b) to the data for the timing range.

Values:  Selecting this allows you to make a series of measurements

from the data.  The measurements can be displayed graphically or

numerically and can be saved to a file.  The measure values menu

has ten options.

Type:  Selecting this allows you to specify the type of measurement

that will be made from the data.  The five measurement types are:

Average Value, Minimum Value, Maximum Value, Point Value, and

Integral Value.  Average value calculates the average of the data

over the timing selected, the mean and standard deviation will be

calculated.  Minimum and Maximum Value find the minimum and maximum

value, respectively for the data over the timing selected.  Point

value calculates the value of the data at a single point in time.

Integral value calculates the integral of the data (area under the

data curve) over the timing selected.

Buffer:  This allows the user to select the buffer whose data you

wish to make the measurements of.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data you wish to make measurements of.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, you wish to make

measurements of.

X Axis Values:  This allows the user to enter the type of values

that will be put on the X axis of the data plot (and which would be

put in the first column of an ASCII data file).  The X axis may be

either the record number or voltage.  For voltage, the values are

entered as such: the voltage of the first record in the series, and

an increment, which is how much the voltage is incremented with

each successive record.

Baseline Timing:  This allows the user to select the range of time

in the data records which is the baseline for measuring the values.

The timing can be selected numerically or by cursor selection.  If

you have selected to make measurements relative to the baseline,

the average value of the data over the baseline timing will be

subtracted from the values measured over the data timing.

Data Timing:  This allows the user to select the range of time in

the data records within which the values will be measured.  The

timing can be selected numerically or by cursor selection.  Note

that if you are measuring at a single point in time, this will be

only a single time point.

Measurements:  This allows the user to specify whether the

measurements that you make should be the actual values of the data

or relative to a baseline measurement.

Go!:  Selecting this causes the measurement operation to proceed.

If you have chosen to graphically select any of the timing, the

screen will clear and allow you to do this.  After the timings have

all been selected, the measurements will be plotted on the screen

and you have several options.  Hit any key to bring up the menus.

File Data:  This allows the user to put the numerical measurements

into an ASCII (text) file for use in other programs.

View Data:  This allows the user to view the numerical data.

Review Plot:  Selecting this clears away the menus so that you can

see the plot unobstructed.

Quit:  Selecting this leaves this menu.

Quit:  Selecting this leaves the measure values menu.

Mean/Variance:  Selecting this allows you to make a mean/variance

plot from your data.  The variance of the data will be plotted vs

the mean.  This is useful for estimating single channel

conductances from whole-cell currents.  The mean/variance plot can

be displayed graphically or numerically and can be saved to a file.

The measure mean/variance menu has seven options.

Buffer:  This allows the user to select the buffer whose data you

wish to make the mean/variance analysis from.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data you wish to make the mean/variance

analysis from.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, you wish to make the

mean/variance analysis from.

Data Timing:  This allows the user to select the range of time in

the data records over which the mean and variance will be measured.

The timing can be selected numerically or by cursor selection.

Subdivisions:  This allows the user to subdivide the timing

specified by Data Timing (above) into multiple, equally sized

subdivisions.  This allows you to generate multiple mean/variance

data points from each record.

Go!:  Selecting this causes the mean/variance measurements to

proceed.  If you have chosen to graphically select any of the

timing, the screen will clear and allow you to do this.  After the

timings have all been selected, the mean/variance plot will appear

on the screen and you have several options.  Hit any key to bring

up the menus.

File Data:  This allows the user to put the mean and variance

measurements into an ASCII (text) file for use in other programs.

View Data:  This allows the user to view the numerical data.

Review Plot:  Selecting this clears away the menus so that you can

see the plot unobstructed.

Quit:  Selecting this leaves this menu.

Quit:  Selecting this leaves the measure mean/variance menu.

Histogram:  Selecting this allows you to make a histogram from your

data.  The frequency of occurrence of data values within the

specified bin width will be plotted.  The histogram can be

displayed graphically or numerically and the data can be saved to

a file.  The histogram menu has nine options.

Buffer:  This allows the user to select the buffer whose data you

wish to make the histogram from.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data you wish to make the histogram from.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, you wish to make the

histogram from.

Timing:  This allows the user to select the range of time in the

data records over which the histogram will be measured.  The timing

can be selected numerically or by cursor selection.

Histogram Range:  This allows the user to select the range of

values within the data records, over which the histogram will be

calculated.

Width of Bin:  This allows the user to select the size of the bin

used in calculating the histogram.  The number and width of the

bins are interrelated with the histogram range.

Number of Bins:  This allows the user to select the number of bins

used for the histogram.  The number and width of the bins are

interrelated with the histogram range.

Go!:  Selecting this causes the histogram calculation to proceed.

If you have chosen to graphically select the timing, the screen

will clear and allow you to do this.  After the timing has been

selected, the histogram plot will appear on the screen and you have

several options.  Hit any key to bring up the menus.

File Data:  This allows the user to put the histogram measurements

into an ASCII (text) file for use in other programs.

View Data:  This allows the user to view the histogram data in a

numerical format.

Review Plot:  Selecting this clears away the menus so that you can

see the histogram plot unobstructed.

Quit:  Selecting this leaves this menu.

Quit:  Selecting this leaves the measure histogram menu.

Power Spectrum:  Selecting this allows the user to generate a power

spectrum from data.  The power spectrum can be displayed

graphically or numerically and the data can be saved to a file.

The power spectrum menu has nine options.

Buffer:  This allows the user to select the buffer whose data you

wish to make the power spectrum from.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data you wish to make the power spectrum

from.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, you wish to make the

power spectrum from.

Timing:  This allows the user to select the range of time in the

data records over which the power spectrum will be measured.  The

timing can be selected numerically or by cursor selection.

Number of Poles:  This allows the user to select the number of

poles (or order) for the power spectrum.  This determines the

sharpness of the spectral features you can measure in the spectrum.

In practice you wants to limit the order to a few times the number

of sharp spectral features that you want to fit.  Limiting it in

this fashion will smooth the spectrum somewhat, but this is often

desirable.  Try values of 10-50 for 1000 to 10000 data points.

Note that the algorithm used can be quirky for large numbers of

poles and will suggest spurious peaks in the spectrum, or might

find multiple peaks for a single true spectral peak.  So try to use

the smallest values.

Frequency Range:  This allows the user to select the frequency

range over which the power spectrum will be calculated.  Note that

the maximum frequency for which you can calculate the power

spectrum is limited to one half the sampling frequency.  The

frequency range and interval are interrelated and limited to 16383

data points.

Freq Interval:  This allows the user to select the frequency

interval for the power spectrum.  The frequency interval and range

are interrelated and limited to 16383 data points.

Go!:  Selecting this cause the power spectrum calculation to

proceed.  If you have chosen to graphically select the timing, the

screen will clear and allow you to do this.  After the timing has

been selected, the power spectrum will be calculated and plotted on

the screen, you then have several options.  Hit any key to bring up

the menus.

File Data:  This allows the user to put the power spectrum

measurements into an ASCII (text) file for use in other programs.

View Data:  This allows the user to view the power spectrum data in

a numerical format.

View Coefficients:  This allows the user to view the coefficients

used in generating the power spectrum.  This probably isn't very

useful to you.

Review Plot:  Selecting this clears away the menus so that you can

see the power spectrum plot unobstructed.

Quit:  Selecting this leaves this menu.

Quit:  Selecting this leaves the measure power spectrum menu.

Input Resistance:  Selecting this allows the user to calculate the

input resistance for data recorded under voltage-clamp.  The input

resistance is best calculated from data that has no

voltage-activated currents.  The input resistance menu has eight

entries:

Buffer:  This allows the user to select the buffer whose data you

wish to calculate the input resistance from.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data you wish to calculate the input

resistance from.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, whose data you wish to

calculate the input resistance from.

V (mV):  This allows the user to specify the voltage range for the

range of records that you wish to calculate the input resistance

from.  If more than one record is specified, you will be prompted

for a voltage and an increment.  The voltage, should be the voltage

for the starting record, the increment specifies how much the

voltage is to be incremented with each successive record.  The

starting voltage should be relative to the holding potential.

Baseline Timing:  This allows the user to select the range of

timing in the data records that is the baseline for calculating the

input resistance.  The baseline timing may be selected either

graphically or numerically.

Data Timing:  This allows the user to select the range of timing in

the data records over which the input resistance will be

calculated.  This timing may be selected either graphically or

numerically.

Go!:  Selecting this item causes the input resistance to be

calculated and displayed in megaohms.  If you have chosen to

graphically select the timing(s), the screen will first clear and

allow you to do this.

Quit:  Selecting this item cancels this operation, the input

resistance is not calculated.

Cell Capacitance:  Selecting this allows the user to calculate the

cell capacitance for data recorded under voltage-clamp.  The

capacitance is measured by calculating the area under a

voltage-clamp current transient.  Data should be leak subtracted

first, no voltage-activated currents should be active in the data.

The cell capacitance menu has eight entries:

Buffer:  This allows the user to select the buffer whose data you

wish to calculate the cell capacitance from.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data you wish to calculate the cell

capacitance from.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, whose data you wish to

calculate the cell capacitance from.

V (mV):  This allows the user to specify the voltage range for the

range of records that you wish to calculate the cell capacitance

from.  If more than one record is specified, you will be prompted

for a voltage and an increment.  The voltage, should be the voltage

for the starting record, the increment specifies how much the

voltage is to be incremented with each successive record.  The

starting voltage should be relative to the holding potential.

Baseline Timing:  This allows the user to select the range of

timing in the data records that is the baseline for calculating the

cell capacitance.  The baseline timing may be selected either

graphically or numerically.

Data Timing:  This allows the user to select the range of timing in

the data records over which the cell capacitance will be

calculated.  This timing should encompass the full width of the

capacitive transient.  The data timing may be selected either

graphically or numerically.

Go!:  Selecting this item causes the cell capacitance to be

calculated and displayed in picofarads.  If you have chosen to

graphically select the timing(s), the screen will first clear and

allow you to do this.

Quit:  Selecting this item cancels this operation, the cell

capacitance is not calculated.

Series Resistance:  Selecting this allows the user to calculate the

series resistance for data recorded under voltage-clamp.  The

series resistance is measured by calculating the height of a

voltage-clamp transient.  Data should be leak-subtracted first.

Note that this method is not the best method to calculate Rs, and

will give an underestimate.  The series resistance menu has eight

entries:

Buffer:  This allows the user to select the buffer whose data you

wish to calculate the series resistance from.

Channel:  This allows the user to select the channel of data in the

selected buffer whose data you wish to calculate the series

resistance from.

Records:  This allows the user to select which data records, on the

selected channel, in the selected buffer, whose data you wish to

calculate the series resistance from.

V (mV):  This allows the user to specify the voltage range for the

range of records that you wish to calculate the series resistance

from.  If more than one record is specified, you will be prompted

for a voltage and an increment.  The voltage, should be the voltage

for the starting record, the increment specifies how much the

voltage is to be incremented with each successive record.  The

starting voltage should be relative to the holding potential.

Baseline Timing:  This allows the user to select the range of

timing in the data records that is the baseline for calculating the

series resistance.  The baseline timing may be selected either

graphically or numerically.

Data Timing:  This allows the user to select the range of timing in

the data records over which the series resistance will be

calculated.  This timing may be selected either graphically or

numerically.

Go!:  Selecting this item causes the series resistance to be

calculated and displayed in megaohms.  If you have chosen to

graphically select the timing(s), the screen will first clear and

allow you to do this.

Quit:  Selecting this item cancels this operation, the series

resistance is not calculated.

Config Menu:

Graphics Display:  Selecting this option allows the user to specify

the type of graphics display installed and the display resolution

that Tack should use.  Normally, the graphics autodetect setting

should be used.  For a given display type to be used, the requisite

display driver file (they have the extension ".bgi") MUST be

present in the same directory as Tack.

Display Type:  Selecting this option allows the user to select a

graphics display type.  Twelve settings are available.

Note:  If your computer does not support the display type that you

have selected, you may well crash the program when you try to

switch display types.

Resolution:  Selecting this option allows the user to select a

display resolution for the selected display type.  Note that some

display types have only one available resolution.

Note:  If your computer's display adapter does not support the

resolution that you have selected, you may well crash the program

when you try to switch display resolutions.

Okay:  Selecting this option causes any changes that have been made

to the graphics display type and resolution to go into effect.

Cancel:  Selecting this option causes any changes that have been

made to the graphics display type and resolution to be cancelled.

Colors:  When this option is selected a color demonstration window

appears and a menu appears from which the user can select the

colors of the various components of the Tack screen.  The various

components are the menu bar, status line, popup boxes and data

window.  For menus, the user can control the background, text,

frame and select letter colors.  For the status line, the user can

control the background and text colors.  For popup boxes, the user

can control the background, text and frame colors.  For the data

window, the user can control the background, frame and axis colors

as well as the colors for the data in up to eight buffers.

Menus:

Status Line:

Popup Boxes:

Data Window:

When the user selects one of these menu options, another menu

appears from which the user can select the color for the elements

of the windows (i.e. frame color of popup window).  Clicking on an

element will select the next available color for that element, this

will be shown in the color demonstration window.  Typing the select

letter of that element will do the same.  Holding the SHIFT key

down while typing the select letter of that element will select the

previous color for that element.  Select Okay when you are done

setting the colors.

Use Defaults:  When this option is selected, the colors of all of

the elements of the Tack display revert to their default settings.

Okay:  Selecting this option causes any changes that have been made

to the color scheme to go into effect.

Cancel:  Selecting this option causes any changes that have been

made to the color scheme to be cancelled.

Font:  Selecting this option allows the user to specify the font

that will be used by Tack.  For all but the default font, the

requisite font file (they have the extension ".chr") MUST be

present in the same directory as Tack.  An example of some text

written in the selected font is displayed.

Font:  Selecting this option allows the user to select the font

family that will be used by Tack.  Eleven font families are

available.  The default font is a bit-mapped font, the others are

stroked (outline) fonts.

Height:  Selecting this option allows the user to set the height of

the font that will be used by Tack.  If the selected font family is

the default, bit-mapped font, the height and the width must be the

same.

Width:  Selecting this option allows the user to set the width of

the font that will be used by Tack.  If the selected font family is

the default, bit-mapped font, the height and the width must be the

same.

Okay:  Selecting this option causes any changes that have been made

to the font to go into effect.

Cancel:  Selecting this option causes any changes that have been

made to the font to be cancelled.

Printer:  Selecting this option allows the user to specify the type

of printer that Tack will use and its configuration.

Printer:  Selecting this option allows the user to specify the type

of printer that is installed.  Six printer options are available:

None:  No printer is attached.

Apple Imagewriter II:  This printer configuration will also work

with a C-Itoh 8510.

IBM Proprinter:  This printer configuration will also work with

many other Epson compatible printers.

HP Laserjet 2:  This printer configuration will also work with many

other PCL-4  compatible printers.

HP Plotter (HPGL):  This printer configuration will also work with

many other HPGL compatible printers.  It is useful to select this

mode and print to a file in order to export data traces for use in

other programs.

HP Laserjet 3:  This printer configuration uses the HPGL mode of

PCL-5. BUT, unless your printer has a bunch of memory it may not

work.  Use Laserjet Series II mode in that case.

Destination:  Selecting this option tells Tack where to find your

printer.  The destination can be to file, or the selected parallel

or serial port.  If the destination is file, then whenever you

print, the print job will go to a file in the format used by the

selected printer.  Printing to file is useful for exporting data

traces in a graphical format, using HPGL files.  If the destination

is a serial port, the menu that appears will allow you to configure

Tack to use the same serial port configuration as the serial

printer you have attached.

Orientation:  Selecting this option allows the user to set the

orientation for the print job, landscape (wide) or portrait

(narrow).

Settings:  Selecting this option allows the user to controls

settings for specific printers.  For dot matrix printers, this

allows you to set whether the print will be bold (two passes) or

normal.  For Laserjet printers, this allows you to set the print

resolution in dots per inch (dpi).

Okay:  Selecting this option causes any changes that have been made

to the printer configuration to be saved.

Cancel:  Selecting this option causes any changes that have been

made to the printer configuration to be cancelled.

Display Options:  Selecting this option allows the user to control

aspects of how data is displayed on the screen.

Show all pts in record:  If this is enabled then Tack will display

all of the data points for a given record on the screen.  That is,

if there are more data points in a record than there are pixels on

the screen horizontally then Tack will show multiple Y points for

each X point.  If this is disabled then Tack will only display one

Y point for each X point.  Disabling this feature causes the

display to draw faster.

Connect the points:  If this is enabled then Tack will connect the

data points on the screen with lines.  This causes the display of

data to look nicer, but significantly slows the display of data.

Grid and Axes:  Selecting this option allows the user to configure

how the grid and axes in the data window(s) are displayed.  There

are five items on this menu:

X Axis:  Selecting this option allows the user to specify where X

axes are to be placed in each data window.  The user can select a

variety of  X axes varying from none to three, appearing at the

top, middle and bottom of each window.

Y Axis:  Selecting this option allows the user to specify where Y

axes are to be placed in each data window.  The user can select a

variety of  Y axes varying from none to three, appearing at the

left, middle and right of each window.

Grid:  Selecting this option allows the user to specify the

appearance of the grid in the data window(s).  The grid can be

horizontal, vertical, both horizontal & vertical or neither.  The

spacing of the grid is determined by the number of divisions (see

next item).

Division:  Selecting this option allows the user to specify the

number of divisions for both the X and Y axes.  The number of

divisions determines where the ticks are placed on the X and Y axes

and the spacing of the grid.

Line Style:  Selecting this option allows the user to specify the

line style for the grid.  The line style is specified by a

hexadecimal number (0 -> FFFF) which specifies how the line is

displayed.  FFFF corresponds to a solid line, while 0 corresponds

to no line.  A value of  0F0F would make a dashed line, while 1

would make a faintly dotted line.

Okay:  Selecting this option exits the display options menu.

Other Options:  Selecting this option allows the user to control

several very important aspects of Tack's operation.  These are

described below:

XMS:  Enabling this feature allows Tack to use memory accessed via

the XMS specification in order to store data acquired before

writing it to disk.  XMS memory is memory above the 1 MB boundary

on PC-AT class computers (80286 and above class microprocessors).

This memory can only be accessed if you have installed the XMS

driver HIMEM.SYS (supplied free of charge by Microsoft), or a

compatible XMS driver (e.g. Desqview's QEMM memory manager) in your

config.sys file.  The use of XMS allows Tack to store data above

the conventional memory area, which allows more room for Tack to

work.  This feature need not be disabled unless you suspect a

problem or else have other needs.

Sounds:  Selecting this toggles the state of sound generation by

Tack.  If sounds are disabled, then Tack will make no noise.

Okay:  Selecting this causes any changes that have been made to the

items on this menu to be saved.

Cancel:  Selecting this causes any changes that have been made to

the items on this menu to be discarded.

Write Config File:  Selecting this option causes the current

configuration data to be written to the Tack.cfg configuration

file.  Only the items that are on controlled by config menu are

stored in this file.  These are:

- Graphics display adapter & resolution

- Colors

- Font

- Printer

- Sounds

- Display Settings (Show All Pts, Connect the Pts, Grid)

- XMS Usage

Conclusion

I hope that you understand all of the features that I have

described here and that Tack serves you well.  I would appreciate

your comments on the program and manual.  I always want to know

about bugs in my program and will endeavor to fix them as speedily

as is possible.  I am not responsible for any loss of data or

damage caused by this program.  I also would value your suggestions

for new features and ways to improve this program.  I can be

reached at the following addresses:

George B. Grant               (617) 625-2419 (Home Phone)

38 Banks St. #2               (617) 495-2599 (Work Phone)

Somerville, MA  02144

ggrant@husc.harvard.edu       (e-mail, over the internet)

or

george@mander.berkeley.edu

72722,3152                    (Compuserve address)