PLOTTER/PRINTER INSTALLATION

& PROBLEM DETERMINATION

A MICRO CADAM* Installation Guide Supplement

April 30, 1993

Copyright (C) 1988 by ALTIUM

An IBM Company

Charlotte, North Carolina 28257

This document shall not be reproduced

nor shall the information therein be

used by or disclosed to others except

as expressly authorized by ALTIUM,

An IBM Company.

ALTIUM is a registered trademark of ALTIUM,

An IBM Company.

MICRO CADAM is a trademark of ALTIUM,

An IBM Company.

Part I - Installation

SERIAL RS-232-C INTERFACE PLOTTERS

Check Hardware Connections

Cable:  Figures 1 & 2 illustrate the required data cable

configurations for stand-alone interface between MICRO CADAM

supported plotters and supported computers.  You should make or

order a cable specifically for your computer rather than trying to

set up a communications link using any spare RS-232-C cable that

you may have.  Because of the many misinterpretations of the

RS-232-C "standards" in the computer industry, a working cable on

one system may prove useless on another.

(COMPUTER) FEMALE DB-9        (COMPUTER) FEMALE DB-25TO

(PLOTTER) MALE DB-25       *           TO(PLOTTER) MALE DB-25

**

1    -----  4                 1     -----  1  SHIELD

2        -----2      2                 ----- 3

3    -----  3                 3     -----  2

4    -----  5 & 6             7     -----  7

5    -----  7                 5, 6, 8       -----

20

6 & 8-----  20

7    -----  8

Figure 1

CALCOMP COLORMASTER, HEWLETT PACKARD, HOUSTON INSTRUMENTS, IBM, &

NICOLET ZETA PLOTTERS

(COMPUTER) FEMALE DB-9        (COMPUTER) FEMALE DB-25

TO (PLOTTER) MALE DB-25                TO DB-25  (PLOTTER) MALE

1    -----  8                 Use the straight 25 to 25

pin cable     2            -----       3     that came with the

plotter and connect  3                 ----- 2      to the port

marked TO DTE HOST.  4                 ----- 20

5    -----  7

6    -----  6

7    -----  4

8                 ----- 5

9    -----  22

Figure 2

CALCOMP 1040GT SERIES PLOTTERS

*HP Cable #24542G for 9-pin serial interface (#24542H for HP 7550A Plotter only)

**HP Cable #13242G for 25-pin serial interface (#17255D for HP 7550A Plotter only)

Identify COM1, COM2 and COM3: To determine serial port addresses,

connect the cable between the computer and plotter and enter

dir>com1 at the DOS prompt.  If the cable is connected to the

serial port addressed as COM1 the system will return the DOS

prompt.  If the cable is not connected to COM1, the error message

"Error reading device COM1, abort, retry, ignore?" will result.

Enter a to let this abort, and then confirm the error by entering

dir>com2.  Return the DOS prompt.  If operating with an IBM PS/2

check for COM3 thru COM8, which are also valid serial port

addresses.

Appearance of a flashing curser (without the DOS prompt),

indicates an address conflict between serial ports -- consult your

computer's Serial/Parallel Adapter documentation to change

addresses.

Check Computer Communications Settings

The config.sys file of the root directory must include the

appropriate plotter device driver statement.The autoexec.bat file

(root directory) should include a path to DOS files, a set and a

mode command statement. These commands tell the system which

computer I/O port will be used and how the data is to be

transmitted to the plotter.   Sample file statements:

config.sys

buffers=20

files=20

shell=c:\command.com  /e:384 ip

device=c:\mcadam\mousesys.sys/g:mouse

device=c:\mcadam\plothp.sys

device=c:\mcadam\gsscgi.sys

break=on

autoexec.bat

cls                                                 echo off

echo micro cadam autoexec.bat boot file

prompt=$p$g

device=c:\mcadam\ibmrga12.sys

path=c:\;c:\dos;c:\mcadam;c:\mcadam\bin;c:\mcadam\gimsample

set mcadam=la03e8 /i4/dmouse /c 02f8 /s050

echo serial plotter port setup

set c:\mcadam\plothp.sys=com1

* mode com1:96,n,8,1,p

cd c:\mcadam

reset

* A common interface mode where:  address=com1;  baud rate=9600;

parity=no/off;  data bits=8;  stop bit=1; automatic restart for

buffer refill=p.   Caution: 9600 is the maximum baud rate.   The

plothp.sys drivers recognize the DOS statement set

orientation=portrait as a command to rotate output 90! when plot

data rotation is defined at 0! in the MICRO CADAM model.  Warning:

only 1 printer/plotter device driver may be loaded, and the

gsscgi.sys device driver statement must be the last device

statement listed.

Check Plotter Settings

External or internal programmable settings on the plotter tell it

how to receive data from the computer (i.e., eavesdrop or

standalone, remote or local, baud rate, odd/even/off parity, data

bits, stop bits).  Plotter communications settings must be set to

match the computer (Reference Part II - Plotter Settings).

PARALLEL INTERFACE PRINTERS

Check Hardware Connections

Data transmission to printers is normally done through the

parallel interface.  Consult your output device manual for set-up

instructions specific to your environment.

Cable:  Most printers use a standard Centronics parallel interface

cable.  Connect the female DB-25 connector to the computer's

parallel port addressed as LPT1 or LPT2 and the other end to the

proper receiving port on the printer.

Identify LPT1 and LPT2:  Determine which parallel port is being

addressed by entering dir>lpt1 at the DOS prompt.  If the cable is

connected to the parallel port addressed as LPT1 the current

directory will be generated on the printer and the system will

return the DOS prompt.  If the cable is not connected to the

parallel port addressed as LPT1, the  error message "Error reading

device LPT1, abort, retry, ignore?" will result.  Enter a to

abort.  Confirm the error by entering dir>lpt2.  This should

return the DOS prompt confirming the parallel port is addressed as

LPT2.

Check Computer Communications Settings

The config.sys file of the root directory must include the

required printer device driver file.  The autoexec.bat file of the

root directory must include a set command (the mode command is not

necessary), the instruction that tells the system which computer

I/O port will be used.  Sample files:

config.sys

device=c:\mcadam\vdidypgd.sys

device=c:\mcadam\epson100.sys

device=c:\mcadam\gsscgi.sys

break=on

autoexec.bat

cls

echo off

echo micro cadam autoexec.bat boot file

prompt=$p$g

path=c:\;c:\dos;c:\mcadam;c:\mcadam/bin;c:\mcadam\gimsampl

set mcadam=/a03e8/i4dmouse/c02f8/s050

set c:\mcadam\epson100.sys=lpt1

Check Printer Settings

Consult the device manufacturer's product support center or

operators manual for set-up instructions specific to your

environment.  Reference Part III - Printer Settings.

Part II - Plotter Settings

MICRO CADAM uses a hardwire or direct  handshake (not XON/XOFF or

ENK/ACK).  The following plotter settings are recommended when

plotting in a stand-alone environment.  Consult the device

manufacturer's product support center or operator manual for

set-up instructions specific to your environment.

CALCOMP COLORMASTER

Driver:  CLRMSTR.SYS

Graphics Mode:  RASTER

Switch 1:  1=UP (ON), 2=DN (OFF), 3=UP (ON), 4=DN (OFF)

Switch 2:  1-4 UP (ON)

CALCOMP PLOTMASTER

Driver:  CLRMSTR.SYS

Graphics Mode:  RASTER

Switch 1:  1=UP (ON), 2=DN (OFF), 3=UP (ON), 4=DN (OFF)

Switch 2:  1-4 UP (ON)

CALCOMP 1023

Driver:  CCP_PCI.SYS Filesize:  21848

Softkey Setup

Setup Submenu:  Host

Host Submenu  Format:  PCI

Data:  8 bit

1 stop

no parity

Baud:  9600

Handshake:  DirCntrl

Sync:  # of Sync/1

Sync char - 2

Chksum:  disabled

End Msg:  3

CALCOMP 104X GT SERIES (1041GT, 1042GT, 1043GT, & 1044GT*)

Driver:  CCP_PCI.SYS

Switch S1:  1=0 (OFF), 2=1 (ON), 3=0 (OFF), 4=1 (ON), 5=0 (OFF),

6=0 (OFF), 7 & 8 NOT APPLICABLE

SoftKey Setup Feature 7

Sub Menu 0: PARITY=0 (none), BITS/CHAR=8

Sub Menu 1: STOP BITS=1, CLOCK=0 (internal)

Sub Menu 2: INTERFACE=0 (serial)

Sub Menu 3: HOST BAUD RATE=9600, MODE=0 (PCI)

Sub Menu 4: TERM MUTING=NO, CKSUM ENABLE=NO

Sub Menu 5: ISOCHRONOUS=NO, EOM=03

Sub Menu 6: DIRECT CONTROL=YES, XON/XOFF=NO

Sub Menu 7: TERM BAUD RATE=9600, DUPLEX=0 (full)

Sub Menu 8: SYNC CODES=1, SYNC CODE VALUE=002

Note:  Specify SET PAPER_SIZE statement as instructed by MICRO

CADAM Installation Guide

HEWLETT PACKARD 7440A

Driver:  PLOTHP.SYS  Filesize:  27228

Switch:  S2=0, S1=0, A4/US=0 or 1, B4=1, B3=0, B2=1, B1=0

HEWLETT PACKARD 7470A

Driver:  PLOTHP.SYS

Switch:  S2=0, S1=0, Y=0, A4/US=0 or 1, B4=1, B3=0, B2=1, B1=0

HEWLETT PACKARD 7475A

Driver:  PLOTHP.SYS

Switch:  S2=0, S1=0, Y=0, US=1, A4/A3=0 or 1, B4=1, B3=0, B2=1,

B1=0

HEWLETT PACKARD 7550A

Driver:  PLOTHP.SYS

SoftKey Setup Features

Menu 1: Mode = ENHANCED

Menu 2: Environment = STANDALONE

Menu 3: Handshake = HARDWIRE

Menu 4: Connection = DIRECT

Menu 5: Duplex = FULL

Menu 6: Connection = DIRECT

Menu 7: Parity = 8 BITS & OFF

Menu 8: Baud Rate = 9600

HEWLETT PACKARD 7570A DRAFTPRO

Driver:  PLOTHP.SYS

Switch 1:  0=OFF, 1=ON, 2=OFF, 3=ON, 4=OFF, 5=OFF, 6=OFF, 7=ON,

8=ON, 9=OFF, 10=OFF,                    11=OFF

HEWLETT PACKARD 7580B & 7585B (Cut-sheet)

Driver:  PLOTHP.SYS

Interface Mode:  RS-232-C

Baud Rate:  9600

Switch 1:  OFF/ODD/DUPLEX-FULL/HARDWIRE/NORMAL

Switch 2:  EXPAND/NORMAL/STANDALONE/NORMAL/NORMAL

HEWLETT PACKARD 7586B (Cut sheet and roll-feed)*

Driver:  PLOTHP.SYS

Interface Mode:  RS-232-C

Baud Rate:  9600

Switch 1:  OFF/ODD/DUPLEX-FULL/HARDWIRE/NORMAL

Switch 2:  NORMAL/EXPAND/NORMAL/STANDALONE/NORMAL/NORMAL

HEWLETT PACKARD 7595A DRAFTMASTER I  (Cut-sheet) &

HEWLETT PACKARD 7596A DRAFTMASTER II (Cut-sheet and roll-feed) *

Driver:  PLOTHP.SYS

from SERIAL SoftKey Setting                     from SERIAL SoftKey

Setting                   from MAIN SoftKey Setting

Dataflow: REMOTE & STANDALONE                   Bypass: OFF

Rotate: OFF

Baud: 9600                Monitor: OFF          Invert: OFF

Parity: 0                 Duplex: FULL          Expand: ON

Hardwire: ON              Auto-Disc: OFF        Emulate: ON

HOUSTON INSTRUMENTS DMP-29 & DMP-50 SERIES (51, 51MP, 52, 52MP,

56, 56A, 56C)

Driver:  HIDMPL.SYS        Filesize:  10376

State 2 Mode: Baud Rate=9600, URAT Parity Bit 8=0,

Handshake=TOGGLE, Pass Through Port Option=TOGGLE

Note:  Driver is hardcoded to communicate through COM1 only.

HOUSTON INSTRUMENTS DMP-40 SERIES (40, 41, 42) --  UNTESTED BY

ALTIUM, An IBM Company.

Driver:  HIDMPL.SYS

State 2 Mode: Baud Rate=9600, URAT Parity Bit 8=0,

Handshake=TOGGLE, Pass Through Port Option=TOGGLE

Note:  Driver hardcoded to COM1 only.

IBM 6180

Driver:  PLOTHP.SYS

Switch:  S2=0, S1=0, Y=0, A4/US=0 or 1, B4=1, B3=0, B2=1, B1=0

IBM 6184

Driver:  PLOTHP.SYS

Switch 1:  0=OFF, 1=ON, 2=OFF, 3=ON, 4=OFF, 5=OFF, 6=OFF, 7=ON,

8=ON, 9=OFF, 10=OFF, 11=OFF

IBM 6186-1 COLOR PLOTTER (Cut-sheet) &

IBM 6186-2 COLOR PLOTTER (Cut-sheet and roll-feed)*

Driver:  PLOTHP.SYS

from SERIAL SoftKey Setting                     from SERIAL SoftKey

Setting                   from MAIN SoftKey Setting

Dataflow: REMOTE & STANDALONE                   Bypass: OFF

Rotate: OFF

Baud: 9600                Monitor: OFF          Invert: OFF

Parity: 0                 Duplex: FULL          Expand: ON

Hardwire: ON              Auto-Disc: OFF        Emulate: ON

IBM 7371

Driver:  PLOTHP.SYS

Switch:  S2=0, S1=0, Y=0, A4/US=0 or 1, B4=1, B3=0, B2=1, B1=0

IBM 7372

Driver:  PLOTHP.SYS

Switch:  S2=0, S1=0, Y=0, US=1, A4/A3=0 or 1, B4=1, B3=0, B2=1, B1=0

IBM 7374

Driver:  PLOTHP.SYS

Interface Mode:  SERIAL

Baud Rate:  9600

Switch 1:  OFF/ODD/DUPLEX-HALF/EXPAND/NORMAL

IBM 7375-1 (Cut-sheet)

Driver:  PLOTHP.SYS

Interface Mode:  SERIAL

Baud Rate:  9600

Switch 1:  OFF/ODD/DUPLEX-HALF/DIRECT CONNECT/NORMAL

Switch 2:  EXPAND/NORMAL/STANDALONE/NORMAL/NORMAL

IBM 7375-2 (Cut-sheet and roll-feed)*

Driver:  PLOTHP.SYS

Interface Mode:  SERIAL

Mode #6 HPGL P67 Option

Driver:  PLOTHP.SYS

SW 01:  1=UP, 2=DN, 3=DN, 4=UP, 5=UP, 6=UP, 7=UP, 8=UP

SW 02:  1=8 UP

SW 03:  1=UP, 2=UP, 3=UP, 4=UP, 5=UP, 6=UP, 7=UP, 8=UP

* CCP_PCI.SYS and PLOTHP.SYS drivers are capable of multiple-frame

plot output on roll-feed models.  Enter SET NFRAMES=X (where X is the

number of 45" frames required) in the DOS environment before the plot

program is executed.  Default frame is 1.

JDL 850 EWS (with GL-Processor Firmware Rev. 2.0 or later)

Driver  PLOTHP.SYS

I/O1:  128K

FUNC:  PLOT

I/O2:  9600, PNON, DB8, STP1, DTR, 32K

PEMU: 758E

PLOT:  PC10

NICOLET ZETA 824 & 836 with P63 Option

Mode #6 HPGL/P63 Option

Driver:  PLOTHP.SYS

SW 01:  1=UP, 2=DN, 3=DN, 4=UP, 5=UP, 6=UP, 7=UP, 8=UP

SW 02:  1=UP, 2=DN, 3=UP, 4=UP, 5=UP, 6=UP, 7=UP, 8=DN

SW 03:  1=UP, 2=DN, 3=DN, 4=UP, 5=UP, 6=DN, 7=DN, 8=DN

WESTERN GRAFTEC MP2300

Driver:  PLOTHP.SYS

RS-232-C Switch S2:  1=ON, 2=OFF, 3=ON, 4=ON, 5=OFF, 6=OFF, 7=OFF,

8=ON, 9=OFF, 10=OFF

Part III - Printer Settings

Consult the device manufacturer's product support center or operator

manual for set-up instructions specific to your environment.  Note

that some printers have options that allow emulation of other

supported devices -- use the device driver required for the printer

being emulated.  Printers require a standard 25 pin parallel cable

unless noted.

EPSON 80 SERIES (RX-80, MX-80, FX-80, & FX-85)

Driver:  EPMX80.SYS

EPSON 100 SERIES (FX-100, RX-100, FX-185)

Driver:  EPMX100.SYS

Note:  The EPMX100.SYS driver produces a maximum plotting frame of

10.25" x 13".

EPSON FX-286 & FX-286E

EPSON Mode

Driver:  EPMX100.SYS

Switch Panel 1:  1=OFF, 2=OFF, 3=OFF, 4=ON, 5=OFF, 6=ON, 7=ON, 8=ON

Switch Panel 2:  1=ON, 2=OFF, 3=OFF, 4=OFF

Note:  The EPMX100.SYS driver produces a maximum plotting frame of

10.25" x 13".

HEWLETT PACKARD THINKJET

Cable:  Figure 1

Driver:  THINKJET.SYS                           Filesize:  41000

Warning: Device currently unsupported - driver too big to load into

RAM memory!

HEWLETT PACKARD LASERJET SERIES II

Driver:  LASERJET         Filesize:  49144

The laserjet.sys recognizes the SET RESOLUTION=XXX command line.  The

XXX may be either 75, 100, 150, or 300.  These numbers are the

resolution in dots per inch (dpi).  The higher the number the clearer

the picture, but the longer the time required to plot.

IBM PROPRINTER 4201 B/W & PROPRINTER XL 4202 B/W

Driver:  IBMGPR.SYS       Filesize:  24700

Switch:  1=OFF, 2=OFF, 3=OFF, 4=OFF, 5=OFF, 6=ON (line feed)

Note:  Driver allows narrow carriage output only!

IBM GRAPHICS PRINTER 5152

Driver:  IBMGPR.SYS

IBM COLOR PRINTER 5182

Driver:  IBMPRCOL.SYS     Filesize:  39876

JDL 850 EWS

Driver:  IBMGPR.SYS (IBM mode) or EPMX100.SYS (Epson mode)

EMUL:  IBM or EPSN

FUNC:  PR-P (Centronics parallel interface)

PLOT:  PC10

OKIDATA MICROLINE 84 & 93

Driver:  OKID84.SYS       Filesize:  25860

OKIDATA MICROLINE 92

Driver:  OKID92.SYS       Filesize:  23552

PRINTRONIX MVP, P300, & P600

Driver:  PRNTRX.SYS       Filesize:  26216

STROBE 100, 200, & 260

Driver:  STROBE.SYS

TOSHIBA P351

Driver:  TOS1350.SYS      Filesize:  31620

Note:  The TOS1350.SYS driver produces a maximum plotting frame of

10.25" x 13".