Using a Super VGA Graphics Board with MICRO CADAM

===== = ===== === ======== ===== ==== ===== =====

The SET VGA statment in your AUTOEXEC.BAT file may need to be

changed to allow MICRO CADAM to operate properly with your

particular graphics board.  If you selected SUPER VGA from

the MCINSTAL program, the following SET statement will appear

in your AUTOEXEC.BAT:

SET VGA=800,600,58h

If you have a PARADISE VGA board, this setting should work

as is.  Other VGA boards will need different settings.

The format of the SET VGA command is as follows:

SET VGA,x,y,mode

X    - is the horizontial resolution

Y    - is the vertical resolution

MODE - is the Video BIOS mode setting value for that intermediate mode

These values can usually be found in the programmer section of the user's

manual for the graphics board.  The values of X, Y, and MODE can be specified

in either decimal or hex.  Hex values must have an "h" appended to the end.

Some example settings are as follows:

Graphics Card               Command

=============               ===================

A11                         SET VGA=640,480,12h

ATI VGA WONDER              SET VGA=800,600,54h

EIZO Professional MD-B07    SET VGA=800,600,58h

EIZO Professional MD-B10    SET VGA=800,600,29h

Genoa SuperVGA HiRes-10*    SET VGA=800,600,79h

Orchid's Designer VGA**     SET VGA=800,600,29h

Paradise VGA Professional   SET VGA=800,600,58h

STB Systems VGA Extra       SET VGA=800,600,29h

Trident TVGA                SET VGA=800,600,5bh

* Valid only on Genoa SuperVGA HiRes-10 BIOS Version 2.0 and above

** Some older Orchid's Designer VGA boards must be modified by

Orchid to run at this mode

For additional information, call your graphics board vendor.