This is the documentation for PMODE 3.0 DPMI/VCPI/XMS/raw protected mode

interface kernel. Copyright (c) 1994, Tran (a.k.a. Thomas Pytel). PMODE is

publicly available and is not confidential or proprietary. I, Thomas Pytel,

reserve all rights to the source code. However, feel free to use or distribute

it in any manner you wish. All I ask, if you use this code in some production,

is credits for it.

------------------------------------------------------------------------------

Contents:

---------

0 - Introduction

0.0 - Disclaimer

0.1 - Description

1 - Overview

1.0 - Initialization and termination

1.1 - Segments, selectors, and descriptors

1.2 - Stacks and mode switching

1.3 - Interrupts

1.4 - Real mode callbacks

1.5 - PMODE specifics

1.6 - PMODE internal stacks

2 - Functions

2.0 - Function 0000h - Allocate Descriptors

2.1 - Function 0001h - Free Descriptor

2.2 - Function 0003h - Get Selector Increment Value

2.3 - Function 0006h - Get Segment Base Address

2.4 - Function 0007h - Set Segment Base Address

2.5 - Function 0008h - Set Segment Limit

2.6 - Function 0009h - Set Descriptor Access Rights

2.7 - Function 000Ah - Create Alias Descriptor

2.8 - Function 000Bh - Get Descriptor

2.9 - Function 000Ch - Set Descriptor

2.10 - Function 000Eh - Get Multiple Descriptors

2.11 - Function 000Fh - Set Multiple Descriptors

2.12 - Function 0200h - Get Real Mode Interrupt Vector

2.13 - Function 0201h - Set Real Mode Interrupt Vector

2.14 - Function 0204h - Get Protected Mode Interrupt Vector

2.15 - Function 0205h - Set Protected Mode Interrupt Vector

2.16 - Function 0300h - Simulate Real Mode Interrupt

2.17 - Function 0301h - Call Real Mode Procedure With Far Return Frame

2.18 - Function 0302h - Call Real Mode Procedure With IRET Frame

2.19 - Function 0303h - Allocate Real Mode Callback Address

2.20 - Function 0304h - Free Real Mode Callback Address

2.21 - Function 0305h - Get State Save/Restore Addresses

2.22 - Function 0306h - Get Raw Mode Switch Addresses

2.23 - Function 0400h - Get Version

2.24 - Function 0500h - Get Free Memory Information

2.25 - Function 0501h - Allocate Memory Block

2.26 - Function 0502h - Free Memory Block

2.27 - Function 0503h - Resize Memory Block

2.28 - Function 050Ah - Get Memory Block Size and Base

2.29 - Function 0900h - Get and Disable Virtual Interrupt State

2.30 - Function 0901h - Get and Enable Virtual Interrupt State

2.31 - Function 0902h - Get Virtual Interrupt State

3 - Miscellaneous

3.0 - Updates

3.1 - Glossary

3.2 - Differences among modes

3.3 - Notes

3.4 - Final word

------------------------------------------------------------------------------

0 - Introduction:

-----------------

This document will not attempt to explain the workings of protected mode. If

you are new to protected mode coding, I suggest you get yourself a good book.

I also suggest you get your hands on some good DPMI documentation as a

reference and background for understanding PMODE. This document is only

intended to explain the workings of PMODE for the purpose of using it directly

or for writing a shell or high level language interface.

0.0 Disclaimer:

---------------

Legal:

I exclude any and all implied warranties, including warranties of

merchantability and fitness for a particular purpose. I make no warranty or

representation, either express or implied, with respect to this source code,

its quality, performance, merchantability, or fitness for a particular

purpose. I shall have no liability for special, incidental, or consequential

damages arising out of or resulting from the use or modification of this

source code.

English:

If you fuck up, its your own problem.

0.1 Description:

----------------

PMODE 3.0 is basically a DOS extender. It allows DOS programs to run in full

protected mode. PMODE will take care of all the system details, the descriptor

tables, extended memory management, interrupts, etc... It does not matter what

kind of system is already in place. DPMI, VCPI, XMS, and a clean system will

all be handled appropriately. If DPMI is in place, PMODE will do basically

nothing, and your code will be talking directly to the DPMI host. But if it is

not, PMODE will provide a subset of DPMI functionality to your code. It is

slightly annoying that most of the DPMI interface uses pairs of 16bit

registers for 32bit values. This goes back to the 80286 support of DPMI.

Code using PMODE can set up its own descriptors. It can run across real

mode, 16bit protected mode, and 32bit protected mode. Full extended memory

management is provided. Blocks of extended memory can be allocated, resized,

and freed. PMODE does not manage low memory, Your code is responsible for the

memory below the 1M boundary. The PMODE kernel is well suited for a shell to

extend its functionality. A shell to provide extended or simplified services

to assembly or high level code.

I wrote PMODE with attention to speed. Wherever I could control it, I tried

to ensure that code using PMODE would run as fast as possible. I also tried to

make sure PMODE is very stable. Under DPMI, your code is a slave to the DPMI

host's whims. Almost definately running at CPL 3, which makes it quite slow.

There is nothing I can do about that. Well, locating and messing with the DPMI

host's system tables might not be too hard, but too unpredictable. Running in

protected mode at CPL 3 is better than real mode CPL 3, because you can avoid

loading segment registers very often (which is slow) by setting up flat

memory. However, if DPMI is not present, you can be sure your code will be

running as fast as it possibly can. Under a VCPI, XMS, or raw system, PMODE

will run your code at CPL 0. There are no I/O permission bitmap checks on port

accesses or task switching. Under an XMS or raw system, real mode calls are

executed in actual real mode rather than the slower V86 mode, which is

actually protected mode at CPL 3. Under VCPI, real mode calls are executed in

V86 mode, which is what the VCPI server normally runs DOS in. Also, under

VCPI, paging is enabled. It is a minor speed degredation factor, but one that

is avoided under XMS or raw.

I've been coding 386 protected mode systems for a couple of years now. I

tried to make PMODE as clean as possible. But it was coded from scratch.

Though I tested it extensively, there is unfortunately always the possibility

of a bug. I am pretty confident it is clean though, because I went over EVERY

line of code when finally done with it. I also plugged it into some old

programs using a previous version of PMODE, which did well to help me find a

few bugs.

------------------------------------------------------------------------------

1 - Overview:

-------------

I adopted the DPMI interface for PMODE to make code that works with PMODE

very portable to other extenders. Only a subset of DPMI is supported, both to

keep the size of the kernel down, and because I am lazy (the latter being the

more important factor). The interface to PMODE is INT 31h in protected mode.

Functions are available for descriptors, interrupt vectors, extended memory,

real mode callbacks, and calling real mode interrupts and procedures. PMODE

works as a subset of DPMI 1.0 rather than DPMI 0.9. This means that error

codes are returned from unsuccessful function calls, and some functions are

available that are not available under DPMI 0.9. Note though, that if the

system is already under DPMI 0.9, PMODE code is not active, so error codes

will not be returned and only DPMI 0.9 functions will be available.

1.0 - Initialization and termination:

-------------------------------------

There are only two functions immediately callable in PMODE. They are

_pm_info and _pm_init. _pm_info returns some information about the current

type of system and the low memory requirements for protected mode. No matter

what the system, DPMI, VCPI, XMS, or raw, a low memory buffer is required for

protected mode operation. The size of this buffer is returned from _pm_info.

Your code is responsible for providing that buffer to _pm_init.

_pm_init switches the system into protected mode. If DPMI is in place, all

that is done is a switch into 32bit protected mode. If the DPMI host does not

support 32bit protected mode, _pm_init will return an error. 32bit protected

mode does not necessarily mean your code has to run in a 32bit segment with

default 32bit instructions, it just means that it is possible. Since DPMI is

defined for 80286 computers, you might even find DPMI that is capable of 32bit

protected mode refusing this request on a 386 system.

_pm_init will return with the carry flag set and an error code in AX if an

error ocurred while trying to switch into protected mode. If the switch to

protected mode was successful, the carry flag will be clear, and the system

will be in protected mode. The CS segment register will have been converted

to a protected mode selector corresponding to a descriptor mapping the same

memory as it did in real mode. Likewise, the DS and SS segment registers will

be converted to selectors. If DS and SS were equal before the call to

_pm_init, the same selector may be returned in both. ES will contain a

selector for your program's PSP. The environment segment at PSP:2ch will also

be converted to a selector if it was a non-zero value before the call to

_pm_init. FS and GS will be returned as 0 (NULL selector). Your code will now

be running in a 16bit protected mode code segment, with full access to the

protected mode INT 31h functions. If the system is DPMI, this is the last your

code will have talked to PMODE, and from now on, will be using the DPMI host

directly.

Both functions are FAR, and the full calling format is as follows:

) _pm_info - Get protected mode info:

In:

None

Out:

AX - return code:

0000h - successful

0001h - no 80386+ detected

0002h - system already in protected mode and no VCPI or DPMI found

0003h - DPMI - host is not 32bit

CF - set on error, if no error:

BX - number of paragraphs needed for protected mode data (may be 0)

CL - processor type:

03h - 80386

04h - 80486

05h - 80586

06h-FFh - reserved for future use

CH - protected mode type:

00h - raw

01h - XMS

02h - VCPI

03h - DPMI

) _pm_init - Initialize protected mode:

In:

ES - real mode segment for protected mode data (ignored if not needed)

Out:

AX - return code:

0000h - successful

0001h - no 80386+ detected

0002h - system already in protected mode and no VCPI or DPMI found

0003h - DPMI - host is not 32bit

0004h - could not enable A20 gate

0005h - DPMI - could not enter 32bit protected mode

0006h - DPMI - could not allocate needed selectors

CF - set on error, if no error:

ESP - high word clear

CS - 16bit selector for real mode CS with limit of 64k

SS - 32bit selector for real mode SS with limit of 64k

DS - 32bit selector for real mode DS with limit of 64k

ES - 32bit selector for PSP with limit of 100h

FS - 0 (NULL selector)

GS - 0 (NULL selector)

The CS, DS, and SS selectors returned from _pm_init can be modified or

freed by your code. The PSP selector and the converted environment selector in

the PSP may not be. There is a special case when the CS selector returned from

_pm_init may not be freed or modified. That is when the FAR CALL to _pm_init

came from the PMODE_TEXT segment. In this case, PMODE will return its own code

selector which maps PMODE_TEXT. This is just a minor optimization provided for

any shell that has its code in PMODE_TEXT to save a descriptor.

To terminate under PMODE, your code must issue an INT 21h function 4ch in

protected mode. Just like in real mode, AL is the return code. Your code

should only terminate from the main stream of execution. That is, do not try

to quit from a protected mode IRQ handler or a real mode callback. Before

termination, your code has the following responsibilities:

) Restore any real mode interrupt vectors which were hooked.

) Free any extended memory blocks that were allocated.

1.1 - Segments, selectors, and descriptors:

-------------------------------------------

As you know (I hope), in protected mode, selectors are used in place of

actual segment values in segment registers. Selectors are basically indexes

into system tables which contain all the information about segments in

descriptors. You can think of selectors as handles to segments. They are

independent of the actual location and size of the segment in memory.

Under PMODE, your code can allocate its own selectors and descriptors. You

can set up code segments, data segments, and your own stack segments. These

segments can be either 16bit, 32bit, or a mix. The best use of the flexibility

of protected mode is to set up very large segments, in effect, eliminating the

need for segmentation. You can set up a code descriptor, and set its size to

4G. Then a data descriptor of that same size and with the same base address.

When all memory is addressable from a single segment, there is little need for

other segments. But they are available, possibly for code modules to be loaded

from disk into seperate segments.

After allocating a descriptor, you code can set it up in one shot, with

Set Descriptor function (000ch). Or you can set the base address, limit, and

access rights/type seperately. You can also allocate a descriptor and have it

automatically set to the same base address and size as another descriptor

using the Create Alias Descriptor function (000ah). You code can also read a

whole descriptor or base address of a descriptor. There is no Get Segment

Limit function because the LSL instruction performs that function. The LAR

instruction returns the access rights/type of a descriptor.

Technically, it is possible for DPMI to deny requests to set up very large

segments for protection reasons. But no DPMI host that I know of does this.

They all do protection at the paging level. DPMI 1.0 specifies a function

that returns an absolute ceiling for large segments. Allowing for flat memory,

but falling short of the full 4G linear address range. I would not worry about

this. If DPMI 1.0 hosts started to deny requests to set segment sizes to 4G,

many protected mode extended programs would cease to function.

When setting up a data descriptor which will be used as a stack segment, be

aware that the B bit will determine whether PUSHes and POPs on that stack use

SP or ESP as the top of stack pointer. However, even if you use a stack with

the B bit clear (using SP), ESP should still be the top of stack pointer.

Which means that when using a 16bit stack segment, the high word of ESP MUST

be clear.

1.2 - Stacks and mode switching:

--------------------------------

Within your main stream of execution, your code can set up its own stack.

But there are times when a stack is provided to your code and your code should

stay on that stack. At those times, your code may switch stacks during

processing, but should return on the same stack it was called. This is during

servicing of hardware interrupts or real mode callbacks in protected mode.

Switching between protected mode and real mode can be accomplished in one of

many ways. In protected mode, the default IRQ handlers switch to real mode to

execute the real mode handler for the specific IRQ that was called. A software

INT instruction in protected mode is also, by default, sent to on to real mode

for processing. There are three specific functions which allow you to call

real mode interrupts and procedures in a much more structured manner. There

are real mode callbacks. These are basically addresses in real mode which,

when called in real mode, transfer control to protected mode routines defined

by your code. And finally, there is raw mode switching. Your code can obtain

the addresses of a real mode routine which will switch the system into

protected mode, and a protected mode routine which will switch the system into

real mode. This is the lowest level, and quickest, method of switching modes.

IRQ and INT redirection is discussed later, as are real mode callbacks.

The INT 31h functions 0300h, 0301h, and 0302h allow your code to call real

mode interrupts of FAR routines. Since protected mode selectors are not valid

in real mode, your must pass the values to load into the segment registers in

real mode, for the interrupt or routine, in a memory structure. This structure

also contains the general registers as you want to pass them to real mode.

Using these INT 31h functions, you can specify a portion of data from the

protected mode stack to be put on the real mode stack for the interrupt or

procedure call. You also do not have to provide a real mode stack. If you set

the SS and SP fields in the register structure to 0, PMODE will provide a real

mode stack for the real mode call. If you prefer, however, you can provide the

stack yourself. Upon return from the real mode call, the register structure

will contain the values that were passed back in the registers from the real

mode interrupt handler or procedure. The CS, IP, SS, and SP fields will remain

unmodified though.

Using the raw mode switching routines is the fastest way to switch between

modes. However, if these functions are to be used, special measures must be

taken to preserve the state of the system. If using raw mode switching, you

must use the state save/restore fuctions whose addresses you can obtain with

INT 31h function 0305h. The state is saved in a buffer you provide. The stack

is a good place for this buffer. Some example code is in order:

buffersize      dd      ?               ; size of state buffer

pmstate         df      ?               ; selector:offset of state routine

pmtorm          df      ?               ; selector:offset of switch routine

rmstate         dd      ?               ; segment:offset of state routine

rmtopm          dd      ?               ; segment:offset of switch routine

; this code gets and stores the addresses of the various routines

mov ax,305h                     ; get addresses of state save/restore

int 31h                         ;  routines

movzx eax,ax                    ; zero high word of EAX

mov buffersize,eax              ; size of state buffer

mov word ptr rmstate[0],cx      ; offset of real mode state routine

mov word ptr rmstate[2],bx      ; segment of real mode state routine

mov dword ptr pmstate[0],edi    ; offset of protected mode routine

mov word ptr pmstate[4],si      ; selector of protected mode routine

mov ax,306h                     ; get addresses of mode switch

int 31h                         ;  routines

mov word ptr rmtopm[0],cx       ; offset of real mode switch routine

mov word ptr rmtopm[2],bx       ; segment of real mode switch routine

mov dword ptr pmtorm[0],edi     ; offset of protected mode routine

mov word ptr pmtorm[4],si       ; selector of protected mode routine

; this code saves the state and jumps to real mode

sub esp,buffersize              ; allocate buffer space on stack

mov edi,esp                     ; set ES:EDI = SS:ESP, buffer address

mov ax,ss

mov es,ax

xor al,al                       ; set AL = 0, save state

call pmstate                    ; save state

mov ax,real_mode_DS_value       ; set values for real mode registers

mov cx,real_mode_ES_value

mov dx,real_mode_SS_value

mov bx,real_mode_SP_value

mov si,real_mode_CS_value

mov di,real_mode_IP_value

jmp pmtorm                      ; switch to real mode

; this would restore the state after a return from real mode

mov edi,esp                     ; set ES:EDI = SS:ESP, buffer address

mov ax,ss

mov es,ax

mov al,1                        ; set AL = 1, restore state

call pmstate                    ; restore state

add esp,buffersize              ; discard stack buffer space

Real mode code to save/restore the state and call protected mode would be

similar, except that EBX would be used to pass a value for ESP, and EDI would

be used to pass EIP rather than IP in switching modes. Also, ES:DI would be

used as the state buffer rather than ES:EDI.

1.3 - Interrupts:

-----------------

When protected mode is first entered, all interrupts except those providing

DPMI functionality are directed to a handler which will pass them on to real

mode. That is, a software INT instruction executed by your code in protected

mode will cause the CPU to be switched to real mode and the interrupt will be

re-issued in real mode. After the interrupt handler returns, the system will

be switched back to protected mode. All general registers (EAX, EBX, ECX, EDX,

ESI, EDI, and EBP) in protected mode are passed on to the real mode handler,

and the general registers and flags are passed back from real mode. The

segment registers are not passed on to real mode since segment registers have

different meanings in protected mode and real mode. This means that you can

call simple interrupt routines which do not require values in segment

registers, such as the keyboard BIOS function 0 to read a character from the

keyboard, by just setting AH to 0 and issuing an INT 16h in protected mode. If

you need to pass segment registers to a real mode interrupt handler, you must

use the INT 31h function 0300h.

IRQs are likewise passed on to real mode. PMODE will not, for the sake of a

little speed increase, pass any registers to or from a real mode IRQ handler.

A real DPMI host probably will pass the general registers just as it would for

a software INT instruction. You may hook a protected mode interrupt vector for

any interrupt, 0-0ffh, and process the interrupt entirely in protected mode if

you wish. Or you can do some processing in protected mode, then chain to the

real mode handler by passing control to the previous handler for the interrupt

your code hooked.

If an IRQ occurs in real mode, by default, it will not be passed on to its

protected mode handler. Rather, it will go directly to its real mode handler.

Under a real DPMI host, the IRQ will probably be sent on to its protected mode

handler first. If you wish send IRQs that occur in real mode to a protected

mode handler, set up a real mode callback for that interrupt vector. Or faster

than that, install your own real mode routine which will switch to your

protected mode code using the raw mode switching routines.

PMODE will provide a real mode stack for both the software INT redirection

and the hardware IRQ redirection to real mode. If a real mode stack is

unavailable because of too many nested calls to real mode, the PC speaker will

be turned on and the machine will be hung. I prefer this to clunky exception

code messing up my nice and pretty extender.

In protected mode, when interrupt handlers are called, the interrupt flag is

not disabled like it is in real mode. This is done only for IRQs and

interrupts 0-7. Handlers for all other interrupts in protected mode must not

assume the interrupt flag has been cleared for them. If they need interrupts

disabled, they must do it themselves.

Under DPMI, the interrupt flag may need to be virtualized. I will spare you

a long explanation because any good DPMI text will give that to you. But I

will give you some rules:

) You must not assume anything about instructions that would normally affect

the interrupt flag. Instructions like POPF and IRETD may have no effect on

the current status of the interrupt flag. Also, PUSHF or an INT may not

store the interrupt flag correctly, so do not trust them to get information

on the interrupt flag.

) The only things you can be sure will affect the interrupt flag are CLI, STI,

and INT 31h functions 0900h and 0901h.

) If you need to learn the current status of the interrupt flag, you must use

one of the INT 31h functions, 0900h to clear and get the status of the

interrupt flag, 0901h to set and get the status of the flag, or 0902h which

simply returns the current value of the interrupt flag.

) Since IRETD may not affect the interrupt flag, you should re-enable

interrupts in a protected mode IRQ handler. This is because the interrupt

flag will have been cleared upon entry to the handler.

When writing interrupt handlers, you must terminate them with an IRETD, not

a simple IRET. For hardware IRQ handlers, you may want to make sure that any

code and data that may be touched by the IRQ handler resides in low memory,

below 1M. This memory is locked under DPMI, and will prevent having to swap

from disk at interrupt time under DPMI hosts which support virtual memory. One

other thing you should remember about IRQ handlers. IRQ 2 is really IRQ 9.

Devices which say they use IRQ 2 are actually using IRQ 9. In real mode, the

BIOS handler for IRQ 9 redirects it to the handler for IRQ 2. There is no such

redirection done in protected mode. So if you wish to write a handler for

IRQ 2 in protected mode, you must put it on IRQ 9. And remember to send the

EOI to the second interrupt controller. The BIOS IRQ 9 handler does that, but

you don't have the BIOS anymore.

1.4 - Real mode callbacks:

--------------------------

Real mode callbacks allow code running in real mode to call protected mode

procedures in a transparent manner. The real mode code thinks it is passing

control to another real mode procedure. This is, in reality, a real mode

callback. Which is basically a small routine which stores the values of the

real mode registers in a structure, then switches to protected mode and passes

control to a protected mode routine you specify.

Callbacks are allocated and freed just like descriptors or memory. When

allocating a callback, your code specifies the address of the protected mode

routine that is to gain control when the real mode callback is called. You

must also specify the selector:offset of a memory structure that is to recieve

the contents of the registers in real mode. The format of this structure is

the same as for INT 31h functions 0300h, 0301h, and 0302h. When the protected

mode routine for a real mode callback gets control, interrupts will be

disabled, and the following registers are defined:

DS:ESI - selector:offset corresponding to real mode SS:SP

ES:EDI - selector:offset of real mode register data structure

SS:ESP - protecterd mode stack provided by PMODE or the DPMI host

The real mode register data structure has the following format:

Offset  Length  Contents

00h     4       EDI

04h     4       ESI

08h     4       EBP

0ch     4       reserved, ignored

10h     4       EBX

14h     4       EDX

18h     4       ECX

1ch     4       EAX

20h     2       CPU status flags

22h     2       ES

24h     2       DS

26h     2       FS

28h     2       GS

2ah     2       IP, undefined

2ch     2       CS, undefined

2eh     2       SP

30h     2       SS

All fields except the CS and IP are filled in with the contents of the real

mode registers when the real mode callback got control. The protected mode

callback procedure can extract its parameters from the register data structure

and/or the real mode stack. Remember that the segment registers contain real

mode segment addresses, not protected mode selectors.

The protected mode callback procedure exits with an IRETD with the address

of the real mode register data structure in ES:EDI. Information can be passed

back to real mode by modifying the contents of the register data structure and

the real mode stack. The protected mode callback routine is responsible for

setting the correct address for the resumption of execution in real mode in

the CS:IP fields of the register data structure. It is also responsible for

updating the SS:SP fields in the register data structure to remove the address

of the calling real mode routine from the real mode stack.

The real mode register data structure and the DS selector used to map the

real mode SS segment are static. This is not a problem if you leave interrupts

disabled throughout the protected mode callback routine. But if you intend to

re-enable interrupts, you must make sure you do not use the DS selector

anymore. If you need to access the real mode stack after enabling interrupts,

you must create an alias descriptor for the DS selector passed to your

callback procedure. Or you can take other measures, but just remember that the

original DS selector is no longer safe after enabling interrupts in a

protected mode callback routine.

Since the real mode register data structure is also static, you should take

special care if you wish to enable interrupts in a callback. Since the data

structure is needed for the exit from the callback, you can not simply

discard it. You should make a copy of the data structure. You can then pass

back the copy when you exit the callback procedure. The ES:EDI upon exit is

not required to be the same as on entry.

1.5 - PMODE specifics:

----------------------

PMODE makes several variables public. These control certain things when

running under VCPI, XMS, or a raw system. Under DPMI, these variables do

absolutely nothing. The size of the low memory protected mode buffer required

for _pm_init is directly affected by these variables. Thus, they must be set

to the same values for the call to _pm_info and _pm_init. You should only

modify these variables before switching into protected mode.

) _pm_pagetables - This specifies the number of page tables you want to have

under VCPI. Each page table requires 4k and maps 4M of linear memory. Page

tables only define linear address space, not actual physical memory. The

amount of extended memory that will be available to your code will be the

lesser of linear and physical memory in the system. Setting a higher number

of page tables will not give you any more physical memory than is available

in the system, but it will give you more linear address space where physical

memory can be mapped. This helps reduce fragmentation of memory when

allocating extended memory blocks under VCPI. Never set this variable to 0,

since the first page table maps the low megabyte of real mode memory.

) _pm_selectors - This is the total number of descriptors you want PMODE to

make available to your code for allocation. The range for this variable is

0 to about 8150. The actual max number of descriptors that can exist in the

global descriptor table, which is where descriptors under PMODE reside, is

8191. But some of these descriptors are used by PMODE. Also, DPMI may not

be able to provide quite as many as 8000 descriptors. Each descriptor takes

up 8 bytes of space in the low memory protected mode buffer.

) _pm_callbacks - This is the number of real mode callbacks you want PMODE to

provide for allocation. Each callback takes up 25 bytes of space. You may

set this variable to 0.

) _pm_rmstacklen - This is the size of the real mode stack, in paragraphs,

that is provided for IRQ and INT redirection to real mode. Also for INT 31h

functions 0300h, 0301h, and 0302h when the SS:SP field in the register

structure is zero.

) _pm_rmstacks - This is the number of real mode stacks you want present in

case of nested calls to real mode. There must be at least one.

) _pm_pmstacklen - This is the size of the protected mode stack, in

paragraphs, to provide for protected mode procedures which handle real mode

callbacks.

) _pm_pmstacks - This is the number of protected mode stacks you want present

in case of nested callbacks. If your code is not going to be using real mode

callbacks, you may set this variable, along with _pm_pmstacklen and

_pm_callbacks, to zero.

1.6 - PMODE internal stacks:

----------------------------

The memory space of the real mode and protected mode stacks used by PMODE

for INT and IRQ redirection and callbacks can be used by your code if you do

your own raw mode switching and need stack space. This is just a minor memory

optimization, especially useful if you will have many nested mode switches

using PMODE and raw mode services. These memory areas are only available under

a VCPI/XMS/raw system. If a DPMI host is already in place, PMODE is not in

control and does not have these memory areas to share with your code, in which

case you will have to allocate any stack buffers you need yourself anyway.

All of PMODE except this is the standard DPMI interface. If you use this

technique I can not guarantee support in future major revisions of PMODE, nor

can you be sure of 'portability' to other extenders. On the other hand, if you

use this method, you will have to handle setting up your own stack buffers

anyway if the system is running under a DPMI host. This will make your code

'portable' since, if ever you move it, you have the stack buffer code right

there. The reason I make this option available in PMODE, is that for a while

now, multiple nested stack areas for PMODE and any external shell for it have

bothered me as an unnecessary waste of memory.

The memory areas of the stacks are defined by four variables made public in

the PMODE segment. They define the base and current top of the entire real

mode and protected mode stack areas (all nested stacks for each mode). The

protected mode base and top are dword pointers relative to the beginning of

memory (0000:0000). The real mode base and top are real mode segment word

values. The tops of the stacks are all that you really need to use the stack

areas, the bases are provided in case you want to check for running out of

nested stacks. The four variables are as follows:

) pmstacktop - This is the linear address of the current top of the protected

mode stack area.

) pmstackbase - This is the linear base address of the entire protected mode

stack area. If ever the value of pmstacktop drops below this value, you ran

out of memory in the protected mode stack buffer.

) rmstacktop - This is the real mode segment value of the current top of the

real mode stack area.

) rmstackbase - This is the real mode segment value of the base of the real

mode stack area. Again, if the value of rmstacktop falls below this value,

you are out of real mode stack buffer memory. This does not necessarily mean

death, since you would be accessing this before you actually switch to real

mode, it simply means there is not enough room in the PMODE stack buffers

to switch to real mode.

The size of the entire real and protected mode stack areas is defined when

you enter protected mode by the variables _pm_rmstacklen, _pm_rmstacks,

_pm_pmstacklen, _pm_pmstacks. The real mode stack area is the one used by

PMODE for INT and IRQ redirection, and INT 31h functions 0300h, 0301h, and

0302h, and its entire size in bytes is _pm_rmstacklen * _pm_rmstacks * 16. The

protected mode stack area is the one used by callbacks and its size is

_pm_pmstacklen * _pm_pmstacks * 16. This means that if you intend to use the

PMODE protected mode stacks, you must make them available through

_pm_pmstacklen and _pm_pmstacks even if you do not use real mode callbacks.

These stack areas are used from the top to bottom as nested mode stacks are

allocated.

An example of use:

; this code loads DX:BX with a valid real mode SS:SP for a raw mode switch

; to real mode assuming DS is a selector mapping the PMODE segment PMODE_TEXT

mov dx,rmstacktop               ; DX = current real mode stack top

mov bx,_pm_rmstacklen           ; BX = length of PMODE real mode stack

sub dx,bx                       ; subtract size of stack from top

cmp dx,rmstackbase              ; check if ran out of memory

jb ran_out_of_RM_stack_memory

mov rmstacktop,dx               ; put the adjusted value back

shl bx,4                        ; adjust stack len from para to bytes

; DX:BX now contains the real mode segment:offset of a valid real mode stack

; as could be used by the raw mode switching services, after the return from

; real mode you must remember to adjust the current stack top

mov ax,_pm_rmstacklen           ; AX = length of real mode stack

add rmstacktop,ax               ; readjust real mode stack top

; this code loads DX:EBX with a valid protected mode SS:ESP for a raw mode

; switch to protected mode assuming DS is PMODE_TEXT

mov ebx,pmstacktop              ; EBX = current protected stack top

mov ecx,ebx                     ; ECX = EBX for adjust

mov eax,_pm_pmstacklen          ; EAX = length of protected mode stack

shl eax,4                       ; convert length from para to bytes

sub ecx,eax                     ; subtract size of stack from top

cmp ecx,pmstackbase             ; check if ran out of memory

jb ran_out_of_PM_stack_memory

mov pmstacktop,ecx              ; put the adjusted value back

; EBX now contains the top of a valid protected mode stack area relative to

; the beginning of memory, you must still load up DX with a valid protected

; mode SS and adjust EBX by the base address of that selector to convert it

; to a valid ESP relative to the base of the stack segment

mov dx,PM_stack_selector        ; DX = SS selector for protected mode

sub ebx,PM_stack_selector_base  ; adjust EBX for a valid ESP

; and ofcourse after the return from protected mode readjust the stack top

mov eax,_pm_pmstacklen          ; EAX = length of protected mode stack

shl eax,4                       ; convert length from para to bytes

add pmstacktop,eax              ; readjust protected mode stack top

You do not have to use _pm_rmstacklen and _pm_pmstacklen for the size of the

stacks you allocate from the stack memory, but you should keep the sizes at

least a dword multiple. Also do not make the stacks too small, at least 128

bytes is a good minimum value.

As I said, under a real DPMI host, PMODE does not initialize these stack

memory areas. Your code would have to do it on its own. But you can use the

same variables to store the base and top of the stack memory areas your code

allocates since PMODE will not be using them as it will not be in control.

This means you can use the same code for raw mode switching and mode stack

allocation. All that will be required is some code at protected mode init time

that will allocate some memory for the stack areas and store their base and

top in the respective variables.

------------------------------------------------------------------------------

2 - Functions:

--------------

PMODE duplicates a subset of DPMI protected mode functions. These functions

are available ONLY in protected through INT 31h. They provide descriptor

services, extended memory services, interrupt services, translation services,

and some other misc things. A function is called by setting AX to the function

code, setting any other registers for the function, and executing an INT 31h.

Upon return, the carry flag will be clear if the function was successful. If

the carry flag is set, the function failed. In this case, an error code will

be placed in AX. However, DPMI 0.9 will not return error codes, just the carry

flag set on errors. All other registers are preserved unless otherwise stated.

2.0 - Function 0000h - Allocate Descriptors:

--------------------------------------------

Allocates one or more descriptors in the client's descriptor table. The

descriptor(s) allocated must be initialized by the application with other

function calls.

In:

AX     = 0000h

CX     = number of descriptors to allocate

Out:

if successful:

AX     = base selector

if failed:

AX     = error code:

8011h - descriptor unavailable

8021h - invalid value (CX = 0) (VCPI/XMS/raw only)

Notes:

) If more that one descriptor was requested, the function returns a base

selector referencing the first of a contiguous array of descriptors. The

selector values for subsequent descriptors in the array can be calculated

by adding the value returned by INT 31h function 0003h.

) The allocated descriptor(s) will be set to expand-up writeable data, with

the present bit set and a base and limit of zero. The privilege level of the

descriptor(s) will match the client's code segment privilege level.

2.1 - Function 0001h - Free Descriptor:

---------------------------------------

Frees a descriptor.

In:

AX     = 0001h

BX     = selector for the descriptor to free

Out:

if failed:

AX     = error code:

8022h - invalid selector

Notes:

) Each descriptor allocated with INT 31h function 0000h must be freed

individually with the function. Even if it was previously allocated as part

of a contiguous array of descriptors.

) Under DPMI 1.0/VCPI/XMS/raw, any segment registers which contain the

selector being freed are zeroed by this function.

2.2 - Function 0003h - Get Selector Increment Value:

----------------------------------------------------

The Allocate Descriptors function (0000h) can allocate an array of contiguous

descriptors, but only return a selector for the first descriptor. The value

returned by this function can be used to calculate the selectors for

subsequent descriptors in the array.

In:

AX     = 0003h

Out:

always successful:

AX     = selector increment value

Notes:

) The increment value is always a power of two.

2.3 - Function 0006h - Get Segment Base Address:

------------------------------------------------

Returns the 32bit linear base address from the descriptor table for the

specified segment.

In:

AX     = 0006h

BX     = selector

Out:

if successful:

CX:DX  = 32bit linear base address of segment

if failed:

AX     = error code:

8022h - invalid selector

Notes:

) Client programs must use the LSL instruction to query the limit for a

descriptor.

2.4 - Function 0007h - Set Segment Base Address:

------------------------------------------------

Sets the 32bit linear base address field in the descriptor for the specified

segment.

In:

AX     = 0007h

BX     = selector

CX:DX  = 32bit linear base address of segment

Out:

if failed:

AX     = error code:

8022h - invalid selector

8025h - invalid linear address (changing the base would cause the

descriptor to reference a linear address range outside that

allowed for DPMI clients) (DPMI 1.0 only)

Notes:

) Under DPMI 1.0/VCPI/XMS/raw, any segment register which contains the

selector specified in register BX will be reloaded. DPMI 0.9 may do this,

but it is not guaranteed.

) I hope you have enough sense not to try to modify your current CS or SS

descriptor.

2.5 - Function 0008h - Set Segment Limit:

-----------------------------------------

Sets the limit field in the descriptor for the specified segment.

In:

AX     = 0008h

BX     = selector

CX:DX  = 32bit segment limit

Out:

if failed:

AX     = error code:

8021h - invalid value (the limit is > 1M, but the low 12 bits are

not set)

8022h - invalid selector

8025h - invalid linear address (changing the base would cause the

descriptor to reference a linear address range outside that

allowed for DPMI clients) (DPMI 1.0 only)

Notes:

) The value supplied to the function in CX:DX is the byte length of the

segment-1.

) Segment limits greater than or equal to 1M must be page aligned. That is,

they must have the low 12 bits set.

) This function has an implicit effect on the "G" bit in the segment's

descriptor.

) Client programs must use the LSL instruction to query the limit for a

descriptor.

) Under DPMI 1.0/VCPI/XMS/raw, any segment register which contains the

selector specified in register BX will be reloaded. DPMI 0.9 may do this,

but it is not guaranteed.

) I hope you have enough sense not to try to modify your current CS or SS

descriptor.

2.6 - Function 0009h - Set Descriptor Access Rights:

----------------------------------------------------

Modifies the access rights field in the descriptor for the specified segment.

In:

AX     = 0009h

BX     = selector

CX     = access rights/type word

Out:

if failed:

AX     = error code:

8021h - invalid value (access rights/type word invalid)

8022h - invalid selector

8025h - invalid linear address (changing the base would cause the

descriptor to reference a linear address range outside that

allowed for DPMI clients) (DPMI 1.0 only)

Notes:

) The access rights/type word passed to the function in CX has the following

format:

Bit: 15  14  13  12  11  10   9   8   7   6   5   4   3   2   1   0

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

| G |B/D| 0 | ? |       ?       | 1 |  DPL  | 1 |C/D|E/C|W/R| A |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+

G   - 0=byte granular, 1=page granular

B/D - 0=default 16bit, 1=default 32bit

DPL - must be equal to caller's CPL

C/D - 0=data, 1=code

E/C - data: 0=expand-up, 1=expand-down

code: must be 0 (non-conforming)

W/R - data: 0=read, 1=read/write

code: must be 1 (readable)

A   - 0=not accessed, 1=accessed

0   - must be 0

1   - must be 1

?   - ignored

) Client programs should use the LAR instruction to examine the access rights

of a descriptor.

) Under DPMI 1.0/VCPI/XMS/raw, any segment register which contains the

selector specified in register BX will be reloaded. DPMI 0.9 may do this,

but it is not guaranteed.

) I hope you have enough sense not to try to modify your current CS or SS

descriptor.

2.7 - Function 000Ah - Create Alias Descriptor:

-----------------------------------------------

Creates a new data descriptor that has the same base and limit as the

specified descriptor.

In:

AX     = 000ah

BX     = selector

Out:

if successful:

AX     = data selector (alias)

if failed:

AX     = error code:

8011h - descriptor unavailable

8022h - invalid selector

Notes:

) The selector supplied to the function may be either a data descriptor or

a code descriptor. The alias descriptor created is always an expand-up

writeable data segment.

) The descriptor alias returned by this function will not track changes to the

original descriptor.

2.8 - Function 000Bh - Get Descriptor:

--------------------------------------

Copies the descriptor table entry for the specified selector into an 8 byte

buffer.

In:

AX     = 000bh

BX     = selector

ES:EDI = selector:offset of 8 byte buffer

Out:

if successful:

buffer pointed to by ES:EDI contains descriptor

if failed:

AX     = error code:

8022h - invalid selector

2.9 - Function 000Ch - Set Descriptor:

--------------------------------------

Copies the contents of an 8 byte buffer into the descriptor for the specified

selector.

In:

AX     = 000ch

BX     = selector

ES:EDI = selector:offset of 8 byte buffer containing descriptor

Out:

if failed:

AX     = error code:

8021h - invalid value (access rights/type word invalid)

8022h - invalid selector

8025h - invalid linear address (changing the base would cause the

descriptor to reference a linear address range outside that

allowed for DPMI clients) (DPMI 1.0 only)

) The descriptors access rights/type word at offset 5 within the descriptor

follows the same format and restrictions as the access rights/type parameter

CX to the Set Descriptor Access Rights function (0009h).

) Under DPMI 1.0/VCPI/XMS/raw, any segment register which contains the

selector specified in register BX will be reloaded. DPMI 0.9 may do this,

but it is not guaranteed.

) I hope you have enough sense not to try to modify your current CS or SS

descriptor or the descriptor of the buffer.

2.10 - Function 000Eh - Get Multiple Descriptors:

-------------------------------------------------

Copies one or more descriptor table entries into a buffer.

In:

AX     = 000eh

CX     = number of descriptors to copy

ES:EDI = selector:offset of a buffer in the following format:

Offset  Length  Contents

00h     2       Selector #1 (set by client)

02h     8       Descriptor #1 (returned by host)

0ah     2       Selector #2 (set by client)

0ch     8       Descriptor #2 (returned by host)

...     ...     ...

Out:

if successful:

buffer contains copies of the descriptors for the specified selectors

if failed:

AX     = error code:

8022h - invalid selector

CX     = number of descriptors successfully copied

Notes:

) If an error occurs because of an invalid selector or descriptor, the

function returns the number of descriptors which were successfully copied

in CX. All of the descriptors which were copied prior to the one that failed

are valid.

) This function is not present under DPMI 0.9.

2.11 - Function 000Fh - Set Multiple Descriptors:

-------------------------------------------------

Copies one or more descriptors from a client buffer into the descriptor table.

In:

AX     = 000fh

CX     = number of descriptors to copy

ES:EDI = selector:offset of a buffer in the following format:

Offset  Length  Contents

00h     2       Selector #1

02h     8       Descriptor #1

0ah     2       Selector #2

0ch     8       Descriptor #2

...     ...     ...

Out:

if failed:

AX     = error code:

8021h - invalid value (access rights/type word invalid)

8022h - invalid selector

8025h - invalid linear address (changing the base would cause the

descriptor to reference a linear address range outside that

allowed for DPMI clients) (DPMI 1.0 only)

CX     = number of descriptors successfully copied

Notes:

) If an error occurs because of an invalid selector or descriptor, the

function returns the number of descriptors which were successfully copied in

CX. All of the descriptors which were copied prior to the one that failed

are valid.

) The descriptors access rights/type word at offset 5 within the descriptor

follows the same format and restrictions as the access rights/type parameter

CX to the Set Descriptor Access Rights (0009h) function.

) Under DPMI 1.0/VCPI/XMS/raw, any segment register which contains a selector

specified in the data structure will be reloaded. DPMI 0.9 may do this,

but it is not guaranteed.

) I hope you have enough sense not to try to modify your current CS or SS

descriptor or the descriptor of the buffer.

) This function is not present under DPMI 0.9.

2.12 - Function 0200h - Get Real Mode Interrupt Vector:

-------------------------------------------------------

Returns the real mode segment:offset for the specified interrupt vector.

In:

AX     = 0200h

BL     = interrupt number

Out:

always successful:

CX:DX  = segment:offset of real mode interrupt handler

Notes:

) The value returned in CX is a real mode segment address, not a protected

mode selector.

2.13 - Function 0201h - Set Real Mode Interrupt Vector:

-------------------------------------------------------

Sets the real mode segment:offset for the specified interrupt vector.

In:

AX     = 0201h

BL     = interrupt number

CX:DX  = segment:offset of real mode interrupt handler

Notes:

) The value passed in CX must be a real mode segment address, not a protected

mode selector. Consequently, the interrupt handler must either reside in

DOS memory (below the 1M boundary) or the client must allocate a real mode

callback address.

2.14 - Function 0204h - Get Protected Mode Interrupt Vector:

------------------------------------------------------------

Returns the address of the current protected mode interrupt handler for the

specified interrupt.

In:

AX     = 0204h

BL     = interrupt number

Out:

always successful:

CX:EDX = selector:offset of protected mode interrupt handler

Notes:

) The value returned in CX is a valid protected mode selector, not a real mode

segment address.

2.15 - Function 0205h - Set Protected Mode Interrupt Vector:

------------------------------------------------------------

Sets the address of the protected mode interrupt handler for the specified

interrupt.

In:

AX     = 0205h

BL     = interrupt number

CX:EDX = selector offset of protected mode interrupt handler

Out:

if failed:

AX     = error code:

8022h - invalid selector

Notes:

) The value passed in CX must be a valid protected mode selector, not a real

mode segment address.

2.16 - Function 0300h - Simulate Real Mode Interrupt:

-----------------------------------------------------

Simulates an interrupt in real mode. The function transfers control to the

address specified by the real mode interrupt vector. The real mode handler

must return by executing an IRET.

In:

AX     = 0300h

BL     = interrupt number

BH     = must be 0

CX     = number of words to copy from the protected mode stack to the real

mode stack

ES:EDI = selector:offset of real mode register data structure in the

following format:

Offset  Length  Contents

00h     4       EDI

04h     4       ESI

08h     4       EBP

0ch     4       reserved, ignored

10h     4       EBX

14h     4       EDX

18h     4       ECX

1ch     4       EAX

20h     2       CPU status flags

22h     2       ES

24h     2       DS

26h     2       FS

28h     2       GS

2ah     2       IP (reserved, ignored)

2ch     2       CS (reserved, ignored)

2eh     2       SP

30h     2       SS

Out:

if successful:

ES:EDI = selector offset of modified real mode register data structure

if failed:

AX     = error code:

8012h - linear memory unavailable (stack)

8013h - physical memory unavailable (stack) (DPMI 1.0 only)

8014h - backing store unavailable (stack) (DPMI 1.0 only)

8021h - invalid value (CX too large) (DPMI 1.0 only)

Notes:

) The CS:IP in the real mode register data structure is ignored by this

function. The appropriate interrupt handler will be called based on the

value passed in BL.

) If the SS:SP fields in the real mode register data structure are zero, a

real mode stack will be provided by the host. Otherwise the real mode SS:SP

will be set to the specified values before the interrupt handler is called.

) The flags specified in the real mode register data structure will be put on

the real mode interrupt handler's IRET frame. The interrupt handler will be

called with the interrupt and trace flags clear.

) Values placed in the segment register positions of the data structure must

be valid for real mode. That is, the values must be paragraph addresses, not

protected mode selectors.

) The target real mode handler must return with the stack in the same state

as when it was called. This means that the real mode code may switch stacks

while it is running, but must return on the same stack that it was called

on and must return with an IRET.

) When this function returns, the real mode register data structure will

contain the values that were returned by the real mode interrupt handler.

The CS:IP and SS:SP values will be unmodified in the data structure.

) It is the caller's responsibility to remove any parameters that were pushed

on the protected mode stack.

2.17 - Function 0301h - Call Real Mode Procedure With Far Return Frame:

-----------------------------------------------------------------------

Simulates a FAR CALL to a real mode procedure. The called procedure must

return by executing a RETF instruction.

In:

AX     = 0301h

BH     = must be 0

CX     = number of words to copy from the protected mode stack to the real

mode stack

ES:EDI = selector:offset of real mode register data structure in the

following format:

Offset  Length  Contents

00h     4       EDI

04h     4       ESI

08h     4       EBP

0ch     4       reserved, ignored

10h     4       EBX

14h     4       EDX

18h     4       ECX

1ch     4       EAX

20h     2       CPU status flags

22h     2       ES

24h     2       DS

26h     2       FS

28h     2       GS

2ah     2       IP

2ch     2       CS

2eh     2       SP

30h     2       SS

Out:

if successful:

ES:EDI = selector offset of modified real mode register data structure

if failed:

AX     = error code:

8012h - linear memory unavailable (stack)

8013h - physical memory unavailable (stack) (DPMI 1.0 only)

8014h - backing store unavailable (stack) (DPMI 1.0 only)

8021h - invalid value (CX too large) (DPMI 1.0 only)

Notes:

) The CS:IP in the real mode register data structure specifies the address of

the real mode procedure to call.

) If the SS:SP fields in the real mode register data structure are zero, a

real mode stack will be provided by the host. Otherwise the real mode SS:SP

will be set to the specified values before the procedure is called.

) Values placed in the segment register positions of the data structure must

be valid for real mode. That is, the values must be paragraph addresses, not

protected mode selectors.

) The target real mode procedure must return with the stack in the same state

as when it was called. This means that the real mode code may switch stacks

while it is running, but must return on the same stack that it was called

on and must return with a RETF and should not clear the stack of any

parameters that were passed to it on the stack.

) When this function returns, the real mode register data structure will

contain the values that were returned by the real mode procedure. The CS:IP

and SS:SP values will be unmodified in the data structure.

) It is the caller's responsibility to remove any parameters that were pushed

on the protected mode stack.

2.18 - Function 0302h - Call Real Mode Procedure With IRET Frame:

-----------------------------------------------------------------

Simulates a FAR CALL with flags pushed on the stack to a real mode procedure.

The real mode procedure must return by executing an IRET instruction or a

RETF 2.

In:

AX     = 0301h

BH     = must be 0

CX     = number of words to copy from the protected mode stack to the real

mode stack

ES:EDI = selector:offset of real mode register data structure in the

following format:

Offset  Length  Contents

00h     4       EDI

04h     4       ESI

08h     4       EBP

0ch     4       reserved, ignored

10h     4       EBX

14h     4       EDX

18h     4       ECX

1ch     4       EAX

20h     2       CPU status flags

22h     2       ES

24h     2       DS

26h     2       FS

28h     2       GS

2ah     2       IP

2ch     2       CS

2eh     2       SP

30h     2       SS

Out:

if successful:

ES:EDI = selector offset of modified real mode register data structure

if failed:

AX     = error code:

8012h - linear memory unavailable (stack)

8013h - physical memory unavailable (stack) (DPMI 1.0 only)

8014h - backing store unavailable (stack) (DPMI 1.0 only)

8021h - invalid value (CX too large) (DPMI 1.0 only)

Notes:

) The CS:IP in the real mode register data structure specifies the address of

the real mode procedure to call.

) If the SS:SP fields in the real mode register data structure are zero, a

real mode stack will be provided by the host. Otherwise the real mode SS:SP

will be set to the specified values before the procedure is called.

) The flags specified in the real mode register data structure will be put on

the real mode procedure's IRET frame. The procedure will be called with the

interrupt and trace flags clear.

) Values placed in the segment register positions of the data structure must

be valid for real mode. That is, the values must be paragraph addresses, not

protected mode selectors.

) The target real mode procedure must return with the stack in the same state

as when it was called. This means that the real mode code may switch stacks

while it is running, but must return on the same stack that it was called

on and must return with an IRET or discard the flags from the stack with a

RETF 2 and should not clear the stack of any parameters that were passed to

it on the stack.

) When this function returns, the real mode register data structure will

contain the values that were returned by the real mode procedure. The CS:IP

and SS:SP values will be unmodified in the data structure.

) It is the caller's responsibility to remove any parameters that were pushed

on the protected mode stack.

2.19 - Function 0303h - Allocate Real Mode Callback Address:

------------------------------------------------------------

Returns a unique real mode segment:offset, known as a "real mode callback",

that will transfer control from real mode to a protected mode procedure.

Callback addresses obtained with this function can be passed by a protected

mode program to a real mode application, interrupt handler, device driver,

TSR, etc... so that the real mode program can call procedures within the

protected mode program.

In:

AX     = 0303h

DS:ESI = selector:offset of protected mode procedure to call

ES:EDI = selector:offset of 32h byte buffer for real mode register data

structure to be used when calling the callback routine.

Out:

if successful:

CX:DX  = segment:offset of real mode callback

if failed:

AX     = error code:

8015h - callback unavailable

Notes:

) A descriptor may be allocated for each callback to hold the real mode SS

descriptor. Real mode callbacks are a limited system resource. A client

should release a callback that it is no longer using.

2.20 - Function 0304h - Free Real Mode Callback Address:

--------------------------------------------------------

Releases a real mode callback address that was previously allocated with the

Allocate Real Mode Callback Address function (0303h).

In:

AX     = 0304h

CX:DX  = segment:offset of real mode callback to be freed

Out:

if failed:

AX     = error code:

8024h - invalid callback address

Notes:

) Real mode callbacks are a limited system resource. A client should release

any callback that it is no longer using.

2.21 - Function 0305h - Get State Save/Restore Addresses:

---------------------------------------------------------

Returns the address of two procedures used to save and restore the state of

the current task's registers in the mode (protected or real) which is not

currently executing.

In:

AX     = 0305h

Out:

always successful:

AX     = size of buffer in bytes required to save state

BX:CX  = segment:offset of real mode routine used to save/restore state

SI:EDI = selector:offset of protected mode routine used to save/restore

state

Notes:

) The real mode segment:offset returned by this function should be called

only in real mode to save/restore the state of the protected mode registers.

The protected mode selector:offset returned by this function should be

called only in protected mode to save/restore the state of the real mode

registers.

) Both of the state save/restore procedures are entered by a FAR CALL with the

following parameters:

AL       = 0 to save state

= 1 to restore state

ES:(E)DI = (selector or segment):offset of state buffer

The state buffer must be at least as large as the value returned in AX by

INT 31h function 0305h. The state save/restore procedures do not modify any

registers. DI must be used for the buffer offset in real mode, EDI must be

used in protected mode.

) Some DPMI hosts and VCPI/XMS/raw will not require the state to be saved,

indicating this by returning a buffer size of zero in AX. In such cases,

that addresses returned by this function can still be called, although they

will simply return without performing any useful function.

) Clients do not need to call the state save/restore procedures before using

INT 31h function 0300h, 0301h, or 0302h. The state save/restore procedures

are provided for clients that use the raw mode switch services only.

2.22 - Function 0306h - Get Raw Mode Switch Addresses:

------------------------------------------------------

Returns addresses that can be called for low level mode switching.

In:

AX     = 0306h

Out:

always successful:

BX:CX  = segment:offset of real to protected mode switch procedure

SI:EDI = selector:offset of protected to real mode switch procedure

Notes:

) The real mode segment:offset returned by this function should be called

only in real mode to switch to protected mode. The protected mode

selector:offset returned by this function should be called only in protected

mode to switch to real mode.

) The mode switch procedures are entered by a FAR JMP to the appropriate

address with the following parameters:

AX    = new DS

CX    = new ES

DX    = new SS

(E)BX = new (E)SP

SI    = new CS

(E)DI = new (E)IP

The processor is placed into the desired mode, and the DS, ES, SS, (E)SP,

CS, and (E)IP registers are updated with the specific values. In other

words, execution of the client continues in the requested mode at the

address provided in registers SI:(E)DI. The values specified to be placed

into the segment registers must be appropriate for the destination mode.

That is, segment addresses for real mode, and selectors for protected mode.

The values in EAX, EBX, ECX, EDX, ESI, and EDI after the mode switch are

undefined. EBP will be preserved across the mode switch call so it can be

used as a pointer. FS and GS will contain zero after the mode switch.

If interrupts are disabled when the mode switch procedure is invoked, they

will not be re-enabled by the host (even temporarily).

) It is up to the client to save and restore the state of the task when using

this function to switch modes. This requires the state save/restore

procedures whose addresses can be obtained with INT 31h function 0305h.

2.23 - Function 0400h - Get Version:

------------------------------------

Returns the version of the DPMI Specification implemented by the DPMI host.

The client can use this information to determine what functions are available.

In:

AX     = 0400h

Out:

always successful:

AH     = DPMI major version as a binary number (VCPI/XMS/raw returns 1)

AL     = DPMI minor version as a binary number (VCPI/XMS/raw returns 0)

BX     = flags:

Bits    Significance

0       0 = host is 16bit (PMODE never runs under one of these)

1 = host is 32bit

1       0 = CPU returned to V86 mode for reflected interrupts

1 = CPU returned to real mode for reflected interrupts

2       0 = virtual memory not supported

1 = virtual memory supported

3-15    reserved

CL     = processor type:

03h = 80386

04h = 80486

05h = 80586

06h-ffh = reserved

DH     = current value of master PIC base interrupt (low 8 IRQs)

DL     = current value of slave PIC base interrupt (high 8 IRQs)

Notes:

) The major and minor version numbers are binary, not BCD. So a DPMI 0.9

implementation would return AH as 0 and AL as 5ah (90).

2.24 - Function 0500h - Get Free Memory Information:

----------------------------------------------------

Returns information about the amount of available memory. Since DPMI clients

could be running in a multitasking environment, the information returned by

this function should be considered advisory.

In:

AX     = 0500h

ES:EDI = selector:offset of 48 byte buffer

Out:

if successful:

buffer is filled with the following information:

if failed:

AX     = error code:

8010h - internal resource unavailable (stack) (XMS only)

Offset  Length  Contents

00h     4       Largest available free block in bytes

04h     2ch     Other fields only supplied by DPMI

Notes:

) Only the first field of the structure is guaranteed to contain a valid

value. Any fields that are not supported by the host will be set to -1

(0ffffffffh) to indicate that the information is not available.

2.25 - Function 0501h - Allocate Memory Block:

----------------------------------------------

Allocates a block of extended memory.

In:

AX     = 0501h

BX:CX  = size of block in bytes (must be non-zero)

Out:

if successful:

BX:CX  = linear address of allocated memory block

SI:DI  = memory block handle (used to resize and free block)

if failed:

AX     = error code:

8010h - internal resource unavailable (stack) (XMS only)

8012h - linear memory unavailable (DPMI 1.0/VCPI only)

8013h - physical memory unavailable

8014h - backing store unavailable (DPMI 1.0 only)

8016h - handle unavailable (DPMI 1.0/XMS only)

8021h - invalid value (BX:CX = 0)

Notes:

) The allocated block is guaranteed to have at least paragraph alignment.

) This function does not allocate any descriptors for the memory block. It is

the responsibility of the client to allocate and initialize any descriptors

needed to access the memory with additional function calls.

) The allocations by this function could be paragraph, kilobyte, or page

aligned. That is, the value you request could be rounded up to the next

paragraph, kilobyte, or page value.

2.26 - Function 0502h - Free Memory Block:

------------------------------------------

Frees a memory block previously allocated with the Allocate Memory Block

function (0501h).

In:

AX     = 0502h

SI:DI  = memory block handle

Out:

if failed:

AX     = error code:

8010h - internal resource unavailable (stack) (XMS only)

8023h - invalid handle

Notes:

) No descriptors are freed by this call. It is the client's responsibility to

free any descriptors that it previously allocated to map the memory block.

Descriptors should be freed before memory blocks.

2.27 - Function 0503h - Resize Memory Block:

--------------------------------------------

Changes the size of a memory block previously allocated with the Allocate

Memory Block function (0501h).

In:

AX     = 0503h

BX:CX  = new size of block in bytes (must be non-zero)

SI:DI  = memory block handle

Out:

BX:CX  = new linear address of memory block

SI:DI  = new memory block handle

if failed:

AX     = error code:

8010h - internal resource unavailable (stack) (XMS only)

8012h - linear memory unavailable (DPMI 1.0/VCPI only)

8013h - physical memory unavailable

8014h - backing store unavailable (DPMI 1.0 only)

8016h - handle unavailable (DPMI 1.0/XMS only)

8021h - invalid value (BX:CX = 0)

8023h - invalid handle

Notes:

) After this function returns successfully, the previous handle for the memory

block is invalid and should not be used anymore.

) It is the client's responsibility to update any descriptors that map the

memory block with the new linear address after resizing the block.

2.28 - Function 050Ah - Get Memory Block Size and Base:

-------------------------------------------------------

Returns the size and base of a memory block that was previously allocated

with the Allocate Memory Block function (0501h).

In:

AX     = 050ah

SI:DI  = memory block handle

Out:

if successful:

BX:CX  = linear address of memory block

SI:DI  = size of memory block (bytes)

if failed:

AX     = error code:

8010h - internal resource unavailable (stack) (XMS only)

8023h - invalid handle

Notes:

) This function is not present under DPMI 0.9.

2.29 - Function 0900h - Get and Disable Virtual Interrupt State:

----------------------------------------------------------------

Disables the virtual interrupt flag and returns the previous state of it.

In:

AX     = 0900h

Out:

always successful:

AL     = 0 if virtual interrupts were previously disabled

AL     = 1 if virtual interrupts were previously enabled

Notes:

) AH is not changed by this function. Therefore the previous state can be

restored by simply executing another INT 31h.

) A client that does not need to know the prior interrupt state can execute

the CLI instruction rather than calling this function. The instruction may

be trapped by a DPMI host and should be assumed to be very slow.

2.30 - Function 0901h - Get and Enable Virtual Interrupt State:

---------------------------------------------------------------

Enables the virtual interrupt flag and returns the previous state of it.

In:

AX     = 0901h

Out:

always successful:

AL     = 0 if virtual interrupts were previously disabled

AL     = 1 if virtual interrupts were previously enabled

Notes:

) AH is not changed by this function. Therefore the previous state can be

retstored by simply executing another INT 31h.

) A client that does not need to know the prior interrupt state can execute

the STI instruction rather than calling this function. The instruction may

be trapped by a DPMI host and should be assumed to be very slow.

2.31 - Function 0902h - Get Virtual Interrupt State:

----------------------------------------------------

Returns the current state of the virtual interrupt flag.

In:

AX     = 0902h

Out:

always successful:

AL     = 0 if virtual interrupts are disabled

AL     = 1 if virtual interrupts are enabled

Notes:

) This function should be used in preference to the PUSHF instruction to

examine the interrupt flag, because the PUSHF instruction returns the

physical interrupt flag rather than the virtualized interrupt flag. On some

DPMI hosts, the physical interrupt flag will always be enabled, even when

the hardware interrupts are not being passed through to the client.

2.32 - Function FFFFh - Special Fluffy Magical Function:

--------------------------------------------------------

Does special fluffy magical things.

In:

AX     = ffffh

Out:

if successful:

AX     = 1234h (normal fluffy magical number)

BX     = 56ijh (special fluffy magical number)

CX     = 89abh (nothing really special)

DX     = cdefh (again, just a boring number)

EX     = return value from beyond

FX     = an acronym for 'effects'

GX     = huh?

HX     = size of special fluffy magical buffer

if failed:

AX     = error code:

8001h - get a grip on reality!

Notes:

) The special fluffy magical buffer must not exceed the special fluffy magical

constant in size, which is defined in the special fluffy magical place.

) Well, maybe it exists... Maybe if you try calling it many many many many

many many many many times in a row it will succeed.

------------------------------------------------------------------------------

3 - Miscellenaous:

------------------

Some final things about PMODE, including some low level tech info if you are

just curious.

3.0 - Updates:

--------------

Here are the changes from any previous versions of PMODE 3.0.

PMODE 3.0:

) First release of PMODE 3.0.

PMODE 3.01:

) Slightly optimized VCPI/XMS/raw mode switching routines.

) Fixed bug with INT 31h translation functions 0300h, 0301h, and 0302h not

returning correct flags in the register structure.

) Fixed bug with software INT redirection calls clearing the interrupt flag.

PMODE 3.02:

) Decreased VCPI mode switching free stack space requirement to 32 bytes.

PMODE 3.03:

) Fixed real mode INT 15h in raw mode, now always returns with the carry flag

clear as it should.

PMODE 3.04:

) Fixed a stupid bug with INT 31h AX=03??h requiring the high word of ECX to

be clear that someone pointed out to me.

) Added (actually just documented) low level access to PMODE's internal real

mode and protected mode stacks.

3.1 - Glossary:

---------------

Bottom up allocation - A method of extended memory allocation which relies on

control blocks specifying the start of free extended memory. This is the

VDISK extended memory allocation scheme.

Client - A program which uses DPMI INT 2fh and INT 31h services to run in

protected mode.

CPL, Current Privilege Level - The privilege level of the currently executing

code.

Descriptor - An 8 byte structure which defines a segment type, its base

address and limit, and the type of access allowed to it.

DPL, Descriptor Privilege Level - The privilege level of a descriptor.

Descriptor protection is based on certain rules of the CPL and DPL of a

descriptor that code may want to access.

DPMI, DOS Protected Mode Interface - An interface for protected mode DOS

programs to manage memory, interrupts, exceptions, debugging registers, and

coprocessor emulation in a well behaved manner which allows them to coexist

with other protected mode programs and operating systems.

Exception - An interrupt that occurs because of some violation of protection

rules.

Extended memory - Memory which lies above the 1M boundary and can only be

addressed in protected mode.

GDT, Global Descriptor Table - A descriptor table which can contain many types

of descriptors besides the regular code and data descriptors. This can

include TSS descriptors and LDT descriptors.

Host - A program which provides DPMI protected mode services.

IDT, Interrupt Descriptor Table - A descriptor table which contains the

gate descriptors to the handlers for the 256 interrupts.

LDT, Local Descriptor Table - A descriptor table which usually contains all

the descriptors of a particular task. Each task in a 386 multitasking system

can have its own LDT whereas there is only one GDT for the entire system.

Linear memory - Address space rather than actual physical RAM of ROM.

Page - A 4k chunk of memory. The 80386 can map any 4k chunk of physical memory

to any linear address. There are also some protection rules that can apply

to pages, such as read-only, or privilege level checking on access.

Page Directory - Sort of a master page table which maps page tables instead of

pages.

Page Table - A 4k table containing 1024 entries for pages. Each page table

maps 4 megabytes of linear memory to physical memory.

Physical memory - The actual physical memory present in a system.

Privilege level - A numeric value representing the freedom of system access

and how much protection applies to code. This value ranges from

0 (most free (godlike)) to 3 (least free (lowly slave)).

RPL, Requestor Privilege Level - The privilege level code requests for a

specific selector access. It is contained in the low two bits of the

selector.

Segment - A specific linear chunk of memory. In real mode, segments are

limited to the first megabyte of memory and are always 64k in length. In

protected mode, a segment can start anywhere in the entire 4 gigabyte

address space of the 80386 and can be that long.

Selector - An index in protected mode into a descriptor table. Selectors are

used in place of segments in protected mode in the segment registers.

Top down allocation - A method of extended memory management which relies on

the BIOS INT 15h function 88h. A program which needs to allocate extended

memory will hook INT 15h and return a smaller extended memory size. The

program is then free to use the memory between the previous top of extended

memory and what it returns as the top of extended memory without worrying

about other programs overwriting its extended memory.

TSS, Task State Segment - A special memory structure used in task switching

and protection.

V86 mode - Actually it is protected mode, running at a privilege level of 3.

Segment registers are used in the same manner as in real mode, with segment

addresses rather than selectors. The advantage is that paging and other

protected tasks can be active, which includes other V86 mode tasks. The

disadvantage is that it is slower than real mode.

VCPI, Virtual Control Program Interface - The predecessor to DPMI. VCPI

extends the EMS interface to allow DOS programs to run in protected mode

in the presence of EMS emulators or other 80386 control programs.

Virtual memory - Extra memory beyond the actual physical memory present in a

system. There is not really any more physical memory in the system, but an

operating system or DPMI host can give that illusion by swapping the

contents of physical memory to and from a disk and mapping that physical

memory to different linear addresses.

XMS, eXtended Memory Specification - A handle based extended memory management

interface.

3.2 - Differences between modes:

--------------------------------

Some differences between DPMI, VCPI, XMS, and raw protected mode:

) DPMI:

Client descriptors reside in a LDT.

VCPI/XMS/raw:

Client descriptors reside in the GDT.

) DPMI:

Code runs at CPL 3.

VCPI/XMS/raw:

Code runs at CPL 0. (Much faster)

) DPMI/VCPI:

Real mode calls are executed in V86 mode.

) XMS/raw:

Real mode calls are executed in real mode. (Much faster)

) DPMI/VCPI:

Paging is enabled.

XMS/raw:

Paging is disabled. (Not really very much faster, but what the hell)

) DPMI:

IRET(D) and POPF(D) may not affect the interrupt flag. PUSHF(D) may not

store the interrupt flag.

VCPI/XMS/raw:

IRET(D) and POPF(D) affect the real interrupt flag. PUSHF(D) stores the

real interrupt flag.

) DPMI:

INTs 1Ch, 23h, and 24h from real mode are sent to protected mode first.

This means if a protected mode handler is installed for these interrupts,

it will get control.

) VCPI/XMS/raw:

DPMI dox are not too clear on the method these interrupts are called in.

Callbacks I would guess. But I am too lazy to investigate. And frankly, I

don't give a shit. So the VCPI/XMS/raw kernel does not support this.

) DPMI:

Seperates exceptions from other low interrupts and IRQs. That is, an

exception 8 would never erroneously go to the handler for IRQ 0.

) VCPI/XMS/raw:

Hey, exceptions are bad, you should not be getting them in the first

place. Reprogramming the interrupt controllers or running the client code

at a lower privilege is not worth the slowdown for me.

) DPMI:

Theoretically, DPMI may refuse a request to set a segment limit to 4G.

I have not yet found a DPMI that will refuse this. They all do protection

at the paging level. And a high limit is necessary for flat mode and

negative offsets.

) VCPI/XMS/raw:

There is no protection, no segment limit or base address will ever be

denied.

) DPMI:

Memory allocation is almost always page granular, but not necessarily.

VCPI:

Memory allocation is page granular (4k chunks).

XMS:

Memory allocation is kilobyte granular.

raw:

Memory allocation is paragraph granular.

) DPMI/VCPI:

Low memory linear addresses may or may not be the actual physical

addresses. Extended memory addresses are almost sure not to be.

XMS/raw:

All linear addresses are physical addresses.

3.3 - Notes:

------------

Here are some misc and low level technical details about PMODE and some

points I want to emphasize. Some of them may seem very obscure, with no real

need to list. But for the sake of thorough documentation, they are here:

) In protected mode, ESP must always be the stack pointer. Meaning, even if

using a 16bit stack segment, the high word of ESP MUST be 0.

) When calling the raw mode switching routine to switch into protected mode,

you must supply the full ESP and EIP. Even if the stack or code segments

being switched to are 16bit.

) If the call to init protected mode was from a segment other than PMODE_TEXT

under DPMI, and a descriptor can not be allocated for that code segment,

immediate termination results. But this should never happen. A DPMI host

that can not supply even one descriptor to its protected mode clients defies

logic. But for the sake of covering all possible screw-ups, this condition

is checked for.

) DPMI 1.0/VCPI/XMS/raw will reload any segment registers for which the

descriptor is changed through an INT 31h function. DPMI 0.9 does not.

(Actually it does, through pushing and then popping any segment registers

it uses. But it is not guaranteed to reload any or all of them).

) DPMI 1.0/VCPI/XMS/raw will zero any segment registers freed with INT 31

function 0001h. DPMI 0.9 may or may not.

) I dont know about DPMI with respect to reloading or freeing a selector which

is currently loaded into SS, but the VCPI/XMS/raw system will not reload or

zero SS. You should not be modifying your stack or code descriptor as you

are using it. This could be bad, even if the INT 31h were handled through a

task gate (which would be slow).

) Remember that DPMI 0.9 does not return error codes as DPMI 1.0/VCPI/XMS/raw

do.

) Reasons for calls to real mode executing in actual real mode rather than

V86 mode (XMS/raw):

) V86 call system can not be used under VCPI very successfully. (Yes, VCPI

runs its real mode in V86. But VCPI MUST be in control in this case.)

) Real mode runs faster than V86 mode.

) INT 15h and XMS extended memory functions will work.

) Other protected mode programs that run DOS functions in real mode will

work.

) It is faster to switch between protected/real mode than protected/V86.

) Use LAR to find out the current CPL for setting descriptor access right.

) You must always set the present bit when setting descriptor access rights.

) Under the VCPI/XMS/raw system, the AVL bit of descriptors is used to keep

track of free and used descriptors. The value you pass for this bit when

setting descriptor access rights will be ignored.

) When switching modes using the raw switching routines, make sure there is

some space on both stacks (real and protected). Specific DPMI requirements

may vary, but 32 bytes is enough for VCPI/XMS/raw.

) In protected mode, remember to use IRETD, not IRET. When DPMI documentation

refers to using IRET, it is actually referring to the 32bit version of the

instruction under 32bit systems, which is IRETD.

) Remember that free memory information is just advisory. A TSR or another

task in a multitasking system might grab some memory in between a call to

INT 31h function 0500h and INT 31h function 0501h, even if you disable

interrupts.

) The _pm_? variables are not checked for validity. So don't set them outside

reasonable bounds. For example, don't ask for 20h real mode stacks of size

1000h paragraphs. This is two megabytes of real mode stack space. This is

way too much, considering that all real mode available low memory

encompasses 640k.

) You should allocate memory in large blocks. Memory space is subject to

fragmentation. Although you can help the situation a little under VCPI by

setting a high number of page tables. This will not increase the physical

memory available, but it will increase the address space available to put

that memory into linear chunks.

) I tried to balance clean, well designed code, with size and speed. True,

some things are not as absolutely optimal as they can be. But the source is

right here. Very clean and commented. If you truly need those last ounces of

speed, feel free to modify it (this does not free you from the obligation of

crediting me for it).

) I can not control what DPMI does. But under XMS/raw, code in protected mode

runs at the fastest speed possible. That is, there is no privilege checking

to get in the way. No exception will rip control away from sensitive

instructions. Not even paging, with its memory references every time a page

table entry is not found in the TLB. Under VCPI, all of this applies except

the paging. Which really isn't that bad.

) Under VCPI, free memory information is validated in function 0500h by

actually allocating that memory and releasing it before passing the

information to your program. This because under some multitasking systems,

the VCPI function for getting the memory available may return information

for the whole system. While the multitasking system may impose allocation

limits on the specific task your code is part of.

) Under VCPI/XMS/raw, INT 31h function 0500h will return only the first field

of the buffer set. All the other fields will be set to 0ffffffffh.

) Under VCPI/XMS/raw, an allocate descriptor INT 31h function 0000h called

with CX = 0 will return error 8021h. DPMI dox dont state this, and I dont

know if DPMI returns an error on CX = 0.

) INT 31h functions 0300h, 0301h, and 0302h will always inform you if there is

not enough real mode stack space. But and IRQ or INT redirection can not.

In this case, the PC speaker will be turned on, and the machine will be

hung. This is better than allowing it to overrun data below it with

unpredictable results. And hey, I dont need no stinkin debug code cluttering

up my nice and pretty extender. I just need it to tell me in case something

like this happenes. If you want debug code, go and hack it in yourself.

) DPMI dox dont state this, but the alias descriptor INT 31h function 000ah

creates is always an expand-up and writeable data descriptor. No matter what

type the source descriptor is.

) You should limit yourself to allocating as few individual memory blocks as

possible. Under XMS, there is usually a strict limit on how many blocks can

be allocated (normally 32).

) DPMI dox state that the field between EBP and EBX should be zero upon an

INT 31h function 0300h, but is ignored by functions 0301h and 0302h. That is

just a stupid typo, the field will be ignored by function 0300h.

) Under XMS an extra 15 bytes will be allocated for possible aligning of the

XMS memory block on a paragraph. Though an XMS block will probably already

be aligned on at least a paragraph boundary, this is not defined in the XMS

standard. And to keep the possibility of problems at nil, this is done.

) Be aware that memory allocation functions under XMS use real mode calls and

real mode stack space defined with _pm_rmstacklen and _pm_rmstacks. If there

is not enough stack space for the call to the real mode XMS driver, error

code 8010h (resource unavailable) will be returned.

) If an XMS memory lock fails, which is used in memory allocation functions,

error 8010h will be returned. A memory lock failure is not due to memory not

being available. But rather, some internal XMS crap. But it should never

happen anyway.

) The raw system checks both INT 15h and the VDISK low to high extended memory

allocation scheme to get its available extended memory area.

) The raw system allocates extended memory on an as-needed basis from the top

down. INT 15h function 88h is hooked and the total amount of memory

allocated using the kernel function 0501h is subtracted from the amount of

memory returned from the previous INT 15h handler. This is so that you can

execute other protected mode programs from within your programs and they

will have extended memory available (if you left any).

) A protected mode IRQ handler or real mode callback must return on the same

stack it was called with.

) A real mode routine called with functions 0300h, 0301h, or 0302h must return

on the same stack it was called with.

) You should make no assumptions about the low memory protected mode data area

needed by PMODE. It can range from very small to very large. And if a DPMI

host is present, it is unpredictable.

) Make sure you do not access, read or write, extended memory outside the

blocks you allocate. Even if there is no physical memory there, you will

probably get exceptions under DPMI/VCPI.

) When setting descriptor access rights, remember that the B bit of stack

descriptors determines whether PUSHes and POPs use SP (B=0) or ESP (B=1).

) If you enable interrupts in a callback, you MUST assume DS is no longer

valid. Even if you are sure your callback will not be re-entered. This is

because PMODE uses the same DS selector for ALL real mode callbacks.

) The reserved field between EBP and EBX in the register structure used during

a callback is used by PMODE to preserve the high word of ESP for real mode.

) You should assume the INT 31h extended memory functions to be slow.

Especially under VCPI, where a call has to be made to the VCPI server for

each 4k of memory allocated or freed, and two calls for each 4k verified.

) Under PMODE, software INT redirection calls only pass back the carry,

parity, aux, zero, sign, and overflow flags from the real mode interrupt

handler.

) When using the raw mode switching routines, keep in mind that they use some

stack space on both the real mode and protected mode stacks. Never set the

destination stack to the same location in memory as the stack being switched

from.

------------------------------------------------------------------------------

3.4 - Final word:

-----------------

I like this latest PMODE a lot. For an extender, it is clean, solid, fast,

and small. It is not limited to assembly code. It does not need control at

startup, but may be initialized at any time. It can be turned into basically

any type of extender. It can work with high level languages. You will probably

also like it because it is free, and the source code is provided. The DPMI

interface assures portability and long life. Enjoy protected mode, and

remember the credits.

L8r...

Tran...