*************************************************************************

How to upgrade your 24oo modem into a 96oo baud.  (US Robotics compatible

of course)

Written By

Death Bringer (CPT/Info-Net/IRA)

06/7/91

Be sure to turn off your modem before you begin.

First of all, you need to go to your nearby electronics store, don't try

to go to Radio Shack, their parts have their own model numbers and you'll

have a hell of a time getting the parts you need.  This upgrading of your

modem will not work on old hayes modems, but they will work on any clone

maker of modems (everex, practical peripherals, anchor, ati, intel, etc.)

It will only work with modems with the Intel Chipset.

This technique widens the band width by using a different chip that is

similiar to the chip already in your modem.

Parts Needed:

16550 UART

L4313545  IC

SC11020CN IC

L8730183  IC

First of all, check to see if your biggest chips are soldered or socketed.

if they are soldered, you'll have to de-solder the chips before you begin.

After you get that done, replace the 8250 UART with the 16550 UART. (16550

UARTs are required for 9600+ speeds).  The other 1 or 2 chip(s) need(s)

to replaced with INTEL L4313545 instead of the chip that is already there.

If your modem has another socket or more, than you are in luck, you can

purchase a chip from US Robotics, and make your modem faster than 9600.

If you don't have the socket, than you can only go 9600.  I'll get the

chip number to you in the next edition and explain how and where to

install it.  I am working on a way for the people (like me) that do not have

the extra socket in their modem to speed up the rates.  I am working on a way

to piggy back the chip.  I'll get back to you.  There should be a 22 pin

chip on your modem also with a model number of SC11005CN or some where close

to it.  Replace it with the SC11020CN.  Also, replace the L8630173 with the

L8730183 chip.  If you have any suggestions or questions, you can locate me

through Compuserve USER ID 74702,97524

*****OPTIONAL, LAP-M Error correction*****

Adding error correction is rather simple.  For this these parts are

necessary:

Potentiometer. This is a 5k audio taper variable resistor.

Capacitor. Any non-polarized 1.0 to 1.5 uf cap should do.

100 ohm resistor - quarter or half watt.

Wire

Solder, soldering iron, etc.

Solder one end of the capacitor to PIN 1 of the phone line input jack

|-------------------|

|                   \ line in         ::

|                   \ phone in        ::

|-----------|||||||||            pin 1^

If your modem is external, the diagram looks like this.

|------------------|

|                 /|

|                 ||RS-232 port

|                 \|                  ::

|                |-|line in           ::

|                |-|             pin 1^

|                |-|phone in

|                |-|

|------------------|

This should be on the back of the modem.

Pin one should the farthest on the bottom left.

Solder the other end of the capacitor to the center lug of the potentiometer

(there are three lugs on this critter). Solder one end of the resistor to the

PIN 4 of the line input jack. Solder the other end of the resistor

to either one of the remaining outside lugs of the potentiometer. Doesn't

matter which one.

-------------------------------------------------------------------------

ADDITION TO ORIGNAL FILE - 6/15/91

Bill McCauley & Dark Spyre/Dr. Brains/Ryan Schwartz/Lamer/Loser

First, a personal recomendation. _THIS WORKS!!!_  I have been plagued with

2400 baud for some time. I hate waiting for the transfer to finish so that

i can use the phone.  Threw the gismo together in about 10 or 15 minutes,

took another five to adjust the pot for best results on my worst

conection, and guess what? No more slow connections!

-------------------------------------------------------------------------

Have Fun!

I am working on cheap ways to install V.32/V.42bis on your modified modem

*************************Updated, 6/17/91********************************

Installing V.42bis:

1) You will need to purchase a 27C512 ROM chip.

2) You will need to purchase a Sony CXK58257P-12L (or equivalent) Ram chip.

3) Install the Rom in the Supervisor (SUP) socket.

4) Install the Ram in the Ram socket.

5) Make up two 3 pin jumpers for P9 and P10 with pins 1 and 2 jumpered

on one and pins 2 and 3 jumpered on the other.  Connect the vacant pin 1

on the one to the vacant pin 3 on the other with a jumper wire.

6) Remove the jumpers on P9 and P10.

7) Install the jumper with pins 1 and 2 connected together on P9.

8) Connect the jumper with pins 2 and 3 connected together on P10.

DeathBringer '91

* (713) 324-2139 * DANSE MACABRE * (713) 324-2139 *